3GPP TSG RAN WG4 Meeting #70	R4-140412

Prague, Czech Republic, 10th – 14th Feb., 2014

- Agenda Item:	9.1

- Source:	LG Electronics

- Title:	TP for UE RX requirements for S-band UE

Document for:	Approval

- Introduction

In the RAN4 #69 meetings, the way forward on the channel arrangements are agreed and interested companies encouraged to provide the required A-MPR and RB restriction simulation results to protect band 34 when S-band and Band 34 deployed in same geometrical regions.

In this contribution, we provide the revised UE receiver RF requirements based on the agreed way forwards.

- References

- R4-137089, “Way forward on how to define a new band for MSS 2GHz,” NTT DOCOMO, Qualcomm, Nokia corporation, Ericsson, KT

- R4-137153, “Way forward on MSS co-existence with Band 34,” Ericsson, NTT DOCOMO

- TS 36.101 v12.2.0

- R4-135186, “UE coexistence analysis to protect Band 34,” LG Electronics

Text proposal for TR 36.861

********************** Start of text proposal to TR 36.861 Chapter 8.2 ***********************

- 8.2    UE RX Requirements

- 8.2.1 REFSENS requirements for S-band UE

New S-band has the same Tx-Rx frequency separation and the total pass bandwidth of S-band is similar with Band 1. Therefore S-band UE’s REFSENS level can be easily as assumed to be the same as that of Band 1. RF simulations are provided to verify this assumption.

Figure 8.2.1-1 shows the simulation results of emission level in S-band DL lower bands when consider 20MHz channel BW with full UL RB allocation in upper edge of UL frequency. The red line indicates the leakage level in S-Band DL lower band.

- 20MHz S-band UE (fc=2000MHz)                      (b) 10MHz S-band UE (fc=2005MHz)

Figure 8.2.1-1 Simulation results of the Tx leakage level into S-band DL

The leakage level in DL band is about -75dBm/MHz, which is further attenuated by Duplexer Tx/Rx isolation level, which is assumed to be 50dB. So the leakage level in DL band is estimated as -125dBm/MHz, which is 18 dB lower than REFSENS. Eq. 8-1 shows the impact of REFSENS in S-band DL induced by S-band Tx leakage.

- REFSENS for S-band UE = Conventional REFSENS + Tx_leakage levels                          (Eq. 8-1)

= -94dBm/CBW +  -125dBm/MHz

= -107dBm/MHz + -125dBm/MHz = - 106.93dBm/MHz

= -93.93dBm/CBW

From Eq. 8-1, we can see that the REFSENS in own Rx band by Tx leakage is negligible.

As a conclusion, REFSENS requirements for S-band UE can be defined as -94dBm/20MHz in Table 8.2.1-1 with the same uplink configurations as in Band1 [2].

Table 8.2.1-1: Reference Sensitivity for QPSK

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| XX |  |  | -100 | -97 | -95.2 | -94 | FDD |

- 8.2.2 Other UE Rx RF requirements

For the receiver requirements other than REFSENS for S-band UE, requirement changes are not expected since there are no critical impacts to determine the UE reception requirements. So we propose as in Table 8.2.2-1.

Table 8.2.2-1: Expected UE Rx RF requirements for S-band UE

| Description | Comments and Description of change |
| --- | --- |
| Adjacent Channel Selectivity | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements that can be larger than or equal to 95% with same condition in section 7.5.1 [2] |
| Blocking characteristics | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements can be larger than or equal to 95% with same condition in section 7.6 [2] |
| Receiver inter-modulation characteristics | No changes since there is no Tx leakage impacts on the S-band DL for the T-put requirements can be larger than or equal to 95% with same condition in section 7.8.1 [2] |

********************** End of text proposal to TR 36.861 Chapter 8.2 ***************************