3GPP TSG-WG4 Meeting #70	R4-140987

Prague, Czech Republic, 10 - 14 February 2014

Agenda item: 	8.1.2

Source: 	QUALCOMM Incorporated

Title: 	Blocking performance for UTRA Band I + SDL 1452 - 1496 MHz

Document for:	Discussion and Approval

# 1	Introduction

The work item on L-band to be used as supplemental downlink (SDL) for both UTRA and E-UTRA was approved [1] for the frequency range from 1452 - 1492 MHz but was further extended to span 1452 - 1496 MHz but optimizing the filter design for the original range.

In this contribution, the blocking performance requirements for Band I + SDL 1452-1496 MHz are provided. Table I summarizes the band combination and number of adjacent DL carriers. A diplexer insertion loss (IL) was assumed as 0.6 dB [3]. A duplexer insertion loss of 4 dB is assumed for Band I and 4.5 dB is assumed for the L-band Rx filter [2]. A combined duplexer and diplexer isolation at the Tx frequency is assumed to be 50 dB. IIP3 = -10 dBm is assumed. These values will be used for the analysis of Rx core requirements in the remainder of the contribution.

Table 1: Band combinations and Additional Insertion Loss for Band I + L-Band

| Scenarios | Band A |  | Band B |  |
| --- | --- | --- | --- | --- |
|  | Band | Number of DL adjacent carriers | Band | Number of DL adjacent carriers |
| Scenario 1 | I | 1 | L-Band (1452-1496 MHz) | 2 |
| Scenario 2 | I | 2 | L-Band (1452-1496 MHz) | 1 |

# 2	Rx Core Requirements

Performance of Adjacent Channel Selectivity (ACS), in-band blocking and intermodulation requirements are analyzed due to the introduction of L-band as SDL for Band I. Requirements will be analyzed based on the cross modulation noise due to the Tx signal and jammer signal(s) for both single uplink and dual uplink cases.

## 2.1	Single Uplink Operation

This section investigates the Rx core requirements for L-band SDL with single uplink operation, including in-band blocking, intermodulation and ACS requirements. For each requirement, the following information is shown:

- Requirements derived from the cross modulation noise

- Existing requirements defined from REFÎor (e.g., REFÎor+3dB or REFÎor+10dB)

- Relative relaxation compared against the existing requirements

  - Positive value means a spec relaxation would be required

  - Negative value means the current spec can be re-used

- Proposed requirements which are the maximum between the requirements from simulation and the existing requirements.

### 2.1.1	Cross Modulation Noise

The cross modulation noise measured from simulation is shown in Table 2.

Table 2: Cross modulation noise with single uplink frequency operation

| Scenario | Scenario 1 |  | Scenario 2 |  |
| --- | --- | --- | --- | --- |
| DL Band | I | L-Band | I | L-Band |
| Band Operation | DB | DB | DB | DB |
| Number of Carriers | 1 | 2 | 2 | 1 |
| Insertion Loss | 4.6 | 5.1 | 4.6 | 5.1 |
| In-band Blocking | -135.64 | -135.19 | -135.64 | -135.19 |
| Intermod | -129.7 | -129.97 | -129.7 | -129.97 |
| ACS Case 1 | -97.24 | -96.74 | -97.24 | -96.74 |
| ACS Case 2 | -69.57 | -69.33 | -69.57 | -69.33 |

### 2.1.2	In-band Blocking

The in-band blocking requirements are shown in Table 3.

Table 3: In-band blocking requirements with single uplink frequency

| Scenario | Scenario 1 |  | Scenario 2 |  |
| --- | --- | --- | --- | --- |
| DL Band | I | L-Band | I | L-Band |
| Band Operation | DB | DB | DB | DB |
| Number of Carriers | 1 | 2 | 2 | 1 |
| Requirements from simulation | -102 | -101 | -102 | -101 |
| Existing Spec (REFÎor+3dB) | -99.7 | -99.7 | -99.7 | -99.7 |
| Relaxation | -2.3 | -1.3 | -2.3 | -1.3 |
| Proposed Requirements | -99.7 | -99.7 | -99.7 | -99.7 |

### 2.1.3	Intermodulation

The intermodulation requirements are shown in Table 4.

Table 4: Intermodulation requirements with single uplink frequency

| Scenario | Scenario 1 |  | Scenario 2 |  |
| --- | --- | --- | --- | --- |
| DL Band | I | L-Band | I | L-Band |
| Band Operation | DB | DB | DB | DB |
| Number of Carriers | 1 | 2 | 2 | 1 |
| Requirements from simulation | -102 | -101 | -102 | -101 |
| Existing Spec (REFÎor +3dB) | -99.7 | -99.7 | -99.7 | -99.7 |
| Relaxation | -2.3 | -1.3 | -2.3 | -1.3 |
| Proposed Requirements | -99.7 | -99.7 | -99.7 | -99.7 |

### 2.1.4	ACS Case 1

The ACS Case 1 requirements are shown in Table 5.

Table 5: ACS Case 1 requirements with single uplink frequency

| Scenario | Scenario 1 |  | Scenario 2 |  |
| --- | --- | --- | --- | --- |
| DL Band | I | L-Band | I | L-Band |
| Band Operation | DB | DB | DB | DB |
| Number of Carriers | 1 | 2 | 2 | 1 |
| Requirements from simulation | -95.1 | -94.1 | -95.1 | -94.1 |
| Existing Spec (REFÎor +14dB) | -88.7 | -88.7 | -88.7 | -88.7 |
| Relaxation | -6.4 | -5.4 | -6.4 | -5.4 |
| Proposed Requirements | -88.7 | -88.7 | -88.7 | -88.7 |

### 2.1.5	ACS Case 2

The ACS Case 2 requirements are shown in Table 6.

Table 6: ACS Case 2 requirements with single uplink frequency

| Scenario | Scenario 1 |  | Scenario 2 |  |
| --- | --- | --- | --- | --- |
| DL Band | I | L-Band | I | L-Band |
| Band Operation | DB | DB | DB | DB |
| Number of Carriers | 1 | 2 | 2 | 1 |
| Requirements from simulation | -68.4 | -67.7 | -68.4 | -67.7 |
| Existing Spec (REFÎor +41dB) | -61.7 | -61.7 | -61.7 | -61.7 |
| Relaxation | -6.7 | -6 | -6.7 | -6 |
| Proposed Requirements | -61.7 | -61.7 | -61.7 | -61.7 |

## 2.2	Dual Uplink Operation

This section investigates the Rx core requirements with dual uplink operation. Scenario 1 is not considered in this section since Band I has only 1 DL carrier and thus cannot support 2 UL carriers.

### 2.2.1	Cross Modulation Noise

The cross modulation noise measured from simulation is shown in Table 7.

Table 7: Cross modulation noise with dual uplink frequency operation

| Scenario | Scenario 2 |  |
| --- | --- | --- |
| DL Band | I | L-Band |
| Band Operation | DB | DB |
| Number of Carriers | 2 | 1 |
| Insertion Loss | 4.6 | 5.1 |
| In-band Blocking | -105.36 | -104.86 |
| Intermod | -96.34 | -95.84 |

### 2.2.2	In-band Blocking

The in-band blocking requirements are shown in Table 8.

Table 8: In-band blocking requirements with dual uplink frequencies

| Scenario | Scenario 2 |  |
| --- | --- | --- |
| DL Band | I | L-Band |
| Band Operation | DB | DB |
| Number of Carriers | 2 | 1 |
| Requirements from simulation | -100 | -99 |
| Existing Spec (REFÎor+3dB) | -99.7 | -99.7 |
| Relaxation | -0.3 | 0.7 |
| Proposed Requirements | -99.7 | -99 |

### 2.2.3	Intermodulation

The intermodulation requirements are shown in Table 9.

Table 9: Intermodulation requirements with dual uplink frequencies

| Scenario | Scenario 2 |  |
| --- | --- | --- |
| DL Band | I | L-Band |
| Band Operation | DB | DB |
| Number of Carriers | 2 | 1 |
| Requirements from simulation | -94.4 | -93.4 |
| Existing Spec (REFÎor +3dB) | -99.7 | -99.7 |
| Relaxation | 5.3 | 6.3 |
| Proposed Requirements | -94.4 | -93.4 |

# 3	Conclusions

In this contribution, the simulation results for UE Rx core requirements due to the introduction of L-Band as supplemental downlink for Band I have been provided. Based on simulation results, the Rx core requirements remain unchanged for the single UL operation. For dual UL operation, the Rx core requirements remain unchanged for in-band blocking requirement for Band I but the in-band blocking requirement for L-band and intermodulation requirements for both Band I and L-band need relaxation.

Proposal 1: Use above simulation results to derive blocking performance requirements for UTRA L-band SDL.

Proposal 2: Agree provided TP in Annex.

# 4	References

3GPP TS 25.101 v9.3.0

R4-140983, “Rx filter insertion loss for UTRA L-band 1452 – 1496 MHz and reference sensitivity for UTRA”, Qualcomm Incorporated

R4-135453, “Diplexer insertion loss for UTRA Band I + SDL 1452 – 1492 MHz,” Qualcomm Incorporated

# Annex	TP to TR 37.814

------------------------------------------------------------ Start of TP -----------------------------------------------------------------

## 8.4			UE In band blocking

A diplexer insertion loss (IL) was assumed as 0.6 dB. A duplexer insertion loss of 4 dB is assumed for Band I and 4.5 dB is assumed for the L-band Rx filter. A combined duplexer and diplexer isolation at the Tx frequency is assumed to be 50 dB. IIP3 = -10 dBm is assumed.

### 8.4.1	Single Uplink Operation

The in-band blocking requirements with single uplink frequency are shown in Table 8.4-1.

Table 8.4-1: In-band blocking requirements with single uplink frequency

| UTRA SDL Configuration | 1b |  | 1c |  |
| --- | --- | --- | --- | --- |
| DL Band | I | L-Band | I | L-Band |
| Band Operation | DB | DB | DB | DB |
| Number of Carriers | 1 | 2 | 2 | 1 |
| Requirements(dBm) | -99.7 | -99.7 | -99.7 | -99.7 |

### 8.4.2	Dual Uplink Operation

The in-band blocking requirements with dual uplink frequencies are shown in Table 8.4-2.

Table 8.4-2: In-band blocking requirements with dual uplink frequencies

| UTRA SDL Configuration | 1c |  |
| --- | --- | --- |
| DL Band | I | L-Band |
| Band Operation | DB | DB |
| Number of Carriers | 2 | 1 |
| Requirements(dBm) | -99.7 | -99 |

## 8.5			UE Intermodulation

A diplexer insertion loss (IL) was assumed as 0.6 dB. A duplexer insertion loss of 4 dB is assumed for Band I and 4.5 dB is assumed for the L-band Rx filter. A combined duplexer and diplexer isolation at the Tx frequency is assumed to be 50 dB. IIP3 = -10 dBm is assumed.

### 8.5.1	Single Uplink Operation

The intermodulation requirements with single uplink frequency are shown in Table 8.5-1.

Table 8.5-1: Intermodulation requirements with single uplink frequency

| UTRA SDL Configuration | 1b |  | 1c |  |
| --- | --- | --- | --- | --- |
| DL Band | I | L-Band | I | L-Band |
| Band Operation | DB | DB | DB | DB |
| Number of Carriers | 1 | 2 | 2 | 1 |
| Requirements(dBm) | -99.7 | -99.7 | -99.7 | -99.7 |

### 8.5.2	Dual Uplink Operation

The intermodulation requirements with dual uplink frequencies are shown in Table 8.5-2.

Table 8.5-2: Intermodulation requirements with dual uplink frequencies

| UTRA SDL Configuration | 1c |  |
| --- | --- | --- |
| DL Band | I | L-Band |
| Band Operation | DB | DB |
| Number of Carriers | 2 | 1 |
| Requirements(dBm) | -94.4 | -93.4 |

## 8.6			UE ACS

A diplexer insertion loss (IL) was assumed as 0.6 dB. A duplexer insertion loss of 4 dB is assumed for Band I and 4.5 dB is assumed for the L-band Rx filter. A combined duplexer and diplexer isolation at the Tx frequency is assumed to be 50 dB. IIP3 = -10 dBm is assumed.

### 8.6.1	Single Uplink Operation

#### 8.6.1.1	ACS Case 1

The ACS Case 1 requirements are shown in Table 8.6-1.

Table 8.6-1: ACS Case 1 requirements with single uplink frequency

| UTRA SDL Configuration | 1b |  | 1c |  |
| --- | --- | --- | --- | --- |
| DL Band | I | L-Band | I | L-Band |
| Band Operation | DB | DB | DB | DB |
| Number of Carriers | 1 | 2 | 2 | 1 |
| Requirements(dBm) | -88.7 | -88.7 | -88.7 | -88.7 |

#### 8.6.1.2	ACS Case 2

The ACS Case 2 requirements are shown in Table 8.6-2.

Table 8.6-2: ACS Case 2 requirements with single uplink frequency

| UTRA SDL Configuration | 1b |  | 1c |  |
| --- | --- | --- | --- | --- |
| DL Band | I | L-Band | I | L-Band |
| Band Operation | DB | DB | DB | DB |
| Number of Carriers | 1 | 2 | 2 | 1 |
| Requirements(dBm) | -61.7 | -61.7 | -61.7 | -61.7 |

## 8.7		UE Reference input power adjustment for a dual band device

<text to be added>

------------------------------------------------------------ End of TP ------------------------------------------------------------------