3GPP TSG-WG4 Meeting #70	R4-141218

# Prague, Czech Republic, 10 - 14 February 2014

Agenda item: 	8.1.2

Source: 	Qualcomm Incorporated

Title: 	Rx filter insertion loss for UTRA L-band 1452 – 1496 MHz and reference sensitivity for UTRA

Document for:	Discussion and Approval

# 1	Introduction

The L-band for Europe is a new band to be used as supplemental downlink (SDL) for both UTRA and E-UTRA. While the work item as originally approved [1] defined the frequency range from 1452 - 1492 MHz, in the RAN#62 it was further approved [2] that the frequency range would be extended to span 1452 - 1496 MHz but optimizing the filter design for the original range. It was also agreed to use the additional insertion loss of 0.6 dB for UE core requirements analysis for UTRA SDL (Band I + L-band). 0.3 dB MOP relaxation and 0 dB REFSENS relaxation was also agreed in [5].

In this contribution, refined filter simulation results for the extended L-band are presented and a proposal for the reference sensitivity for UTRA L-band and insertion loss assumption for blocking performance for UTRA L-band is provided. It also provides corresponding TP to TR 37.814.

# 2	Discussion

## 2.1	Rx filter insertion loss

In [3] and [4], filter simulation results from two vendors were provided. In addition to the data available in [3] and [4], new refined simulation results for the insertion loss for extended L-band are summarized in Table 1 below, where vendor A and B come from [4] and [3], and vendors C and D are new data presented in this contribution. Vendor C and Vendor D provided new simulation results compared to [6] considering potential additional SDL scenarios discussed in [1]. They confirmed that they can still meet the same IL as shown in [6].

It is proposed to take the maximum IL among Vendor C and Vendor D, since they are more updated results compared to Vendor A/B results. In this case, the Rx passband insertion loss is 2.5 dB for 1452-1492 MHz and 2.6 dB for 1452-1496 MHz. Considering a 0.1 dB difference, it is proposed to take a universal 2.5 dB IL assumption for the extended L-band. Compared to a common Band I duplexer which may have an insertion loss of 2 – 2.5 dB, this filter exhibits an additional insertion loss of up to 0.5 dB. In order to make progress, we propose to adopt the insertion loss for UTRA L-band Rx filter same as Band I duplexer insertion loss.

Proposal 1: Agree on the use of insertion loss for UTRA L-band Rx filter same as Band I duplexer insertion loss for UE core requirements analysis for UTRA SDL (Band I + L-Band).

Table 1: Summary of insertion loss simulation data for L-band SDL Rx filter

|  | Insertion loss for L-band (1452 - 1492 MHz) | Insertion loss for L-band (1452 - 1496 MHz) |
| --- | --- | --- |
| Vendor A [3] | 2.8 dB | 3.3 dB |
| Vendor B [2] | 2.4 dB | 2.8 dB |
| Vendor C | 2.0 dB | 2.0 dB |
| Vendor D | 2.5 dB | 2.6 dB |
| Vendor E [7] | 2.5dB | 2.6dB |

## 2.2	Reference sensitivity

Reference sensitivity is impacted by transmitter noise and front-end insertion loss among other factors. For the L-band SDL, the transmitter is in a different band. Isolation is provided by the duplexer in the uplink band as well as the diplexer. For the UTRA, when aggregated with Band I, it is anticipated that transmitter noise can be neglected for reference sensitivity.

Compared to Band I, due to additional IL of up to 0.5 dB in the L-band filter, it is expected that the REFSENS might be degraded by up to 0.5 dB. However, during RAN4#69, this was not agreeable between companies. In order to make progress, we propose to adopt Band I REFSENS for L-band.

Proposal 2: Agree on the UTRA L-band REFSENS same as Band I.

Based on the previous agreement in [5] and proposal 2, the proposed REFSENS for UTRA SDL (Band I + L-band) is provided below. Note that L-band is labelled as “L” since the numbering needs to be done for the specification.

Table 7.2B: Test parameters for reference sensitivity, additional requirement for DB-DC-HSDPA.

| DB-DC-HSDPA configuration | DL Band | UL Band | Unit | HS-PDSCH_Ec <REFSENS> | <REFÎor> |
| --- | --- | --- | --- | --- | --- |
| 1 | I | I | dBm/3.84 MHz | -113 | -102.7 |
|  | VIII |  | dBm/3.84 MHz | -110 | -99.7 |
|  | I | VIII | dBm/3.84 MHz | -113 | -102.7 |
|  | VIII |  | dBm/3.84 MHz | -110 | -99.7 |
| 2 | II | II | dBm/3.84 MHz | -110 | -99.7 |
|  | IV |  | dBm/3.84 MHz | -112 | -101.7 |
|  | II | IV | dBm/3.84 MHz | -110 | -99.7 |
|  | IV |  | dBm/3.84 MHz | -112 | -101.7 |
| 3 | I | I | dBm/3.84 MHz | -113 | -102.7 |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 |
|  | I | V | dBm/3.84 MHz | -113 | -102.7 |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 |
| 4 | I | I | dBm/3.84 MHz | -112 | -101.7 |
|  | XI |  | dBm/3.84 MHz | -112 | -101.7 |
|  | I | XI | dBm/3.84 MHz | -112 | -101.7 |
|  | XI |  | dBm/3.84 MHz | -112 | -101.7 |
| 5 | II | II | dBm/3.84 MHz | -111 | -100.7 |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 |
|  | II | V | dBm/3.84 MHz | -111 | -100.7 |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 |
| 6 | I | I | dBm/3.84 MHz | -113 | -102.7 |
|  | L |  | dBm/3.84 MHz | -113 | -102.7 |
| NOTE 1 For Power class 3 and 3bis this shall be at the maximum output power NOTE 2 For Power class 4 this shall be at the maximum output power |  |  |  |  |  |

Table 7.2D: Test parameters for reference sensitivity, additional requirement for dual band 4C-HSDPA.

| Dual band 4C-HSDPA configuration | DL Band | UL Band | Unit | HS-PDSCH_Ec <REFSENS> | <REFÎor> | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1, I-2-VIII-2, I-1-VIII-2 | I | I | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | VIII |  | dBm/3.84 MHz | -110 | -99.7 | Minimum |
|  | I | VIII | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | VIII |  | dBm/3.84 MHz | -110 | -99.7 | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | dBm/3.84 MHz | -110 | -99.7 | Minimum |
|  | IV |  | dBm/3.84 MHz | -112 | -101.7 | Minimum |
|  | II | IV | dBm/3.84 MHz | -110 | -99.7 | Minimum |
|  | IV |  | dBm/3.84 MHz | -112 | -101.7 | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 | Minimum |
|  | I | V | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 | Minimum |
| II-1-V-2 | II | II | dBm/3.84 MHz | -111 | -100.7 | Minimum |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 | Minimum |
|  | II | V | dBm/3.84 MHz | -111 | -100.7 | Minimum |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 | Minimum |
| I-1-L-2 I-2-L-1 | I | I | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | L |  | dBm/3.84 MHz | -113 | -102.7 | Minimum |
| NOTE 1 For Power class 3, 3bis and 4, this shall be at the maximum output power |  |  |  |  |  |  |

# 3	Conclusion

In this contribution, additional updated filter data for L-band Rx filter is provided to determine the appropriate reference sensitivity specification for UTRA L-band. Based on this data, we propose a reference sensitivity value that is the same as Band I.

Proposal 1: Agree on the use of insertion loss for UTRA L-band Rx filter same as Band I duplexer insertion loss for UE core requirements analysis for UTRA SDL (Band I + L-Band).

Proposal 2: Agree on the UTRA L-band REFSENS same as Band I.

Proposal 3: Agree provided TP in Annex.

# 4	Reference

- RP-130829, "New Work Item Proposal: L-band for Supplemental Downlink in E-UTRA and UTRA,” Orange

- RP-132041, " Revised Work Item proposal: L-band for Supplemental Downlink in E-UTRA and UTRA," KDDI

- R4-134829, "UE RF characteristic on SDL L-band," KDDI, KYOCERA

- R4-134831, "More on SDL L-band's UE RF characteristic," KDDI

- R4-135453, “Diplexer insertion loss for UTRA Band I + SDL 1452 – 1492 MHz,” Qualcomm Incorporated

- R4-136745, “Rx filter insertion loss for UTRA L-band 1452 – 1492 1496 MHz and reference sensitivity for UTRA”, Qualcomm Incorporated

- R4-135959, “One more UE filter data on SDL L-band, KDDI

# Annex	TP to TR 37.814

------------------------------------------------------------ Start of TP -----------------------------------------------------------------

# 8	Study of UTRA specific issues, Band I + L-band

Recognizing that at 1452 – 1492 MHz, the L-band is a mid-frequency band, we can take advantage of the extensive study already conducted for carrier aggregation with mid-bands.

UTRA supplemental downlink (Band I + L-band combination) is similar to the combination of E-UTRA Band 1 with Band 21 (1447.9 MHz – 1510.9 MHz). The diplexer data is provided in Table 8-1 where it can be seen that the average insertion loss is approximately 0.5 – 0.6 dB.

Table 8-1: Summary of diplexer insertion loss data

|  |  | Vendors |  |  |  |  |  |  |  |  | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 |  | 2 | 3 |  | 4 |  | 5 |  |  |
|  |  | Min | Max | removed | Min | Max | Min | Max | Min | Max | (dB) |
|  |  | (dB) | (dB) |  | (dB) | (dB) | (dB) | (dB) | (dB) | (dB) |  |
| IL | B21 Tx | - | 0.43 |  | - | 0.64 | - | 0.3 | - | 0.59 | 0.5 |
|  | B21 Rx | - | 0.45 |  | - | 0.64 | - | 0.3 | - | 0.62 | 0.5 |
|  | B1 Tx | - | 0.65 |  | - | 0.76 | - | 0.5 | - | 0.49 | 0.6 |
|  | B1 Rx | - | 0.5 |  | - | 0.76 | - | 0.5 | - | 0.52 | 0.6 |

Furthermore, it is observed that for this CA configuration for E-UTRA, the allocation of insertion loss to reference sensitivity and maximum output power relaxations are such that RIB = 0 and TIB = 0.3.

On the other hand, a similar band combination was also considered for DB-DC-HSDPA. DB-DC-HSDPA configuration 4, which consists of UTRA Band I and Band XI (1427.9 MHz – 1495.9 MHz), assumes the insertion loss of 1 dB for both Band I and Band XI based on the typical insertion loss (0.8 dB for Band XI and 0.9 dB for Band I) identified in previous DB-DC-HSDPA study. For this DB-DC-HSDPA configuration, 1 dB MOP and REFSENS relaxation was used.

It is expected that the additional insertion loss for a diplexer for UTRA supplemental downlink (Band I and L-band) would not be worse than either option (E-UTRA Band 1+21 or UTRA Band I+XI). Therefore, 0.6 dB of additional insertion loss due to the diplexer will be assumed for the analysis for UE core requirements analysis for UTRA supplemental downlink (Band I + L-Band) .

Simulation results for the insertion loss for L-band are summarized in Table 8-2. Compared to a commercially available Band I duplexer which may have an insertion loss of 2-2.5 dB, these simulated filters exhibits an additional insertion loss of up to 0.5 dB in some cases. The insertion loss for UTRA L-band Rx filter will be assumed to be the same as Band I duplexer insertion loss for UE core requirements for UTRA supplemental downlink (Band I + L-Band). Thus, L-band REFSSENS is the same as for Band I.

Table 8-2: Summary of insertion loss simulation data for L-band SDL Rx filter

|  | Insertion loss for L-band spec (1452 - 1492 MHz) | Insertion loss for L-band spec (1452 - 1496 MHz) |
| --- | --- | --- |
| Vendor A | 2.8 dB | 3.3 dB |
| Vendor B | 2.4 dB | 2.8 dB |
| Vendor C | 2.0 dB | 2.0 dB |
| Vendor D | 2.5 dB | 2.6 dB |
| Vendor E | 2.5 dB | 2.6 dB |

## 8.1 	UE MOP

The same values of relaxation for the lower tolerance of maximum output power shown in Table 8.1-1 are adopted as used in E-UTRA Band 1+21 for the corresponding DB-DC-HSDPA and dual band 4C-HSDPA combinations between Band I and L-Band.

Table 8.1-1: Allowed adjustment to maximum output power lower tolerance for UTRA Band I + L-Band

| UTRA SDL Configuration | Anchor Band (DL and UL) |  | Supplemental Band (DL only) |  | Maximum allowed adjustment in lower side of tolerance (dB) |
| --- | --- | --- | --- | --- | --- |
|  | Band | Bandwidth | Band | Bandwidth |  |
| 1a | I (2.1 GHz) | 5 MHz | L-band | 5 MHz | -0.3 |
| 1b | I (2.1 GHz) | 5 MHz | L-band | 10 MHz | -0.3 |
| 1c | I (2.1 GHz) | 10 MHz | L-band | 5 MHz | -0.3 |
| NOTE: The maximum allowed adjustment in lower side tolerance of maximum output power is only applicable for the anchor band. |  |  |  |  |  |

## 8.2	UE REFSENS

The same values of relaxation for the reference sensitivity shown in Table 8.2-1 are adopted as used in E-UTRA Band 1+21 for the corresponding DB-DC-HSDPA and dual band 4C-HSDPA combinations between Band I and L-Band.

Table 8.2-1: Allowed desense for UTRA Band I + L-Band

| UTRA SDL Configuration | Anchor Band (DL and UL) |  | Supplemental Band (DL only) |  | Allowed de-sensitization (dB) |
| --- | --- | --- | --- | --- | --- |
|  | Band | Bandwidth | Band | Bandwidth |  |
| 1a | I (2.1 GHz) | 5 MHz | L-band | 5 MHz | 0 |
| 1b | I (2.1 GHz) | 5 MHz | L-band | 10 MHz | 0 |
| 1c | I (2.1 GHz) | 10 MHz | L-band | 5 MHz | 0 |

Reference sensitivity is impacted by transmitter noise and front-end insertion loss among other factors. For the L-band SDL, the transmitter is in a different band. Isolation is provided by the duplexer in the uplink band as well as the diplexer. For the UTRA, when aggregated with Band I, it is anticipated that transmitter noise can be neglected for reference sensitivity.

Based on Table 8.2-1 and UTRA L-band REFSENS proposal, the REFSENS for UTRA SDL (Band I + L-band) is provided below. Note that L-band is labelled as “L” since the numbering needs to be done for the specification.

Table 7.2B: Test parameters for reference sensitivity, additional requirement for DB-DC-HSDPA.

| DB-DC-HSDPA configuration | DL Band | UL Band | Unit | HS-PDSCH_Ec <REFSENS> | <REFÎor> |
| --- | --- | --- | --- | --- | --- |
| 1 | I | I | dBm/3.84 MHz | -113 | -102.7 |
|  | VIII |  | dBm/3.84 MHz | -110 | -99.7 |
|  | I | VIII | dBm/3.84 MHz | -113 | -102.7 |
|  | VIII |  | dBm/3.84 MHz | -110 | -99.7 |
| 2 | II | II | dBm/3.84 MHz | -110 | -99.7 |
|  | IV |  | dBm/3.84 MHz | -112 | -101.7 |
|  | II | IV | dBm/3.84 MHz | -110 | -99.7 |
|  | IV |  | dBm/3.84 MHz | -112 | -101.7 |
| 3 | I | I | dBm/3.84 MHz | -113 | -102.7 |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 |
|  | I | V | dBm/3.84 MHz | -113 | -102.7 |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 |
| 4 | I | I | dBm/3.84 MHz | -112 | -101.7 |
|  | XI |  | dBm/3.84 MHz | -112 | -101.7 |
|  | I | XI | dBm/3.84 MHz | -112 | -101.7 |
|  | XI |  | dBm/3.84 MHz | -112 | -101.7 |
| 5 | II | II | dBm/3.84 MHz | -111 | -100.7 |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 |
|  | II | V | dBm/3.84 MHz | -111 | -100.7 |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 |
| 6 | I | I | dBm/3.84 MHz | -113 | -102.7 |
|  | L |  | dBm/3.84 MHz | -113 | -102.7 |
| NOTE 1 For Power class 3 and 3bis this shall be at the maximum output power NOTE 2 For Power class 4 this shall be at the maximum output power |  |  |  |  |  |

Table 7.2D: Test parameters for reference sensitivity, additional requirement for dual band 4C-HSDPA.

| Dual band 4C-HSDPA configuration | DL Band | UL Band | Unit | HS-PDSCH_Ec <REFSENS> | <REFÎor> | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1, I-2-VIII-2, I-1-VIII-2 | I | I | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | VIII |  | dBm/3.84 MHz | -110 | -99.7 | Minimum |
|  | I | VIII | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | VIII |  | dBm/3.84 MHz | -110 | -99.7 | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | dBm/3.84 MHz | -110 | -99.7 | Minimum |
|  | IV |  | dBm/3.84 MHz | -112 | -101.7 | Minimum |
|  | II | IV | dBm/3.84 MHz | -110 | -99.7 | Minimum |
|  | IV |  | dBm/3.84 MHz | -112 | -101.7 | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 | Minimum |
|  | I | V | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 | Minimum |
| II-1-V-2 | II | II | dBm/3.84 MHz | -111 | -100.7 | Minimum |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 | Minimum |
|  | II | V | dBm/3.84 MHz | -111 | -100.7 | Minimum |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 | Minimum |
| I-1-L-2 I-2-L-1 | I | I | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | L |  | dBm/3.84 MHz | -113 | -102.7 | Minimum |
| NOTE 1 For Power class 3, 3bis and 4, this shall be at the maximum output power |  |  |  |  |  |  |

------------------------------------------------------------ End of TP ------------------------------------------------------------------