3GPP TSG-RAN WG4 Meeting #70 	R4-141262

# Prague, Czech Republic, 10-14 February 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.101 | CR | 1026 | rev | - | Current version: | 10.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial corrections to UE RF core requirements in TS 25.101 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10 |  |  |  |  |  |  | Date: |  | 2014-02-10 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Some normative notes in UE core requirements were put in outside the table by mistake. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some normative notes are moved withint the frame of the table and merged into the existing notes which were added later. Additional minor editorial modifications also has been done. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion on Note and reference to note will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.1, 7.4.2.1, 7.4.2.2, 7.4.3.1, 7.4.3.2, 7.5.1, 7.5.2, 7.5.3, 7.6.1, 7.6.1A, 7.6.1B, 7.6.1C.1, 7.6.1C.2, 7.6.1D.1, 7.6.1D.2, 7.6.2, 7.6.2A, 7.6.2B, 7.6.2C, 7.6.2D, 7.6.3, 7.6.3A, 7.6.3B, 7.6.3C.2, 7.6.3D.1, 7.6.3D.2, 7.7.1, 7.7.2, 7.8.1, 7.8.1A, 7.8.1B, 7.8.1C.1, 7.8.1C.2, 7.8.1D.1, 7.8.1D.2, 7.8.2, 7.8.2A, 7.8.2B, 7.8.2D.1, 7.8.2D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR 34.121-1 CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change -----------------------------------------------------------

## 7.4	Maximum input level

This is defined as the maximum mean power received at the UE antenna port, at which the specified BER performance shall be met.

### 7.4.1	Minimum requirement for DPCH reception

The BER shall not exceed 0.001 for the parameters specified in Table 7.3.

Table 7.3: Maximum input level

| Parameter | Unit | Level |
| --- | --- | --- |
|  | dB | -19 |
| Îor | dBm/3.84 MHz | -25 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 1 |
| NOTE 1: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. |  |  |

NOTE:	Since the spreading factor is large (10log(SF)=21dB), the majority of the total input signal consists of the OCNS interference. The structure of OCNS signal is defined in Annex C.3.2.

### 7.4.2	Minimum requirement for HS-PDSCH reception

#### 7.4.2.1	Minimum requirement for 16QAM

The requirements are specified in terms of a minimum information bit throughput R for the DL reference channel H-Set 1 (16QAM version) specified in Annex A.7.1.1 with the addition of the parameters in Table 7.3A and the downlink physical channel setup according to table C.8.

Using this configuration the throughput shall meet or exceed the minimum requirements specified in table 7.3B.

# Table 7.3A Test parameters for maximum input level

| Parameter |  |  | Unit |  | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase reference |  |  |  |  |  | P-CPICH |  |  |
| Îor |  |  |  | dBm/3.84 MHz |  | -25 |  |  |
| UE transmitted mean power |  |  |  | dBm |  | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 2 |  |  |
| DPCH_Ec/Ior |  |  |  | dB |  | -13 |  |  |
| HS-SCCH_1_Ec/Ior |  |  |  | dB |  | -13 |  |  |
| Redundancy and constellation version |  |  |  |  |  | 6 |  |  |
| Maximum number of HARQ transmissions |  |  |  |  |  | 1 |  |  |
| NOTE 1: The HS-SCCH and corresponding HS-PDSCH shall be transmitted continuously with constant power but the HS-SCCH shall only use the identity of the UE under test every third TTI NOTE 2: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. |  |  |  |  |  |  |  |  |

# Table 7.3B Minimum requirement

| HS-PDSCH (dB) | T-put (kbps) |
| --- | --- |
| -3 | 700 |

#### 7.4.2.2	Minimum requirement for 64QAM

The requirements are specified in terms of a minimum information bit throughput R for the DL reference channel H-Set 8 specified in Annex A.7.1.8. with the addition of the parameters in Table 7.3C and the downlink physical channel setup according to table C.8.

Using this configuration the throughput shall meet or exceed the minimum requirements specified in table 7.3D.

# Table 7.3C Test parameters for maximum input level

| Parameter |  |  | Unit |  |  | Value |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |
| Îor |  |  | dBm/3.84 MHz | -25 |  |  |  |
| UE transmitted mean power |  |  | dBm | 0 |  |  |  |
| DPCH_Ec/Ior |  |  | dB | -13 |  |  |  |
| HS-SCCH_1_Ec/Ior |  |  | dB | -13 |  |  |  |
| Redundancy and constellation version |  |  |  | 6 |  |  |  |
| Maximum number of HARQ transmissions |  |  |  | 1 |  |  |  |
| NOTE 1: The HS-SCCH and corresponding HS-PDSCH shall be transmitted continuously with constant power but the HS-SCCH shall only use the identity of the UE under test every third TTI |  |  |  |  |  |  |  |

# Table 7.3D Minimum requirement

| HS-PDSCH (dB) | T-put (kbps) |
| --- | --- |
| -2 | 11800 |

### 7.4.3	Additional requirement for DC-HSDPA and DB-DC-HSDPA

#### 7.4.3.1	Additional requirement for 16QAM

The additional requirements are specified in terms of a minimum information throughput per cell R with the DL reference channel H-Set 1 (16QAM version) specified in Annex A7.1.1, with the addition of the parameters in Table 7.3E, and the downlink physical channel setup according to table C.8, applied to both cells simultaneously. Using this configuration the throughput shall meet or exceed the minimum requirements specified in table 7.3F.

# Table 7.3E Test parameters for maximum input level

| Parameter |  |  | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |
| Îor |  |  |  | dBm/3.84 MHz | -25 |  |  |
| UE transmitted mean power |  |  |  | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 2 |  |  |
| DPCH_Ec/Ior |  |  |  | dB | -13 |  |  |
| HS-SCCH_1_Ec/Ior |  |  |  | dB | -13 |  |  |
| Redundancy and constellation version |  |  |  |  | 6 |  |  |
| Maximum number of HARQ transmissions |  |  |  |  | 1 |  |  |
| NOTE 1: The HS-SCCH and corresponding HS-PDSCH shall be transmitted continuously with constant power but the HS-SCCH shall only use the identity of the UE under test every third TTI NOTE 2: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. |  |  |  |  |  |  |  |

# Table 7.3F Minimum requirement

| HS-PDSCH (dB) | T-put (kbps) |
| --- | --- |
| -3 | 700 |

#### 7.4.3.2	Additional requirement for 64QAM

The additional requirements are specified in terms of a minimum information throughput per cell R with the DL reference channel H-Set 8 specified in Annex A7.1.8, with the addition of the parameters in Table 7.3G, and the downlink physical channel setup according to table C.8, applied to both cells simultaneously. Using this configuration the throughput shall meet or exceed the minimum requirements specified in table 7.3H.

# Table 7.3G Test parameters for maximum input level

| Parameter |  |  | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Phase reference |  | P-CPICH |
| Îor |  |  |  | dBm/3.84 MHz | -25 |  |  |
| UE transmitted mean power |  |  |  | dBm | 0 |  |  |
| DPCH_Ec/Ior |  |  |  | dB | -13 |  |  |
| HS-SCCH_1_Ec/Ior |  |  |  | dB | -13 |  |  |
| Redundancy and constellation version |  |  |  |  | 6 |  |  |
| Maximum number of HARQ transmissions |  |  |  |  | 1 |  |  |
| NOTE 1: The HS-SCCH and corresponding HS-PDSCH shall be transmitted continuously with constant power but the HS-SCCH shall only use the identity of the UE under test every third TTI |  |  |  |  |  |  |  |

# Table 7.3H Minimum requirement

| HS-PDSCH (dB) | T-put (kbps) |
| --- | --- |
| -2 | 11800 |

-------------------------------------------------- Unchanged sections omitted ------------------------------------------------------

## 7.5	Adjacent Channel Selectivity (ACS)

Adjacent Channel Selectivity (ACS) is a measure of a receiver’s ability to receive a W-CDMA signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

### 7.5.1	Minimum requirement

The UE shall fulfill the minimum requirement specified in Table 7.4 for all values of an adjacent channel interferer up to -25 dBm.

However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5 where the BER shall not exceed 0.001.

# Table 7.4: Adjacent Channel Selectivity

| Unit | ACS |
| --- | --- |
| dB | 33 |

# Table 7.5: Test parameters for Adjacent Channel Selectivity

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| DPCH_Ec | dBm/3.84 MHz | <REFSENS> + 14 dB | <REFSENS> + 41 dB |
| Îor | dBm/3.84 MHz | <REFÎor> + 14 dB | REFÎor> + 41 dB |
| Ioac mean power (modulated) | dBm | -52 | -25 |
| Fuw (offset) | MHz | +5 or -5 | +5 or -5 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 3 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE3 |
| NOTE 1: The Ioac (modulated) signal consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: <REFSENS> and <REFÎor> refers to the DPCH_Ec<REFSENS> and the DPCH<REFÎor> as specified in Table 7.2. NOTE 3: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. |  |  |  |

### 7.5.2	Additional requirement for DC-HSDPA and DB-DC-HSDPA

The UE shall fulfill the additional requirement specified in Table 7.5A for all values of an adjacent channel interferer up to -25 dBm.

However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5B, where the HS-PDSCH BLER shall not exceed 0.1.

# Table 7.5A: Adjacent Channel Selectivity

| Unit | ACS |
| --- | --- |
| dB | 33 |

# Table 7.5B: Test parameters for Adjacent Channel Selectivity

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| HS-PDSCH_Ec | dBm/3.84 MHz | <REFSENS> + 14 dB | <REFSENS> + 41 dB |
| Îor | dBm/3.84 MHz | <REFÎor> + 14 dB | <REFÎor> + 41 dB |
| Ioac mean power (modulated) | dBm | -52 | -25 |
| Fuw (offset) (NOTE 2) | MHz | +5 or -5 | +5 or -5 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 4 |
| NOTE 1: The Ioac (modulated) signal consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For DC-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency used and positive offset refers to the assigned channel frequency of the highest carrier frequency used. For DB-DC-HSDPA, offset refers to the assigned channel frequencies of the individual cells. NOTE 3: <REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2A for DC-HSDPA and Table 7.2B for DB-DC-HSDPA. NOTE 4: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. |  |  |  |

### 7.5.3	Additional requirement for single band/dual band 4C-HSDPA

The UE shall fulfill the additional requirement specified in Table 7.5C for all values of an adjacent channel interferer up to -25 dBm.

However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5D and the requirements are given in Table 7.5E and Table 7.5EA for single band 4C-HSDPA and in 7.5F and 7.5G for dual band 4C-HSDPA, where the HS-PDSCH BLER shall not exceed 0.1.

The ACS requirement for single band/dual-band 4C-HSDPA is not applicable for dual uplink operation.

# Table 7.5C: Adjacent Channel Selectivity

| Rx Parameter | Unit | Number of adjacent downlink carriers in a band |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 |
| ACS | dB | 33 | 33 | 33 | 33 |

# Table 7.5D: Test parameters for Adjacent Channel Selectivity

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Ioac mean power (modulated) | dBm | -52 | -25 |
| Fuw (offset) (NOTE 2) | MHz | +5 or -5 | +5 or -5 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |
| NOTE 1: The Ioac (modulated) signal consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: Negative offset refers to the assigned channel frequency of the lowest carrier frequenc(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequenc(ies) in each band. |  |  |  |

# Table 7.5E: Single band 4C-HSDPA requirements for Adjacent Channel Selectivity, Case 1

| Single band 4C-HSDPA Configuration | DL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- |
| I-3 | I | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2C for single band 4C-HSDPA. |  |  |  |  |

Table 7.5EA: Single band 4C-HSDPA requirements for Adjacent Channel Selectivity, Case 2

| Single band 4C-HSDPA Configuration | DL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- |
| I-3 | I | <REFSENS>+41 dB | <REFÎor>+41 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2C for single band 4C-HSDPA. |  |  |  |  |

# Table 7.5F: Dual band 4C-HSDPA requirements for Adjacent Channel Selectivity, Case 1

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |
|  | VIII |  | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |
|  | I | VIII | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |
|  | VIII |  | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |
|  | IV |  | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |
|  | II | IV | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |
|  | IV |  | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |
|  | V |  | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |
|  | I | V | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |
|  | V |  | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2D for dual band 4C-HSDPA. |  |  |  |  |  |  |

# Table 7.5G: Dual band 4C-HSDPA requirements for Adjacent Channel Selectivity, Case 2

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | <REFSENS>+41 dB | <REFÎor>+41 dB | Minimum |
|  | VIII |  | <REFSENS>+41 dB | <REFÎor>+41 dB | Minimum |
|  | I | VIII | <REFSENS>+41 dB | <REFÎor>+41 dB | Minimum |
|  | VIII |  | <REFSENS>+41 dB | <REFÎor>+41 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+41 dB | <REFÎor>+41 dB | Minimum |
|  | IV |  | <REFSENS>+41 dB | <REFÎor>+41 dB | Minimum |
|  | II | IV | <REFSENS>+41 dB | <REFÎor>+41 dB | Minimum |
|  | IV |  | <REFSENS>+41 dB | <REFÎor>+41 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | <REFSENS>+41 dB | <REFÎor>+41 dB | Minimum |
|  | V |  | <REFSENS>+41 dB | <REFÎor>+41 dB | Minimum |
|  | I | V | <REFSENS>+41 dB | <REFÎor>+41 dB | Minimum |
|  | V |  | <REFSENS>+41 dB | <REFÎor>+41 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2D for dual band 4C-HSDPA. |  |  |  |  |  |  |

## 7.6	Blocking characteristics

The blocking characteristic is a measure of the receiver’s ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occur.

### 7.6.1	Minimum requirement (In-band blocking)

The BER shall not exceed 0.001 for the parameters specified in Table 7.6. In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

Table 7.6: In-band blocking

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| DPCH_Ec | dBm/3.84 MHz | <REFSENS>+3 dB |  |
| Îor | dBm/3.84 MHz | <REFÎor> + 3 dB |  |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band II operation) | MHz | 1922.4 f 1997.6 | 1915 f 2005 |
| Fuw (Band III operation) | MHz | 1797.4 f 1887.6 | 1790 f 1895 |
| Fuw (Band IV operation) | MHz | 2102.4 f 2162.6 | 2095 f 2170 |
| Fuw (Band V operation) | MHz | 861.4 f 901.6 | 854 f 909 |
| Fuw (Band VI operation) | MHz | 867.4 f 892.6 (NOTE 2) | 860 f 900 (NOTE 2) |
| Fuw (Band VII operation) | MHz | 2612.4 f 2697.6 | 2605 f 2705 |
| Fuw (Band VIII operation) | MHz | 917.4 f 967.6 | 910 f 975 |
| Fuw (Band IX operation) | MHz | 1837.4 f 1887.4 | 1829.9 f 1894.9 |
| Fuw (Band X operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band XI operation) | MHz | 1468.4 f 1503.4 | 1460.9 f 1510.9 |
| Fuw (Band XII operation) | MHz | 721.4 f 753.6 | 714 f 761 |
| Fuw (Band XIII operation) | MHz | 738.4 f 763.6 | 731 f 771 |
| Fuw (Band XIV operation) | MHz | 750.4 f 775.6 | 743 f 783 |
| Fuw (Band XIX operation) | MHz | 867.4 f 897.6 | 860 f 905 (NOTE 2) |
| Fuw (Band XX operation) | MHz | 783.4 f 828.6 | 776 f 836 |
| Fuw (Band XXI operation) | MHz | 1488.4 f 1518.4 | 1480.9 f 1525.9 (NOTE 2) |
| Fuw (Band XXII operation) | MHz | 3502.4 f 3597.6 | 3495 f 3605 |
| Fuw (Band XXV operation) | MHz | 1922.4 f 2002.6 | 1915 f 2010 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 4 |  |
| NOTE 1: Iblocking (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For Band VI, Band XIX and Band XXI, the unwanted interfering signal does not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band. NOTE 3: <REFSENS> and <REFÎor> refer to the DPCH_Ec<REFSENS> and the DPCH<REFÎor> as specified in Table 7.2. NOTE 4: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. |  |  |  |

### 7.6.1A	Additional requirement for DC-HSDPA and DB-DC-HSDPA (In-band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.6A. In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

Table 7.6A: In-band blocking for DC-HSDPA and DB-DC-HSDPA

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| HS-PDSCH_Ec | dBm/3.84 MHz | <REFSENS>+3 dB |  |
| Îor | dBm/3.84 MHz | <REFÎor> + 3 dB |  |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 3) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band II operation) | MHz | 1922.4 f 1997.6 | 1915 f 2005 |
| Fuw (Band III operation) | MHz | 1797.4 f 1887.6 | 1790 f 1895 |
| Fuw (Band IV operation) | MHz | 2102.4 f 2162.6 | 2095 f 2170 |
| Fuw (Band V operation) | MHz | 861.4 f 901.6 | 854 f 909 |
| Fuw (Band VI operation) | MHz | 867.4 f 892.6 (NOTE 2) | 860 f 900 (NOTE 2) |
| Fuw (Band VII operation) | MHz | 2612.4 f 2697.6 | 2605 f 2705 |
| Fuw (Band VIII operation) | MHz | 917.4 f 967.6 | 910 f 975 |
| Fuw (Band IX operation) | MHz | 1837.4 f 1887.4 | 1829.9 f 1894.9 |
| Fuw (Band X operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band XI operation) | MHz | 1468.4 f 1503.4 | 1460.9 f 1510.9 |
| Fuw (Band XII operation) | MHz | 721.4 f 753.6 | 714 f 761 |
| Fuw (Band XIII operation) | MHz | 738.4 f 763.6 | 731 f 771 |
| Fuw (Band XIV operation) | MHz | 750.4 f 775.6 | 743 f 783 |
| Fuw (Band XIX operation) | MHz | 867.4 f 897.6 | 860 f 905 (NOTE 2) |
| Fuw (Band XX operation) | MHz | 783.4 f 828.6 | 776 f 836 |
| Fuw (Band XXI operation) | MHz | 1488.4 f 1518.4 | 1480.9 f 1525.9 (NOTE 2) |
| Fuw (Band XXII operation) | MHz | 3502.4 f 3597.6 | 3495 f 3605 |
| Fuw (Band XXV operation) | MHz | 1922.4 f 2002.6 | 1915 f 2010 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 5 |  |
| NOTE 1: Iblocking (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For Band VI, Band XIX and Band XXI, the unwanted interfering signal does not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band. NOTE 3: For DC-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency used and positive offset refers to the assigned channel frequency of the highest carrier frequency used. For DB-DC-HSDPA, offset refers to the assigned channel frequencies of the individual cells. NOTE 4: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2A for DC-HSDPA and Table 7.2B for DB-DC-HSDPA. NOTE 5: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. |  |  |  |

### 7.6.1B	Additional requirement for DC-HSUPA (In-band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.6B and Table 7.6C. In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

Table 7.6B: In-band blocking for DC-HSUPA

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 3) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band II operation) | MHz | 1922.4 f 1997.6 | 1915 f 2005 |
| Fuw (Band III operation) | MHz | 1797.4 f 1887.6 | 1790 f 1895 |
| Fuw (Band IV operation) | MHz | 2102.4 f 2162.6 | 2095 f 2170 |
| Fuw (Band V operation) | MHz | 861.4 f 901.6 | 854 f 909 |
| Fuw (Band VI operation) | MHz | 867.4 f 892.6 (NOTE 2) | 860 f 900 (NOTE 2) |
| Fuw (Band VII operation) | MHz | 2612.4 f 2697.6 | 2605 f 2705 |
| Fuw (Band VIII operation) | MHz | 917.4 f 967.6 | 910 f 975 |
| Fuw (Band IX operation) | MHz | 1837.4 f 1887.4 | 1829.9 f 1894.9 |
| Fuw (Band X operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band XI operation) | MHz | 1468.4 f 1503.4 | 1460.9 f 1510.9 |
| Fuw (Band XII operation) | MHz | 721.4 f 753.6 | 714 f 761 |
| Fuw (Band XIII operation) | MHz | 738.4 f 763.6 | 731 f 771 |
| Fuw (Band XIV operation) | MHz | 750.4 f 775.6 | 743 f 783 |
| Fuw (Band XIX operation) | MHz | 867.4 f 897.6 | 860 f 905 (NOTE 2) |
| Fuw (Band XX operation) | MHz | 783.4 f 828.6 | 776 f 836 (NOTE 2) |
| Fuw (Band XXI operation) | MHz | 1488.4 f 1518.4 | 1480.9 f 1525.9 (NOTE 2) |
| Fuw (Band XXII operation) | MHz | 3502.4 f 3597.6 | 3495 f 3605 |
| Fuw (Band XXV operation) | MHz | 1922.4 f 2002.6 | 1915 f 2010 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 4 |  |
| NOTE 1: Iblocking (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For Band VI, Band XIX and Band XXI, the unwanted interfering signal does not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band. NOTE 3: For DC-HSUPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency used and positive offset refers to the assigned channel frequency of the highest carrier frequency used. NOTE 4: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. |  |  |  |

Table 7.6C: Reference input powers for in-band blocking, DC-HSUPA.

| Operating Band | Unit | HS-PDSCH_Ec | Îor |
| --- | --- | --- | --- |
| I | dBm/3.84 MHz | -110 | -99.7 |
| II | dBm/3.84 MHz | -108 | -97.7 |
| III | dBm/3.84 MHz | -107 | -96.7 |
| IV | dBm/3.84 MHz | -110 | -99.7 |
| V | dBm/3.84 MHz | -104.3 | -94 |
| VI | dBm/3.84 MHz | -104.7 | -94.4 |
| VII | dBm/3.84 MHz | -108 | -97.7 |
| VIII | dBm/3.84 MHz | -101.1 | -90.8 |
| IX | dBm/3.84 MHz | -109 | -98.7 |
| X | dBm/3.84 MHz | -110 | -99.7 |
| XI | dBm/3.84 MHz | -101.4 | -91.1 |
| XII | dBm/3.84 MHz | N/A | N/A |
| XIII | dBm/3.84 MHz | N/A | N/A |
| XIV | dBm/3.84 MHz | N/A | N/A |
| XIX | dBm/3.84 MHz | -104.7 | -94.4 |
| XX |  |  |  |  | dBm/3.84 MHz | TBD | TBD |
| XXI | dBm/3.84 MHz | -101.4 | -91.1 |
| XXII | dBm/3.84 MHz | -107 | -96.7 |
| XXV | dBm/3.84 MHz | -106.5 | -96.2 |
| NOTE 1 For the UE which supports both Band III and Band IX operating frequencies, the reference sensitivity level of TBD dBm <REF_Ec,in-band> shall apply for Band IX. The corresponding <REFÎor,in-band> is TBD dBm NOTE 2 For the UE which supports both Band XI and Band XXI operating frequencies, the reference input power level is FFS. NOTE 3 For the UE which supports DB-DC-HSDPA configuration in Table 5.0aA the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 7.12. NOTE 4 For the UE which supports dual band 4C-HSDPA configuration in Table 5.0aC the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 7.13. |  |  |  |

### 7.6.1C	Additional requirement for single band 4C-HSDPA (In-band blocking)

#### 7.6.1C.1	Single uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.6D and Table 7.6E. In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

Table 7.6D: Test parameters for in-band blocking, single band 4C-HSDPA, single uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 2) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |
| NOTE 1: Iblocking (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For single band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequencies, and positive offset refers to the assigned channel frequency of the highest carrier frequencies. |  |  |  |

Table 7.6E: In-band blocking requirements, single band 4C-HSDPA, single uplink operation

| Single band 4C-HSDPA Configuration | DL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- |
| I-3 | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2C for single band 4C-HSDPA. |  |  |  |  |

#### 7.6.1C.2	Dual uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.6F and Table 7.6G. In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

Table 7.6F: Test parameters for in-band blocking, single band 4C-HSDPA, dual uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 2) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| NOTE 1: Iblocking (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For single band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequencies, and positive offset refers to the assigned channel frequency of the highest carrier frequencies. |  |  |  |

Table 7.6G: In-band blocking requirements, single band 4C-HSDPA, dual uplink operation

| Single band 4C-HSDPA Configuration | DL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UE transmitted mean power (dBm) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-3 | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| NOTE 1 For the UE which supports DB-DC-HSDPA configuration in Table 5.0aA the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 7.12. NOTE 2 For the UE which supports dual band 4C-HSDPA configuration in Table 5.0aC the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 7.13. |  |  |  |  |  |

### 7.6.1D	Additional requirement for dual band 4C-HSDPA (In-band blocking)

#### 7.6.1D.1	Single uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.6H and Table 7.6I. In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

Table 7.6H: Test parameters for in-band blocking, dual band 4C-HSDPA, single uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 2) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band II operation) | MHz | 1922.4 f 1997.6 | 1915 f 2005 |
| Fuw (Band IV operation) | MHz | 2102.4 f 2162.6 | 2095 f 2170 |
| Fuw (Band V operation) | MHz | 861.4 f 901.6 | 854 f 909 |
| Fuw (Band VIII operation) | MHz | 917.4 f 967.6 | 910 f 975 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |
| NOTE 1: Iblocking (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequenc(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequenc(ies) in each band. |  |  |  |  |

Table 7.6I: In-band blocking requirements, dual band 4C-HSDPA, single uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | VIII | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | II | IV | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | V | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2D for dual band 4C-HSDPA. |  |  |  |  |  |

#### 7.6.1D.2	Dual uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.6J and Table 7.6K. In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

Table 7.6J: Test parameters for in-band blocking, dual band 4C-HSDPA, dual uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 2) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band II operation) | MHz | 1922.4 f 1997.6 | 1915 f 2005 |
| Fuw (Band IV operation) | MHz | 2102.4 f 2162.6 | 2095 f 2170 |
| Fuw (Band V operation) | MHz | 861.4 f 901.6 | 854 f 909 |
| Fuw (Band VIII operation) | MHz | 917.4 f 967.6 | 910 f 975 |
| NOTE 1: Iblocking (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequenc(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequenc(ies) in each band. |  |  |  |  |

Table 7.6K: In-band blocking requirements, dual band 4C-HSDPA, dual uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UE transmitted mean power (dBm) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | VIII |  | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-1-IV-2 | II | IV | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-1 | II | II | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-2 | II | II | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | II | IV | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-1-V-2 | I | V | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -103.2 | -92.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-1 | I | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -108 | -97.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-2 | I | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -108 | -97.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | I | V | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -103.2 | -92.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |

### 7.6.2	Minimum requirement (Out-of-band blocking)

The BER shall not exceed 0.001 for the parameters specified in Table 7.7. Out-of-band blocking is defined for an unwanted interfering signal falling more than 15 MHz below or above the UE receive band.

For Table 7.7 in frequency range 1, 2 and 3, up to 24 exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 7.7 Spurious response are applicable.

For Table 7.7 in frequency range 4, up to 8 exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 7.7 Spurious response are applicable

Table 7.7: Out of band blocking

| Parameter | Unit | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 |
| --- | --- | --- | --- | --- | --- |
| DPCH_Ec | dBm / 3.84 MHz | <REFSENS>+3 dB | <REFSENS>+3 dB | <REFSENS>+3 dB | <REFSENS> +3 dB |
| Îor | dBm / 3.84 MHz | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB |
| Iblocking (CW) | dBm | -44 | -30 | -15 | -15 |
| Fuw (Band I operation) | MHz | 2050<f <2095 2185<f <2230 | 2025 <f 2050 2230 f <2255 | 1< f 2025 2255f<12750 | - |
| Fuw (Band II operation) | MHz | 1870<f <1915 2005<f <2050 | 1845 <f 1870 2050 f <2075 | 1< f 1845 2075f<12750 | 1850 f 1910 |
| Fuw (Band III operation) | MHz | 1745 <f <1790 1895<f <1940 | 1720 <f 1745 1940f < 1965 | 1< f 1720 1965f<12750 | - |
| Fuw (Band IV operation) | MHz | 2050< f <2095 2170< f <2215 | 2025< f 2050 2215 f < 2240 | 1< f 2025 2240f<12750 | - |
| Fuw (Band V operation) | MHz | 809< f <854 909< f <954 | 784< f 809 954 f < 979 | 1< f 784 979f<12750 | 824 f 849 |
| Fuw (Band VI operation) | MHz | 815 < f < 860 900 < f < 945 | 790 < f 815 945 f < 970 | 1 < f 790 970 f < 12750 | - |
| Fuw (Band VII operation) | MHz | 2570 < f < 2605 2705 < f < 2750 | na 2750 f < 2775 | 1 < f 2570 2775 f < 12750 | - |
| Fuw (Band VIII operation) | MHz | 865 < f < 910 975 < f < 1020 | 840 < f 865 1020 f < 1045 | 1 < f 840 1045 f < 12750 | - |
| Fuw (Band IX operation) | MHz | 1784.9 < f < 1829.9 1894.9 < f < 1939.9 | 1759.9 < f 1784.9 1939.9 f < 1964.9 | 1 < f 1759.9 1964.9 f < 12750 | - |
| Fuw (Band X operation) | MHz | 2050 < f < 2095 2185 < f < 2230 | 2025 < f 2050 2230 f < 2255 | 1 < f 2025 2255 f< 12750 | - |
| Fuw (Band XI operation) | MHz | 1415.9 < f < 1460.9 1510.9 < f < 1555.9 | 1390.9 < f 1415.9 1555.9 f < 1580.9 | 1 < f 1390.9 1580.9 f < 12750 | - |
| Fuw (Band XII operation) | MHz | 669 < f < 714 761 < f < 806 | 644 < f 669 806 f < 831 | 1 < f 644 831 f< 12750 | 699 f 716 |
| Fuw (Band XIII operation) | MHz | 686 < f < 731 771 < f < 816 | 61 < f 686 816 f < 841 | 1 < f 661 841 f< 12750 | 776 f 788 |
| Fuw (Band XIV operation) | MHz | 698 < f < 743 783 < f < 828 | 673 < f 698 828 f < 853 | 1 < f 673 853 f< 12750 | 788 f 798 |
| Fuw (Band XIX operation) | MHz | 815 < f < 860 905 < f < 950 | 790 < f 815 950 f < 975 | 1 < f 790 975 f < 12750 | - |
| Fuw (Band XX operation) | MHz | 731< f <776 836< f <881 | 706 < f 731 881 f < 906 | 1 < f 706 906 f < 12750 | - |
| Fuw (Band XXI operation) | MHz | 1435.9 < f < 1480.9 1525.9 < f < 1570.9 | 1410.9 < f 1435.9 1570.9 f < 1595.9 | 1 < f 1410.9 1595.9 f < 12750 | - |
| Fuw (Band XXII operation) | MHz | 3450 <f <3495 3605<f <3650 | 3425 <f 3450 3650f < 3675 | 1< f 3425 3675f<12750 | - |
| Fuw (Band XXV operation) | MHz | 1870<f <1915 2010<f <2055 | 1845 <f 1870 2055 f <2080 | 1< f 1845 2080f<12750 | 1850 f 1915 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 3 |  |  |  |
| Band I operation | For 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band II operation | For 1915f 2005 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied |  |  |  |  |
| Band III operation | For 1790f 1895 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band IV operation | For 2095f2170 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band V operation | For 854f909 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band VI operation | For 860f900 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band VII operation | For 2605 f 2705 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band VIII operation | For 910 f 975 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band IX operation | For 1829.9f 1894.9 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band X operation | For 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band XI operation | For 1460.9f 1510.9 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band XII operation | For 714 f 761 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band XIII operation | For 731 f 771 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band XIV operation | For 743 f 783 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band XIX operation | For 860f905 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band XX operation | For 776f836 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band XXI operation | For 1480.9f 1525.9 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. |  |  |  |  |
| Band XXII operation | For 3495 f 3605 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied. NOTE 3 |  |  |  |  |
| Band XXV operation | For 1915f 2010 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied |  |  |  |  |
| NOTE 1: <REFSENS> and <REFÎor> refer to the DPCH_Ec<REFSENS> and the DPCH<REFÎor> as specified in Table 7.2. NOTE 2: For the UE which supports both Band XI and Band XXI operating frequencies, the Out of band blocking is FFS. NOTE 3: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. |  |  |  |  |  |  |

### 7.6.2A	Additional requirement for DC-HSDPA (Out-of-band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.7AA. Out-of-band band blocking is defined for an unwanted interfering signal falling more than 15 MHz below or above the UE receive band.

For Table 7.7AA in frequency range 1, 2 and 3, up to 24 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 7.7 Spurious response are applicable.

For Table 7.7AA in frequency range 4, up to 8 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 7.7 Spurious response are applicable.

# Table 7.7AA: Out of band blocking for DC-HSDPA

| Parameter |  |  | Unit |  | Frequency range 1 |  | Frequency range 2 | Frequency range 3 | Frequency range 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HS-PDSCH_Ec |  |  | dBm / 3.84 MHz |  | <REFSENS>+3 dB |  | <REFSENS>+3 dB | <REFSENS>+3 dB | <REFSENS> +3 dB |  |
| Îor |  |  | dBm / 3.84 MHz |  | <REFÎor> + 3 dB |  | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB |  |
| Iblocking (CW) |  |  | dBm |  | -44 |  | -30 | -15 | -15 |  |
| Fuw (Band I operation) |  |  | MHz |  | 2050<f <2095 2185<f <2230 |  | 2025 <f 2050 2230 f <2255 | 1< f 2025 2255f<12750 | - |  |
| Fuw (Band II operation) |  |  | MHz |  | 1870<f <1915 2005<f <2050 |  | 1845 <f 1870 2050 f <2075 | 1< f 1845 2075f<12750 | 1850 f 1910 |  |
| Fuw (Band III operation) |  |  | MHz |  | 1745 <f <1790 1895<f <1940 |  | 1720 <f 1745 1940f < 1965 | 1< f 1720 1965f<12750 | - |  |
| Fuw (Band IV operation) |  |  | MHz |  | 2050< f <2095 2170< f <2215 |  | 2025< f 2050 2215 f < 2240 | 1< f 2025 2240f<12750 | - |  |
| Fuw (Band V operation) |  |  | MHz |  | 809< f <854 909< f <954 |  | 784< f 809 954 f < 979 | 1< f 784 979f<12750 | 824 f 849 |  |
| Fuw (Band VI operation) |  |  | MHz |  | 815 < f < 860 900 < f < 945 |  | 790 < f 815 945 f < 970 | 1 < f 790 970 f < 12750 | - |  |
| Fuw (Band VII operation) |  |  | MHz |  | 2570 < f < 2605 2705 < f < 2750 |  | na 2750 f < 2775 | 1 < f 2570 2775 f < 12750 | - |  |
| Fuw (Band VIII operation) |  |  | MHz |  | 865 < f < 910 975 < f < 1020 |  | 840 < f 865 1020 f < 1045 | 1 < f 840 1045 f < 12750 | - |  |
| Fuw (Band IX operation) |  |  | MHz |  | 1784.9 < f < 1829.9 1894.9 < f < 1939.9 |  | 1759.9 < f 1784.9 1939.9 f < 1964.9 | 1 < f 1759.9 1964.9 f < 12750 | - |  |
| Fuw (Band X operation) |  |  | MHz |  | 2050 < f < 2095 2185 < f < 2230 |  | 2025 < f 2050 2230 f < 2255 | 1 < f 2025 2255 f< 12750 | - |  |
| Fuw (Band XI operation) |  |  | MHz |  | 1415.9 < f < 1460.9 1510.9 < f < 1555.9 |  | 1390.9 < f 1415.9 1555.9 f < 1580.9 | 1 < f 1390.9 1580.9 f < 12750 | - |  |
| Fuw (Band XII operation) |  |  | MHz |  | 669 < f < 714 761 < f < 806 |  | 643 < f 669 806 f < 831 | 1 < f 644 831 f< 12750 | 699 f 716 |  |
| Fuw (Band XIII operation) |  |  | MHz |  | 686 < f < 731 771 < f < 816 |  | 61 < f 686 816 f < 841 | 1 < f 661 841 f< 12750 | 776 f 788 |  |
| Fuw (Band XIV operation) |  |  | MHz |  | 698 < f < 743 783 < f < 828 |  | 673 < f 698 828 f < 853 | 1 < f 673 853 f< 12750 | 788 f 798 |  |
| Fuw (Band XIX operation) |  |  | MHz |  | 815 < f < 860 905 < f < 950 |  | 790 < f 815 950 f < 975 | 1 < f 790 975 f < 12750 | - |  |
| Fuw (Band XX operation) |  |  | MHz |  | 731< f <776 836< f <881 |  | 706 < f 731 881 f < 906 | 1 < f 706 906 f < 12750 | - |  |
| Fuw (Band XXI operation) |  |  | MHz |  | 1435.9 < f < 1480.9 1525.9 < f < 1570.9 |  | 1410.9 < f 1435.9 1570.9 f < 1595.9 | 1 < f 1410.9 1595.9 f < 12750 | - |  |
| Fuw (Band XXII operation) |  |  | MHz |  | 3450 <f <3495 3605<f <3650 |  | 3425 <f 3450 3650f < 3675 | 1< f 3425 3675f<12750 | - |  |
| Fuw (Band XXV operation) |  |  | MHz |  | 1870<f <1915 2010<f <2055 |  | 1845 <f 1870 2055 f <2080 | 1< f 1845 2080f<12750 | 1850 f 1915 |  |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 3 |  |  |  |
| Band I operation |  | For 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band II operation |  | For 1915f 2005 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied |  |  |  |  |  |  |  |
| Band III operation |  | For 1790f 1895 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band IV operation |  | For 2095f2170 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band V operation |  | For 854f909 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band VI operation |  | For 860f900 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band VII operation |  | For 2605 f 2705 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band VIII operation |  | For 910 f 975 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band IX operation |  | For 1829.9f 1894.9 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band X operation |  | For 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band XI operation |  | For 1460.9f 1510.9 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band XII operation |  | For 714 f 761 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band XIII operation |  | For 731 f 771 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band XIV operation |  | For 743 f 783 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band XIX operation |  | For 860f905 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band XX operation |  | For 776f836 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band XXI operation |  | For 1480.9f 1525.9 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |  |  |
| Band XXII operation |  | For 3495 f 3605 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. NOTE 3 |  |  |  |  |  |  |  |
| Band XXV operation |  | For 1915f 2010 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied |  |  |  |  |  |  |  |
| NOTE 1: <REFSENS> and <REFÎor> refer to the DPCH_Ec<REFSENS> and the DPCH<REFÎor> as specified in Table 7.2A. NOTE 2: For the UE which supports both Band XI and Band XXI operating frequencies, the Out of band blocking is FFS. NOTE 3: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. |  |  |  |  |  |  |  |  |  |  |  |

### 7.6.2B	Additional requirement for DB-DC-HSDPA (Out-of-band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.7AB. Out-of-band blocking is defined for an unwanted interfering signal falling at frequencies outside of frequency regions defined as the UE receive bands extended by 15 MHz at their lower and upper ends. For Table 7.7AB in frequency range 1, 2 and 3, up to 24 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 7.7 Spurious response are applicable.

For Table 7.7AB in frequency range 4, up to 8 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 7.7 Spurious response are applicable.

Table 7.7AB: Out of band blocking for DB-DC-HSDPA

| Parameter | Unit | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 |
| --- | --- | --- | --- | --- | --- |
| HS-PDSCH_Ec | dBm / 3.84 MHz | <REFSENS>+3 dB | <REFSENS>+3 dB | <REFSENS>+3 dB | <REFSENS> +3 dB |
| Îor | dBm / 3.84 MHz | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB |
| Iblocking (CW) | dBm | -44 | -30 | -15 | -15 |
| Fuw (DB-DC-HSDPA Configuration 1) | MHz | 865< f <910 975< f <1020 2050< f <2095 2185< f <2230 | 840< f 865 1020 f <1045 2025< f 2050 2230 f <2255 | 1< f 840 1045 f <2025 2255< f 12750 | - |
| Fuw (DB-DC-HSDPA Configuration 2) | MHz | 1870< f <1915 2005< f <2095 2170< f <2215 | 1845< f 1870 2215 f <2240 | 1< f 1845 2240 f <12750 | 1850 f 1910 |
| Fuw (DB-DC-HSDPA Configuration 3) | MHz | 809< f <854 909< f <954 2050< f <2095 2185< f <2230 | 784< f 809 954 f < 979 2025< f 2050 2230 f <2255 | 1< f 784 979 f <2025 2255< f 12750 | 824 f 849 |
| Fuw (DB-DC-HSDPA Configuration 4) | MHz | 1415.9 < f < 1460.9 1510.9 < f < 1555.9 2050<f <2095 2185<f <2230 | 1390.9 < f 1415.9 1555.9 f < 1580.9 2025 <f 2050 2230 f <2255 | 1 < f 1390.9 1580.9 f < 2025 2255f<12750 | - |
| Fuw (DB-DC-HSDPA Configuration 5) | MHz | 809< f <854 909< f <954 1870<f <1915 2005<f <2050 | 784< f 809 954 f < 979 1845 <f 1870 2050 f <2075 | 1< f 784 979< f 1845 2075f<12750 | 824 f 849 1850 f 1910 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |  |  |
| DB-DC-HSDPA Configuration 1 | For 910f 975 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |
| DB-DC-HSDPA Configuration 2 | For 1915f 2005 MHz and 2095f 2070 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |
| DB-DC-HSDPA Configuration 3 | For 854f909 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |
| DB-DC-HSDPA Configuration 4 | For 1460.9f 1510.9 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |
| DB-DC-HSDPA Configuration 5 | For 854f909 MHz and 1915f 2005 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |
| NOTE: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2B. |  |  |  |  |  |

### 7.6.2C	Additional requirement for single band 4C-HSDPA (Out-of-band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.7AC and Table 7.7AD. Out-of-band band blocking is defined for an unwanted interfering signal falling more than 15 MHz below or above the UE receive band. The requirement is not applicable for dual uplink operation.

For Table 7.7AC in frequency range 1, 2 and 3, up to 24 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 7.7 Spurious response are applicable.

For Table 7.7AC in frequency range 4, up to 8 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 7.7 Spurious response are applicable.

# Table 7.7AC: Test parameters for out of band blocking, single band 4C-HSDPA

| Parameter | Unit | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 |
| --- | --- | --- | --- | --- | --- |
| Iblocking (CW) | dBm | -44 | -30 | -15 | -15 |
| Fuw (Single band 4C-HSDPA Configuration I-3) | MHz | 2050<f <2095 2185<f <2230 | 2025 <f 2050 2230 f <2255 | 1< f 2025 2255f<12750 | - |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |  |  |
| Single band 4C-HSDPA Configuration I-3 | For 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1C.1 shall be applied. |  |  |  |  |

# Table 7.7AD: Out of band blocking requirements, single band 4C-HSDPA

| Singe band 4C-HSDPA Configuration | Parameter | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-3 | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB |  |
| NOTE: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2C. |  |  |  |  |  |  |

### 7.6.2D	Additional requirement for dual band 4C-HSDPA (Out-of-band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.7AE and Table 7.7AF. Out-of-band blocking is defined for an unwanted interfering signal falling at frequencies outside of frequency regions defined as the UE receive bands extended by 15 MHz at their lower and upper ends. The requirement is not applicable for dual uplink operation.

For Table 7.7AF in frequency range 1, 2 and 3, up to 24 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 7.7 Spurious response are applicable.

For Table 7.7AF in frequency range 4, up to 8 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 7.7 Spurious response are applicable.

# Table 7.7AE: Test parameters for out of band blocking, dual band 4C-HSDPA

| Parameter | Unit | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 |
| --- | --- | --- | --- | --- | --- |
| Iblocking (CW) | dBm | -44 | -30 | -15 | -15 |
| Fuw (Dual band 4C-HSDPA Configuration I-2-VIII-1, I-3-VIII-1) | MHz | 865< f <910 975< f <1020 2050< f <2095 2185< f <2230 | 840< f 865 1020 f <1045 2025< f 2050 2230 f <2255 | 1< f 840 1045 f <2025 2255< f 12750 | - |
| Fuw (Dual band 4C-HSDPA Configuration II-1-IV-2, II-2-IV-1, II-2-IV-2) | MHz | 1870< f <1915 2005< f <2095 2170< f <2215 | 1845< f 1870 2215 f <2240 | 1< f 1845 2240 f <12750 | 1850 f 1910 |
| Fuw (Dual band 4C-HSDPA Configuration I-1-V-2, I-2-V-1, I-2-V-2) | MHz | 809< f <854 909< f <954 2050< f <2095 2185< f <2230 | 784< f 809 954 f < 979 2025< f 2050 2230 f <2255 | 1< f 784 979 f <2025 2255< f 12750 | 824 f 849 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |  |  |
| Dual band 4C-HSDPA Configuration I-2-VIII-1, I-3-VIII-1 | For 910f 975 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1D.1 shall be applied. |  |  |  |  |
| Dual band 4C-HSDPA Configuration II-1-IV-2, II-2-IV-1, II-2-IV-2 | For 1915f 2005 MHz and 2095f 2070 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1D.1 shall be applied. |  |  |  |  |
| Dual band 4C-HSDPA Configuration I-1-V-2, I-2-V-1, I-2-V-2 | For 854f909 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1D.1 shall be applied. |  |  |  |  |

# Table 7.7AF: Out of band blocking requirements, dual band 4C-HSDPA

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | Parameter | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | VIII |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
|  | I | VIII | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | VIII |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | IV |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
|  | II | IV | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | IV |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | V |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
|  | I | V | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | V |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2D. |  |  |  |  |  |  |  |  |

### 7.6.3	Minimum requirement (Narrow band blocking)

The BER shall not exceed 0.001 for the parameters specified in Table 7.7A. This requirement is measure of a receiver’s ability to receive a W-CDMA signal at its assigned channel frequency in the presence of an unwanted narrow band interferer at a frequency, which is less than the nominal channel spacing.

Table 7.7A: Narrow band blocking characteristics

| Parameter | Unit | Band II, IV, V, X, XXV | Band III, VIII, XII, XIII, XIV |
| --- | --- | --- | --- |
| DPCH_Ec | dBm/3.84 MHz | <REFSENS> + 10 dB | <REFSENS> + 10 dB |
| Îor | dBm/3.84 MHz | <REFÎor> + 10 dB | <REFÎor> + 10 dB |
| Iblocking (GMSK) | dBm | -57 | -56 |
| Fuw (offset) | MHz | 2.7 | 2.8 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |
| NOTE 1: Iblocking (GMSK) is an interfering signal as defined in TS 45.004 [6] NOTE 2: <REFSENS> and <REFÎor> refer to the DPCH_Ec<REFSENS> and the DPCH<REFÎor> as specified in Table 7.2. |  |  |  |

### 7.6.3A	Additional requirement for DC-HSDPA and DB-DC-HSDPA (Narrow band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.7B. This requirement is measure of a receiver’s ability to receive a W-CDMA signal at its assigned channel frequency in the presence of an unwanted narrow band interferer at a frequency, which is less than the nominal channel spacing.

Table 7.7B: Narrow band blocking characteristics for DC-HSDPA

| Parameter | Unit | Band II, IV, V, X, XXV | Band III, VIII, XII, XIII, XIV |
| --- | --- | --- | --- |
| HS-PDSCH_Ec | dBm/3.84 MHz | <REFSENS> + 10 dB | <REFSENS> + 10 dB |
| Îor | dBm/3.84 MHz | <REFÎor> + 10 dB | <REFÎor> + 10 dB |
| Iblocking (GMSK) | dBm | -57 | -56 |
| Fuw (offset) (NOTE 2) | MHz | 2.7 | 2.8 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |
| NOTE 1: Iblocking (GMSK) is an interfering signal as defined in TS 45.004 [6] NOTE 2: For DC-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency used and positive offset refers to the assigned channel frequency of the highest carrier frequency used. For DB-DC-HSDPA, offset refers to the assigned channel frequencies of the individual cells. NOTE 3: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2A for DC-HSDPA and Table 7.2B for DB-DC-HSDPA. |  |  |  |

### 7.6.3B	Additional requirement for DC-HSUPA (Narrow band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.7C and Table 7.7D. This requirement is measure of a receiver’s ability to receive a W-CDMA signal at its assigned channel frequency in the presence of an unwanted narrow band interferer at a frequency, which is less than the nominal channel spacing.

Table 7.7C: Narrow band blocking characteristics for DC-HSUPA

| Parameter | Unit | Band II, IV, V, X, XXV | Band III, VIII, XII, XIII, XIV |
| --- | --- | --- | --- |
| Iblocking (GMSK) | dBm | -57 | -56 |
| Fuw (offset) (NOTE 2) | MHz | 2.7 | 2.8 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |
| NOTE 1: Iblocking (GMSK) is an interfering signal as defined in TS 45.004 [6] NOTE 2: For DC-HSUPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency used and positive offset refers to the assigned channel frequency of the highest carrier frequency used. |  |  |  |

Table 7.7D: Reference input powers for narrow-band blocking, DC-HSUPA.

| Operating Band | Unit | HS-PDSCH_Ec | Îor |
| --- | --- | --- | --- |
| II | dBm/3.84 MHz | -101 | -90.7 |
| III | dBm/3.84 MHz | -100 | -89.7 |
| IV | dBm/3.84 MHz | -102.8 | -92.5 |
| V | dBm/3.84 MHz | -100.9 | -90.6 |
| VIII | dBm/3.84 MHz | -98.5 | -88.2 |
| X | dBm/3.84 MHz | -102.8 | -92.5 |
| XII | dBm/3.84 MHz | N/A | N/A |
| XIII | dBm/3.84 MHz | N/A | N/A |
| XIV | dBm/3.84 MHz | N/A | N/A |
| XXV | dBm/3.84 MHz | -99.5 | -89.2 |
| NOTE 1 For the UE which supports DB-DC-HSDPA configuration in Table 5.0aA the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 7.12. NOTE 2 For the UE which supports dual band 4C-HSDPA configuration in Table 5.0aC the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 7.13. |  |  |  |

## 7.6.3C	Additional requirement for single band 4C-HSDPA (Narrow band blocking)

### 7.6.3D	Additional requirement for dual band 4C-HSDPA (Narrow band blocking)

This requirement is measure of a receiver’s ability to receive a W-CDMA signal at its assigned channel frequency in the presence of an unwanted narrow band interferer at a frequency, which is less than the nominal channel spacing.

#### 7.6.3D.1	Single uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.7E and Table 7.7F.

# Table 7.7E: Test parameters for narrow band blocking characteristics, dual band 4C-HSDPA, single uplink operation

| Parameter | Unit | Band II, IV, V | Band VIII |
| --- | --- | --- | --- |
| Iblocking (GMSK) | dBm | -57 | -56 |
| Fuw (offset) (NOTE 2) | MHz | 2.7 | 2.8 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |
| NOTE 1: Iblocking (GMSK) is an interfering signal as defined in TS 45.004 [6] NOTE 2: For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequenc(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequenc(ies) in each band. |  |  |  |

Table 7.7F: Narrow band blocking requirements, dual band 4C-HSDPA, single uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | VIII | I | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
|  | VIII | VIII | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
|  | IV |  | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
|  | II | IV | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
|  | IV |  | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | V | I | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
|  | V | V | <REFSENS>+10 dB | <REFÎor>+10 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2D for dual band 4C-HSDPA. |  |  |  |  |  |

#### 7.6.3D.2	Dual uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.7G and Table 7.7H.

Table 7.7G: Test parameters for narrow band blocking characteristics for dual band 4C-HSDPA, dual uplink operation

| Parameter | Unit | Band II, IV, V | Band VIII |
| --- | --- | --- | --- |
| Iblocking (GMSK) | dBm | -57 | -56 |
| Fuw (offset) (NOTE 2) | MHz | 2.7 | 2.8 |
| NOTE 1: Iblocking (GMSK) is an interfering signal as defined in TS 45.004 [6] NOTE 2: For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequenc(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequenc(ies) in each band. |  |  |  |

Table 7.7H: Narrow band blocking requirements, dual band 4C-HSDPA, dual uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UE transmitted mean power (dBm) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | VIII | I | -100 | -89.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-1-IV-2 | II | IV | -100 | -89.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-1 | II | II | -100 | -89.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-2 | II | II | -100 | -89.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | II | IV | -100 | -89.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-1-V-2 | V | V | -99.8 | -89.5 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-1 | V | I | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-2 | V | I | -101 | -90.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V | V | -99.8 | -89.5 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |

## 7.7	Spurious response

### 7.7.1	Minimum requirement

Spurious response is a measure of the receiver’s ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in subclause 7.6.2 is not met.

The BER shall not exceed 0.001 for the parameters specified in Table 7.8.

# Table 7.8: Spurious Response

| Parameter | Unit | Level |
| --- | --- | --- |
| DPCH_Ec | dBm/3.84 MHz | <REFSENS> +3 dB |
| Îor | dBm/3.84 MHz | <REFÎor> +3 dB |
| Iblocking (CW) | dBm | -44 |
| Fuw | MHz | Spurious response frequencies |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 2 |
| NOTE 1: <REFSENS> and <REFÎor> refer to the DPCH_Ec<REFSENS> and the DPCH<REFÎor> as specified in Table 7.2. NOTE 2: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. |  |  |

### 7.7.2	Additional requirement for DC-HSDPA, DB-DC-HSDPA and single band/dual band 4C-HSDPA

Spurious response is a measure of the receiver’s ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in subclause 7.6.2A, 7.6.2B, 7.6.2C or 7.6.2D is not met.

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.8A. The requirement is not applicable for dual uplink operation.

# Table 7.8A: Spurious Response

| Parameter | Unit | Level |
| --- | --- | --- |
| HS-PDSCH_Ec | dBm/3.84 MHz | <REFSENS> +3 dB |
| Îor | dBm/3.84 MHz | <REFÎor> +3 dB |
| Iblocking (CW) | dBm | -44 |
| Fuw | MHz | Spurious response frequencies |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) Note 1 |
| NOTE 1: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2A for DC-HSDPA, Table 7.2B for DB-DC-HSDPA, Table 7.2C for single band 4C-HSDPA, Table 7.2D for dual band 4C-HSDPA. NOTE 2: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. |  |  |

## 7.8	Intermodulation characteristics

Third and higher order mixing of the two interfering RF signals can produce an interfering signal in the band of the desired channel. Intermodulation response rejection is a measure of the capability of the receiver to receiver a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

### 7.8.1	Minimum requirement

The BER shall not exceed 0.001 for the parameters specified in Table 7.9.

# Table 7.9: Receive intermodulation characteristics

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| DPCH_Ec | dBm/3.84 MHz | <REFSENS> +3 dB |  |
| Îor | dBm/3.84 MHz | <REFÎor> +3 dB |  |
| Iouw1 (CW) | dBm | -46 |  |
| Iouw2 mean power (modulated) | dBm | -46 |  |
| Fuw1 (offset) | MHz | 10 | -10 |
| Fuw2 (offset) | MHz | 20 | -20 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 3 |  |
| NOTE 1: Iouw2 (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: <REFSENS> and <REFÎor> refer to the DPCH_Ec<REFSENS> and the DPCH<REFÎor> as specified in Table 7.2. NOTE 3: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. |  |  |  |

### 7.8.1A	Additional requirement for DC-HSDPA and DB-DC-HSDPA

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.9AA.

# Table 7.9AA: Receive intermodulation characteristics

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| HS-PDSCH_Ec | dBm/3.84 MHz | <REFSENS> +3 dB |  |
| Îor | dBm/3.84 MHz | <REFÎor> +3 dB |  |
| Iouw1 (CW) | dBm | -46 |  |
| Iouw2 mean power (modulated) | dBm | -46 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 10 | -10 |
| Fuw2 (offset) (NOTE 2) | MHz | 20 | -20 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 4 |  |
| NOTE 1: Iouw2 (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For DC-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency used and positive offset refers to the assigned channel frequency of the highest carrier frequency used. For DB-DC-HSDPA, offset refers to the assigned channel frequencies of the individual cells. NOTE 3: <REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2A for DC-HSDPA and Table 7.2B for DB-DC-HSDPA. NOTE 4: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. |  |  |  |

### 7.8.1B	Additional requirement for DC-HSUPA

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.9AB and Table 7.9AC.

Table 7.9AB: Receive intermodulation characteristics

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -46 |  |
| Iouw2 mean power (modulated) | dBm | -46 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 10 | -10 |
| Fuw2 (offset) (NOTE 2) | MHz | 20 | -20 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 3 |  |
| NOTE 1: Iouw2 (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For DC-HSUPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency used and positive offset refers to the assigned channel frequency of the highest carrier frequency used. NOTE 3: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. |  |  |  |

Table 7.9AC: Reference input powers for intermod, DC-HSUPA.

| Operating Band | Unit | HS-PDSCH_Ec | Îor |
| --- | --- | --- | --- |
| I | dBm/3.84 MHz | -105 | -94.7 |
| II | dBm/3.84 MHz | -105.3 | -95 |
| III | dBm/3.84 MHz | -104.1 | -93.8 |
| IV | dBm/3.84 MHz | -105 | -94.7 |
| V | dBm/3.84 MHz | -102 | -91.7 |
| VI | dBm/3.84 MHz | -102.2 | -91.9 |
| VII | dBm/3.84 MHz | -105.3 | -95 |
| VIII | dBm/3.84 MHz | -99.8 | -89.5 |
| IX | dBm/3.84 MHz | -104.6 | -94.3 |
| X | dBm/3.84 MHz | -105 | -94.7 |
| XI | dBm/3.84 MHz | -100 | -89.7 |
| XII | dBm/3.84 MHz | N/A | N/A |
| XIII | dBm/3.84 MHz | N/A | N/A |
| XIV | dBm/3.84 MHz | N/A | N/A |
| XIX | dBm/3.84 MHz | -102.2 | -91.9 |
| XX |  |  |  |  | dBm/3.84 MHz | TBD | TBD |
| XXI | dBm/3.84 MHz | -100 | -89.7 |
| XXII | dBm/3.84 MHz | -104.1 | -93.8 |
| XXV |  |  |  |  | dBm/3.84 MHz | -103.5 | -93.2 |
| NOTE 1 For the UE which supports both Band III and Band IX operating frequencies, the reference sensitivity level of TBD dBm <REF_Ec,intermod> shall apply for Band IX. The corresponding <REFÎor,intermod> is TBD dBm NOTE 2 For the UE which supports both Band XI and Band XXI operating frequencies, the reference input power level is FFS. NOTE 3 For the UE which supports DB-DC-HSDPA configuration in Table 5.0aA the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 7.12. NOTE 4 For the UE which supports dual band 4C-HSDPA configuration in Table 5.0aC the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 7.13. |  |  |  |

### 7.8.1C	Additional requirement for single band 4C-HSDPA

#### 7.8.1C.1	Single uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.9AD and Table 7.9AE.

# Table 7.9AD: Test parameters for receive intermodulation characteristics, single band 4C-HSDPA, single uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -46 |  |
| Iouw2 mean power (modulated) | dBm | -46 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 10 | -10 |
| Fuw2 (offset) (NOTE 2) | MHz | 20 | -20 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |
| NOTE 1: Iouw2 (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For single band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequencies, and positive offset refers to the assigned channel frequency of the highest carrier frequencies. |  |  |  |

Table 7.9AE: Intermodulation requirements, single band 4C-HSDPA, single uplink operation

| Single band 4C-HSDPA Configuration | DL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- |
| I-3 | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2C for single band 4C-HSDPA. |  |  |  |  |

#### 7.8.1C.2	Dual uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.9AF and Table 7.9AG.

Table 7.9AF: Receive intermodulation characteristics for single band 4C-HSDPA, dual uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -46 |  |
| Iouw2 mean power (modulated) | dBm | -46 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 10 | -10 |
| Fuw2 (offset) (NOTE 2) | MHz | 20 | -20 |
| NOTE 1: Iouw2 (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For single band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequencies, and positive offset refers to the assigned channel frequency of the highest carrier frequencies. |  |  |  |

Table 7.9AG: Intermodulation requirements, single band 4C-HSDPA, dual uplink operation

| Single band 4C-HSDPA Configuration | DL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UE transmitted mean power (dBm) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-3 | I | -105 | -94.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| NOTE 1 For the UE which supports DB-DC-HSDPA configuration in Table 5.0aA the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 7.12. NOTE 2 For the UE which supports dual band 4C-HSDPA configuration in Table 5.0aC the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 7.13. |  |  |  |  |  |

### 7.8.1D	Additional requirement for dual band 4C-HSDPA

#### 7.8.1D.1	Single uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.9AH and Table 7.9AI.

# Table 7.9AH: Test parameters for receive intermodulation characteristics, dual band 4C-HSDPA, single uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -46 |  |
| Iouw2 mean power (modulated) | dBm | -46 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 10 | -10 |
| Fuw2 (offset) (NOTE 2) | MHz | 20 | -20 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |
| NOTE 1: Iouw2 (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequenc(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequenc(ies) in each band. |  |  |  |

Table 7.9AI: Intermodulation requirements, dual band 4C-HSDPA, single uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | VIII | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | II | IV | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | V | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2D for dual band 4C-HSDPA. |  |  |  |  |  |

#### 7.8.1D.2	Dual uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.9AJ and Table 7.9AK.

Table 7.9AJ: Receive intermodulation characteristics for dual band 4C-HSDPA, dual uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -46 |  |
| Iouw2 mean power (modulated) | dBm | -46 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 10 | -10 |
| Fuw2 (offset) (NOTE 2) | MHz | 20 | -20 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |
| NOTE 1: Iouw2 (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequenc(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequenc(ies) in each band. |  |  |  |

Table 7.9AK: Intermodulation requirements, dual band 4C-HSDPA, dual uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UE transmitted mean power (dBm) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | -104.2 | -93.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | VIII |  | -103.6 | -93.3 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-1-IV-2 | II | IV | -103.1 | -92.8 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -103.4 | -93.1 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-1 | II | II | -103.1 | -92.8 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -103.4 | -93.1 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-2 | II | II | -103.1 | -92.8 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -103.4 | -93.1 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | II | IV | -103.1 | -92.8 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -103.4 | -93.1 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-1-V-2 | I | V | -104.2 | -93.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -101.1 | -90.8 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-1 | I | I | -104.2 | -93.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -103.9 | -93.6 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-2 | I | I | -104.2 | -93.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -103.9 | -93.6 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | I | V | -104.2 | -93.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -101.1 | -90.8 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |

### 7.8.2	Minimum requirement (Narrow band)

The BER shall not exceed 0.001 for the parameters specified in Table 7.9A.

# Table 7.9A: Receive intermodulation characteristics

| Parameter | Unit | Band II, IV, V, X, XXV |  | Band III, VIII, XII, XIII, XIV |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| DPCH_Ec | dBm/3.84 MHz | <REFSENS>+ 10 dB |  | <REFSENS>+ 10 dB |  |  |
| Îor | dBm/3.84 MHz | <REFÎor> + 10 dB |  | <REFÎor> +10 dB |  |  |
| Iouw1 (CW) | dBm | -44 |  | -43 |  |  |
| Iouw2 (GMSK) | dBm | -44 |  | -43 |  |  |
| Fuw1 (offset) | MHz | 3.5 | -3.5 | 3.6 | -3.6 |  |
| Fuw2 (offset) | MHz | 5.9 | -5.9 | 6.0 | -6.0 |  |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |  |  |  |
| NOTE 1: Iouw2 (GMSK) is an interfering signal as defined in TS 45.004 [6]. NOTE 2: <REFSENS> and <REFÎor> refer to the DPCH_Ec<REFSENS> and the DPCH<REFÎor> as specified in Table 7.2. |  |  |  |  |  |

### 7.8.2A	Additional requirement for DC-HSDPA and DB-DC-HSDPA (Narrow band)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.9B.

# Table 7.9B: Receive intermodulation characteristics

| Parameter | Unit | Band II, IV, V, X, XXV |  | Band III, VIII, XII, XIII, XIV |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| HS-PDSCH_Ec | dBm/3.84 MHz | <REFSENS>+ 10 dB |  | <REFSENS>+ 10 dB |  |  |
| Îor | dBm/3.84 MHz | <REFÎor> + 10 dB |  | <REFÎor> +10 dB |  |  |
| Iouw1 (CW) | dBm | -44 |  | -43 |  |  |
| Iouw2 (GMSK) | dBm | -44 |  | -43 |  |  |
| Fuw1 (offset) (NOTE 2) | MHz | 3.5 | -3.5 | 3.6 | -3.6 |  |
| Fuw2 (offset) (NOTE 2) | MHz | 5.9 | -5.9 | 6.0 | -6.0 |  |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |  |  |  |
| NOTE 1: Iouw2 (GMSK) is an interfering signal as defined in TS 45.004 [6]. NOTE 2: For DC-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency used and positive offset refers to the assigned channel frequency of the highest carrier frequency used. For DB-DC-HSDPA, offset refers to the assigned channel frequencies of the individual cells. NOTE3: <REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2A for DC-HSDPA and Table 7.2B for DB-DC-HSDPA. |  |  |  |  |  |

### 7.8.2B	Additional requirement for DC-HSUPA (Narrow band)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.9C and Table 7.9D.

Table 7.9C: Receive intermodulation characteristics

| Parameter | Unit | Band II, IV, V, X,XXV |  | Band III, VIII, XII, XIII, XIV |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -44 |  | -43 |  |  |
| Iouw2 (GMSK) | dBm | -44 |  | -43 |  |  |
| Fuw1 (offset) (NOTE 2) | MHz | 3.5 | -3.5 | 3.6 | -3.6 |  |
| Fuw2 (offset) (NOTE 2) | MHz | 5.9 | -5.9 | 6.0 | -6.0 |  |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |  |  |  |
| NOTE 1: Iouw2 (GMSK) is an interfering signal as defined in TS 45.004 [6]. NOTE 2: For DC-HSUPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency used and positive offset refers to the assigned channel frequency of the highest carrier frequency used. |  |  |  |  |  |

Table 7.9D: Reference input powers for intermodulation, narrow-band, DC-HSUPA.

| Operating Band | Unit | HS-PDSCH_Ec | Îor |
| --- | --- | --- | --- |
| II | dBm/3.84 MHz | -86.9 | -76.6 |
| III | dBm/3.84 MHz | -85.7 | -75.4 |
| IV | dBm/3.84 MHz | -86.9 | -76.6 |
| V | dBm/3.84 MHz | -86.9 | -76.6 |
| VIII | dBm/3.84 MHz | -85.6 | -75.3 |
| X | dBm/3.84 MHz | -86.9 | -76.6 |
| XII | dBm/3.84 MHz | N/A | N/A |
| XIII | dBm/3.84 MHz | N/A | N/A |
| XIV | dBm/3.84 MHz | N/A | N/A |
| XXV |  |  |  |  | dBm/3.84 MHz | -84.7 | -74.4 |
| NOTE 1 For the UE which supports DB-DC-HSDPA configuration in Table 5.0aA the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 7.12. NOTE 2 For the UE which supports dual band 4C-HSDPA configuration in Table 5.0aC the < HS-PDSCH_Ec > and < Îor > are allowed to be increased by an amount defined in Table 7.13. |  |  |  |

### 7.8.2C	Additional requirement for single band 4C-HSDPA (Narrow band)

### 7.8.2D	Additional requirement for dual band 4C-HSDPA (Narrow band)

#### 7.8.2D.1	Single uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.9E and Table 7.9F.

Table 7.9E: Test parameters for receive narrow-band intermodulation characteristics, dual band 4C-HSDPA, single uplink operation

| Parameter | Unit | Band II, IV, V |  | Band VIII |  |
| --- | --- | --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -44 |  | -43 |  |
| Iouw2 (GMSK) | dBm | -44 |  | -43 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 3.5 | -3.5 | 3.6 | -3.6 |
| Fuw2 (offset) (NOTE 2) | MHz | 5.9 | -5.9 | 6.0 | -6.0 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |  |  |
| NOTE 1: Iouw2 (GMSK) is an interfering signal as defined in TS 45.004 [6]. NOTE 2: For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequenc(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequenc(ies) in each band. |  |  |  |  |  |

Table 7.9F: Narrow-band intermodulation requirements, dual band 4C-HSDPA, single uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | VIII | I | <REFSENS>+16.6 dB | <REFÎor>+16.6 dB | Minimum |
|  | VIII | VIII | <REFSENS>+16.6 dB | <REFÎor>+16.6 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+17 dB | <REFÎor>+17 dB | Minimum |
|  | IV |  | <REFSENS>+18.9 dB | <REFÎor>+18.9 dB | Minimum |
|  | II | IV | <REFSENS>+17 dB | <REFÎor>+17 dB | Minimum |
|  | IV |  | <REFSENS>+18.9 dB | <REFÎor>+18.9 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | V | I | <REFSENS>+17 dB | <REFÎor>+17 dB | Minimum |
|  | V | V | <REFSENS>+17 dB | <REFÎor>+17 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2D for dual band 4C-HSDPA. |  |  |  |  |  |  |

#### 7.8.2D.2	Dual uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.9G and Table 7.9H.

Table 7.9G: Test parameters for receive narrow-band intermodulation characteristics, dual band 4C-HSDPA, dual uplink operation

| Parameter | Unit | Band II, IV, V |  | Band VIII |  |
| --- | --- | --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -44 |  | -43 |  |
| Iouw2 (GMSK) | dBm | -44 |  | -43 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 3.5 | -3.5 | 3.6 | -3.6 |
| Fuw2 (offset) (NOTE 2) | MHz | 5.9 | -5.9 | 6.0 | -6.0 |
| NOTE 1: Iouw2 (GMSK) is an interfering signal as defined in TS 45.004 [6]. NOTE 2: For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequenc(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequenc(ies) in each band. |  |  |  |  |  |  |

Table 7.9H: Narrow-band intermodulation requirements, dual band 4C-HSDPA, dual uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UE transmitted mean power (dBm) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | VIII | I | -84.7 | -74.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-1-IV-2 | II | IV | -84.7 | -74.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -84.7 | -74.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-1 | II | II | -84.7 | -74.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -84.7 | -74.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-2 | II | II | -84.7 | -74.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -84.7 | -74.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | II | IV | -84.7 | -74.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -84.7 | -74.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-1-V-2 | V | V | -85.7 | -75.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-1 | V | I | -85.7 | -75.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-2 | V | I | -85.7 | -75.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V | V | -85.7 | -75.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |

------------------------------------------------------------- End of change ------------------------------------------------------------