3GPP TSG-RAN WG4 Meeting #70bis 	R4-141486

# San Jose Del Cabo, Mexico, 31 Mar - 4 Apr 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.101 | CR | 1030 | rev | - | Current version: | 12.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE RF core requirements for UTRA Band I + SDL 1452 - 1496 MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_UTRA_SDL_bandL-Core |  |  |  |  |  |  | Date: |  | 2014-03-31 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | UE RF core requirements need to be introduced for Band I + supplemental DL in L-band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE Tx and Rx core RF requirements for UTRA Band I + SDL 1452 – 1496 MHz are introduced in TS 25.101. - New band XXXII has been introduced as a DL only band for 1452 – 1496 MHz. The following note has been added: “Restricted to UTRA operation with dual band configuration. This supplemental downlink band shall be paired with the other FDD (UL/DL) frequency band as dual band configuration.” - DB-DC-HSDPA configuration 6 and dual band 4C-HSDPA configurations I-1-XXXII-2 and I-2-XXXII-1 have been introduced. - Core requirements for DB-DC-HSDPA configuration 6 and dual band 4C-HSDPA configurations I-1-XXXII-2 and I-2-XXXII-1 have been introduced. - For in-band blocking requirements for DC-HSDPA and DB-DC-HSDPA, non-applicability of requirements to Band XXXII for DC-HSDPA has been explicityly stated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE RF core reuqirements are missing for UTRA Band I + SDL 1452 – 1496 MHz. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.3, 5.4.3, 5.4.4, 6.2.1, 6.6.3.1, 6.6.3.1A, 7.3.3, 7.3.5, 7.5.3, 7.6.1A, 7.6.1D.1, 7.6.1D.2, 7.6.2B, 7.6.2D, 7.8.1D.1, 7.8.1D.2, 7.9.1, 7.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR 34.121-1 CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | L-band is the first supplemental DL band in TS 25.101. Since the UE capability for L-band as a single band cannot be signalled, relevant DB-DC-HSDPA and dual band 4C-HSDPA requirements need to be introduced for UE Tx and Rx core requirements. |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

# 5	Frequency bands and channel arrangement

## 5.1	General

The information presented in this subclause is based on a chip rate of 3.84 Mcps.

NOTE:	Other chip rates may be considered in future releases.

## 5.2	Frequency bands

a)	UTRA/FDD is designed to operate in the following paired bands:

Table 5.0: UTRA FDD frequency bands

| Operating Band | UL Frequencies UE transmit, Node B receive | DL frequencies UE receive, Node B transmit |
| --- | --- | --- |
| I | 1920 - 1980 MHz | 2110 -2170 MHz |
| II | 1850 -1910 MHz | 1930 -1990 MHz |
| III | 1710-1785 MHz | 1805-1880 MHz |
| IV | 1710-1755 MHz | 2110-2155 MHz |
| V | 824 - 849 MHz | 869-894 MHz |
| VI | 830-840 MHz | 875-885 MHz |
| VII | 2500-2570 MHz | 2620-2690 MHz |
| VIII | 880 - 915 MHz | 925 - 960 MHz |
| IX | 1749.9-1784.9 MHz | 1844.9-1879.9 MHz |
| X | 1710-1770 MHz | 2110-2170 MHz |
| XI | 1427.9 - 1447.9 MHz | 1475.9 - 1495.9 MHz |
| XII | 699 – 716 MHz | 729 – 746 MHz |
| XIII | 777 - 787 MHz | 746 - 756 MHz |
| XIV | 788 – 798 MHz | 758 – 768 MHz |
| XV | Reserved | Reserved |
| XVI | Reserved | Reserved |
| XVII | Reserved | Reserved |
| XVIII | Reserved | Reserved |
| XIX | 830 – 845MHz | 875 – 890 MHz |
| XX | 832 – 862 MHz | 791 – 821 MHz |
| XXI | 1447.9 – 1462.9 MHz | 1495.9 – 1510.9 MHz |
| XXII | 3410 – 3490 MHz | 3510 – 3590 MHz |
| XXV | 1850 – 1915 MHz | 1930 – 1995 MHz |
| XXVI | 814 – 849 MHz | 859 – 894 MHz |
| XXXII1 | N/A | 1452-1496 MHz |
| NOTE1: Restricted to UTRA operation with dual band configuration. This supplemental downlink band shall be paired with the other FDD (UL/DL) frequency band as dual band configuration. |  |  |

b)	Deployment in other frequency bands is not precluded

c) DB-DC-HSDPA is designed to operate in the following configurations:

Table 5.0aA DB-DC-HSDPA configurations

| DB-DC-HSDPA Configuration | UL Band | DL Band A | DL Band B |
| --- | --- | --- | --- |
| 1 | I or VIII | I | VIII |
| 2 | II or IV | II | IV |
| 3 | I or V | I | V |
| 4 | I or XI | I | XI |
| 5 | II or V | II | V |
| 6 | I | I | XXXII |

d)	Single band 4C-HSDPA is designed to operate in the following configurations:

Table 5.0aB Single band 4C-HSDPA configurations

| Single band 4C-HSDPA Configuration | Operating Band | Number of DL carriers |
| --- | --- | --- |
| I-3 | I | 3 |
| II-3 | II | 3 |
| II-4 | II | 4 |
| NOTE: Single band 4C-HSDPA configuration is numbered as (X-M) where X denotes the operating band and M denotes the number of DL carriers. |  |  |

e)	Dual band 4C-HSDPA is designed to operate in the following configurations:

Table 5.0aC Dual band 4C-HSDPA configurations

| Dual band 4C-HSDPA Configuration | UL Band | DL Band A | Number of DL carriers in Band A | DL Band B | Number of DL carriers in Band B |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 | I or VIII | I | 2 | VIII | 1 |
| I-2-VIII-2 | I or VIII | I | 2 | VIII | 2 |
| I-1-VIII-2 | I or VIII | I | 1 | VIII | 2 |
| I-3-VIII-1 | I or VIII | I | 3 | VIII | 1 |
| II-1-IV-2 | II or IV | II | 1 | IV | 2 |
| II-2-IV-1 | II or IV | II | 2 | IV | 1 |
| II-2-IV-2 | II or IV | II | 2 | IV | 2 |
| I-1-V-2 | I or V | I | 1 | V | 2 |
| I-2-V-1 | I or V | I | 2 | V | 1 |
| I-2-V-2 | I or V | I | 2 | V | 2 |
| II-1-V-2 | II or V | II | 1 | V | 2 |
| I-1-XXXII-2 | I | I | 1 | XXXII | 2 |
| I-2-XXXII-1 | I | I | 2 | XXXII | 1 |
| NOTE: Dual band 4C-HSDPA configuration is numbered as (X-M-Y-N) where X denotes the DL Band A, M denotes the number DL carriers in the DL Band A, Y denotes the DL Band B, and N denotes the number of DL carriers in the DL Band B |  |  |  |  |  |

f)	Single band 8C-HSDPA is designed to operate in the following configurations:

Table 5.0aD Single band 8C-HSDPA configurations

| Single band 8C-HSDPA Configuration | Operating Band | Number of DL carriers |
| --- | --- | --- |
| I-8 | I | 8 |
| NOTE: Single band 8C-HSDPA configuration is numbered as (X-M) where X denotes the operating band and M denotes the number of DL carriers. |  |  |

g)	Single band NC-4C-HSDPA is designed to operate in the following configurations:

Table 5.0aE Single band NC-4C-HSDPA configurations

| Single band NC-4C-HSDPA Configuration | Operating Band | Number of DL carriers in one subblock | Gap between subblocks [MHz] | Number of DL carriers in the other subblock |
| --- | --- | --- | --- | --- |
| I-1-5-1 | I | 1 | 5 | 1 |
| I-2-5-1 | I | 2 | 5 | 1 |
| I-3-10-1 | I | 3 | 10 | 1 |
| IV-1-5-1 | IV | 1 | 5 | 1 |
| IV-2-10-1 | IV | 2 | 10 | 1 |
| IV-2-15-2 | IV | 2 | 15 | 2 |
| IV-2-20-1 | IV | 2 | 20 | 1 |
| IV-2-25-2 | IV | 2 | 25 | 2 |
| NOTE: Single band NC-4C-HSDPA configuration is numbered as (X-M-Y-N) where X denotes the operating band, M denotes the number of DL carriers in one subblock, Y denotes the gap between subblocks in MHz and N denotes the number of DL carriers in the other subblock. M and N can be switched |  |  |  |  |

## 5.3	TX-RX frequency separation

a)	UTRA/FDD is designed to operate with the following TX-RX frequency separation

Table 5.0A: TX-RX frequency separation

| Operating Band | TX-RX frequency separation |
| --- | --- |
| I | 190 MHz |
| II | 80 MHz. |
| III | 95 MHz. |
| IV | 400 MHz |
| V | 45 MHz |
| VI | 45 MHz |
| VII | 120 MHz |
| VIII | 45 MHz |
| IX | 95 MHz |
| X | 400 MHz |
| XI | 48 MHz |
| XII | 30 MHz |
| XIII | 31 MHz |
| XIV | 30 MHz |
| XIX | 45 MHz |
| XX | 41 MHz |
| XXI | 48 MHz |
| XXII | 100 MHz |
| XXV | 80 MHz |
| XXVI | 45MHz |
| XXXII | N/A |

b)	UTRA/FDD can support both fixed and variable transmit to receive frequency separation.

c)	The use of other transmit to receive frequency separations in existing or other frequency bands shall not be precluded.

d)	When configured to operate on dual cells in the DL with a single UL frequency, the TX-RX frequency separation in Table 5.0A shall be applied for the serving HS-DSCH cell. For bands XII, XIII and XIV, the TX-RX frequency separation in Table 5.0A shall be the minimum spacing between the UL and either of the DL carriers.

e)	When configured to operate on dual cells in both the DL and UL, the TX-RX frequency separation in Table 5.0A shall be applied to the primary UL frequency and DL frequency of the serving HS-DSCH cell, and to the secondary UL frequency and the frequency of the secondary serving HS-DSCH cell respectively.

f)	When configured to operate on single/dual band 4C-HSDPA or single band 8C-HSDPA or single band NC-4C-HSDPA with a single UL frequency, the TX-RX frequency separation in Table 5.0A shall be applied for the DL frequency of the serving HS-DSCH cell. When configured to operate on single/dual band 4C-HSDPA or single band 8C-HSDPA or single band NC-4C-HSDPA with dual UL frequencies, the TX-RX frequency separation in Table 5.0A shall be applied to the primary UL frequency and DL frequency of the serving HS-DSCH cell, and to the secondary UL frequency and the frequency of the 1st secondary serving HS-DSCH cell respectively.

g)	For bands XII, XIII and XIV, all the requirements in TS 25.101 are applicable only for a single uplink carrier frequency, however dual cell uplink operation may be considered in future releases.

## 5.4	Channel arrangement

### 5.4.1	Channel spacing

The nominal channel spacing is 5 MHz, but this can be adjusted to optimise performance in a particular deployment scenario. In DC-HSDPA and DB-DC-HSDPA mode, the UE receives two cells simultaneously. In context of DC-HSDPA and DB-DC-HSDPA, a cell is characterized by a combination of scrambling code and a carrier frequency, see [21.905].

### 5.4.2	Channel raster

The channel raster is 200 kHz, for all bands which means that the centre frequency must be an integer multiple of 200 kHz. In addition a number of additional centre frequencies are specified according to table 5.1A, which means that the centre frequencies for these channels are shifted 100 kHz relative to the general raster.

### 5.4.3	Channel number

The carrier frequency is designated by the UTRA Absolute Radio Frequency Channel Number (UARFCN). For each operating Band, the UARFCN values are defined as follows:

Uplink:		NU = 5 * (FUL - FUL_Offset),	for the carrier frequency range FUL_low FUL  FUL_high

Downlink:	ND = 5 * (FDL - FDL_Offset),	for the carrier frequency range FDL_low FDL  FDL_high

For each operating Band, FUL_Offset, FUL_low FUL_high, FDL_Offset,, FDL_lowand FDL_high are defined in Table 5.1 for the general UARFCN. For the additional UARFCN, FUL_Offset, FDL_Offset and the specific FUL and FDL are defined in Table 5.1A.

# Table 5.1: UARFCN definition (general)

| Band | UPLINK (UL) UE transmit, Node B receive |  |  | DOWNLINK (DL) UE receive, Node B transmit |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | UARFCN formula offset FUL_Offset [MHz] | Carrier frequency (FUL) range [MHz] |  | UARFCN formula offset FDL_Offset [MHz] | Carrier frequency (FDL) range [MHz] |  |
|  |  | FUL_low | FUL_high |  | FDL_low | FDL_high |
| I | 0 | 1922.4 | 1977.6 | 0 | 2112.4 | 2167.6 |
| II | 0 | 1852.4 | 1907.6 | 0 | 1932.4 | 1987.6 |
| III | 1525 | 1712.4 | 1782.6 | 1575 | 1807.4 | 1877.6 |
| IV | 1450 | 1712.4 | 1752.6 | 1805 | 2112.4 | 2152.6 |
| V | 0 | 826.4 | 846.6 | 0 | 871.4 | 891.6 |
| VI | 0 | 832.4 | 837.6 | 0 | 877.4 | 882.6 |
| VII | 2100 | 2502.4 | 2567.6 | 2175 | 2622.4 | 2687.6 |
| VIII | 340 | 882.4 | 912.6 | 340 | 927.4 | 957.6 |
| IX | 0 | 1752.4 | 1782.4 | 0 | 1847.4 | 1877.4 |
| X | 1135 | 1712.4 | 1767.6 | 1490 | 2112.4 | 2167.6 |
| XI | 733 | 1430.4 | 1445.4 | 736 | 1478.4 | 1493.4 |
| XII | -22 | 701.4 | 713.6 | -37 | 731.4 | 743.6 |
| XIII | 21 | 779.4 | 784.6 | -55 | 748.4 | 753.6 |
| XIV | 12 | 790.4 | 795.6 | -63 | 760.4 | 765.6 |
| XIX | 770 | 832.4 | 842.6 | 735 | 877.4 | 887.6 |
| XX | -23 | 834.4 | 859.6 | -109 | 793.4 | 818.6 |
| XXI | 1358 | 1450.4 | 1460.4 | 1326 | 1498.4 | 1508.4 |
| XXII | 2525 | 3412.4 | 3487.6 | 2580 | 3512.4 | 3587.6 |
| XXV | 875 | 1852.4 | 1912.6 | 910 | 1932.4 | 1992.6 |
| XXVI | -291 | 816.4 | 846.6 | -291 | 861.4 | 891.6 |
| XXXII | N/A |  |  | 135,8 | 1454.4 | 1493.6 |

# Table 5.1A: UARFCN definition (additional channels)

| Band | UPLINK (UL) UE transmit, Node B receive |  | DOWNLINK (DL) UE receive, Node B transmit |  |
| --- | --- | --- | --- | --- |
|  | UARFCN formula offset FUL_Offset [MHz] | Carrier frequency [MHz] (FUL) | UARFCN formula offset FDL_Offset [MHz] | Carrier frequency [MHz] (FDL) |
| I | - | - | - | - |
| II | 1850.1 | 1852.5, 1857.5, 1862.5, 1867.5, 1872.5, 1877.5, 1882.5, 1887.5, 1892.5, 1897.5, 1902.5, 1907.5 | 1850.1 | 1932.5, 1937.5, 1942.5, 1947.5, 1952.5, 1957.5, 1962.5, 1967.5, 1972.5, 1977.5, 1982.5, 1987.5 |
| III | - | - | - | - |
| IV | 1380.1 | 1712.5, 1717.5, 1722.5, 1727.5, 1732.5, 1737.5 1742.5, 1747.5, 1752.5 | 1735.1 | 2112.5, 2117.5, 2122.5, 2127.5, 2132.5, 2137.5, 2142.5, 2147.5, 2152.5 |
| V | 670.1 | 826.5, 827.5, 831.5, 832.5, 837.5, 842.5 | 670.1 | 871.5, 872.5, 876.5, 877.5, 882.5, 887.5 |
| VI | 670.1 | 832.5, 837.5 | 670.1 | 877.5, 882.5 |
| VII | 2030.1 | 2502.5, 2507.5, 2512.5, 2517.5, 2522.5, 2527.5, 2532.5, 2537.5, 2542.5, 2547.5, 2552.5, 2557.5, 2562.5, 2567.5 | 2105.1 | 2622.5, 2627.5, 2632.5, 2637.5, 2642.5, 2647.5, 2652.5, 2657.5, 2662.5, 2667.5, 2672.5, 2677.5, 2682.5, 2687.5 |
| VIII | - | - | - | - |
| IX | - | - | - | - |
| X | 1075.1 | 1712.5, 1717.5, 1722.5, 1727.5, 1732.5, 1737.5, 1742.5, 1747.5, 1752.5, 1757.5, 1762.5, 1767.5 | 1430.1 | 2112.5, 2117.5, 2122.5, 2127.5, 2132.5, 2137.5, 2142.5, 2147.5, 2152.5, 2157.5, 2162.5, 2167.5 |
| XI | - | - | - | - |
| XII | -39.9 | 701.5, 706.5, 707.5, 712.5, 713.5 | -54.9 | 731.5, 736.5, 737.5, 742.5, 743.5 |
| XIII | 11.1 | 779.5, 784.5 | -64.9 | 748.5, 753.5 |
| XIV | 2.1 | 790.5, 795.5 | -72.9 | 760.5, 765.5 |
| XIX | 755.1 | 832.5, 837.5, 842.5 | 720.1 | 877.5, 882.5, 887.5 |
| XX | - | - | - | - |
| XXI | - | - | - | - |
| XXII | - | - | - | - |
| XXV | 639.1 | 1852.5, 1857.5, 1862.5,1867.5, 1872.5, 1877.5, 1882.5, 1887.5, 1892.5, 1897.5, 1902.5, 1907.5, 1912.5 | 674.1 | 1932.5, 1937.5, 1942.5, 1947.5, 1952.5, 1957.5, 1962.5, 1967.5, 1972.5, 1977.5, 1982.5, 1987.5, 1992.5 |
| XXVI | -325.9 | 816.5, 821.5, 826.5, 827.5, 831.5, 832.5, 836.5, 837.5, 841.5, 842.5, 846.5 | -325.9 | 861.5, 866.5, 871.5, 872.5, 876.5, 877.5, 881.5, 882.5, 886,5, 887.5, 891.5 |
| XXXII | N/A |  | - | - |

### 5.4.4	UARFCN

The following UARFCN range shall be supported for each paired band

Table 5.2: UTRA Absolute Radio Frequency Channel Number

| Band |  | Uplink (UL) UE transmit, Node B receive |  |  |  | Downlink (DL) UE receive, Node B transmit |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | General |  | Additional |  | General |  | Additional |  |
| I |  | 9612 to 9888 |  | - |  | 10562 to 10838 |  | - |  |
| II |  | 9262 to 9538 |  | 12, 37, 62, 87, 112, 137, 162, 187, 212, 237, 262, 287 |  | 9662 to 9938 |  | 412, 437, 462, 487, 512, 537, 562, 587, 612, 637, 662, 687 |  |
| III |  | 937 to 1288 |  | - |  | 1162 to 1513 |  | - |  |
| IV |  | 1312 to 1513 |  | 1662, 1687, 1712, 1737, 1762, 1787, 1812, 1837, 1862 |  | 1537 to 1738 |  | 1887, 1912, 1937, 1962, 1987, 2012, 2037, 2062, 2087 |  |
| V |  | 4132 to 4233 |  | 782, 787, 807, 812, 837, 862 |  | 4357 to 4458 |  | 1007, 1012, 1032, 1037, 1062, 1087 |  |
| VI |  | 4162 to 4188 |  | 812, 837 |  | 4387 to 4413 |  | 1037, 1062 |  |
| VII |  | 2012 to 2338 |  | 2362, 2387, 2412, 2437, 2462, 2487, 2512, 2537, 2562, 2587, 2612, 2637, 2662, 2687 |  | 2237 to 2563 |  | 2587, 2612, 2637, 2662, 2687, 2712, 2737, 2762, 2787, 2812, 2837, 2862, 2887, 2912 |  |
| VIII |  | 2712 to 2863 |  | - |  | 2937 to 3088 |  | - |  |
| IX |  | 8762 to 8912 |  | - |  | 9237 to 9387 |  | - |  |
| X |  | 2887 to 3163 |  | 3187, 3212, 3237, 3262, 3287, 3312, 3337, 3362, 3387, 3412, 3437, 3462 |  | 3112 to 3388 |  | 3412, 3437, 3462, 3487, 3512, 3537, 3562, 3587, 3612, 3637, 3662, 3687 |  |
| XI |  | 3487 to 3562 |  | - |  | 3712 to 3787 |  | - |  |
| XII |  | 3617 to 3678 |  | 3707, 3732, 3737, 3762, 3767 |  | 3842 to 3903 |  | 3932, 3957, 3962, 3987, 3992 |  |
| XIII |  | 3792 to 3818 |  | 3842, 3867 |  | 4017 to 4043 |  | 4067, 4092 |  |
| XIV |  | 3892 to 3918 |  | 3942, 3967 |  | 4117 to 4143 |  | 4167, 4192 |  |
| XIX |  | 312 to 363 |  | 387, 412, 437 |  | 712 to 763 |  | 787, 812, 837 |  |
| XX |  | 4287 to 4413 |  | - |  | 4512 to 4638 |  | - |  |
| XXI |  | 462 to 512 |  | - |  | 862 to 912 |  | - |  |
| XXII |  | 4437 to 4813 |  | - |  | 4662 to 5038 |  | - |  |
| XXV |  | 4887 to 5188 |  | 6067, 6092, 6117, 6142, 6167, 6192, 6217, 6242, 6267, 6292, 6317, 6342, 6367 |  | 5112 to 5413 |  | 6292, 6317, 6342, 6367, 6392, 6417, 6442, 6467, 6492, 6517, 6542, 6567, 6592 |  |
| XXVI |  | 5537 to 5688 |  | 5712, 5737, 5762, 5767, 5787, 5792, 5812, 5817, 5837, 5842, 5862 |  | 5762 to 5913 |  | 5937, 5962, 5987, 5992, 6012, 6017, 6037, 6042, 6062, 6067, 6087 |  |
| XXXII |  | N/A |  |  |  | 6593 to 6789 |  | - |  |

NOTE:	If the UE is on a network with Mobile Country Code set to Japan then it may assume that any DL UARFCN sent by the network from the overlapping region of Band V and Band VI is from Band VI. If the UE is on a network with a Mobile Country Code other than Japan then it may assume that any DL UARFCN sent by the network from the overlapping region of Band V and Band VI is from Band V.

-------------------------------------------------- Unchanged sections omitted ------------------------------------------------------

## 6.2	Transmit power

### 6.2.1	UE maximum output power

The following Power Classes define the nominal maximum output power. The nominal power defined is the broadband transmit power of the UE, i.e. the power in a bandwidth of at least (1+α) times the chip rate of the radio access mode. The period of measurement shall be at least one timeslot. For DC-HSUPA, the nominal transmit power is defined by the sum of the broadband transmit power of each carrier in the UE.

# Table 6.1: UE Power Classes

| Operating Band | Power Class 1 |  | Power Class 2 |  | Power Class 3 |  | Power Class 3bis |  | Power Class 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Power (dBm) | Tol (dB) | Power (dBm) | Tol (dB) | Power (dBm) | Tol (dB) | Power (dBm) | Tol (dB) | Power (dBm) | Tol (dB) |
| Band I | +33 | +1/-3 | +27 | +1/-3 | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band II | - | - | - | - | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band III | - | - | - | - | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band IV | - | - | - | - | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band V | - | - | - | - | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band VI | - | - | - | - | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band VII | - | - | - | - | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band VIII | - | - | - | - | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band IX | - | - | - | - | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band X | - | - | - | - | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band XI | - | - | - | - | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band XII | - | - | - | - | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band XIII | - | - | - | - | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band IV | - | - | - | - | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band XIX |  |  |  |  | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band XX |  |  |  |  | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band XXI |  |  |  |  | +24 | +1/-3 | 23 | +2/-2 | +21 | +2/-2 |
| Band XXII | - | - | - | - | +24 | +1/-4.5 | 23 | +2/-3.5 | +21 | +2/-3.5 |
| Band XXV |  |  |  |  | +24 | +1/-4 | 23 | +2/-3 | +21 | +2/-3 |
| Band XXVI (Note 1) | - | - | - | - | +24 | +1/-4 | 23 | +2/-3 | +21 | +2/-3 |
| NOTE 1 For the UE which supports both Band V and Band XXVI operating frequencies, the UE maximum output power of Band V shall apply for Band XXVI when the carrier frequency of the assigned UTRA channel is within 824-845 MHz. |  |  |  |  |  |  |  |  |  |  |

NOTE:	The tolerance allowed for the nominal maximum output power applies even for the multi-code DPDCH transmission mode.

For the UE which supports DB-DC-HSDPA configuration in Table 6.1aB, the lower side of the tolerance in Table 6.1 is allowed to be adjusted by the amount given in Table 6.1aB for the applicable bands.

Table 6.1aB Allowed adjustment in lower side of tolerance for UE which supports DB-DC-HSDPA

| DB-DC-HSDPA Configuration |  |  | Maximum allowed adjustment in lower side of tolerance (dB) |  |  | Applicable bands |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | -0.3 |  |  | I, VIII |  |  |
| 2 |  |  | -1 |  |  | II, IV |  |  |
| 3 |  |  | -0.3 |  |  | I, V |  |  |
| 4 |  |  | -1 |  |  | I, XI |  |  |
| 5 |  |  | -0.3 |  |  | II, V |  |  |
| 6 | -0.3 |  | I |
| NOTE: The requirements reflect what can be achieved with the present state of the art technology. They shall be reconsidered when the state of the art technology progresses. |  |  |  |  |  |  |  |  |

For the UE which supports dual band 4C-HSDPA configuration in Table 6.1aC, the lower side of the tolerance in Table 6.1 is allowed to be adjusted by the amount given in Table 6.1aC for the applicable bands.

Table 6.1aC Allowed adjustment in lower side of tolerance for UE which supports dual band 4C-HSDPA

| Dual Band 4C-HSDPA Configuration | Maximum allowed adjustment in lower side of tolerance (dB) | Applicable bands |
| --- | --- | --- |
| I-2-VIII-1, I-3-VIII-1, I-2-VIII-2, I-1-VIII-2 | -0.3 | I, VIII |
| II-1-IV-2, II-2-IV-1, II-2-IV-2 | -1 | II, IV |
| I-1-V-2, I-2-V-1, I-2-V-2 | -0.3 | I, V |
| II-1-V-2 | -0.3 | II, V |
| I-1-XXXII-2, I-2-XXXII-1 | -0.3 | I |
| NOTE: The requirements reflect what can be achieved with the present state of the art technology. They shall be reconsidered when the state of the art technology progresses. |  |  |

For the UE which supports E-UTRA inter-band carrier aggregation, the lower side of the tolerance in Table 6.1 is allowed to be decreased by the amount given in Table 6.2.5A-3 of TS 36.101[11] for those UTRA operating bands corresponding to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations.  The tolerance in Table 6.2.5A-3 of TS 36.101[11] does not apply to supported UTRA operating bands with frequency range below 1 GHz that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations when such bands are belonging only to band combination(s) where one band is <1GHz and another band is >1.7GHz and there is no harmonic relationship between the low band UL and high band DL.

In case the UE supports DB-DC-HSDPA or dual band 4C-HSDPA configurations and one or more of the E-UTRA inter-band carrier aggregation configurations listed in Table 6.2.5A-3 of TS36.101[11] with a UTRA operating band that belongs to UTRA and E-UTRA carrier aggregation configurations, then

-	When the UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the applicable tolerances, truncated to one decimal place for that operating band among the supported DB-DC-HSDPA, dual band 4C-HSDPA, and E-UTRA CA configurations, with the DB-DC-HSDPA, dual carrier 4C-HSDPA, and E-UTRA CA configurations counted separately. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported carrier aggregation configurations involving such band shall be applied

-	When the UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum tolerance that  applies for that operating band among the supported DB-DC-HSDPA, dual band 4C-HSDPA, and E-UTRA CA configurations.

-------------------------------------------------- Unchanged sections omitted ------------------------------------------------------

### 6.6.3	Spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emission, intermodulation products and frequency conversion products, but exclude out of band emissions.

The frequency boundary and the detailed transitions of the limits between the requirement for out band emissions and spectrum emissions are based on ITU-R Recommendations SM.329 [2].

#### 6.6.3.1	Minimum requirement

These requirements are only applicable for frequencies, which are greater than 12.5 MHz away from the UE centre carrier frequency.

# Table 6.12: General spurious emissions requirements

| Frequency Bandwidth | Measurement Bandwidth | Minimum requirement | Note |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | 1 kHz | -36 dBm |  |
| 150 kHz f < 30 MHz | 10 kHz | -36 dBm |  |
| 30 MHz f < 1000 MHz | 100 kHz | -36 dBm |  |
| 1 GHz f < 12.75 GHz | 1 MHz | -30 dBm |  |
| 12.75 GHz f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | 1 MHz | -30 dBm | Note 1 |
| NOTE 1: Applies only for Band XXII. |  |  |  |

# Table 6.13: Additional spurious emissions requirements

| Operating Band | Frequency Bandwidth | Measurement Bandwidth | Minimum requirement |
| --- | --- | --- | --- |
| I | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1452 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm * |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz <f< 1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2570 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm |
| II | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 758 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2690 MHz | 1 MHz | -50 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm** |
| III | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f  890 MHz | 3.84 MHz | -60 dBm ***** |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * - 60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1452  f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1496 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm ***** |
|  | 1805 MHz f 1880 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm ***** |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2570 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm ** |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm ** |
| IV | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2690 MHz | 1 MHz | -50 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm** |
| V | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 859 MHz f 869 MHz | 1 MHz | -27 dBm |
|  | 869 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2690 MHz | 1 MHz | -50 dBm** |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm |
| VI | 758 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f < 875 MHz | 1 MHz | -37 dBm |
|  | 875 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
| VII | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 758 MHz f 791 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 869 MHz | 1 MHz | -50 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1452  f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm * |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 2590 MHz  f  2620 MHz | 3.84 MHz | -50 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm |
| VIII | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 860 MHz f 890 MHz | 1 MHz | -37 dBm **** |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz 3.84 MHz | -79 dBm * -60 dBm |
|  | 1452  f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1496 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm **** |
|  | 1805 MHz < f  1830 MHz | 100 kHz 3.84 MHz | -71 dBm ** & * -60 dBm ** |
|  | 1830 MHz < f  1880 MHz | 100 kHz 3.84 MHz | -71 dBm * -60 dBm |
|  | 1884.5 MHz f 1915.7 MHz | 300 kHz | -41 dBm **** |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2570 MHz  f  2640 MHz | 3.84 MHz | -60 dBm |
|  | 2640 MHz < f  2690 MHz | 3.84 MHz | -60 dBm ** |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm ** |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm ** |
| IX | 758 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
| X | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm ** |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm ** |
| XI | 758 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
| XII | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm |
| XIII | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 763 MHz f 775 MHz | 6.25 kHz | [TBD] dBm*** |
|  | 793 MHz f 805 MHz | 6.25 kHz | [TBD] dBm*** |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm** |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm** |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm |
| XIV | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 769 MHz f 775 MHz | 6.25 kHz | [TBD] dBm *** |
|  | 799 MHz f 805 MHz | 6.25 kHz | [TBD] dBm *** |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm |
| XIX | 758 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f < 875 MHz | 1 MHz | -37 dBm |
|  | 875 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
| XX | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1452  f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm * |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 2570 MHz  f  2620 MHz | 3.84 MHz | -60 dBm** |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm ** |
| XXI | 758 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 1 MHz | -35 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
| XXII | 758 MHz f 791 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1452  f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm * |
|  | 1880 MHz  f  1920 MHz | 3.84 MHz | -60 dBm |
|  | 2010 MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2300 MHz  f  2400 MHz | 3.84 MHz | -60 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 2570 MHz  f  2620 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3525 MHz | 1 MHz | -40 dBm |
|  | 3525 MHz  f  3590 MHz | 1 MHz | -50 dBm |
|  | 3600 MHz  f  3800 MHz | 3.84 MHz | -50 dBm |
| XXV | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 3.84 MHz | -60 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz f 3800 MHz | 1 MHz | -50 dBm ** |
| XXVI | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 799 MHz | 1 MHz | -50 dBm |
|  | 799 MHz f 803 MHz | 1 MHz | -40 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz f 1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1839.9 MHz f 1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2010 MHz f 2025 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz f 2400 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm ** |
|  | 3400 MHz f 3800 MHz | 1 MHz | -50 dBm |
| Note * The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 6.12 are permitted for each UARFCN used in the measurement Note ** The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, measurements with a level up to the applicable requirements defined in Table 6.12 are permitted for each UARFCN used in the measurement due to 2nd, 3rd and 4th harmonic spurious emissions Note *** This requirement is applicable also for frequencies, which are between 2.5 MHz and 12.5 MHz away from the UE centre carrier frequency. Note **** This requirement is applicable only when transmission is made between 900MHz to 915MHz. Note ***** This requirement is applicable only when transmission is made between 1744.9 MHz to 1784.9 MHz |  |  |  |

##### 6.6.3.1.1	Additional requirement with a guard band

These requirements are applicable only for frequencies which are greater than Fguard MHz away from the UE transmit carrier frequency.

# Table 6.13a: Additional spurious emissions requirements with a guard band

| Operating Band | Frequency Bandwidth | Measurement Bandwidth | Minimum requirement | Guard Band (Fguard MHz) |
| --- | --- | --- | --- | --- |
| XXVI | 806 MHz f 813.5 MHz | 6.25 kHz | -42 dBm | TBD |
|  | 806 MHz f 816 MHz | 6.25 kHz | -42 dBm | TBD |
|  | 851 MHz f 859 MHz | 1 MHz | -32 dBm | TBD |
|  | 851 MHz f 859 MHz | 6.25 kHz | -53 dBm | TBD |

#### 6.6.3.1A	Additional requirement for DC-HSUPA

The requirements in Table 6.12A are only applicable for frequencies, which are greater than 20 MHz away from the centre of the assigned carrier frequencies when dual adjacent carriers are assigned on the uplink.

Table 6.12A: General spurious emissions requirements for DC-HSUPA

| Frequency Bandwidth | Measurement Bandwidth | Minimum requirement | Note |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | 1 kHz | -36 dBm |  |
| 150 kHz f < 30 MHz | 10 kHz | -36 dBm |  |
| 30 MHz f < 1000 MHz | 100 kHz | -36 dBm |  |
| 1 GHz f < 12.75 GHz | 1 MHz | -30 dBm |  |
| 12.75 GHz f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | 1 MHz | -30 dBm | Note 1 |
| NOTE 1: Applies only for Band XXII. |  |  |  |

The requirements in Table 6.13A are only applicable for frequencies, which are greater than 25 MHz away from the centre of the assigned frequencies when dual adjacent carriers are assigned on the uplink.

Table 6.13A: Additional spurious emissions requirements for DC-HSUPA

| Operating Band | Frequency Bandwidth | Measurement Bandwidth | Minimum requirement |
| --- | --- | --- | --- |
| I | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1452 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm * |
|  | 1844.9 MHz  f  1879.9 MHz | 3.84 MHz | -55 dBm |
|  | 1884.5 MHz <f< 1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2570 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm |
| II | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 758 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2620 MHz | 1 MHz | -50 dBm |
| III | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 869 MHz | 1 MHz | -50 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * - 60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1452  f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz f 1880 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2570 MHz  f  2620 MHz | 3.84 MHz | -60 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm ** |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm ** |
| IV | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2690 MHz | 1 MHz | -50 dBm |
| V | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 3.84 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 859 MHz f 869 MHz | 1 MHz | -27 dBm |
|  | 869 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2620 MHz | 1 MHz | -50 dBm ** |
| VI | 860 MHz  f < 875 MHz | 1 MHz | -37 dBm |
|  | 875 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1844.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
| VII | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 758 MHz f 791 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 869 MHz | 1 MHz | -50 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1452  f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm * |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 2590 MHz  f  2620 MHz | 1 MHz | -37 dBm |
| VIII | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -57 dBm *, *** -50 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz 3.84 MHz | -79 dBm * -60 dBm |
|  | 1452  f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz < f  1830 MHz | 100 kHz 3.84 MHz | -71 dBm ** & * -60 dBm ** |
|  | 1830 MHz < f  1880 MHz | 100 kHz 3.84 MHz | -71 dBm * -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2570 MHz f 2620 MHz | 3.84 MHz | -60 dBm |
|  | 2620 MHz  f  2640 MHz | 3.84 MHz | -60 dBm |
|  | 2640 MHz < f  2690 MHz | 3.84 MHz | -60 dBm ** |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm ** |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm ** |
| IX | 758 MHz  f  803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1844.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
| X | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
| XI | 758 MHz  f  803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1844.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
| XIX | 758 MHz  f  803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f < 875 MHz | 1 MHz | -30 dBm |
|  | 875 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1844.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
|  | 2595 MHz  f  2645 MHz | 1 MHz | -50 dBm |
| XX | 811 MHz f 821 MHz | 3.84 MHz | -50 dBm *** |
|  | 791 MHz f 811 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1452  f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm * |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 2570 MHz  f  2620 MHz | 3.84 MHz | -60 dBm ** |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm ** |
| XXII | 758 MHz  f  791 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 869 MHz | 1 MHz | -50 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1452  f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm * |
|  | 1880 MHz  f  1920 MHz | 3.84 MHz | -60 dBm |
|  | 2010 MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2300 MHz  f  2400 MHz | 3.84 MHz | -60 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 2570 MHz  f  2620 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3525 MHz | 1 MHz | -40 dBm |
|  | 3525 MHz  f  3590 MHz | 1 MHz | -50 dBm |
|  | 3600 MHz f  3800 MHz | 3.84 MHz | -50 dBm |
| XXV | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 3.84 MHz | -60 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm |
|  | 3400 MHz f 3800 MHz | 1 MHz | -50 dBm |
| XXVI | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 799 MHz | 1 MHz | -50 dBm |
|  | 799 MHz f 803 MHz | 1 MHz | -40 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz f 1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1844.9 MHz f 1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f 1915.7 MHz | 300 kHz | -41 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2010 MHz f 2025 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz f 2400 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm ** |
|  | 3400 MHz f 3800 MHz | 1 MHz | -50 dBm |
| Note * The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 6.12 are permitted for each UARFCN used in the measurement Note ** The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, measurements with a level up to the applicable requirements defined in Table 6.12 are permitted for each UARFCN used in the measurement due to 2nd, 3rd and 4th harmonic spurious emissions Note *** This requirement is applicable also for frequencies, which are between 5 MHz and 25 MHz away from the UE centre carrier frequency. |  |  |  |

##### 6.6.3.1A.1	Additional requirement with a guard band for DC-HSUPA

These requirements are applicable only for frequencies which are greater than Fguard MHz away from the UE transmit center carrier frequency.

# Table 6.13B: Additional spurious emissions requirements with a guard band

| Operating Band | Frequency Bandwidth | Measurement Bandwidth | Minimum requirement | Guard Band (Fguard MHz) |
| --- | --- | --- | --- | --- |
| XXVI | 806 MHz f 813.5 MHz | 6.25 kHz | -42 dBm | TBD |
|  | 806 MHz f 816 MHz | 6.25 kHz | -42 dBm | TBD |
|  | 851 MHz f 859 MHz | 1 MHz | -32 dBm | TBD |
|  | 851 MHz f 859 MHz | 6.25 kHz | -53 dBm | TBD |

#### 6.6.3.1B	Additional requirement for UL OLTD

For UE with two active transmit antenna connectors in UL OLTD operation, the requirements specified in sub-clause 6.6.3.1 apply at each transmit antenna connector.

#### 6.6.3.1C	Additional requirement for UL CLTD

For UE with two active transmit antenna connectors in UL CLTD activation state 1, the spectrum emission requirements specified in sub-clause 6.6.3.1 apply at each transmit antenna connector.

For UE configured in UL CLTD activation state 2 or activation state 3, the spectrum emission requirements in sub-clause 6.6.3.1 apply at the active transmit antenna connector.

#### 6.6.3.1D	Additional requirement for UL MIMO

For UE with two active transmit antenna connectors in UL MIMO operation, the spectrum emission requirements specified in sub-clause 6.6.3.1 apply at each transmit antenna connector.

-------------------------------------------------- Unchanged sections omitted ------------------------------------------------------

### 7.3.3	Additional requirement for DB-DC-HSDPA

For all requirements listed in Table 7.2.B, corresponding to the specific DB-DC-HSDPA configuration(s) supported by the UE, (see Table 5.0aA), the BLER measured on each individual cell shall not exceed 0.1.

Table 7.2B: Test parameters for reference sensitivity, additional requirement for DB-DC-HSDPA.

| DB-DC-HSDPA configuration | DL Band | UL Band | Unit | HS-PDSCH_Ec <REFSENS> | <REFÎor> |
| --- | --- | --- | --- | --- | --- |
| 1 | I | I | dBm/3.84 MHz | -113 | -102.7 |
|  | VIII |  | dBm/3.84 MHz | -110 | -99.7 |
|  | I | VIII | dBm/3.84 MHz | -113 | -102.7 |
|  | VIII |  | dBm/3.84 MHz | -110 | -99.7 |
| 2 | II | II | dBm/3.84 MHz | -110 | -99.7 |
|  | IV |  | dBm/3.84 MHz | -112 | -101.7 |
|  | II | IV | dBm/3.84 MHz | -110 | -99.7 |
|  | IV |  | dBm/3.84 MHz | -112 | -101.7 |
| 3 | I | I | dBm/3.84 MHz | -113 | -102.7 |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 |
|  | I | V | dBm/3.84 MHz | -113 | -102.7 |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 |
| 4 | I | I | dBm/3.84 MHz | -112 | -101.7 |
|  | XI |  | dBm/3.84 MHz | -112 | -101.7 |
|  | I | XI | dBm/3.84 MHz | -112 | -101.7 |
|  | XI |  | dBm/3.84 MHz | -112 | -101.7 |
| 5 | II | II | dBm/3.84 MHz | -111 | -100.7 |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 |
|  | II | V | dBm/3.84 MHz | -111 | -100.7 |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 |
| 6 | I | I | dBm/3.84 MHz | -113 | -102.7 |
|  | XXXII |  | dBm/3.84 MHz | -113 | -102.7 |
| NOTE 1 For Power class 3 and 3bis this shall be at the maximum output power NOTE 2 For Power class 4 this shall be at the maximum output power |  |  |  |  |  |

-------------------------------------------------- Unchanged sections omitted ------------------------------------------------------

### 7.3.5	Additional requirement for dual band 4C-HSDPA

For all requirements listed in Table 7.2D, corresponding to the specific dual band 4C-HSDPA configuration(s) supported by the UE, (see Table 5.0aC), the BLER measured on each individual cell shall not exceed 0.1.

Note:	The reference sensitivity level <REFSENS> requirement for dual band 4C-HSDPA is not applicable for dual uplink operation. However, there might be a substantial Rx de-sensitization for the UE operating in bands which have less than 80 MHz Tx-Rx frequency separation, transmitting on more than one uplink frequency, at maximum power.

Table 7.2D: Test parameters for reference sensitivity, additional requirement for dual band 4C-HSDPA.

| Dual band 4C-HSDPA configuration | DL Band | UL Band | Unit | HS-PDSCH_Ec <REFSENS> | <REFÎor> | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 I-2-VIII-2 I-1-VIII-2 | I | I | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | VIII |  | dBm/3.84 MHz | -110 | -99.7 | Minimum |
|  | I | VIII | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | VIII |  | dBm/3.84 MHz | -110 | -99.7 | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | dBm/3.84 MHz | -110 | -99.7 | Minimum |
|  | IV |  | dBm/3.84 MHz | -112 | -101.7 | Minimum |
|  | II | IV | dBm/3.84 MHz | -110 | -99.7 | Minimum |
|  | IV |  | dBm/3.84 MHz | -112 | -101.7 | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 | Minimum |
|  | I | V | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 | Minimum |
| II-1-V-2 | II | II | dBm/3.84 MHz | -111 | -100.7 | Minimum |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 | Minimum |
|  | II | V | dBm/3.84 MHz | -111 | -100.7 | Minimum |
|  | V |  | dBm/3.84 MHz | -111 | -100.7 | Minimum |
| I-1-XXXII-2 I-2-XXXII-1 | I | I | dBm/3.84 MHz | -113 | -102.7 | Minimum |
|  | XXXII |  | dBm/3.84 MHz | -113 | -102.7 | Minimum |
| NOTE 1 For Power class 3, 3bis and 4, this shall be at the maximum output power |  |  |  |  |  |  |

-------------------------------------------------- Unchanged sections omitted ------------------------------------------------------

### 7.5.3	Additional requirement for single band/dual band 4C-HSDPA

The UE shall fulfill the additional requirement specified in Table 7.5C for all values of an adjacent channel interferer up to -25 dBm.

However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5D and the requirements are given in Table 7.5E and Table 7.5EA for single band 4C-HSDPA and in 7.5F and 7.5G for dual band 4C-HSDPA, where the HS-PDSCH BLER shall not exceed 0.1.

The ACS requirement for single band/dual-band 4C-HSDPA is not applicable for dual uplink operation.

# Table 7.5C: Adjacent Channel Selectivity

| Rx Parameter | Unit | Number of adjacent downlink carriers in a band |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 |
| ACS | dB | 33 | 33 | 33 | 33 |

# Table 7.5D: Test parameters for Adjacent Channel Selectivity

| Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- |
| Ioac mean power (modulated) | dBm | -52 | -25 |
| Fuw (offset) (NOTE 2) | MHz | +5 or -5 | +5 or -5 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |
| NOTE 1: The Ioac (modulated) signal consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: Negative offset refers to the assigned channel frequency of the lowest carrier frequenc(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequenc(ies) in each band. |  |  |  |

# Table 7.5E: Single band 4C-HSDPA requirements for Adjacent Channel Selectivity, Case 1

| Single band 4C-HSDPA Configuration | DL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- |
| I-3 | I | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |
| II-3, II-4 | II | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2C for single band 4C-HSDPA. |  |  |  |  |

Table 7.5EA: Single band 4C-HSDPA requirements for Adjacent Channel Selectivity, Case 2

| Single band 4C-HSDPA Configuration | DL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- |
| I-3 | I | <REFSENS>+41 dB | <REFÎor>+41 dB | Minimum |
| II-3, II-4 | II | <REFSENS>+41 dB | <REFÎor>+41 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2C for single band 4C-HSDPA. |  |  |  |  |

# Table 7.5F: Dual band 4C-HSDPA requirements for Adjacent Channel Selectivity, Case 1

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |  |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 I-2-VIII-2 I-1-VIII-2 | I | I | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
|  | VIII |  | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
|  | I | VIII | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
|  | VIII |  | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
|  | IV |  | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
|  | II | IV | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
|  | IV |  | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
|  | V |  | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
|  | I | V | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
|  | V |  | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
| II-1-V-2 | II | II | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
|  | V |  | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
|  | II | V | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
|  | V |  | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
| I-1-XXXII-2 I-2-XXXII-1 | I | I | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
|  | XXXII |  | <REFSENS>+14 dB | <REFÎor>+14 dB | Minimum |  |
| NOTE: <REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2D for dual band 4C-HSDPA. |  |  |  |  |  |

# Table 7.5G: Dual band 4C-HSDPA requirements for Adjacent Channel Selectivity, Case 2

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) |  |  | Îor (dBm/3.84MHz) | UL-DL carrier separation |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 I-2-VIII-2 I-1-VIII-2 | I | I | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
|  | VIII |  | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
|  | I | VIII | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
|  | VIII |  | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
|  | IV |  | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
|  | II | IV | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
|  | IV |  | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
|  | V |  | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
|  | I | V | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
|  | V |  | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
| II-1-V-2 | II | II | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
|  | V |  | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
|  | II | V | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
|  | V |  | <REFSENS>+41 dB |  |  | <REFÎor>+41 dB |  | Minimum |  |  |
| I-1-XXXII-2 I-2-XXXII-1 | I | I |  | <REFSENS>+41 dB | <REFÎor>+41 dB |  |  |  | Minimum |  |
|  | XXXII |  |  | <REFSENS>+41 dB | <REFÎor>+41 dB |  |  |  | Minimum |  |
| NOTE: <REFSENS> and <REFÎor> refers to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2D for dual band 4C-HSDPA. |  |  |  |  |  |  |  |  |  |

-------------------------------------------------- Unchanged sections omitted ------------------------------------------------------

### 7.6.1A	Additional requirement for DC-HSDPA and DB-DC-HSDPA (In-band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.6A. In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

Table 7.6A: In-band blocking for DC-HSDPA and DB-DC-HSDPA

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| HS-PDSCH_Ec | dBm/3.84 MHz | <REFSENS>+3 dB |  |
| Îor | dBm/3.84 MHz | <REFÎor> + 3 dB |  |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 3) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band II operation) | MHz | 1922.4 f 1997.6 | 1915 f 2005 |
| Fuw (Band III operation) | MHz | 1797.4 f 1887.6 | 1790 f 1895 |
| Fuw (Band IV operation) | MHz | 2102.4 f 2162.6 | 2095 f 2170 |
| Fuw (Band V operation) | MHz | 861.4 f 901.6 | 854 f 909 |
| Fuw (Band VI operation) | MHz | 867.4 f 892.6 (NOTE 2) | 860 f 900 (NOTE 2) |
| Fuw (Band VII operation) | MHz | 2612.4 f 2697.6 | 2605 f 2705 |
| Fuw (Band VIII operation) | MHz | 917.4 f 967.6 | 910 f 975 |
| Fuw (Band IX operation) | MHz | 1837.4 f 1887.4 | 1829.9 f 1894.9 |
| Fuw (Band X operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band XI operation) | MHz | 1468.4 f 1503.4 | 1460.9 f 1510.9 |
| Fuw (Band XII operation) | MHz | 721.4 f 753.6 | 714 f 761 |
| Fuw (Band XIII operation) | MHz | 738.4 f 763.6 | 731 f 771 |
| Fuw (Band XIV operation) | MHz | 750.4 f 775.6 | 743 f 783 |
| Fuw (Band XIX operation) | MHz | 867.4 f 897.6 | 860 f 905 (NOTE 2) |
| Fuw (Band XX operation) | MHz | 783.4 f 828.6 | 776 f 836 |
| Fuw (Band XXI operation) | MHz | 1488.4 f 1518.4 | 1480.9 f 1525.9 (NOTE 2) |
| Fuw (Band XXII operation) | MHz | 3502.4 f 3597.6 | 3495 f 3605 |
| Fuw (Band XXV operation) | MHz | 1922.4 f 2002.6 | 1915 f 2010 |
| Fuw (Band XXVI operation) | MHz | 851.4 f 901.6 | 844 f 909 |
| Fuw (Band XXXII operation) | MHz | 1444.4 f 1503.6 (NOTE 6) | 1437 f 1511 (NOTE 6) |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) NOTE 5 |  |
| NOTE 1: Iblocking (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For Band VI, Band XIX and Band XXI, the unwanted interfering signal does not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band. NOTE 3: For DC-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequency used and positive offset refers to the assigned channel frequency of the highest carrier frequency used. For DB-DC-HSDPA, offset refers to the assigned channel frequencies of the individual cells. NOTE 4: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2A for DC-HSDPA and Table 7.2B for DB-DC-HSDPA. NOTE 5: The UE transmitted mean power shall be reduced by 0.5dB for a UE operating in band XXII. NOTE 6: Band XXXII is restricted to UTRA operation with dual band configuration. DC-HSDPA test is not applicable for Band XXXII. |  |  |  |

-------------------------------------------------- Unchanged sections omitted ------------------------------------------------------

### 7.6.1D	Additional requirement for dual band 4C-HSDPA (In-band blocking)

#### 7.6.1D.1	Single uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.6H and Table 7.6I. In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

Table 7.6H: Test parameters for in-band blocking, dual band 4C-HSDPA, single uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 2) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band II operation) | MHz | 1922.4 f 1997.6 | 1915 f 2005 |
| Fuw (Band IV operation) | MHz | 2102.4 f 2162.6 | 2095 f 2170 |
| Fuw (Band V operation) | MHz | 861.4 f 901.6 | 854 f 909 |
| Fuw (Band VIII operation) | MHz | 917.4 f 967.6 | 910 f 975 |
| Fuw (Band XXXII operation) |  | MHz | 1444.4 f 1503.6 | 1437 f 1511 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |
| NOTE 1: Iblocking (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequenc(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequenc(ies) in each band. |  |  |  |

Table 7.6I: In-band blocking requirements, dual band 4C-HSDPA, single uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 I-2-VIII-2 I-1-VIII-2 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | VIII | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | II | IV | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | V | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| II-1-V-2 | II | II | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | II | V | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| I-1-XXXII-2 I-2-XXXII-1 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | XXXII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2D for dual band 4C-HSDPA. |  |  |  |  |  |

#### 7.6.1D.2	Dual uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.6J and Table 7.6K. In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band.

Table 7.6J: Test parameters for in-band blocking, dual band 4C-HSDPA, dual uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iblocking mean power (modulated) | dBm | -56 | -44 |
| Fuw offset (NOTE 2) |  | =10 MHz | -15 MHz & 15 MHz |
| Fuw (Band I operation) | MHz | 2102.4 f 2177.6 | 2095 f 2185 |
| Fuw (Band II operation) | MHz | 1922.4 f 1997.6 | 1915 f 2005 |
| Fuw (Band IV operation) | MHz | 2102.4 f 2162.6 | 2095 f 2170 |
| Fuw (Band V operation) | MHz | 861.4 f 901.6 | 854 f 909 |
| Fuw (Band VIII operation) | MHz | 917.4 f 967.6 | 910 f 975 |
| Fuw (Band XXXII operation) |  | MHz | 1444.4 f 1503.6 | 1437 f 1511 |
| NOTE 1: Iblocking (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequenc(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequenc(ies) in each band. |  |  |  |

Table 7.6K: In-band blocking requirements, dual band 4C-HSDPA, dual uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UE transmitted mean power (dBm) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | VIII |  | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-VIII-2 | I | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | VIII |  | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | I | VIII | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | VIII |  | -99.7 | -89.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-1-VIII-2 | I | VIII | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | VIII |  | -99.7 | -89.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-1-IV-2 | II | IV | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-1 | II | II | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-2 | II | II | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | II | IV | -107 | -96.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -109 | -98.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-1-V-2 | I | V | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -103.2 | -92.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-1 | I | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -108 | -97.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-2 | I | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -108 | -97.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | I | V | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -103.2 | -92.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-1-V-2 | II | V | -108 | -97.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -103.1 | -92.8 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-XXXII-1 | I | I | -110 | -99.7 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | XXXII |  | -109.3 | -99.0 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |

-------------------------------------------------- Unchanged sections omitted ------------------------------------------------------

### 7.6.2B	Additional requirement for DB-DC-HSDPA (Out-of-band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.7AB. Out-of-band blocking is defined for an unwanted interfering signal falling at frequencies outside of frequency regions defined as the UE receive bands extended by 15 MHz at their lower and upper ends. For Table 7.7AB in frequency range 1, 2 and 3, up to 24 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 7.7 Spurious response are applicable.

For Table 7.7AB in frequency range 4, up to 8 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 7.7 Spurious response are applicable.

Table 7.7AB: Out of band blocking for DB-DC-HSDPA

| Parameter | Unit | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 |  |
| --- | --- | --- | --- | --- | --- | --- |
| HS-PDSCH_Ec | dBm / 3.84 MHz | <REFSENS>+3 dB | <REFSENS>+3 dB | <REFSENS>+3 dB | <REFSENS> +3 dB |  |
| Îor | dBm / 3.84 MHz | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB |  |
| Iblocking (CW) | dBm | -44 | -30 | -15 | -15 |  |
| Fuw (DB-DC-HSDPA Configuration 1) | MHz | 865< f <910 975< f <1020 2050< f <2095 2185< f <2230 | 840< f 865 1020 f <1045 2025< f 2050 2230 f <2255 | 1< f 840 1045 f <2025 2255< f 12750 | - |  |
| Fuw (DB-DC-HSDPA Configuration 2) | MHz | 1870< f <1915 2005< f <2095 2170< f <2215 | 1845< f 1870 2215 f <2240 | 1< f 1845 2240 f <12750 | 1850 f 1910 |  |
| Fuw (DB-DC-HSDPA Configuration 3) | MHz | 809< f <854 909< f <954 2050< f <2095 2185< f <2230 | 784< f 809 954 f < 979 2025< f 2050 2230 f <2255 | 1< f 784 979 f <2025 2255< f 12750 | 824 f 849 |  |
| Fuw (DB-DC-HSDPA Configuration 4) | MHz | 1415.9 < f < 1460.9 1510.9 < f < 1555.9 2050<f <2095 2185<f <2230 | 1390.9 < f 1415.9 1555.9 f < 1580.9 2025 <f 2050 2230 f <2255 | 1 < f 1390.9 1580.9 f < 2025 2255f<12750 | - |  |
| Fuw (DB-DC-HSDPA Configuration 5) | MHz | 809< f <854 909< f <954 1870<f <1915 2005<f <2050 | 784< f 809 954 f < 979 1845 <f 1870 2050 f <2075 | 1< f 784 979< f 1845 2075f<12750 | 824 f 849 1850 f 1910 |  |
| Fuw (DB-DC-HSDPA Configuration 6) | MHz | 1392< f <1437 1511< f <1556 2050< f <2095 2185< f <2230 | 1367< f 1392 1556 f <1581 2025< f 2050 2230 f <2255 | 1< f 1367 1581 f <2025 2255< f 12750 | - |  |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |  |  |  |
| DB-DC-HSDPA Configuration 1 | For 910f 975 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |
| DB-DC-HSDPA Configuration 2 | For 1915f 2005 MHz and 2095f 2070 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |
| DB-DC-HSDPA Configuration 3 | For 854f909 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |
| DB-DC-HSDPA Configuration 4 | For 1460.9f 1510.9 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |
| DB-DC-HSDPA Configuration 5 | For 854f909 MHz and 1915f 2005 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |
| DB-DC-HSDPA Configuration 6 | For 1437f1511 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1A shall be applied. |  |  |  |  |  |
| NOTE: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2B. |  |  |  |  |  |  |

-------------------------------------------------- Unchanged sections omitted ------------------------------------------------------

### 7.6.2D	Additional requirement for dual band 4C-HSDPA (Out-of-band blocking)

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.7AE and Table 7.7AF. Out-of-band blocking is defined for an unwanted interfering signal falling at frequencies outside of frequency regions defined as the UE receive bands extended by 15 MHz at their lower and upper ends. The requirement is not applicable for dual uplink operation.

For Table 7.7AF in frequency range 1, 2 and 3, up to 24 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 7.7 Spurious response are applicable.

For Table 7.7AF in frequency range 4, up to 8 exceptions per received cell are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1 MHz step size. For these exceptions the requirements of clause 7.7 Spurious response are applicable.

# Table 7.7AE: Test parameters for out of band blocking, dual band 4C-HSDPA

| Parameter |  | Unit | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Iblocking (CW) |  | dBm | -44 | -30 | -15 | -15 |  |
| Fuw (Dual band 4C-HSDPA Configuration I-2-VIII-1, I-3-VIII-1, I-2-VIII-2, I-1-VIII-2) |  | MHz | 865< f <910 975< f <1020 2050< f <2095 2185< f <2230 | 840< f 865 1020 f <1045 2025< f 2050 2230 f <2255 | 1< f 840 1045 f <2025 2255< f 12750 | - |  |
| Fuw (Dual band 4C-HSDPA Configuration II-1-IV-2, II-2-IV-1, II-2-IV-2) |  | MHz | 1870< f <1915 2005< f <2095 2170< f <2215 | 1845< f 1870 2215 f <2240 | 1< f 1845 2240 f <12750 | 1850 f 1910 |  |
| Fuw (Dual band 4C-HSDPA Configuration I-1-V-2, I-2-V-1, I-2-V-2) |  | MHz | 809< f <854 909< f <954 2050< f <2095 2185< f <2230 | 784< f 809 954 f < 979 2025< f 2050 2230 f <2255 | 1< f 784 979 f <2025 2255< f 12750 | 824 f 849 |  |
| Fuw (Dual band 4C-HSDPA Configuration II-1-V-2) |  | MHz | 809< f <854 909< f <954 1870< f <1915 2005< f <2050 | 784< f 809 954 f < 979 1845< f 1870 2050 f <2075 | 1< f 784 979 f <1845 2075< f 12750 | 824 f 849, 1850 f 1910 |  |
| Fuw (Dual band 4C-HSDPA Configuration I-1-XXXII-2, I-2-XXXII-1) |  |  | MHz | 1392< f <1437 1511< f <1556 2050< f <2095 2185< f <2230 | 1367< f 1392 1556 f <1581 2025< f 2050 2230 f <2255 | 1< f 1367 1581 f <2025 2255< f 12750 | - |  |
| UE transmitted mean power |  | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |  |  |  |
| Dual band 4C-HSDPA Configuration I-2-VIII-1, I-3-VIII-1, I-2-VIII-2, I-1-VIII-2 |  | For 910f 975 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1D.1 shall be applied. |  |  |  |  |  |
| Dual band 4C-HSDPA Configuration II-1-IV-2, II-2-IV-1, II-2-IV-2 |  | For 1915f 2005 MHz and 2095f 2070 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1D.1 shall be applied. |  |  |  |  |  |
| Dual band 4C-HSDPA Configuration I-1-V-2, I-2-V-1, I-2-V-2 |  | For 854f909 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1D.1 shall be applied. |  |  |  |  |  |
| Dual band 4C-HSDPA Configuration II-1-V-2 |  | For 854f909 MHz and 1915 f 2005 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1D.1 shall be applied. |  |  |  |  |  |
| Dual band 4C-HSDPA Configuration I-1-XXXII-2, I-2-XXXII-1 | For 1437f 1511 MHz and 2095f 2185 MHz, the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.2 and subclause 7.6.1D.1 shall be applied. |  |  |  |  |

# Table 7.7AF: Out of band blocking requirements, dual band 4C-HSDPA

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | Parameter | Frequency range 1 | Frequency range 2 | Frequency range 3 | Frequency range 4 | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 I-2-VIII-2 I-1-VIII-2 | I | I | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | VIII |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
|  | I | VIII | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | VIII |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | IV |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
|  | II | IV | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | IV |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | V |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
|  | I | V | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | V |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
| II-1-V-2 | II | II | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | V |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
|  | II | V | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | V |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
| I-1-XXXII-2 I-2-XXXII-1 | I | I | HS-PDSCH_Ec (dBm/3.84MHz) | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | <REFSENS> +3 dB | Minimum |
|  | XXXII |  | Îor (dBm/3.84MHz) | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | <REFÎor> + 3 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2D. |  |  |  |  |  |  |  |  |

-------------------------------------------------- Unchanged sections omitted ------------------------------------------------------

### 7.8.1D	Additional requirement for dual band 4C-HSDPA

#### 7.8.1D.1	Single uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.9AH and Table 7.9AI.

# Table 7.9AH: Test parameters for receive intermodulation characteristics, dual band 4C-HSDPA, single uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -46 |  |
| Iouw2 mean power (modulated) | dBm | -46 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 10 | -10 |
| Fuw2 (offset) (NOTE 2) | MHz | 20 | -20 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |
| NOTE 1: Iouw2 (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequenc(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequenc(ies) in each band. |  |  |  |

Table 7.9AI: Intermodulation requirements, dual band 4C-HSDPA, single uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 I-2-VIII-2 I-1-VIII-2 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | VIII | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | VIII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | II | II | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | II | IV | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | IV |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| I-1-V-2 I-2-V-1 I-2-V-2 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | I | V | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| II-1-V-2 | II | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | II | V | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | V |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| I-1-XXXII-2 I-2-XXXII-1 | I | I | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
|  | XXXII |  | <REFSENS>+3 dB | <REFÎor>+3 dB | Minimum |
| NOTE: <REFSENS> and <REFÎor> refer to the HS-PDSCH_Ec<REFSENS> and the HS-PDSCH<REFÎor> as specified in Table 7.2D for dual band 4C-HSDPA. |  |  |  |  |  |

#### 7.8.1D.2	Dual uplink operation

The BLER measured on each individual cell shall not exceed 0.1 for the parameters specified in Table 7.9AJ and Table 7.9AK.

Table 7.9AJ: Receive intermodulation characteristics for dual band 4C-HSDPA, dual uplink operation

| Parameter | Unit | Level |  |
| --- | --- | --- | --- |
| Iouw1 (CW) | dBm | -46 |  |
| Iouw2 mean power (modulated) | dBm | -46 |  |
| Fuw1 (offset) (NOTE 2) | MHz | 10 | -10 |
| Fuw2 (offset) (NOTE 2) | MHz | 20 | -20 |
| UE transmitted mean power | dBm | 20 (for Power class 3 and 3bis) 18 (for Power class 4) |  |
| NOTE 1: Iouw2 (modulated) consists of the common channels needed for tests as specified in Table C.7 and 16 dedicated data channels as specified in Table C.6. NOTE 2: For dual band 4C-HSDPA, negative offset refers to the assigned channel frequency of the lowest carrier frequenc(ies) in each band, and positive offset refers to the assigned channel frequency of the highest carrier frequenc(ies) in each band. |  |  |  |

Table 7.9AK: Intermodulation requirements, dual band 4C-HSDPA, dual uplink operation

| Dual band 4C-HSDPA Configuration | DL Band | UL Band | HS-PDSCH_Ec (dBm/3.84MHz) | Îor (dBm/3.84MHz) | UE transmitted mean power (dBm) | UL-DL carrier separation |
| --- | --- | --- | --- | --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 | I | I | -104.2 | -93.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | VIII |  | -103.6 | -93.3 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-VIII-2 | I | I | -104.2 | -93.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | VIII |  | -103.6 | -93.3 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | I | VIII | -104.8 | -94.5 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | VIII |  | -98.7 | -88.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-1-VIII-2 | I | VIII | -104.8 | -94.5 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | VIII |  | -98.7 | -88.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-1-IV-2 | II | IV | -103.1 | -92.8 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -103.4 | -93.1 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-1 | II | II | -103.1 | -92.8 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -103.4 | -93.1 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-2-IV-2 | II | II | -103.1 | -92.8 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -103.4 | -93.1 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | II | IV | -103.1 | -92.8 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | IV |  | -103.4 | -93.1 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-1-V-2 | I | V | -104.2 | -93.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -101.1 | -90.8 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-1 | I | I | -104.2 | -93.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -103.9 | -93.6 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-V-2 | I | I | -104.2 | -93.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -103.9 | -93.6 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | I | V | -104.2 | -93.9 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -101.1 | -90.8 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| II-1-V-2 | II | V | -104.4 | -94.1 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | V |  | -101.1 | -90.8 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
| I-2-XXXII-1 | I | I | -104.7 | -94.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |
|  | XXXII |  | -103.7 | -93.4 | 20 (for Power class 3 and 3bis) 18 (for Power class 4) | Minimum |

-------------------------------------------------- Unchanged sections omitted ------------------------------------------------------

## 7.9	Spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector. The spurious emission is verified per antenna connector with the other(s) terminated.

### 7.9.1	Minimum requirement

The power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table 7.10 and Table 7.11

Table 7.10: General receiver spurious emission requirements

| Frequency Band | Measurement Bandwidth | Maximum level | Note |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | -57 dBm |  |
| 1GHz f 12.75 GHz | 1 MHz | -47 dBm |  |
| 12.75GHz f 5th harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 dBm | Note 1 |
| NOTE 1: Applies only for Band XXII. |  |  |  |

Table 7.11: Additional receiver spurious emission requirements

| Band | Frequency Band |  | MeasurementBandwidth | Maximum level | Note |
| --- | --- | --- | --- | --- | --- |
| I | 462.5 MHz f 467.5 MHz |  | 1 MHz | -50 dBm |  |
|  | 703 MHz f 803 MHz |  | 1 MHz | -50 dBm |  |
|  | 791 MHz  f  821 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 852 MHz f 859 MHz |  | 1 MHz | -50 dBm |  |
|  | 859 MHz  f  894 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 921 MHz  f < 925 MHz |  | 100 kHz | -60 dBm * |  |
|  | 925 MHz f 935 MHz |  | 100 kHz 3.84MHz | -67 dBm * -60 dBm |  |
|  | 935 MHz < f  960 MHz |  | 100 kHz | -79 dBm * |  |
|  | 1805 MHz  f  1880 MHz |  | 100 kHz | -71 dBm * |  |
|  | 1452 MHz f  1510.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1839.9 MHz f  1879.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1920 MHz f 1980 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 2496 MHz f 2570 MHz |  | 1 MHz | -50 dBm |  |
|  | 2570 MHz f 2690 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 3510 MHz  f  3590 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 3400 MHz  f  3800 MHz |  | 1 MHz | -50 dBm |  |
| II | 717 MHz f 728 MHz |  | 1 MHz | -50 dBm |  |
|  | 729 MHz f 746 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 746 MHz f 756 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 758 MHz f 768 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 768 MHz f 803 MHz |  | 1 MHz | -50 dBm |  |
|  | 852 MHz f 859 MHz |  | 1 MHz | -50 dBm |  |
|  | 859 MHz  f 894 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1850 MHz f 1915 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 1930 MHz f 1990 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 1990 MHz f 1995 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2350 MHz f 2360 MHz |  | 1 MHz | -50 dBm |  |
|  | 2496 MHz f 2690 MHz |  | 1 MHz | -50 dBm |  |
| III | 462.5 MHz f 467.5 MHz |  | 1 MHz | -50 dBm |  |
|  | 703 MHz f 803 MHz |  | 1 MHz | -50 dBm |  |
|  | 791 MHz  f  821 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 852 MHz f 869 MHz |  | 1 MHz | -50 dBm |  |
|  | 860 MHz f  890 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 921 MHz  f < 925 MHz |  | 100 kHz | -60 dBm* |  |
|  | 925 MHz f 935 MHz |  | 100 kHz 3.84 MHz | -67 dBm* -60 dBm |  |
|  | 935 MHz < f  960 MHz |  | 100 kHz | -79 dBm* |  |
|  | 1452 MHz f  1510.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1710 MHz f 1785 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 1805 MHz f 1880 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 1884.5 MHz f 1915.7 MHz |  | 3.84 MHz | -41 dBm |  |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2496 MHz f 2570 MHz |  | 1 MHz | -50 dBm |  |
|  | 2570 MHz f 2690 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 3510 MHz  f  3590 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 3400 MHz  f  3800 MHz |  | 1 MHz | -50 dBm |  |
| IV | 717 MHz f 728 MHz |  | 1 MHz | -50 dBm |  |
|  | 729 MHz f 746 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 746 MHz f 756 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 758 MHz f 768 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 768 MHz f 803 MHz |  | 1 MHz | -50 dBm |  |
|  | 852 MHz f 859 MHz |  | 1 MHz | -50 dBm |  |
|  | 859 MHz  f < 894 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1710 MHz  f < 1755 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 1930 MHz f 1995 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 2350 MHz f 2360 MHz |  | 1 MHz | -50 dBm |  |
|  | 2496 MHz f 2690 MHz |  | 1 MHz | -50 dBm |  |
| V | 462.5 MHz f 467.5 MHz |  | 1 MHz | -50 dBm |  |
|  | 717 MHz f 728 MHz |  | 1 MHz | -50 dBm |  |
|  | 703 MHz f 803 MHz |  | 1 MHz | -50 dBm |  |
|  | 729 MHz f 746 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 746 MHz f 756 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 758 MHz f 768 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 824 MHz  f 849 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 859 MHz f 869 MHz |  | 1 MHz | -27 dBm |  |
|  | 869 MHz  f < 894 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 1930 MHz f 1995 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2350 MHz f 2360 MHz |  | 1 MHz | -50 dBm |  |
|  | 2496 MHz f 2690 MHz |  | 1 MHz | -50 dBm |  |
| VI | 758 MHz f 803 MHz |  | 1 MHz | -50 dBm |  |
|  | 815 MHz f 830 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 830 MHz f 840 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 840 MHz f 845 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 860 MHz f  875 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 875 MHz f  885 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 885 MHz f  890 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 945 MHz  f 960 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1475.9 MHz f  1510.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1839.9 MHz f  1879.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2545 MHz f 2575 MHz |  | 1 MHz | -50 dBm |  |
|  | 2595 MHz  f  2645 MHz |  | 1 MHz | 50 dBm |  |
| VII | 462.5 MHz f 467.5 MHz |  | 1 MHz | -50 dBm |  |
|  | 717 MHz f 728 MHz |  | 1 MHz | -50 dBm |  |
|  | 758 MHz f 791 MHz |  | 1 MHz | -50 dBm |  |
|  | 791 MHz  f < 821 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 852 MHz f 869 MHz |  | 1 MHz | -50 dBm |  |
|  | 921 MHz  f < 925 MHz |  | 100 kHz | -60 dBm * |  |
|  | 925 MHz f 935 MHz |  | 100 kHz -3.84 MHz | -67 dBm * -60 dBm |  |
|  | 935 MHz < f  960 MHz |  | 100 kHz | -79 dBm * |  |
|  | 1452  f 1496 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1805 MHz  f  1880 MHz |  | 100 kHz | -71 dBm * |  |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2350 MHz f 2360 MHz |  | 1 MHz | -50 dBm |  |
|  | 2500 MHz f 2570 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 2570 MHz f 2620 MHz |  | 1 MHz | -60 dBm |  |
|  | 2620 MHz f 2690 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 3510 MHz  f  3590 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 3400 MHz  f  3800 MHz |  | 1 MHz | -50 dBm |  |
| VIII | 462.5 MHz f 467.5 MHz |  | 1 MHz | -50 dBm |  |
|  | 703 MHz f 803 MHz |  | 1 MHz | -50 dBm |  |
|  | 791 MHz  f < 821 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 860 MHz  f < 890 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 880 MHz f 915 MHz |  | 3.84 MHz | -60 dBm | UE in URA_PCH, Cell_PCH and idle state |
|  | 921 MHz  f < 925 MHz |  | 100 kHz | -60 dBm * |  |
|  | 925 MHz f 935MHz |  | 100 kHz 3.84 MHz | -67 dBm * -60 dBm | UE receive band |
|  | 935 MHz < f  960 MHz |  | 100 kHz | -79 dBm * | UE receive band |
|  | 1452 MHz f  1510.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1805 MHz < f  1880 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2110 MHz  f  2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2496 MHz f 2570MHz |  | 1 MHz | -50 dBm |  |
|  | 2570 MHz  f  2690 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 3510 MHz  f  3590 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 3400 MHz  f  3800 MHz |  | 1 MHz | -50 dBm |  |
| IX | 758 MHz f 803 MHz |  | 1 MHz | -50 dBm |  |
|  | 860 MHz f  890 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 945 MHz  f 960 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1475.9 MHz f  1510.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1749.9 MHz f 1784.9 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 1839.9 MHz f  1879.9 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2545 MHz f 2575 MHz |  | 1 MHz | -50 dBm |  |
|  | 2595 MHz  f  2645 MHz |  | 1 MHz | -50 dBm |  |
| X | 717 MHz f 728 MHz |  | 1 MHz | -50 dBm |  |
|  | 729 MHz f 746 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 746 MHz f 756 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 758 MHz f 768 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 768 MHz f 803 MHz |  | 1 MHz | -50 dBm |  |
|  | 852 MHz f 859 MHz |  | 1 MHz | -50 dBm |  |
|  | 859 MHz  f < 894 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1710 MHz  f < 1770 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 1930 MHz f 1995 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
| XI | 758 MHz f 803 MHz |  | 1 MHz | -50 dBm |  |
|  | 860 MHz f  890 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 945 MHz  f 960 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1427.9 MHz f 1447.9 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 1447.9 MHz f 1462.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1475.9 MHz f  1495.9 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 1495.9 MHz f  1510.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1839.9 MHz f  1879.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2545 MHz f 2575 MHz |  | 1 MHz | -50 dBm |  |
|  | 2595 MHz  f  2645 MHz |  | 1 MHz | -50 dBm |  |
| XII | 699 MHz f 716 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 728 MHz f 746 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 746 MHz f 756 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 758 MHz f 768 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 852 MHz f 859 MHz |  | 1 MHz | -50 dBm |  |
|  | 859 MHz  f < 894 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1930 MHz f 1995 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2350 MHz f 2360 MHz |  | 1 MHz | -50 dBm |  |
|  | 2496 MHz f 2690 MHz |  | 1 MHz | -50 dBm |  |
| XIII | 717 MHz f 728 MHz |  | 1 MHz | -50 dBm |  |
|  | 729 MHz f 746 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 746 MHz f 756 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 758 MHz f 768 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 776 MHz f 788 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 852 MHz f 859 MHz |  | 1 MHz | -50 dBm |  |
|  | 859 MHz  f < 894 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1930 MHz f 1995 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2350 MHz f 2360 MHz |  | 1 MHz | -50 dBm |  |
|  | 2496 MHz f 2690 MHz |  | 1 MHz | -50 dBm |  |
| XIV | 717 MHz f 728 MHz |  | 1 MHz | -50 dBm |  |
|  | 729 MHz f 746 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 746 MHz f 756 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 758 MHz f 768 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 788 MHz f 798 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 852 MHz f 859 MHz |  | 1 MHz | -50 dBm |  |
|  | 859 MHz  f < 894 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1930 MHz f 1995 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2350 MHz f 2360 MHz |  | 1 MHz | -50 dBm |  |
|  | 2496 MHz f 2690 MHz |  | 1 MHz | -50 dBm |  |
| XIX | 758 MHz f 803 MHz |  | 1 MHz | -50 dBm |  |
|  | 815 MHz f 830 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 830 MHz f 845 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 860 MHz f  875 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 875 MHz f  890 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 945 MHz  f 960 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1475.9 MHz f  1510.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1839.9 MHz f  1879.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2545 MHz f 2575 MHz |  | 1 MHz | -50 dBm |  |
|  | 2595 MHz  f  2645 MHz |  | 1 MHz | -50 dBm |  |
| XX | 791 MHz  f < 821 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 832 MHz f 862 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 921 MHz  f < 925 MHz |  | 100 kHz | -60 dBm* |  |
|  | 925 MHz f 935 MHz |  | 100 kHz 3.84 MHz | -67 dBm* -60 dBm |  |
|  | 935 MHz < f  960 MHz |  | 100 kHz | -79 dBm* |  |
|  | 1452  f 1496 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1805 MHz f 1880 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2570 MHz f 2620 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2620 MHz f 2690 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 3400 MHz  f  3800 MHz |  | 3.84 MHz | -60 dBm |  |
| XXI | 758 MHz f 803 MHz |  | 1 MHz | -50 dBm |  |
|  | 860 MHz f  890 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 945 MHz  f 960 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1427.9 MHz f 1447.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1447.9 MHz f 1462.9 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 1475.9 MHz f  1495.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1495.9 MHz f  1510.9 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 1839.9 MHz f  1879.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2545 MHz f 2575 MHz |  | 1 MHz | -50 dBm |  |
|  | 2595 MHz  f  2645 MHz |  | 1 MHz | -50 dBm |  |
| XXII | 758 MHz f 803 MHz |  | 1 MHz | -50 dBm |  |
|  | 791 MHz  f < 821 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 852 MHz f 869 MHz |  | 1 MHz | -50 dBm |  |
|  | 921 MHz  f < 925 MHz |  | 100 kHz | -60 dBm* |  |
|  | 925 MHz f 935 MHz |  | 100 kHz 3.84 MHz | -67 dBm* -60 dBm |  |
|  | 935 MHz < f  960 MHz |  | 100 kHz | -79 dBm* |  |
|  | 1452  f 1496 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1805 MHz f 1880 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1880 MHz  f  1920 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2010 MHz  f  2025 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2110 MHz  f  2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2300 MHz  f  2400 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2570 MHz  f  2620 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2620 MHz f 2690 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 3410 MHz  f  3490 MHz |  | 3.84 MHz | -60 dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 3510 MHz  f  3590 MHz |  | 3.84 MHz | -60 dBm | UE receive band |
|  | 3600 MHz  f  3800 MHz |  | 3.84 MHz | -50 dBm |  |
| XXV | 717 MHz f 728 MHz |  | 1 MHz | -50 dBm |  |
|  | 729 MHz f 746 MHz |  | 3.84 MHz | [-60] dBm |  |
|  | 746 MHz f 756 MHz |  | 3.84 MHz | [-60] dBm |  |
|  | 758 MHz f 768 MHz |  | 3.84 MHz | [-60] dBm |  |
|  | 852 MHz f 859 MHz |  | 1 MHz | -50 dBm |  |
|  | 859 MHz  f 894 MHz |  | 3.84 MHz | [-60] dBm |  |
|  | 1850 MHz f 1915 MHz |  | 3.84 MHz | [-60] dBm | UE transmit band in URA_PCH, Cell_PCH and idle state |
|  | 1930 MHz f 1995 MHz |  | 3.84 MHz | [-60] dBm | UE receive band |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | [-60] dBm |  |
|  | 2180 MHz f 2200 MHz |  | 1 MHz | [-50] dBm |  |
|  | 2350 MHz f 2360 MHz |  | 1 MHz | -50 dBm |  |
|  | 2496 MHz f 2690 MHz |  | 1 MHz | [-50] dBm |  |
|  | 3400 MHz f 3800 MHz |  | 1 MHz | [-50] dBm |  |
| XXVI | 462.5 MHz f 467.5 MHz |  | 1 MHz | -50 dBm |  |
|  | 717 MHz f 728 MHz |  | 1 MHz | -50 dBm |  |
|  | 758 MHz f 799 MHz |  | 1 MHz | -50 dBm |  |
|  | 799 MHz f 803 MHz |  | 1 MHz | -40 dBm |  |
|  | 729 MHz f 756 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 758 MHz f 768 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 859 MHz f 894 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 945 MHz  f  960 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1475.9 MHz f 1510.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1525 MHz f 1559 MHz |  | 1 MHz | -50 dBm |  |
|  | 1839.9 MHz f 1879.9 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 1884.5 MHz f 1919.6 MHz |  | 300 kHz | -41 dBm |  |
|  | 1930 MHz f 1995 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2010 MHz f 2025 MHz |  | 1 MHz | -50 dBm |  |
|  | 2110 MHz f 2170 MHz |  | 3.84 MHz | -60 dBm |  |
|  | 2180 MHz f 2200 MHz |  | 1 MHz | -50 dBm |  |
|  | 2300 MHz f 2400 MHz |  | 1 MHz | -50 dBm |  |
|  | 2496 MHz f 2690 MHz |  | 1 MHz | -50 dBm ** |  |
|  | 3400 MHz f 3800 MHz |  | 1 MHz | -50 dBm |  |
| XXXII |  | 791 MHz  f < 821 MHz | 3.84 MHz | -60 dBm |  |
|  |  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm* |  |
|  |  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm* -60 dBm |  |
|  |  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm* |  |
|  |  | 1452 MHz < f  1496 MHz | 3.84 MHz | -60 dBm | UE receive band |
|  |  | 1805 MHz f 1880 MHz | 3.84 MHz | -60 dBm |  |
|  |  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |  |
|  |  | 2570 MHz f 2620 MHz | 3.84 MHz | -60 dBm |  |
|  |  | 2620 MHz f 2690 MHz | 3.84 MHz | -60 dBm |  |
|  |  | 3400 MHz  f  3510 MHz | 1 MHz | -50 dBm |  |
|  |  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |  |
|  |  | 3590 MHz  f  3800 MHz | 1 MHz | -50 dBm |  |
| Note * The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 7.10 are permitted for each UARFCN used in the measurement |  |  |  |  |  |

## 7.10	Reference input power adjustment for a dual band device

For the UE which supports DB-DC-HSDPA configuration in Table 5.0aA, the reference input powers (HS-PDSCH_Ec and Îor) of core requirements specified in subclause 7.6.1B, 7.6.1C.2, 7.6.3B, 7.8.1B, 7.8.1C.2, and 7.8.2B are allowed to be increased by the amount given in Table 7.12 for the applicable bands.

# Table 7.12: Allowed increase of HS-PDSCH Ec and Îor for UE which supports DB-DC-HSDPA.

| DB-DC-HSDPA Configuration | Allowed increase of HS-PDSCH Ec and Îor (dB) | Applicable bands |
| --- | --- | --- |
| 1 | 0.5 | I, VIII |
| 2 | 1 | II, IV |
| 3 | 0.5 | I, V |
| 4 | 1 | I, XI |
| 5 | 0.5 | II, V |
| 6 | 0.6 | I |

For the UE which supports dual band 4C-HSDPA configuration in Table 5.0aC, the reference input powers (HS-PDSCH_Ec and Îor) of core requirements specified in subclause 7.6.1B, 7.6.1C.2, 7.6.3B, 7.8.1B, 7.8.1C.2, and 7.8.2B are allowed to be increased by the amount given in Table 7.13 for the applicable bands.

# Table 7.13: Allowed increase of HS-PDSCH Ec and Îor for UE which supports dual band 4C-HSDPA.

| Dual Band 4C-HSDPA Configuration | Allowed increase of HS-PDSCH Ec and Îor (dB) | Applicable bands |
| --- | --- | --- |
| I-2-VIII-1 I-3-VIII-1 I-1-VIII-2 I-2-VIII-2 | 0.5 | I, VIII |
| II-1-IV-2 II-2-IV-1 II-2-IV-2 | 1 | II, IV |
| I-1-V-2 I-2-V-1 I-2-V-2 | 0.5 | I, V |
| II-1-V-2 | 0.5 | II, V |
| I-1-XXXII-2 I-2-XXXII-1 | 0.6 | I |

------------------------------------------------------------- End of change ------------------------------------------------------------