3GPP TSG-RAN WG4 (Radio) Meeting #70bis	R4-141886

San Jose Del Cabo, Mexico, 31 March – 4 April 2014

Agenda Item:	7.42.2

Source: 	Alcatel-Lucent, KDDI

Title: 	TP for TR 36.853: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (1 + 18 + 28)

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 1, Band 18 and Band 28 was approved in RAN#63 [1]. One objective of the WI is to specify the band-combination specific Radio Frequency (RF) requirements for inter-band Carrier Aggregation (CA) of Band 1, Band 18 and Band 28 with 1 Uplink (UL) active in one of these bands.

The impacts of harmonics and IMD products caused by LTE-A BS supporting 3DL CA of Band (1 + 18 + 28) to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to record the findings in the Technical Report for 3 Band Carrier Aggregation with Single Uplink [3].

# 2.	Text Proposal

<New section>

##### 6.x.1	Co-existence studies for 1 UL/3 DL

The 3rd order IMD products caused in the BS by transmitting of Band 1, Band 18 and Band 28 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.x.1-1 below:

Table 6.x.1-1: Band 1, Band 18 and Band 28 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f2-low | f2-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 803 | 860 | 875 | 2110 | 2170 |
| 3rd order IMD products | (f3-low – f1-high – f2-high) |  |  | (f3-high – f1-low – f2-low) |  |  |
| IMD frequency limits (MHz) | 432 |  |  | 552 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 1993 |  |  | 2113 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 2167 |  |  | 2287 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 3728 |  |  | 3848 |  |  |

It can be seen from Table 6.x.1-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (1 + 18 + 28) may fall into the BS receive band of Bands 23, 31, 34 and 43. Bands 23, 31 and 34 are not intended for use in the same geographical area as Bands 1, 18 and 28. Therefore, the focus here is on the 3rd order IMD products falling into Band 43.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 43 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (1 + 18 + 28) BS transmitter does not share the same antenna with Band 43 BS receiver.

Therefore, it is recommended that Band (1 + 18 + 28) BS transmitter should not share the same antenna with Band 43 BS receiver to prevent BS receiver desensitization, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 43 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of section>

# References

[1]	RP-140451, “New WI: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 1, Band 18 and Band 28”, KDDI.

[2]	R4-141884, “Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (1 + 18 + 28)”, Alcatel-Lucent, KDDI.

[3]	R4-137075, “Draft New Technical Report 'LTE Advanced 3 Band Carrier Aggregation (3DL/1UL)'”, AT&T.