3GPP TSG-RAN WG4 (Radio) Meeting #70bis	R4-141939

San Jose Del Cabo, Mexico, 31 March – 4 April 2014

Agenda Item:	7.42.2

Source: 	Alcatel-Lucent, AT&T

Title: 	TP for TR 36.853: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Contiguous or Non-contiguous Carrier Aggregation of Band 12

Document for:	Approval

# 1.	Introduction

The revised WI proposal to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 12 and Band 12 was approved in RAN#62 [1]. One objective of the WI is to specify the band-combination specific Radio Frequency (RF) requirements for Band 12 intra-band CA (2DL/1UL) scenario.

The impacts of harmonics and IMD products caused by LTE-A BS supporting intra-band contiguous or non-contiguous CA of this band to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to record the findings in the Technical Report for 3 Band Carrier Aggregation with Single Uplink [3].

# 2.	Text Proposal

<New section>

##### 6.x.1	Co-existence studies for 1 UL/2 DL

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting contiguous or non-contiguous Band 12 DL carriers can be calculated as shown in Table 6.x.1-1 below:

Table 6.x.1-1: Band 12 DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 729 | 746 |
| 2nd harmonics frequency limits (MHz) | 1458 | 1492 |
| 3rd harmonics frequency limits (MHz) | 2187 | 2238 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -17 | 17 |
| 3rd order IMD products | (2* f-low – f-high) | (2*f-high – f-low) |
| IMD frequency limits (MHz) | 712 | 763 |

It can be seen from Table 6.x.1-1 that the 2nd harmonics of BS transmitting in Band 12 may fall into the BS receive band of Band 21, while the 3rd IMD products of BS supporting CA of Band 12 may fall into the BS receive band of Bands 12, 17, 28 and 44. Note that the calculation in Table 6.x.1-1 assumes the BS is transmitting with the whole 17 MHz DL frequency of Band 12. If the BS is only transmitting up to 15 MHz (10 + 5 or 5 + 10 MHz) contiguous carriers in Band 12, then the 3rd IMD products may only fall into the BS receive band of the Bands 12 and 17 under the transmit configurations shown in Table 6.x.1-2  below:

Table 6.x.1-2: Band 12 BS contiguous carriers transmit configurations with 3rd IMD within Bands 12 and 17 BS receive band

| 1st CC channel bandwidth (MHz) | 2nd CC channel bandwidth (MHz) | Lower edge of 15 MHz DL frequency block (MHz) | Lower edge of IMD frequency limits (MHz) |
| --- | --- | --- | --- |
| 5 | 10 | 729 – 730.9 | 714 – 715.9 |
| 10 | 5 | 729 – 730.9 | 714 – 715.9 |

The case if the BS is transmitting up to 15 MHz (10 + 5 or 5 + 10 MHz) non-contiguous carriers in Band 12 is studied as follows. Assume that the 2 non-contiguous CC transmitted by the Band 12 BS are a and b MHz channel bandwidth and c and d MHz from the edges of Band 12 DL frequency band as shown in Figure 6.x.1-1 below:

f2-high

f2-low

f1-high

f1-low

1st CC

2nd CC

729

746

c MHz

d MHz

a MHz

b MHz

MHz

Figure 6.x.1-1: Non-contiguous CC transmitted by the Band 12 BS

And the corresponding BS receive blocks are as shown in Figure 6.x.1-2 below:

f1-low

f1-high

f2-low

f2-high

1st CC

2nd CC

699

716

c MHz

d MHz

a MHz

b MHz

MHz

Figure 6.x.1-2: Non-contiguous CC received by the Band 12 BS

Then the 3rd order IMD products caused in the BS by transmitting the 2 CC can be calculated as shown in Table 6.x.1-3  below:

Table 6.x.1-3: Band 12 BS DL 3rd order IMD products

| 3rd order IMD products | IMD frequency limits (MHz) |
| --- | --- |
| (2*f1-low – f2-high) | 712 + 2*c + d |
| (2*f1-high – f2-low) | 712 + 2*c + d + 2*a + b |
| (2*f2-low – f1-high) | 763 – 2*d – c – 2*b – a |
| (2*f2-high – f1-low) | 763 – 2*d – c |
| (f1-low – f2-high + f2-low) | 729 + c – b |
| (f1-high + f2-high – f2-low) | 729 + c + b + a |
| (f2-low – f1-high + f1-low) | 746 – d – a – b |
| (f2-high + f1-high – f1-low) | 746 – d + a |

Comparing the IMD frequency limits in Table 6.x.1-3 with the BS receive blocks in Figure 6.x.1-2, it can be deduced that the 3rd order IMD products caused in the BS by transmitting the 2 CC will only fall into the BS own receive blocks of Band 12 under one of the transmit configurations shown in Table 6.x.1-4 below:

Table 6.x.1-4: Band 12 BS non-contiguous carriers transmit configurations with 3rd IMD within Band 12 BS own receive blocks

| 1st CC channel bandwidth (a MHz) | 2nd CC channel bandwidth (b MHz) | 1st and 2nd CC position (c and d MHz) |
| --- | --- | --- |
| 5 | 5 | c + d < 2 |
| 5 | 10 | c + d < 2 |
| 10 | 5 | c + d < 2 |

Therefore, it is recommended that Band 12 BS transmitters supporting contiguous or non-contiguous CA should not share the same antenna with other Band 12 or 17 BS receiver, or the own Band 12 BS receiver, if the aforementioned BS transmit configurations are used, in order to not to cause Band 12 or 17 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of section>

# References

[1]	RP-131654, “New WI proposal: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 12 and Band 12”, AT&T.

[2]	R4-141835, “Harmonics and Intermodulation Products caused by LTE Advanced Contiguous or Non-contiguous Carrier Aggregation of Band 12”, Alcatel-Lucent, AT&T.

[3]	R4-137075, “Draft New Technical Report 'LTE Advanced 3 Band Carrier Aggregation (3DL/1UL)'”, AT&T.