3GPP TSG-RAN WG4 RAN4 #70Bis                                      R4-142055

San Jose del Cabo, Mexico, 31 Mar - 4 Apr, 2014

Agenda item:	7.14.2.2

Source: 	Samsung

Title:			  On blind detection feasibility of parameter combination for DMRS-based TM

Document for:	Discussion

# 1	Introduction

In 3GPP RAN #63 meeting, it was agreed to set up a work item targeted on specifying advanced non-linear receiver in Rel-12, based on the extensive study during study item. Consequently, RAN4 are expected to identify and agree on the parameter combinations that could be blindly detected, including:

- Presence or absence of interference

- Transmission modes (TM)

- For DMRS-based TMs: DMRS ports, modulation order, Virtual cell ID, nSCID, Cell ID, CRS ports, and MBSFN pattern

- For CRS-based TMs: PMI, RI, modulation order, Cell ID, CRS ports, and MBSFN pattern, ρA

- CFI (if not coordinated and required by receiver implementation)

In this paper, we provide our views and analysis on dynamic parameters for DMRS based TM, including

- Presence or absence of interference

- For DMRS-based TMs: DMRS ports, modulation order, nSCID

The analysis on dynamic parameters for CRS-based TM are provided in companion papers[1].

# Simulation Assumptions

In this contribution, we focus on the blind detection performance of R-ML receiver. Especially, we evaluate the performance loss caused by

- Blind detection of interference presence or absence

For blind detection of interference presence or absence, the estimation can be done based on the measured covariance matrix of received signal.

Note that for the interference ON/OFF detection, it is not only to evaluate the performance loss caused by ON->OFF misdetection for R-ML receiver when interference presence, but also necessary to guarantee enough good performance of false alarm, i.e. OFF->ON misdetection, when interference is absence.

- Blind detection of modulation

For blind detection of Modulation, a similar classification algorithm as in [2] can be applied.

- Blind detection of DMRS ports and nSCID

Furthermore, we evaluated the performance under TM9 with cell ID [0, 6, 1] with a set of interference profiles listed below. Detailed assumptions are shown in Annex.

- Interference profiles:  (1) INR1 = 7.77dB, INR2 = 2.29dB; (2) INR1 = 13.91dB, INR2 = 3.34dB

- Interference combinations:

Table 1 Simulated combinations

| TM (Serving/I1/I2) | Interference cell RI | Interference cell MCS | Serving cell RI | Serving cell MCS |
| --- | --- | --- | --- | --- |
| TM9/TM9/TM9 ON/ON/ON pattern | 1 | 5 | 1 | {5} |
|  |  |  |  | {14} |
|  | 1 | 14 | 1 | {5} |
|  |  |  |  | {14} |
|  | 2 | 5 | 1 | {5} |
|  |  |  |  | {14} |
|  | 2 | 14 | 1 | {5} |
|  |  |  |  | {14} |

# Simulation Results

In this contribution, we focus on the performance evaluation of R-ML receiver without losing the generality. Especially, three types of R-ML receiver are evaluated:

- Genie-aided R-ML receiver

- R-ML receiver with blind detection of modulation format

- R-ML receiver with joint blind detection of modulation, interference presence, DMRS port and n_SCID.

- Additionally, the performance of MMSE-IRC receiver is also presented as a baseline receiver.

The throughput performances of all cases are listed in Figure 1 to Figure 4.

- Figure 1: Performance under Rank 1 interference and INR1 = 7.77dB, INR2 = 2.29dB

- Figure 2: Performance under Rank 2 interference and INR1 = 7.77dB, INR2 = 2.29dB

- Figure 3: Performance under Rank 1 interference and INR1 = 13.91dB, INR2 = 3.34dB

- Figure 4: Performance under Rank 2 interference and INR1 = 13.91dB, INR2 = 3.34dB

Figure 1: Performance under Rank 1 interference and INR1 = 7.77dB, INR2 = 2.29dB

Figure 2: Performance under Rank 2 interference and INR1 = 7.77dB, INR2 = 2.29dB

Figure 3: Performance under Rank 1 interference and INR1 = 13.91dB, INR2 = 3.34dB

Figure 4: Performance under Rank 2 interference and INR1 = 13.91dB, INR2 = 3.34dB

# Observations

Based on the simulation results presented in Section 3, the performance for different receivers under different interference profiles are summarized in Table 1 . Furthermore, the average performance gain is also provided compared with MMSE-IRC receiver.

Table 1: Required SNR @ 70% Throughput [dB]

| INF Level 1 + Rank 1 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} | AVG Gain | Max Gain |
| --- | --- | --- | --- | --- | --- | --- |
| MMSE | 5.7 | 5.8 | 12.6 | 12.6 |  |  |
| R-ML without BD | 3.7 | 5.0 | 11.4 | 12.3 | 1.1 | 2.1 |
| R-ML with MF BD (1RB) | 3.7 | 5.4 | 11.5 | 12.7 | 0.9 | 2.1 |
| R-ML with MF+INF ON/OFF +DMRS Port and nSCID BD (1RB) | 3.7 | 5.5 | 11.5 | 12.7 | 0.8 | 2.0 |
| R-ML with MF BD (3RB) | 3.7 | 5.4 | 11.4 | 12.7 | 0.9 | 2.1 |
| R-ML with MF+INF ON/OFF +DMRS Port and nSCID BD (3RB) | 3.7 | 5.4 | 11.4 | 12.7 | 0.9 | 2.1 |
| INF Level 1 + Rank 2 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} | AVG Gain | Max Gain |
| MMSE | 5.9 | 5.9 | 12.7 | 12.7 |  |  |
| R-ML without BD | 5.0 | 5.7 | 12.5 | 13.0 | 0.3 | 0.9 |
| R-ML with MF BD (1RB) | 5.0 | 5.8 | 12.5 | 13.1 | 0.2 | 0.9 |
| R-ML with MF+INF ON/OFF +DMRS Port and nSCID BD (1RB) | 5.0 | 5.9 | 12.5 | 13.1 | 0.2 | 0.8 |
| R-ML with MF BD (3RB) | 5.0 | 5.9 | 12.5 | 13.1 | 0.2 | 0.9 |
| R-ML with MF+INF ON/OFF +DMRS Port and nSCID BD (3RB) | 5.0 | 5.9 | 12.5 | 13.1 | 0.2 | 0.9 |
| INF Level 2 + Rank 1 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} | AVG Gain | Max Gain |
| MMS | 7.6 | 7.7 | 14.3 | 14.3 |  |  |
| R-ML without BD | 3.9 | 5.8 | 10.9 | 13.4 | 2.5 | 3.8 |
| R-ML with MF BD (1RB) | 3.9 | 6.0 | 11.2 | 13.8 | 2.3 | 3.8 |
| R-ML with MF+INF ON/OFF +DMRS Port and nSCID BD (1RB) | 3.9 | 6.0 | 11.2 | 13.8 | 2.2 | 3.7 |
| R-ML with MF BD (3RB) | 3.9 | 5.9 | 11.0 | 13.6 | 2.4 | 3.8 |
| R-ML with MF+INF ON/OFF +DMRS Port and nSCID BD (3RB) | 3.9 | 5.9 | 11.0 | 13.6 | 2.4 | 3.8 |
| INF Level 2 + Rank 2 INF | MCS{5,5} | MCS{5,14} | MCS{14,5} | MCS{14,14} | AVG Gain | Max Gain |
| MMSE | 9.6 | 9.6 | 16.3 | 16.3 |  |  |
| R-ML without BD | 5.1 | 8.2 | 13.6 | 15.9 | 2.2 | 4.4 |
| R-ML with MF BD (1RB) | 5.2 | 8.7 | 13.8 | 16.4 | 1.9 | 4.3 |
| R-ML with MF+INF ON/OFF +DMRS Port and nSCID BD (1RB) | 5.4 | 8.7 | 13.9 | 16.4 | 1.8 | 4.2 |
| R-ML with MF BD (3RB) | 5.2 | 8.6 | 13.8 | 16.3 | 1.9 | 4.3 |
| R-ML with MF+INF ON/OFF +DMRS Port and nSCID BD (3RB) | 5.2 | 8.6 | 13.8 | 16.3 | 1.9 | 4.3 |

Based on the results, it is observed that for TM9 2Tx case,

- Compared with genie-aided R-ML receiver, blind detection of modulation format suffers marginal performance loss, i.e. up to 0.3dB in terms of average performance loss, under all simulated interference scenario.

- On top of modulation format blind detection, blind detection of interference presence, DMRS port and n_SCID doesn't show noticeable performance loss.

- BD with 3RB granularity and 1RB granularity doesn't show noticeable performance difference.

- In addition, Figure 5 provide the results when both interference cell are turn off. The results confirm no false alarm issue (i.e. OFF->ON misdetection) for interference absence detection.

- With joint blind detection of modulation, interference presence, DMRS port and n_SCID, R-ML receiver provide more than 4dB performance gain for high interference level and 2dB for low interference level with rank 1 case. compared with MMSE-IRC receiver.

  - Noted, because both interference cell are turn on in simulation, the interference level seen by UE is underestimated, i.e. the interference level could be even higher in realistic network.

Figure 5: Performance when both interference cell are turn off

Based on the analysis above, our conclusion is:

Conclusion: for TM9-TM9 case with 2Tx case, the joint blind detection of modulation, interference presence, DMRS port and n_SCID is feasible, and R-ML receiver with blind detection still provide promising performance gain.

# Conclusion

In this contribution, we investigate the performance of blind detection of dynamic parameters for TM9 mode,

Our observations are:

- Compared with genie-aided R-ML receiver, blind detection of modulation format suffer marginal performance loss, i.e. up to 0.3dB in terms of average performance loss, under all simulated interference scenario.

- On top of modulation format blind detection, blind detection of interference presence, DMRS port and n_SCID doesn't show noticeable performance loss.

- BD with 3RB granularity and 1RB granularity doesn't show noticeable performance difference.

- In addition, Figure 5 provide the results when both interference cell are turn off. The results confirm no false alarm issue (i.e. OFF->ON misdetection) for interference absence detection.

- With joint blind detection of modulation, interference presence, DMRS port and n_SCID, R-ML receiver provide more than 4dB performance gain for high interference level and 2dB for rank 1 with low interference level.

Based on the observation above, it is concluded for TM9-TM9 case with 2Tx case, the joint blind detection of modulation, interference presence, DMRS port and n_SCID is feasible, and R-ML receiver with blind detection still provide promising performance gain.

# Reference

- R4-142054, On blind detection feasibility of parameter combination for CRS-based TM, Samsung

- D. Bai, et al, “Near ML Modulation Classification”, in Proc. of IEEE VTC 2012, pp.1-5.

# Annex

Table 5-1: Simulation Assumptions

| Parameter | Value |
| --- | --- |
| System bandwidth | 10 MHz |
| RB allocation | 6 |
| Cell ID | [0, 6, 1] |
| Transmission mode on Serving cell | TM4 |
| Transmission mode on Interference cell | TM4 |
| MIMO configuration | 2x2 and low correlation |
| Channel model and Doppler frequency for target and interference cells | EPA 5Hz Use different channel seed for between cells |
| CRS configuration | 2 CRS ports. CRS is colliding between serving cell and interference cells |
| PA | -3dB |
| PB | 0dB |
| CSI-RS configuration | None |
| Channel Estimation | DMRS-IC |
| PMI | Random PMI |
| H-ARQ | 8 HARQ processes |
| PCFICH | CFI = 2 |
| PCFICH/PDCCH detection | Not considered |