3GPP TSG-RAN WG4 RAN4 #70bis                                     R4 -142068

San Jose Del Cabo, Mexico, 31 Mar – 4 Apr, 2014

Agenda item:	6.1.1

Source: 	Samsung

Title: 	Power imbalance for intra-band Non-contiguous CA

Document for:	Discussion

- Introduction

In previous RAN4 meetings, the intra-band Non-contiguous CA with shared LNA architecture were discussed[1-4]. One of the key issues is that define the reasonable maximum allowed power imbalance between DL CCs, especially with non-collocated deployment, in order to be taken as assumption in performance test. As mentioned in former contributions, the received signal with larger power was taken as interference signal to another receiver signal with smaller power, so one option for the power imbalance values was that follow the ACS/in-band emission requirement in single carrier. Before final decision is made in RAN4 meeting, value of the maximum allowed power imbalance level will be varied due to additional consideration, so in this contribution, the possible value ranges of the maximum power imbalance are enumerated and analyzed by taking receiver performance into account.

- Discussion

The reference UE architecture is defined with dual receiver chain but with shared LNA as shown in Fig.1. Compared to dual LNAs architecture, single LNA architecture has simpler front-end configuration, less insertion loss and coupling loss. However, it causes power imbalance issue for shared LNA and dual receiver chains. As pointed in[1], when the two component carriers are collocated, then the power imbalance between 2 DL CCs can be ignored, so the current test cases for CA assuming equal power per CC can be reused since each CC suffers from the similar path losses. But for non-collocated deployment scenario, especially when UE is located at the cell edge of a component carrier as well as closer to the center of a small cell in another carrier, the power imbalance between PCC and SCC is not ignored.

Fig.1 Reference UE architecture for intra-band Non-contiguous CA

As former contribution mentioned, due to 2 downlink CCs shared LNA, so the signal received by one CC could be taken as the interference signal to the signal received by another CC, and worst case is that when the signal with larger power is considered as interference signal. Hence the power imbalance between these two CCs depends on whether the signal received with smaller power could still meet the receiver performance requirements such as ACS and in-band blocking. . In order to summary all possible value ranges of the maximum allowed power imbalance, the breakpoint values should be obtained.

- Value A: -25dBm-MIN(REFSENSPCC, REFSENSSCC)

For intra-band Non-contiguous CA, the extreme large power imbalance exists when one CC works at the maximum input level while another CC work at the REFSENS level. For example, the specification indicates that the maximum input level per CC for CA class B is -25dBm/MHz, meanwhile the REFSENS level is defined assuming QPSK in current specification. Thus the extreme large power imbalance is the difference between -25dBm/MHz and the related REFSENS level. Due to the DL CC which receive signal with larger power level could be either PCC or SCC, so the extreme value for power difference assuming QPSK demodulation are derived as -25dBm-MIN(REFSENSPCC, REFSENSSCC). And according to the REFSENS values defined in TS36.101, Value A is approximately 70-75dB. If for a higher modulation 64QAM, excess demodulation margin should be added.

- Value B: power imblance level according to single carrier in-band blocking requirement

Beside the extreme case as ValueA, if take the signal with large power as interference signal to the signal with smaller power, there are two minimum requirement for this interference level in specification which are in-band blocking and ACS. And ACS is the for the worst case that interference signal is adjacently close to wanted signal, while in-band blocking requirment is for the signal which is shift within 15MHz away from the wanted signal. Given meet the fixed gap condition, power imbalance level is derived based on the interference level suffer from signal with large power to signal with smaller power meet the in-band blocking requirment in single carrier, as noted Value B. Take reference case 5+20MHz of CA_3A-3A.

- Case1: PCC 5MHz + SCC 20MHz with higher power level

Assuming 20MHz SCC has stronger receive power than 5MHz PCC. Then SCC is taken as a block signal for PCC, and because the REFSENS level on Band 3 with 5MHz is -97dBm/5MHz,with limitation of in-band blocking/ACS requirements the maximum power imbalance between 2CCs is -44dBm/5MHz-(REFSENS/5MHz +6dB) =47dB to support QPSK demodulation.

Fig.2 Reference case on intra-band Non-contiguous CA Band 3 with 5+20MHz and sub-block gap of 15MHz

- Case2: PCC 20MHz + SCC 5MHz with higher power level

Assuming 5MHz SCC has stronger receive power than 20MHz PCC. REFSENS level on Band 3 with 20MHz is -91dBm/20MHz, thereby the maximum allowed power imbalance is -44dBm/5MHz-(REFSENS/20MHz +9dB-10log4) =44dB to support QPSK demodulation with limitation of current in-band blocking requirement.

Fig.3 Reference case on intra-band Non-contiguous CA Band 3 with 5+20MHz and sub-block gap of 15MHz

Therefore, Value B is evaluated by meeting the in-band requirements with certain sub-blcok gap. It is band specific and bandwidth specific since REFSENS is band specific while minimum in-band requirement is bandwidth specifc.

- Value C: power imblance level according to single carrier ACS requirement

As discussed above, ACS requirement is for the worst case when interference signal adjacently close to the wanted signal. For intra-band NC CA, that equal to DL PCC is quite close to DL SCC. At this moment, the maximum input power imbalance value could evaluated based on single carrier ACS requirement which is only bandwidth specific.

| CC with lower receiver power | 5MHz | 10MHz | 15MHz | 20MHz |
| --- | --- | --- | --- | --- |
| Value C | 31.5dB | 31.5dB | 28.5dB | 25.5dB |

In case there still not define the final value XdB for the maximum power imbalance between 2 CCs, maybe take additional aspect into account, then all the possible value ranges based Value A, B and C are summrized here and pro/cons for each range is also analyzed here.

Option1: X > Value A

Pros: Allow quite large power imbalance

Cons: UE cannot work.

Since if XdB is too large, then it exceeds dynamic range of LNA and AGC.

Option2: Value B < X ≤ Value A

Pros: Allow relatively large power imbalance

Cons: Have to introduce new in-band blocking requirement in specification. Also exist REFSEN degradation problem for CC with weaker receiver power.

Option2 indicate that interference level suffer from CC with higher receiver power to CC with lower receiver power exceed the minimum in-band blocking requirement. So given this condition, additional in-band blocking requirement should be introduced into specification, and it spends time and energy on this study for this new WI. Meanwhile, there also exists another problem that CC with lower power would suffer from the excess noise generated by LNA, because LNA is expected to configure with high gain to achieve low NF, if LNA gain switch from stronger CC to weaker CC,   it leads to non-optimal LNA gain and generate excess noise inside LNA to CC with weaker signal.

Option3: Value C ≤ X ≤ Value B

Pros: Won’t lead to specification change

Cons: Limitation that combined with certain sub-block gap

If the maximum allowed power imbalance is within Option3, it should combined with certain sub-blcok gap which follow the in-band blocking test in specification. Otherwise without gap limitation, if sub-block gap is too small that 2CCs are considered as adjacent channel, then additional ACS requirement is still needed.

Option4: X < Value C

Pros: No additional ACS/in-band blocking requirements without any limitation

Cons: Too small power imbalance that may not meet operator’s requirement

Option4 is the strictest value range of power imbalance level. Different from Option3 with sub-block gap limitation, Option4 won’t lead any change to specification without any limitation condition. However, such strict power imbalance limit the operators’ network allocation.

Proposal: The maximum allowed power imbalance level of intra-band Non-contiguous CA with non-collocated applies within range of Option3 with certain sub-block gap limitation in terms of UE performance and cost.

- Conclusion

In this contribution, all possible value ranges for the maximum power imbalance are enumerated, and the related pros and cons for each value range are also analyzed. Our proposal is as follow

Proposal: The maximum allowed power imbalance level of intra-band Non-contiguous CA with non-collocated applies within range of Option3 with certain sub-block gap limitation in terms of UE performance and cost.

- Reference

- R4-134947, “Intra-band non-contiguous CA deployment scenario and UE performance requirement considerations”, Ericsson.

- R4-134774, “Receiver timing window requirement for intra-band non-contiguous CA”, Qualcomm Incorporation.

- R4-136482, “Non-contiguous Intra-band CA power imbalance and timing difference”, Broadcom Corporation.

- R4-136632, “Consideration from RF perspective on intra-band NC CA with non-collocation deployment”, Ericsson.