3GPP TSG-RAN WG4 RAN4 #70bis                                    R4 -142073

San Jose Del Cabo, Mexico, 31Mar - 4 Apr, 2014

Agenda item:	5.4

Source: 	Samsung

Title: 			  Simulation results for TM9 FeICIC test

Document for:	   Discussion

1	Introduction

Currently, demodulation and CSI performance requirements for FeICIC WI were only introduced based on CRS mode. It’s recognized CRS-IC can also bring performance gain DMRS mode i.e TM9. In order to verify UE can also support CRS-IC in TM9 mode, a way forward was agreed in last meeting:

- PDSCH demodulation test cases for TM9 FeICIC in Rel 11 spec will be introduced as part of TEI11

  - Both for FDD and TDD

- Reuse existing Rel11 FeICIC test case parameters for:

  - Noc levels, Timing and Frequency offsets, ABS pattern, etc.

  - Other options are not precluded.

In this contribution, we provide initial simulation results and analysis for FDD TM9 FeICIC test.

- Analysis

2.1 Simulation assumption

As proposed in WF, existing FeICIC test case parameters can be reused here. Furthermore, for TM9 specific test parameters, legacy TM9 demodulation test case parameters can be reused. A detailed test parameters for simulation assumption was given in table 1, and some key parameters summarized below:

- TM9 specific parameters:

  - NZP CSI-RS configuration: Port 7,8 TCSI-RS / ∆CSI-RS 5/2, configuration 0

  - ZP CSI-RS configuration: Not configured

  - Beamforming model:

    - Option 1, random precoding as Annex B.4.1

    - Option 2, following PMI as Annex B.4.3 with wideband PMI

- FeICIC specific parameters:

  - ABS pattern: 11000000

  - Timing offset: 3us, -1us

  - Frequency offset: 300Hz, -100Hz

  - INR: 12dB, 10dB

  - Cell ID configuration:

    - Option1: CN

    - Option2: NC

- Common parameters:

  - MCS: 16QAM 1/2 Rank1

  - Antenna MIMO correlation and fading channel: 2*2

    - Option1: EVA5 Low

    - Option2: EPA5 High

Test Parameters for TM9-Single-Layer (2 CSI-RS ports) – Non-MBSFN ABS

| Parameter |  |  | Unit |  | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  |  | dB |  | 0 |  | -3 |  | -3 |  |
|  |  |  | dB |  | 0 (Note 1) |  | -3 (Note 1) |  | -3 (Note 1) |  |
|  |  |  | dB |  | -3 |  | N/A |  | N/A |  |
| at antenna port |  |  | dBm/15kHz |  | -98 (Note 2) |  | N/A |  | N/A |  |
|  |  |  | dBm/15kHz |  | -98 (Note 3) |  | N/A |  | N/A |  |
|  |  |  | dBm/15kHz |  | -93 (Note 4) |  | N/A |  | N/A |  |
|  |  |  | dB |  | Reference Value in Table 2 |  | 12 |  | 10 |  |
| BWChannel |  |  | MHz |  | 10 |  | 10 |  | 10 |  |
| Subframe Configuration |  |  |  |  | Non-MBSFN |  | Non-MBSFN |  | Non-MBSFN |  |
| Time Offset between Cells |  |  | s |  | N/A |  | 3 |  | -1 |  |
| Frequency shift between Cells |  |  | Hz |  | N/A |  | 300 |  | -100 |  |
| Cell Id |  |  |  |  | 0 |  | Option 1: 126 Option 2: 1 |  | Option 1: 1 Option 2: 126 |  |
| Cell-specific reference signals |  |  |  |  | Antenna ports 0,1 |  |  |  |  |  |
| CSI reference signals |  |  |  |  | Antenna ports 15,16 |  | N/A |  | N/A |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | Subframes |  | 5 / 2 |  | N/A |  | N/A |  |
| CSI reference signal configuration |  |  |  |  | 8 |  | N/A |  | N/A |  |
| ZeroPowerCSI-RS bitmap |  |  | bitmap |  | 0000000000000000 |  | N/A |  | N/A |  |
| ABS pattern (Note 5) |  |  |  |  | N/A |  | 11000000 11000000 11000000 11000000 11000000 |  | 11000000 11000000 11000000 11000000 11000000 |  |
| RLM/RRM Measurement Subframe Pattern (Note 6) |  |  |  |  | 10000000 10000000 10000000 10000000 10000000 |  | N/A |  | N/A |  |
| CSI Subframe Sets (Note7) |  | CCSI,0 |  |  | 11000000 11000000 11000000 11000000 11000000 |  | N/A |  | N/A |  |
|  |  | CCSI,1 |  |  | 00111111 00111111 00111111 00111111 00111111 |  | N/A |  | N/A |  |
| Number of control OFDM symbols |  |  |  |  | 2 |  | Note 8 |  | Note 8 |  |
| PDSCH transmission mode |  |  |  |  | 9-1layer |  | Note 9 |  | Note 9 |  |
| Beamforiming Mode |  |  |  |  | Option1: Annex B.4.1 Option2: Annex B.4.3 |  | N/A |  | N/A |  |
| Peporting mode |  |  |  |  | PUSCH 3-1 |  | N/A |  | N/A |  |
| CodeBookSubsetRestriction bitmap |  |  |  |  | 1111 |  | N/A |  | N/A |  |
| Cyclic prefix |  |  |  |  | Normal |  | Normal |  | Normal |  |

2.2 Simulation results

Based on proposed test setup in last chapter, simulation results for different combinations were given in this chapter. Both performances with CRS-IC and without CRS-IC were evaluated to verify CRS-IC gain for certain test configurations.

Figure 1 and figure 2 gives absolute throughput performance vs. SNR with different NC cell-ID configurations (NC and CN) for different combinations of channel model (EPA5 High and EVA5 Low) and precoding model(following PMI and random precoding).

| Figure 1: Throughput Vs SNR for NC cases |  |
| --- | --- |
| Figure 2: Throughput Vs SNR for CN cases |  |

Based on the simulation results, reference SNR at 70% relative throughput point with CRS-IC on/off and CRS-IC gain were summarized in table 1.

Table 1: Reference SNR point at 70% relative TP and SNR offset with CRS-IC on/off

| Fading channel & Cell ID configuration | EPA5Hz High |  | EVA5 Low |  |
| --- | --- | --- | --- | --- |
|  | NC | CN | NC | CN |
| Following UE wideband PMI |  |  |  |  |
| SNR at 70% TP with CRS on | 3.1 dB | 3.1 dB | 3.5 dB | 3.5 dB |
| SNR at 70% TP with CRS off | 8.5 dB | 7.0 dB | 8.4 dB | 6. 9 dB |
| SNR offset | 5.4 dB | 3.9 dB | 4.9 dB | 3.4 dB |
| Fandom precoding |  |  |  |  |
| SNR at 70% TP with CRS on | 7.4 dB | 7.4 dB | 5.0 dB | 5.0 dB |
| SNR at 70% TP with CRS off | 13.0 dB | 11.5 dB | 9.7 dB | 8.3 dB |
| SNR offset | 6.6 dB | 4.1 dB | 4.7 dB | 3.3 dB |

Based on above summary, we have such observations:

Observation 1: Performance gap for CRS-IC on/off under fading channel EPA5Hz with high correlation is larger than EVA5Hz with low correlation.

Observation 2: Performance gap for CRS-IC on/off with cell ID combination (0,1,126) is larger than the cell ID configuration (0,126,1) case.

Observation 3: For beamforming model, the performance gap for CRS-IC on/off with random precoding is larger than the case with following UE reported PMI precoding.

Observation 4: The performance difference for CRS-IC on/off is 3.3dB ~ 6.6 dB depending on the parameters combination including cell ID, fading channel, and precoding model.

Based on such analysis and observations, such detailed test case design given:

Proposal1: Reusing the parameters from existing FeICIC test case and legacy TM9 demodulation test cases to introduce TM9 FeICIC test cases i.e. such common parameters can be used:

- MCS: 16QAM Rank1

- Channel and antenna correlation: EPA5Hz , 2*2 High

- Beamforming model: Random precoding (Annex B.4.1)

- Cell-ID configuration: 0, 1,126

- Conclusion

In this contribution, simulation results for FeICIC TM9 test were evaluated based on proposed test configuration, and such observation were given:

Observation 1: Performance gap for CRS-IC on/off under fading channel EPA5Hz with high correlation is larger than EVA5Hz with low correlation.

Observation 2: Performance gap for CRS-IC on/off with cell ID combination (0,1,126) is larger than the cell ID configuration (0,126,1) case.

Observation 3: For beamforming model, the performance gap for CRS-IC on/off with random precoding is larger than the case with following UE reported PMI precoding.

Observation 4: The performance difference for CRS-IC on/off is 3.3dB ~ 6.6 dB depending on the parameters combination including cell ID, fading channel, and precoding model.

Based on the observations and analysis, such detailed test case design were proposed :

Proposal1: Reusing the parameters from existing FeICIC test case and legacy TM9 demodulation test cases to introduce TM9 FeICIC test cases i.e. such common parameters can be used:

- MCS: 16QAM Rank1

- Channel and antenna correlation: EPA5Hz , 2*2 High

- Beamforming model: Random precoding (Annex B.4.1)

- Cell-ID confgiration: 0,1,126

- Reference

- R4-141091, “Way Forward on FeICIC TM9 Test Case” Qualcomm, Intel, Ericsson