3GPP TSG-WG4 Meeting #70bis	R4-141485

# San Jose Del Cabo, Mexico, 31 Mar - 4 Apr 2014

Agenda item: 	8.1.2

Source: 	Qualcomm Incorporated

Title: 	Remaining UE core requirements for UTRA Band I + SDL 1452 – 1496 MHz

Document for:	Discussion and Approval

# 1	Introduction

This contribution addresses the remaining UE core requirements for UTRA L-band SDL. It also provides corresponding TP to TR 37.814.

# 2	Discussion

## 2.1	Spurious emissions

UE spurious emissions requirements in TR 37.814 v0.3.0 needs to be updated with respect to the updated WID to with upper 4 MHz extended. Updated UE spurious emissions requirements are presented in Annex. It should be noted that, for UE Tx core requirements, there is no requirement for L-band since it is a supplemental DL band. However, UE spurious emissions requirements are needed to protect L-band itself for UE Rx core requirements. Additional UE Rx spurious emissions requirements for L-band are also presented in Annex.

## 2.2	UE reference input power adjustment for a dual band device

For the UE which supports DB-DC-HSDPA or dual band 4C-HSDPA, the reference input powers for DC-HSUPA requirements need to be adjusted. Typically the amount of adjustment has been the same as the additional insertion loss itself. Therefore, it is proposed that the allowed increase of reference input power be the same as the agreed additional insertion loss due to a diplexer.

# 3	Conclusion

This contribution has addressed the remaining UE core requirements for UTRA L-band SDL. It also provides corresponding TP to TR 37.814 for approval.

Proposal: Agree provided TP in Annex.

# 4	Reference

- RP-132041, " Revised Work Item proposal: L-band for Supplemental Downlink in E-UTRA and UTRA," KDDI

- R4-140811, “TR 37.814: L-band for Supplemental Downlink in E-UTRA and UTRA,” Ericsson, Orange

# Annex	TP to TR 37.814

------------------------------------------------------------ Start of TP -----------------------------------------------------------------

# 6	List of band specific issues for the L-band for Supplemental DL

- General

  - Co-existence with other systems around/within the band;

  - Co-existence with other 3GPP bands in Region 1;

- Specific E-UTRA: Band 20+L-band

  - UE REFSENS;

  - UE MOP;

- Specific UTRA: Band I+L-band

  - UE REFSENS;

  - UE MOP;

  - UE blocking performance;

  - UE Reference input power adjustment for a dual band device

-------------------------------------------------- Unchanged text omitted ------------------------------------------------------------

## 8.1 	UE MOP

The same values of relaxation for the lower tolerance of maximum output power shown in Table 8.1-1 are adopted as used in E-UTRA Band 1+21 for the corresponding DB-DC-HSDPA and dual band 4C-HSDPA combinations between Band I and L-Band.

Table 8.1-1: Allowed adjustment to maximum output power lower tolerance for UTRA Band I + L-Band

| UTRA SDL Configuration | Anchor Band (DL and UL) |  | Supplemental Band (DL only) |  | Maximum allowed adjustment in lower side of tolerance (dB) |
| --- | --- | --- | --- | --- | --- |
|  | Band | Bandwidth | Band | Bandwidth |  |
| 1a | I (2.1 GHz) | 5 MHz | L-band | 5 MHz | -0.3 |
| 1b | I (2.1 GHz) | 5 MHz | L-band | 10 MHz | -0.3 |
| 1c | I (2.1 GHz) | 10 MHz | L-band | 5 MHz | -0.3 |
| NOTE: The maximum allowed adjustment in lower side tolerance of maximum output power is only applicable for the anchor band. |  |  |  |  |  |

-------------------------------------------------- Unchanged text omitted ------------------------------------------------------------

## 8.3	UE emissions for co-existence

Table 8.3-1 contains the updates required in the Additional spurious emissions for co-existence table in TS25.101. This is needed for SC and DC-HSUPA.

# Table 8.3-1: Additional spurious emissions requirements for UTRA

| Operating Band | Frequency Bandwidth | Measurement Bandwidth | Minimum requirement |
| --- | --- | --- | --- |
| I | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1452 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm * |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz <f< 1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2570 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm |
| II | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 758 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2690 MHz | 1 MHz | -50 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm** |
| III | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f  890 MHz | 3.84 MHz | -60 dBm ***** |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * - 60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1452 MHz < f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm ***** |
|  | 1805 MHz f 1880 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm ***** |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2570 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm ** |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm ** |
| IV | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2690 MHz | 1 MHz | -50 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm** |
| V | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 859 MHz f 869 MHz | 1 MHz | -27 dBm |
|  | 869 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz  f  2690 MHz | 1 MHz | -50 dBm** |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm |
| VI | 758 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f < 875 MHz | 1 MHz | -37 dBm |
|  | 875 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
| VII | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 758 MHz f 791 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 869 MHz | 1 MHz | -50 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1452 MHz < f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm * |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 2590 MHz  f  2620 MHz | 3.84 MHz | -50 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm |
| VIII | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 703 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 860 MHz f 890 MHz | 1 MHz | -37 dBm **** |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz 3.84 MHz | -79 dBm * -60 dBm |
|  | 1452 MHz < f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm **** |
|  | 1805 MHz < f  1830 MHz | 100 kHz 3.84 MHz | -71 dBm ** & * -60 dBm ** |
|  | 1830 MHz < f  1880 MHz | 100 kHz 3.84 MHz | -71 dBm * -60 dBm |
|  | 1884.5 MHz f 1915.7 MHz | 300 kHz | -41 dBm **** |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2496 MHz  f  2570 MHz | 1 MHz | -50 dBm |
|  | 2570 MHz  f  2640 MHz | 3.84 MHz | -60 dBm |
|  | 2640 MHz < f  2690 MHz | 3.84 MHz | -60 dBm ** |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm ** |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm ** |
| IX | 758 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
| X | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm ** |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm ** |
| XI | 758 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
| XII | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm |
| XIII | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 763 MHz f 775 MHz | 6.25 kHz | [TBD] dBm*** |
|  | 793 MHz f 805 MHz | 6.25 kHz | [TBD] dBm*** |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm** |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm** |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm |
| XIV | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 769 MHz f 775 MHz | 6.25 kHz | [TBD] dBm *** |
|  | 799 MHz f 805 MHz | 6.25 kHz | [TBD] dBm *** |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm |
| XIX | 758 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f < 875 MHz | 1 MHz | -37 dBm |
|  | 875 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
| XX | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1452 MHz < f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm * |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 2570 MHz  f  2620 MHz | 3.84 MHz | -60 dBm** |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz  f  3800 MHz | 1 MHz | -50 dBm ** |
| XXI | 758 MHz f 803 MHz | 1 MHz | -50 dBm |
|  | 860 MHz  f  890 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz  f  1510.9 MHz | 1 MHz | -35 dBm |
|  | 1839.9 MHz  f  1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2545 MHz  f  2575 MHz | 1 MHz | -50 dBm |
| XXII | 758 MHz f 791 MHz | 1 MHz | -50 dBm |
|  | 791 MHz f 821 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm * |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm * -60 dBm |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm * |
|  | 1452 MHz < f  1496 MHz | 3.84 MHz | -60 dBm |
|  | 1805 MHz  f  1880 MHz | 100 kHz | -71 dBm * |
|  | 1880 MHz  f  1920 MHz | 3.84 MHz | -60 dBm |
|  | 2010 MHz  f  2025 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz  f  2170 MHz | 3.84 MHz | -60 dBm |
|  | 2300 MHz  f  2400 MHz | 3.84 MHz | -60 dBm |
|  | 2620 MHz  f  2690 MHz | 3.84 MHz | -60 dBm |
|  | 2570 MHz  f  2620 MHz | 3.84 MHz | -60 dBm |
|  | 3510 MHz  f  3525 MHz | 1 MHz | -40 dBm |
|  | 3525 MHz  f  3590 MHz | 1 MHz | -50 dBm |
|  | 3600 MHz  f  3800 MHz | 3.84 MHz | -50 dBm |
| XXV | 462.5 MHz f 467.5 MHz | 1 MHz | -50 dBm |
|  | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 746 MHz | 3.84 MHz | -60 dBm |
|  | 746 MHz f 756 MHz | 3.84 MHz | -60 dBm |
|  | 758 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 852 MHz f 859 MHz | 1 MHz | -50 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 3.84 MHz | -60 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2350 MHz f 2360 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |
|  | 3400 MHz f 3800 MHz | 1 MHz | -50 dBm ** |
| XXVI | 717 MHz f 728 MHz | 1 MHz | -50 dBm |
|  | 729 MHz f 768 MHz | 3.84 MHz | -60 dBm |
|  | 768 MHz f 799 MHz | 1 MHz | -50 dBm |
|  | 799 MHz f 803 MHz | 1 MHz | -40 dBm |
|  | 859 MHz f 894 MHz | 3.84 MHz | -60 dBm |
|  | 945 MHz  f  960 MHz | 3.84 MHz | -60 dBm |
|  | 1475.9 MHz f 1510.9 MHz | 3.84 MHz | -60 dBm |
|  | 1525 MHz f 1559 MHz | 1 MHz | -50 dBm |
|  | 1839.9 MHz f 1879.9 MHz | 3.84 MHz | -60 dBm |
|  | 1884.5 MHz f  1915.7 MHz | 300 kHz | -41 dBm |
|  | 1930 MHz f 1995 MHz | 3.84 MHz | -60 dBm |
|  | 2010 MHz f 2025 MHz | 1 MHz | -50 dBm |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |
|  | 2180 MHz f 2200 MHz | 1 MHz | -50 dBm |
|  | 2300 MHz f 2400 MHz | 1 MHz | -50 dBm |
|  | 2496 MHz f 2690 MHz | 1 MHz | -50 dBm ** |
|  | 3400 MHz f 3800 MHz | 1 MHz | -50 dBm |
| Note * The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 6.12 are permitted for each UARFCN used in the measurement Note ** The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, measurements with a level up to the applicable requirements defined in Table 6.12 are permitted for each UARFCN used in the measurement due to 2nd, 3rd and 4th harmonic spurious emissions Note *** This requirement is applicable also for frequencies, which are between 2.5 MHz and 12.5 MHz away from the UE centre carrier frequency. Note **** This requirement is applicable only when transmission is made between 900MHz to 915MHz. Note ***** This requirement is applicable only when transmission is made between 1744.9 MHz to 1784.9 MHz |  |  |  |

Table 8.3-2 contains the updates required for UE Rx core requirements in addition to the changes in Table 8.3-1. Note that L-band is labelled as “L” since the numbering needs to be done for the specification.

Table 8.3-2: Additional Rx spurious emissions requirements for UTRA

| Band | Frequency Band | MeasurementBandwidth | Maximum level | Note |
| --- | --- | --- | --- | --- |
| L | 791 MHz  f < 821 MHz | 3.84 MHz | -60 dBm |  |
|  | 921 MHz  f < 925 MHz | 100 kHz | -60 dBm* |  |
|  | 925 MHz f 935 MHz | 100 kHz 3.84 MHz | -67 dBm* -60 dBm |  |
|  | 935 MHz < f  960 MHz | 100 kHz | -79 dBm* |  |
|  | 1452 MHz < f  1496 MHz | 3.84 MHz | -60 dBm | UE receive band |
|  | 1805 MHz f 1880 MHz | 3.84 MHz | -60 dBm |  |
|  | 2110 MHz f 2170 MHz | 3.84 MHz | -60 dBm |  |
|  | 2570 MHz f 2620 MHz | 3.84 MHz | -60 dBm |  |
|  | 2620 MHz f 2690 MHz | 3.84 MHz | -60 dBm |  |
|  | 3400 MHz  f  3510 MHz | 1 MHz | -50 dBm |  |
|  | 3510 MHz  f  3590 MHz | 3.84 MHz | -60 dBm |  |
|  | 3590 MHz  f  3800 MHz | 1 MHz | -50 dBm |  |
| Note * The measurements are made on frequencies which are integer multiples of 200 kHz. As exceptions, up to five measurements with a level up to the applicable requirements defined in Table 7.10 are permitted for each UARFCN used in the measurement |  |  |  |  |

-------------------------------------------------- Unchanged text omitted ------------------------------------------------------------

## 8.5		UE Reference input power adjustment for a dual band device

For the UE which supports DB-DC-HSDPA or dual band 4C-HSDPA, the reference input powers for DC-HSUPA requirements need to be adjusted. Typically the amount of adjustment has been the same as the additional insertion loss itself. Therefore, the allowed increase of reference input power in Table 8.5-1 and Table 8.5-2 is adopted. Note that L-band is labelled as “L” since the numbering needs to be done for the specification.

# Table 8.5-1: Allowed increase of HS-PDSCH Ec and Îor for UE which supports DB-DC-HSDPA.

| DB-DC-HSDPA Configuration | Allowed increase of HS-PDSCH Ec and Îor (dB) | Applicable bands |
| --- | --- | --- |
| 6 | 0.6 | I |

# Table 8.5-2: Allowed increase of HS-PDSCH Ec and Îor for UE which supports dual band 4C-HSDPA.

| Dual Band 4C-HSDPA Configuration | Allowed increase of HS-PDSCH Ec and Îor (dB) | Applicable bands |
| --- | --- | --- |
| I-1-L-2 I-2-L-1 | 0.6 | I |

------------------------------------------------------------ End of TP ------------------------------------------------------------------