3GPP TSG-RAN WG4 Meeting #70bis                                                                   R4-141832

# San Jose Del Cabo, Mexico, 31 Mar - 4 Apr, 2014

Source:	LG Electronics

Title:	Link performance for NAICS receiver with blind detection under DMRS based transmission mode

Agenda item:	7.14.2.1

Document for:	Discussion

Introduction

In last RAN Plenary, Work Item for NAICS [1] was approved. RAN4’s objective is to identify and agree on the parameter combinations that could be blindly detected jointly, including if under any subset restriction for any parameters. Considering parameters for blind detection are as follows:

    - Presence or absence of interference

    - Transmission modes (TM)

    - For DMRS-based TMs: DMRS ports, modulation order, Virtual cell ID, nSCID, Cell ID, CRS ports, and MBSFN pattern

    - For CRS-based TMs: PMI, RI, modulation order, Cell ID, CRS ports, and MBSFN pattern, ρA

    - CFI (if not coordinated and required by receiver implementation)

To cancel or suppress aggressor PDSCH, higher-layer signaling of parameters are beneficial to reduce the blind detection complexity or performance degradation and desirable blind detection parameters to reduce scheduling restriction and signaling overhead were itemized in last RAN1 meeting [2]. In this contribution, we discuss blind detection for dynamic parameters and provide link level performance for blind detection under DMRS based transmission mode.

Performance for blind detection

. For DMRS based transmission mode, some candidate parameters for blind detection are as follows:

    - Transmission modes (TM)

    - DMRS ports, Presence or absence of interference

    - Modulation order

    - Virtual cell ID

    - nSCID

In this contribution, we evaluate blind detection performance for combination of DMRS ports, nSCID, and modulation order under follow interference conditions:

| TM | Serving MCS | Interference MCS |  | INR |  | Interference pattern |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Interference 1 | Interference 2 | Interference 1 | Interference 2 | Interference 1 | Interference 2 |
| 9 | 5 | 5 | 5 | 7.68 | 2.16 | ON | ON/OFF |
|  |  | 14 | 14 |  |  |  |  |
|  |  | 25 | 25 |  |  |  |  |
|  | 14 | 5 | 5 |  |  |  |  |
|  |  | 14 | 14 |  |  |  |  |
|  |  | 25 | 25 |  |  |  |  |

Those parameters are blindly detected per PRB pair. By working assumption in last RAN4 meeting, maximum 3 layers and one interference cell to cancel/suppress PDSCH interference are considered. For TM blind detection, it is assumed that DMRS based transmission mode for interference cell is known.

[Simulation results are to be added ]

Discussion for further simulation assumption

To analyze and align company observations on blind detection performance for dynamic parameters, common simulation assumption should be considered. Based on agreement in the last meeting, we propose the following simulation assumptions for fair comparison among companies from next meeting (Table 3-1~3-3).

Table 3-1 Simulation assumption for DMRS-based transmission (Common parameters)

| Common Parameters |  | Unit | Serving | I1 | I2 |
| --- | --- | --- | --- | --- | --- |
| Downlink power allocation |  | dB | 0 | 0 | 0 |
|  |  | dB | 0 | 0 | 0 |
|  | Pc | dB | -3 | -3 | -3 |
| BWChannel |  | MHz | 10 | 10 | 10 |
| Cell Id |  |  | 0 | 6 | 1 |
| Propagation channel |  |  | EPA5 |  |  |
| Correlation and antenna configuration |  |  | 2X2 Low |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |  |  |
| CSI reference signals |  |  | Antenna ports 15, 16 |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ICSI-RS |  |  | [TBD] | [TBD] | [TBD] |
| MCS |  |  | 5 | 5 | 5 |
|  |  |  |  | 14 | 14 |
|  |  |  | 14 |  |  |
|  |  |  |  | 25 | 25 |
| Transmission mode |  |  | 9 | 9 | 9 |
| PDSCH Allocation |  | PRBs | 1 (Note1) | 50 | 50 |
| Interference pattern |  |  |  | ON | ON |
|  |  |  |  |  | OFF |
| Number of control OFDM symbols |  |  | 2 | 2 | 2 |
| Note1: to avoid channel effect, middle PRB should be considered. For example, selected PRB index 20 among PRB index 0~49. Note2: the precoder shall be updated in each TTI (1 ms granularity) Note3: no timing and frequency offset |  |  |  |  |  |

For low (5~25%) SINR geometry and RU 40%,

Table 3-2 Simulation parameters for low SINR geometry

| Parameters | Unit | Serving | I1 | I2 |
| --- | --- | --- | --- | --- |
| E/Noc | dB | Sweep E/Noc to cover SINR range | 50% I1/Noc | Conditional median I2/Noc |
|  |  |  | 80% I1/Noc | Conditional median I2/Noc |
| Rank |  | 1 | 1 | 1 |
|  |  |  | 2 | 2 |
| Note1: Both layers of rank2 transmissions use the same MCS |  |  |  |  |

For medium (40~60%) SINR geometry and RU 40%,

Table 3-3 Simulation parameters for medium SINR geometry

| Parameters | Unit | Serving | I1 | I2 |
| --- | --- | --- | --- | --- |
| E/Noc | dB | Sweep E/Noc to cover SINR range | 50% I1/Noc | Conditional median I2/Noc |
| Rank |  | 2 | 1 | 1 |
| Note1: Both layers of rank2 transmissions use the same MCS |  |  |  |  |

Based on above simulation assumption, we suggest that blind detection performance for combination of dynamic parameters is evaluated, and SNR gain and loss at 70% of maximum throughput in comparison with baseline IRC and genie-aid NAICS throughput can be considered for evaluation metric.

Conclusion

In this contribution, we provide link level simulation results for blind detection performance under DMRS-based transmission mode, and our observations are as follows:

.- Observation:

And we propose common simulation assumptions to have fair comparison among companies on blind detection performance for dynamic parameters as Table 3-1~3-3. For evaluation metric, SNR gain and loss at 70% of maximum throughput in comparison with baseline IRC and genie-aid NAICS throughput can be considered.

.

Reference

- RP-140519, “New work item proposal for network assistance interference cancellation and suppression for LTE,” MediaTek Inc.

- 3GPP TR 36.866 V2.0.0.