3GPP TSG-RAN WG4 (Radio) Meeting #70bis	R4-141902

San Jose Del Cabo, Mexico, 31 March – 4 April 2014

Agenda Item:	7.42.2

Source: 	Alcatel-Lucent, KDDI

Title: 	TP for TR 36.851: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (18 + 28)

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 1, Band 18 and Band 28  was approved in RAN#63 [1]. One objective of the WI is to specify the band-combinations specific Radio Frequency (RF) requirements for Band 18 and Band 28 which are not yet specified.

The impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting carrier aggregation of this band combination to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to record the findings in the Inter-band Carrier Aggregation Technical Report [3].

# 2.	Text Proposal

<Start of change>

# 1	Scope

The present document is a technical report for Inter-band Carrier Aggregation under Rel-12 time frame. The purpose is to gather the relevant background information and studies in order to address Inter-band Carrier Aggregation requirements.

This TR covers relevant background information and studies in order to address Inter-band Carrier Aggregation requirements for the Rel-12 band combinations in table 1-1.

Table 1-1: Release 12 inter-band carrier aggregation combinations

| WI code | WI title | Class |
| --- | --- | --- |
| LTE_CA_B1_B7 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 7 | A3 |
| LTE_CA_B2_B4 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 4 | A4 |
| LTE_CA_B23_B29 | LTE Advanced inter-band Carrier Aggregation of Band 23 and Band 29 | A1 |
| LTE_CA_B3_B26 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 26 | A1 |
| LTE_CA_B3_B19 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 19 | A1 |
| LTE_CA_B1_B8 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 8 | A1 |
| LTE_CA_B3_B28 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 28 | A1 |
| LTE_CA_B1_B26 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 26 | A1 |
| LTE_CA_B39_B41 | LTE Advanced inter-band Carrier Aggregation of Band 39 and Band 41 | A3 |
| LTE_CA_B2_B12 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 12 | A1 |
| LTE_CA_B2_B13 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 13 | A1 |
| LTE_CA_B19_B21 | LTE Advanced inter-band Carrier Aggregation of Band 19 and Band 21 | A5 |
| LTE_CA_B12_B25 | LTE Advanced inter-band Carrier Aggregation of Band 12 and Band 25 | A1 |
| LTE_CA_B2_B5 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 5 | A1 |
| LTE_CA_B1_B18 | Additional bandwidth combination set for LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 18 | A1 |
| LTE_CA_B7_B28 | LTE Advanced inter-band Carrier Aggregation of Band 7 and Band 28 | A1 |
| LTE_CA_B5_B25 | LTE Advanced inter-band Carrier Aggregation of Band 5 and Band 25 | A1 |
| LTE_CA_B1_B11 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 11 | A5 |
| LTE_CA_B8_B11 | LTE Advanced inter-band Carrier Aggregation of Band 8 and Band 11 | A5 |
| LTE_CA_B5_B7 | LTE Advanced inter-band Carrier Aggregation of Band 5 and Band 7 | A1 |
| LTE_CA_B1_B3 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 3 | A3 |
| LTE_CA_B1_B28 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 28 | A2 |
| LTE_CA_B4_B27 | LTE Advanced inter-band Carrier Aggregation of Band 4 and Band 27 | A1 |
| LTE_CA_B3_B27 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 27 | A1 |
| LTE_CA_B8_B27 | LTE Advanced inter-band Carrier Aggregation of Band 8 and Band 27 | A3 |

The scope of the report has been further extended [6] to cover the 3 band Carrier Aggregation combinations that result in single UL configuration. These combinations are listed in table 1-2

Table 1-2: Release 12 inter-band carrier aggregation combinations

| WI code | WI title | Class |
| --- | --- | --- |
| LTE_CA_B2_B5_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 5 and Band 30 | A1 |
| LTE_CA_B2_B29_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 29 and Band 30 | A1 |
| LTE_CA_B1_B5_B7 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 5 and Band 7 | A1 |
| LTE_CA_B2_B12_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 12 and Band 30 | A1 |
| LTE_CA_B1_B3_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 3 and Band 20 | A1 |
| LTE_CA_B1_B7_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 7 and Band 20 | A1 |
| LTE_CA_B7_B8_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 7, Band 8 and Band 20 | A3 |
| LTE_CA_B1_B18_B28 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 18 and Band 28 | - |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

<New section>

##### 6.3.x.1.2.1	Co-existence studies for 1 UL/2 DL

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 18 and restricted (703 – 733 MHz UL and 758 – 788 MHz DL) Band 28 DL carriers can be calculated as shown in Table 6.3.x.1.2.1-1 below:

Table 6.3.x.1.2.1-1: Band 18 and restricted Band 28 DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 758 | 788 | 860 | 875 |
| 2nd harmonics frequency limits (MHz) | 1516 | 1576 | 1720 | 1750 |
| 3rd harmonics frequency limits (MHz) | 2274 | 2364 | 2580 | 2625 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 72 | 117 | 1618 | 1663 |
| 3rd order IMD products | (f2-low – 2*f1-high) | (f2-high – 2*f1-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 641 | 716 | 932 | 992 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 2376 | 2451 | 2478 | 2538 |
| 3rd order IMD products | (f1-low – f2-high + f2-low) | (f1-high + f2-high – f2-low) | (f2-low – f1-high + f1-low) | (f2-high + f1-high – f1-low) |
| IMD frequency limits (MHz) | 743 | 803 | 830 | 905 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 743 | 803 | 850 | 885 |

It can be seen from Table 6.3.x.1.2.1-1 that the 2nd harmonics of BS transmitting in Band 18 may fall into the BS receive band of Bands 3, 4, 9 and 10, and the 3rd harmonics of BS transmitting in Band 18 and restricted Band 28 may fall into the BS receive band of Bands 30, 38, 40 and 41, while the 2nd IMD products caused by BS supporting CA of Band 18 and restricted Band 28 may fall into the BS receive band of Band 24, and the 3rd IMD products may fall into the BS receive band of Bands 5, 6, 7, 8, 12, 13, 14, 17, 19, 20, 26, 28, 40, 41 and 44. Note that the calculation in Table 6.3.x.1.2.1-1 (except the last row) assumes the BS is transmitting with the whole 15 MHz DL frequency of Band 18 and the whole 30 MHz DL frequency of restricted Band 28. If the BS is only transmitting an up to 15 MHz DL in Band 18 and an up to 10 MHz DL in restricted Band 28 as stated in the WIDS, then the 3rd IMD products may only fall into the BS receive band of the Bands 7, 8, 12, 13, 14, 17, 20, 28, 40, 41 and 44 as shown in the last row in Table 6.3.x.1.2.1-1. Note that the 3rd IMD products will not fall into the BS receive frequency range within restricted Band 28, or the BS receive frequency range within Band 8 (900 – 915 MHz) that is allocated in Japan. Also the 3rd IMD products may only fall into the BS receive band of certain frequency range within Band 28 under the transmit configurations shown in Table 6.3.x.1.2.1-2 below.

Table 6.3.x.1.2.1-2: Band (18 + 28) BS transmit configurations with 3rd IMD within Band 28 BS receive band

| Band 18 DL channel bandwidth (MHz) | Band 28 DL channel bandwidth (MHz) | Lower edge of Band 28 DL frequency block (MHz) | Lower edge of IMD frequency limits (MHz) |
| --- | --- | --- | --- |
| 15 | 5 or 10 | 758 – 762.9 | 743 – 747.9 |

It should be noted that Bands 4, 7, 10, 12, 13, 14, 17, 20, 24, 30, 38, 40 and 44 are not intended for use in the same geographical area as Bands 18 and 28, and the 3rd IMD products caused by BS supporting carrier aggregation of Band 18 and Band 28 will not fall into the BS receive frequency range within Band 8 that is allocated in Japan. Therefore, the focus here will be on the harmonics and IMD falling into Bands 3, 9, 28 and 41.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 3, 9 or 41 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 18 and 28 BS transmitters do not share the same antenna with Band 3, 9 or 41 BS receiver.

On the other hand, it is recommended that Bands 18 and 28 BS transmitters should not share the same antenna with Band 3, 9 or 41 BS receiver, or Band 28 BS receiver for the affected frequency ranges if the aforementioned BS transmit configurations are used, in order to prevent BS receiver desensitization, unless the antenna path meets very stringent harmonics and 3rd order PIM specification so that the harmonics and PIM will not cause Band 3, 9, 28 or 41BS receiver desensitization.

<End of change>

# References

[1]	RP-140451, “New WI: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 1, Band 18 and Band 28”, KDDI.

[2]	R4-141897, “Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (18 + 28)”, Alcatel-Lucent, KDDI.

[3]	R4-141475, “TR 36.851 V0.10.0: Rel-12 Inter-band Carrier Aggregation”, Ericsson.