3GPP TSG-RAN WG4 (Radio) Meeting #70bis	R4-141932

San Jose Del Cabo, Mexico, 31 March – 4 April 2014

Agenda Item:	7.42.2

Source: 	Alcatel-Lucent, AT&T

Title: 	TP for TR 36.853: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (2 + 12 + 12)

Document for:	Approval

# 1.	Introduction

The revised WI proposal to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 12 and Band 12 was approved in RAN#62 [1]. One objective of the WI is to specify the band-combination specific Radio Frequency (RF) requirements for inter-band Carrier Aggregation (CA) of Band 2, Band 12 and Band 12 with 1 Uplink (UL) active in one of these bands.

The impacts of harmonics and IMD products caused by LTE-A BS supporting 3DL CA of Band (2 + 12 + 12) to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to record the findings in the Technical Report for 3 Band Carrier Aggregation with Single Uplink [3].

# 2.	Text Proposal

<New section>

##### 6.x.1	Co-existence studies for 1 UL/3 DL

The 3rd order IMD products caused in the BS by transmitting of Band 2, Band 12 and Band 12 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.x.1-1 below:

Table 6.x.1-1: Band 2, Band 12 and Band 12 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 729 | 746 | 1930 | 1990 |
| 3rd order IMD products | (f3-low – f1-high – f2-high) |  |  | (f3-high – f1-low – f2-low) |  |  |
| IMD frequency limits (MHz) | 438 |  |  | 532 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 1913 |  |  | 2007 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 1913 |  |  | 2007 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 3388 |  |  | 3482 |  |  |

It can be seen from Table 6.x.1-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (2 + 12 + 12) may fall into the BS receive band of Bands 1, 22, 23, 25, 31, 33, 36, 37, 39 and 42. However, Bands 1, 31, 33 and 39 are not intended for use in the same geographical area as Bands 2 and 12. Moreover, co-location of Band (2 + 12 + 12) transmitter and Band 36 or 37 transceiver implies FDD/TDD co-location on adjacent frequencies which requires the use of certain site-engineering solutions to avoid mutual interference. Therefore, the focus here is on the 3rd order IMD products falling into Bands 22, 23, 25 and 42.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 22 or 42 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (2 + 12 + 12) BS transmitter does not share the same antenna with Band 22 or 42 BS receiver. But there is not a large frequency gap between Band 2 DL and Bands 23 and 25 UL, and hence Band 23 or 25 BS receiver desensitization may still be an issue.

Therefore, it is recommended that Band (2 + 12 + 12) BS transmitter should not share the same antenna with Band 22, 23, 25 or 42 BS receiver to prevent BS receiver desensitization, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 22, 23, 25 or 42 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of section>

# References

[1]	RP-131654, “New WI proposal: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 12 and Band 12”, AT&T.

[2]	R4-141928, “Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (2 + 12 + 12)”, Alcatel-Lucent, AT&T.

[3]	R4-137075, “Draft New Technical Report 'LTE Advanced 3 Band Carrier Aggregation (3DL/1UL)'”, AT&T.