3GPP TSG-RAN WG4 (Radio) Meeting #70bis	R4-141943

San Jose Del Cabo, Mexico, 31 March – 4 April 2014

Agenda Item:	7.42.2

Source: 	Alcatel-Lucent, AT&T

Title: 	TP for TR 36.851: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (4 + 12)

Document for:	Approval

# 1.	Introduction

The WI proposal to support the Additional bandwidth combinations for LTE Advanced inter-band Carrier Aggregation to support certain 3DL fallback was approved in RAN#63 [1]. One objective of the WI is to specify the band-combinations specific Radio Frequency (RF) requirements for the new bandwidth combinations for CA_4A-12A.

The impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting carrier aggregation of this new bandwidth combinations to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to record the findings in the Inter-band Carrier Aggregation Technical Report [3].

# 2.	Text Proposal

<Start of change>

# 1	Scope

The present document is a technical report for Inter-band Carrier Aggregation under Rel-12 time frame. The purpose is to gather the relevant background information and studies in order to address Inter-band Carrier Aggregation requirements.

This TR covers relevant background information and studies in order to address Inter-band Carrier Aggregation requirements for the Rel-12 band combinations in table 1-1.

Table 1-1: Release 12 inter-band carrier aggregation combinations

| WI code | WI title | Class |
| --- | --- | --- |
| LTE_CA_B1_B7 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 7 | A3 |
| LTE_CA_B2_B4 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 4 | A4 |
| LTE_CA_B23_B29 | LTE Advanced inter-band Carrier Aggregation of Band 23 and Band 29 | A1 |
| LTE_CA_B3_B26 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 26 | A1 |
| LTE_CA_B3_B19 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 19 | A1 |
| LTE_CA_B1_B8 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 8 | A1 |
| LTE_CA_B3_B28 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 28 | A1 |
| LTE_CA_B1_B26 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 26 | A1 |
| LTE_CA_B39_B41 | LTE Advanced inter-band Carrier Aggregation of Band 39 and Band 41 | A3 |
| LTE_CA_B2_B12 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 12 | A1 |
| LTE_CA_B2_B13 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 13 | A1 |
| LTE_CA_B19_B21 | LTE Advanced inter-band Carrier Aggregation of Band 19 and Band 21 | A5 |
| LTE_CA_B12_B25 | LTE Advanced inter-band Carrier Aggregation of Band 12 and Band 25 | A1 |
| LTE_CA_B2_B5 | LTE Advanced inter-band Carrier Aggregation of Band 2 and Band 5 | A1 |
| LTE_CA_B1_B18 | Additional bandwidth combination set for LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 18 | A1 |
| LTE_CA_B7_B28 | LTE Advanced inter-band Carrier Aggregation of Band 7 and Band 28 | A1 |
| LTE_CA_B5_B25 | LTE Advanced inter-band Carrier Aggregation of Band 5 and Band 25 | A1 |
| LTE_CA_B1_B11 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 11 | A5 |
| LTE_CA_B8_B11 | LTE Advanced inter-band Carrier Aggregation of Band 8 and Band 11 | A5 |
| LTE_CA_B5_B7 | LTE Advanced inter-band Carrier Aggregation of Band 5 and Band 7 | A1 |
| LTE_CA_B1_B3 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 3 | A3 |
| LTE_CA_B1_B28 | LTE Advanced inter-band Carrier Aggregation of Band 1 and Band 28 | A2 |
| LTE_CA_B4_B27 | LTE Advanced inter-band Carrier Aggregation of Band 4 and Band 27 | A1 |
| LTE_CA_B3_B27 | LTE Advanced inter-band Carrier Aggregation of Band 3 and Band 27 | A1 |
| LTE_CA_B8_B27 | LTE Advanced inter-band Carrier Aggregation of Band 8 and Band 27 | A3 |
| LTE_CA_3DL_FB_BW | Additional bandwidth combinations for LTE Advanced inter-band Carrier Aggregation to support 3DL fallback | - |

The scope of the report has been further extended [6] to cover the 3 band Carrier Aggregation combinations that result in single UL configuration. These combinations are listed in table 1-2

Table 1-2: Release 12 inter-band carrier aggregation combinations

| WI code | WI title | Class |
| --- | --- | --- |
| LTE_CA_B2_B5_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 5 and Band 30 | A1 |
| LTE_CA_B2_B29_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 29 and Band 30 | A1 |
| LTE_CA_B1_B5_B7 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 5 and Band 7 | A1 |
| LTE_CA_B2_B12_B30 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 12 and Band 30 | A1 |
| LTE_CA_B1_B3_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 3 and Band 20 | A1 |
| LTE_CA_B1_B7_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 1, Band 7 and Band 20 | A1 |
| LTE_CA_B7_B8_B20 | LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 7, Band 8 and Band 20 | A3 |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

<New section>

##### 6.1.X.1.2.1	Co-existence studies for 1 UL/2 DL

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of Band 4 and Band 12 DL carriers can be calculated as shown in Table 6.1.X.1.2.1-1 below:

Table 6.1.X.1.2.1-1: Band 4 and Band 12 DL harmonics and IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 729 | 746 | 2110 | 2155 |
| 2nd harmonics frequency limits (MHz) | 1458 | 1492 | 4220 | 4310 |
| 3rd harmonics frequency limits (MHz) | 2187 | 2238 | 6330 | 6465 |
| 2nd order IMD products | (f2-low – f1-high) | (f2-high – f1-low) | (f2-low + f1-low) | (f2-high + f1-high) |
| IMD frequency limits (MHz) | 1364 | 1426 | 2839 | 2901 |
| 3rd order IMD products | (f2-low – 2*f1-high) | (f2-high – 2*f1-low) | (2*f2-low – f1-high) | (2* f2-high – f1-low) |
| IMD frequency limits (MHz) | 618 | 697 | 3474 | 3581 |
| 3rd order IMD products | (2*f1-low + f2-low) | (2*f1-high + f2-high) | (2*f2-low + f1-low) | (2*f2-high + f1-high) |
| IMD frequency limits (MHz) | 3568 | 3647 | 4949 | 5056 |
| 3rd order IMD products | (f1-low – f2-high + f2-low) | (f1-high + f2-high – f2-low) | (f2-low – f1-high + f1-low) | (f2-high + f1-high – f1-low) |
| IMD frequency limits (MHz) | 684 | 791 | 2093 | 2172 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1-low – f2-BWmax) | (f1-high + f2- BWmax) | (f2-low – f1- BWmax) | (f2-high + f1- BWmax) |
| IMD frequency limits (MHz) | 709 | 766 | 2100 | 2165 |

It can be seen from Table 6.1.X.1.2.1-1 that the 2nd harmonics of BS transmitting in Band 12 may fall into the BS receive band of Band 21, while the 3rd IMD products caused by BS supporting CA of Band 4 and Band 12 may fall into the BS receive band of Bands 12, 13, 14, 17, 22, 28, 42, 43 and 44. Note that the calculation in Table 6.1.X.1.2.1-1 (except the last row) assumes the BS is transmitting with the whole 45 MHz DL frequency of Band 4 and the whole 17 MHz DL frequency of Band 12. If the BS is only transmitting an up to 20 MHz DL in Band 4 and an up to 10 MHz DL in Band 1 as stated in the WIDS, then the 3rd IMD products may only fall into the BS receive band of the Bands 12, 17, 22, 28, 42, 43 and 44 as shown in the last row in Table 6.1.X.1.2.1-1, and the 3rd IMD products will not fall into the BS own (up to 10 MHz) receive block within Band 12, and may only fall into the BS receive band of certain frequency range within Band 12 and Band 17 under the transmit configurations shown in Table 6.1.X.1.2.1-2 below.

Table 6.1.X.1.2.1-2: Band (4 + 12) BS transmit configurations with 3rd IMD within Bands 12 and 17 BS receive band

| Band 4 DL channel bandwidth (MHz) | Band 12 DL channel bandwidth (MHz) | Lower edge of Band 12 DL frequency block (MHz) | Lower edge of IMD frequency limits (MHz) |
| --- | --- | --- | --- |
| 15 | 5 or 10 | 729 – 730.9 | 714 – 715.9 |
| 20 | 5 or 10 | 729 – 735.9 | 709 – 715.9 |

It should be noted that Bands 21, 28 and 44 are not intended for use in the same geographical area as Bands 4 and 12. Therefore, the focus here will be on the IMD falling into Bands 12, 17, 22, 42 and 43.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 22, 42 or 43 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Bands 4 and 12 BS transmitters do not share the same antenna with Band 22, 42 or 43 BS receiver.

On the other hand, it is recommended that Bands 4 and 12 BS transmitters should not share the same antenna with Band 22, 42 or 43 BS receiver, or Band 12 or 17 BS receiver for the affected frequency ranges if the aforementioned BS transmit configurations are used, in order to prevent BS receiver desensitization, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 12, 17, 22, 42 or 43BS receiver desensitization.

<End of change>

# References

[1]	RP-140454, “New WI proposal: Additional bandwidth combinations for LTE Advanced inter-band Carrier Aggregation to support 3DL fallback”, Qualcomm Incorporated.

[2]	R4-141942, “Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (4 + 12)”, Alcatel-Lucent, AT&T.

[3]	R4-141475, “TR 36.851 V0.10.0: Rel-12 Inter-band Carrier Aggregation”, Ericsson.