3GPP TSG-RAN WG4 #70bis	R4-142049

San Jose Del Cabo, Mexico, 31 Mar - 4 Apr, 2014

Source: 	Ericsson

Title:	TM9 test for FeICIC

Agenda Item:	5.4

Document for:	Discussion

# Introduction

In RAN4#70 meeting, the group agreed to have TM9 test for FeICIC in TEI 11 [1]. The group had the following agreements:

- PDSCH demodulation test cases for TM9 FeICIC in Rel 11 spec will be introduced as part of TEI11

  - Both for FDD and TDD

- Reuse existing Rel11 FeICIC test case parameters for:

  - Noc levels, Timing and Frequency offsets, ABS pattern, etc.

  - Other options are not precluded.

The details about what tests shall be introduced and how to test are still open. In this contribution, we try to address our view on how to set up the TM9 test in FeICIC.

# Test case discussion for TM9

In the current specification, there are several test cases to cover TM9. One of TM9 test cases is about dual-layer spatial multiplexing, as shown in 8.3.1.2 of 36.101. The purpose of this test case is to verify that the UE correctly estimate SNR. For TM9 demodulation, the following principle has been agreed in RAN4:

The noise and interference estimation shall be based on DMRS, not based on CRS.

However, under the current FeICIC RAN4 test setup, if UE makes short cut and estimates the noise and interference based on CRS, UE can still pass the demodulation requirements, even show advantage to the UE who follows the above agreements in the test.

For example, in the current FeICIC RAN4 setup, one cell is with colliding CRS and one cell is with non-colliding CRS. In the ABS subframes, zero-power ABS is assumed. The received signal at the CRS RE is:

rcrs=h1s1+h2s2+n,

where h1 is the channel of serving cell, and h2 is the channel of aggressor cell with colliding CRS, and n is the white noise, s1 and s2 are the transmitted signal from the serving cell and the aggressor cell with colliding CRS, respectively. With ideal CRS-IC, the post-CRS-IC signal at the CRS RE is:

rcrspost_CRS_ic=h1s1+n

If the CRS-based noise/interference is used, the estimated noise and interference estimation is Rnn after CRS-IC is performed, where Rnn is the covariance matrix of the noise.

At the data REs, the received signal after CRS interference cancelation is:

rdata=h1s1+n

The experienced noise and interference for data is also Rnn, which is the same as that observed at CRS REs. Based on the above derivation, we can see that UE can get coincidentally correct covariance estimation for noise and interference when UE estimates the noise and interference covariance based on CRS. Further, for CRS-based noise and interference estimation, more samples can be used. Hence, it will lead to even better performance for the receiver whose noise and interference estimation is based on CRS. One example is shown in Figure 1.  In this example, most of parameters are reused from TM6 test case designed for FeICIC defined in 36.101 except the transmission mode. In this example, TM9 is used. The detail transmission parameters are listed in Appendix A. In the legend, “CN” stands for “the strongest cell is with colliding CRS and the second cell is with non-colliding CRS”. From Figure 1, we can see that CRS-based noise/interference estimation can achieve even better performance than DMRS-based noise/interference estimation scheme. With this setup, the UE with short cut can beat the UE with the right behaviour. It is not desirable from test setup point of view.

Figure 1: The performance for TM9 with the similar setup as TM6

In practical network when TM9 is deployed, if the UE estimates noise and interference based on CRS, the performance will be poorer. The noise and interference in practical network is much more complex than RAN4 setup. The noise and interference may come from the neighbour pico cells; it may come from the secondary strongest macro cells that are configured as normal subframes in the envisioned subframes; it may come from the cells with non-zero power ABS even the subframes are indicated as ABS subframes from neighbour cells via X2. In these scenarios, UE will get very poor performance in practical network even though UE can get pretty good performance in the RAN4 test case.

In summary, if TM9 reuses the similar setup as TM6 in FeICIC context, the UE with CRS-based noise and interference estimation will show advantage to the UE with DMRS-based noise and interference estimation. In other words, the UE with the right behaviour will be penalized. In order to have proper TM9 test, the following principle shall be followed:

- Proposal 1: For TM9 demodulation, DMRS-based noise and interference estimation shall NOT be penalized and the receiver with CRS-based noise and interference estimation can NOT show any advantage to that with DMRS-based noise and interference estimation in FeICIC context.

In order to fulfil the above principle, we can extend the current TM9 SNR test case designed for non-FeICIC scenarios into FeICIC scenario. We can have the following settings:

- Two aggressor cells are configured, one is with colliding CRS and one is non-colliding CRS

- For the cell with colliding CRS case, INR1=12 dB and for the cell with non-colliding CRS case, INR2=10 dB

- The time offset is [-3 1] us and the frequency offset is [-300 100] Hz for the two aggressor cells, respectively.

- The ABS pattern can be

11000000 11000000 11000000 11000000 11000000

- For the first aggressor cell, in the ABS subframes, PDSCH EPRE is x-dB lower than CRS EPRE, x can be:

[ 9dB   6dB  3dB]

For the second aggressor cell, in the ABS subframes, zero power ABS is configured.

- 2x2 ETU5 low can be used as the channel for all the cells (both aggressor cells and serving cells)

- Dual-layer spatial multiplexing + QPSK ½ can be used as the FRC channel.

Other details parameters are listed in Table 2. The FeICIC specific parameters are marked as “red”.

Based on the above setting, the Interference + Noise level imbalance between DM-RSs and CRS after CRS-IC, denoted as C,  is:

Clin=11+INR1linx|lin

With different PDSCH EPRE, the interference + noise level imbalances between DM-RS and CRS are listed in Table 1.

Table 1: The interference + noise level imbalance between DM-RSs and CRS after CRS-IC

| x | C |
| --- | --- |
| 9 | -4.76435 |
| 6 | -6.97323 |
| 3 | -9.51497 |

Based the above setup, link level simulation results are provided to compared the performance of CRS-based (wrong behavior) and DMRS-based (right behavior), as shown in Figure 2 ~ Figure 4.

Table 2: Test Parameters for Testing CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations

| # parameter |  | # Unit | # Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | # Cell 1 | # Cell 2 and Cell 3 |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 | Antenna ports 0 and 1 |
|  |  | dBm/15kHz | -98 (Note 2) | N/A |
|  |  | dBm/15kHz | -98 (Note 3) | N/A |
|  |  | dBm/15kHz | -93 (Note 4) | N/A |
| Cell ID |  |  | 0 | Cell 2: 126 Cell 3: 1 |
| CSI reference signals |  |  | Antenna ports 15,16 | NA |
| Beamforming model |  |  | Annex B.4.2 | NA |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | NA |
| CSI reference signal configuration |  |  | 8 | NA |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0000000000000000 | NA |
| at antenna port |  | dBm/15kHz | -98 | -98 |
|  |  |  | Reference Value in Table 8.3.1.2-2 | Cell2: 12dB Cell 3: 10 dB |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | Cell 2: 3 Cell 3: -1 |
| Frequency shift between Cells |  | Hz | N/A | Cell 2: 300 Cell 3: 100 |
| ABS pattern (Note 5) |  |  | N/A | 11000000 11000000 11000000 11000000 11000000 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) | NA |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | NA |
| Simultaneous transmission |  |  | No | NA |
| PDSCH transmission mode |  |  | 9 | One is with PDSCH transmission but with de-boost data and one is blanking |

Table 3: Minimum performance for CDM-multiplexed DM RS (FRC) with multiple CSI-RS configurations

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 10 MHz QPSK 1/2 | R.51 FDD | OP.1 FDD | ETU5 | 2x2 Low | 70 | TBD | 2-8 |

Figure 2: The performance of TM9 with 2-layer, QPSK ½, and strongest aggressor cell PDSCH with 9 dB de-boost

Figure 3: The performance of TM9 with 2-layer, QPSK ½, and strongest aggressor cell PDSCH with 6 dB de-boost

Figure 4: The performance of TM9 with 2-layer, QPSK ½, and strongest aggressor cell PDSCH with 3 dB de-boost

From Figure 2 ~ Figure 4, we can see the performance of UE with wrong behavior is much worse than that with right behavior with this setup. Hence, it is more valuable to have TM9 with the above setup.

- Proposal 2: Parameters listed in Table 1 can be set as TM9 test setup in FeICIC and interested companies are encouraged to provide link level simulation results based on the simulation assumption listed in Table 2 and Table 3 in the next meeting.

# Conclusion

In this paper, we give our view on the TM9 test in the FeICIC context. We have the following proposals:

- Proposal 1: For TM9 demodulation, DMRS-based noise and interference estimation shall NOT be penalized and the receiver with CRS-based noise and interference estimation can NOT show any advantage to that with DMRS-based noise and interference estimation in FeICIC context.

- Proposal 2: Parameters listed in Table 1 can be set as TM9 test setup in FeICIC and interested companies are encouraged to provide link level simulation results based on the simulation assumption listed in Table 2 and Table 3 in the next meeting.

# Appendix A

Table 4: Parameters for Testing CDM-multiplexed DM RS (dual layer) with multiple CSI-RS configurations

| # parameter |  | # Unit | # Test 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | # Cell 1 | # Cell 2 and Cell 3 |
| Downlink power allocation |  | dB | 0 | 0 |
|  |  | dB | 0 (Note 1) | 0 |
|  |  | dB | -3 | -3 |
| Cell-specific reference signals |  |  | Antenna ports 0 and 1 | Antenna ports 0 and 1 |
|  |  | dBm/15kHz | -98 (Note 2) | N/A |
|  |  | dBm/15kHz | -98 (Note 3) | N/A |
|  |  | dBm/15kHz | -93 (Note 4) | N/A |
| Cell ID |  |  | 0 | Cell 2: 126 Cell 3: 1 |
| CSI reference signals |  |  | Antenna ports 15,16 | NA |
| Beamforming model |  |  | Annex B.4.2 | NA |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  | Subframes | 5 / 2 | NA |
| CSI reference signal configuration |  |  | 8 | NA |
| Zero-power CSI-RS configuration ICSI-RS / ZeroPowerCSI-RS bitmap |  | Subframes / bitmap | 3 / 0000000000000000 | NA |
| at antenna port |  | dBm/15kHz | -98 | -98 |
|  |  |  | Reference Value in Table 8.3.1.2-2 | Cell2: 12dB Cell 3: 10 dB |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Time Offset between Cells |  | s | N/A | Cell 2: 3 Cell 3: -1 |
| Frequency shift between Cells |  | Hz | N/A | Cell 2: 300 Cell 3: 100 |
| ABS pattern (Note 5) |  |  | N/A | 11000000 11000000 11000000 11000000 11000000 |
| Symbols for unused PRBs |  |  | OCNG (Note 2) | NA |
| Number of allocated resource blocks (Note 2) |  | PRB | 50 | NA |
| Simultaneous transmission |  |  | No | NA |
| PDSCH transmission mode |  |  | 9 | blank |
| Note 1: Note 2: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PDSCH per virtual UE; the data transmitted over the OCNG PDSCHs shall be uncorrelated pseudo random data, which is QPSK modulated. |  |  |  |  |

Table 5: Minimum Performance Single-Layer Spatial Multiplexing (FRC)– Non-MBSFN ABS

| Test Number | Reference Channel | OCNG Pattern |  |  | Propagation Conditions (Note1) |  |  | Correlation Matrix and Antenna Configuration (Note 2) | Reference Value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Cell 1 | Cell 2 | Cell 3 | Cell 1 | Cell 2 | Cell 3 |  | Fraction of Maximum Throughput (%) | SNR (dB) (Note 3) |  |
| 1 | R.51 FDD | OP.1FDD | OP.1FDD | OP.1FDD | EPA5 | EPA5 | EPA5 | 2x2 low | 70 | [6.1] | 2-8 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: The correlation matrix and antenna configuration apply for Cell 1, Cell 2 and Cell 3. Note 3: SNR corresponds to of cell 1. |  |  |  |  |  |  |  |  |  |  |  |

# Reference

R4-141091, Way Forward on FeICIC TM9 Test Case, Qualcomm, Intel, Ericsson

R4-135773, Way forward on SNR estimation for TM9, Ericsson, Samsung