3GPP TSG-RAN WG4 RAN4 #70bis                                    R4 -142361

San Jose Del Cabo, Mexico, 31st Mar – 4th Apr, 2014

Agenda item:	7.5.1

Source: 	Samsung, Alcatel-Lucent, Huawei, ZTE, Qualcomm

Title: 			  Simulation assumptions for eDL-MIMO single PMI test

Document for:	   Approval

1	Introduction

During FeDL-MIMO ad hoc, there were discussions on the PMI test for 4Tx enhanced codebook. Regarding single PMI test case, such agreements were reached:

Task #1: To down select from PUCCH 1-1 submode 1, 2 (rank 1) and PUCCH 2-1 (rank 2) for single PMI tests

- PUCCH 1-1 submode 1, EPA 5Hz, XP High with Beam-S (Model given in R4-141077)

- [QPSK 1/2 or 16 QAM ½]

In this paper, test parameters were listed as baseline for further discussion and simulation evaluation by capturing companies’ proposals.

- Simulation assumption for single PMI test

The requirement for transmission mode 9 with R.12 4Tx codebook are specified in terms of the ratio

In the definition of γ, for single PMI test is [TBD] of the maximum throughput obtained at  using the precoders configured according to the UE reports, and is the throughput measured at with random precoding. For random precoding, random wideband W1 and random wideband W2 is selected in each subframe.

2.1 Simulation assumption for FDD

Table 1 Test parameters for single PMI test of PUCCH1-1 (FDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Propagation channel |  |  | EPA5 |
| PDSCH PRBs |  |  | 50 |
| Precoding granularity |  | PRB | 50 |
| Correlation and antenna configuration |  |  | XP High 4 x 2 |
| Beamforming model |  |  | Annex B.4.3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ICSI-RS |  |  | 5/ 1 |
| CSI-RS reference signal configuration |  |  | 6 |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 FFFF 0000 FFFF |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUCCH 1-1 submode1 |
| Reporting periodicity |  | ms | Npd = 5 |
| PMI delay (Note 2) |  | ms | TBD |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |
| PUCCH Report Type for CQI/second PMI |  |  | 2b |
| Physical channel for RI reporting |  |  | PUSCH |
| PUCCH Report Type for RI/ first PMI |  |  | 5 |
| cqi-pmi-ConfigurationIndex |  |  | [4] |
| ri-ConfigIndex |  |  | [1] |
| Reporting periodicity |  | ms | Npd = 5 |
| Modulation and Coded rate |  |  | Option1: QPSK 1/2 Option2: 16QAM 1/2 |
| OCNG Pattern |  |  | OP.1 FDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity) Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. Note 4: PDSCH _RA= 0 dB, PDSCH_RB= 0 dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver Note 5: Only subframes 1,2,3,4,6,7,8, and 9 are allocated to avoid PBCH and synchronization signal overhead Note 6: Randomization of the principle beam direction shall be used |  |  |  |

2.2 Simulation assumption for TDD

Table 2 Test parameters for single PMI test of PUCCH1-1 (TDD)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Transmission mode |  |  | 9 |
| Uplink downlink configuration |  |  | 1 |
| Special subframe configuration |  |  | 4 |
| Propagation channel |  |  | EPA5 |
| PDSCH PRBs |  |  | 50 |
| Precoding granularity |  | PRB | 50 |
| Correlation and antenna configuration |  |  | XP High 4 x 2 |
| Beamforming model |  |  | Annex B.4.3 |
| Cell-specific reference signals |  |  | Antenna ports 0,1 |
| CSI reference signals |  |  | Antenna ports 15,…,18 |
| CSI-RS periodicity and subframe offset TCSI-RS / ICSI-RS |  |  | 5/ 4 |
| CSI-RS reference signal configuration |  |  | 6 |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 FFFF 0000 FFFF |
| Downlink power allocation |  | dB | 0 |
|  |  | dB | 0 |
|  | Pc | dB | -3 |
|  |  | dB[mW/15kHz] | -98 |
| Reporting mode |  |  | PUCCH 1-1 submode1 |
| Reporting periodicity |  | ms | Npd = 5 |
| PMI delay (Note 2) |  | ms | TBD |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note 3) |
| PUCCH Report Type for CQI/second PMI |  |  | 2b |
| Physical channel for RI reporting |  |  | PUSCH |
| PUCCH Report Type for RI/ first PMI |  |  | 5 |
| cqi-pmi-ConfigurationIndex |  |  | [4] |
| ri-ConfigIndex |  |  | [1] |
| Reporting periodicity |  | ms | Npd = 5 |
| Modulation and Coded rate |  |  | Option1: QPSK 1/2 Option2: 16QAM 1/2 |
| OCNG Pattern |  |  | OP.1 TDD |
| Max number of HARQ transmissions |  |  | 4 |
| Redundancy version coding sequence |  |  | {0,1,2,3} |
| ACK/NACK feedback mode |  |  | Multiplexing |
| Note 1: For random precoder selection, the precoder shall be updated in each TTI (1 ms granularity) Note 2: If the UE reports in an available uplink reporting instance at subrame SF#n based on PMI estimation at a downlink SF not later than SF#(n-4), this reported PMI cannot be applied at the eNB downlink before SF#(n+4). Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. Note 4: PDSCH _RA= 0 dB, PDSCH_RB= 0 dB in order to have the same PDSCH and OCNG power per subcarrier at the receiver Note 5: Only subframes 4 and 9 are allocated to avoid PBCH and synchronization signal overhead. Note 6: Randomization of the principle beam direction shall be used |  |  |  |