3GPP TSG-RAN WG4 Meeting #70bis 	R4-142384

# San Jose Del Cabo, Mexico, 31 Mar - 4 Apr 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.133 | CR | 1342 | rev | 1 | Current version: | 12.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE RRM performance requirements for UTRA Band I + SDL 1452 - 1496 MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_UTRA_SDL_bandL-Core |  |  |  |  |  |  | Date: |  | 2014-03-31 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Measurement performance requirements for L-band needs to be introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Measurement performance requirements for L-band are introduced with respect to the agreed REFSENS for L-band. Note has been added to clarify that measurements performance requirements for L-band are applicable only with dual band configuration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Measurement performance requirements for L-band are missing. Isolated impact analysis Not needed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1, 9.1.1, 9.1.2, 9.1.3, 9.1.7, 9.1.8, B.3.1, B.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS/TR 34.121-1 CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | UE Rx-Tx time difference requirements are not specified for L-band since L-band is being introduced as a supplemental DL band. |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

# 9	Measurements Performance Requirements

One of the key services provided by the physical layer is the measurement of various quantities which are used to trigger or perform a multitude of functions. Both the UE and the UTRAN are required to perform a variety of measurements. The physical layer measurement model and a complete list of measurements is specified in TS 25.302 "Services Provided by Physical Layer". The physical layer measurements for FDD are described and defined in TS25.215 "Physical layer - Measurements (FDD)". In this clause for each measurement the relevant requirements on the measurement period, reporting range, granularity and performance in terms of accuracy are specified.

Since the UE reference sensitivity requirements are different depending on supported band, this is noted in each case with definition of the range Io for each frequency band. Definitions of each frequency bands can be found in TS 25.101.

The accuracy requirements in this clause are applicable for AWGN radio propagation conditions.

## 9.1	Measurement Performance for UE

The requirements in this clause are applicable for a UE:

-	in state CELL_DCH and/or state CELL_FACH.

-	performing measurements according to section 8.

-	that is synchronised to the cell that is measured.

The reported measurement result after layer 1 filtering shall be an estimate of the average value of the measured quantity over the measurement period. The reference point for the measurement result after layer 1 filtering is referred to as point B in the measurement model described in TS25.302.

The accuracy requirements in this clause are valid for the reported measurement result after layer 1 filtering. The accuracy requirements are verified from the measurement report at point D in the measurement model having the layer 3 filtering disabled.

NOTE:	It needs to be clarified how the accuracy requirements shall be handled when the UE is measuring on cells using IPDL.

NOTE:	For the UE which supports both Band III and Band IX operating frequencies, the measurement performance requirements for Band III shall apply to the multi-band UE.

NOTE:	The requirements for Band XXXII are applicable only with dual band configuration (e.g., DB-DC-HSDPA or dual band 4C-HSDPA).

### 9.1.1	CPICH RSCP

NOTE:	This measurement is for handover evaluation, DL open loop power control, UL open loop power control and for the calculation of pathloss.

#### 9.1.1.1	Intra frequency measurements accuracy

The measurement period for CELL_DCH state can be found in sub clause 8.1.2.2. The measurement period for CELL_FACH state can be found in sub clause 8.4.2.2.

##### 9.1.1.1.1	Absolute accuracy requirement

The accuracy requirements in table 9.1 are valid under the following conditions:

CPICH_RSCP1|dBm.according to Annex B.3.1 for a corresponding Band

# Table 9.1: CPICH_RSCP Intra frequency absolute accuracy

| Parameter | Accuracy [dB] |  | Conditions |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Normal condition | Extreme condition | Operating bands | Io [dBm/3,84MHz] |  |
|  |  |  |  | Minimum Io | Maximum Io |
| CPICH_RSCP | 6 | 9 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -70 |
|  |  |  | II, V and VII | -92 | -70 |
|  |  |  | XXV and XXVI | -90.5 (Note 1) | -70 |
|  |  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -70 |
|  |  |  | IX | -93 | -70 |
|  | 8 | 11 | Note 2 | -70 | -50 |
| NOTE 1: The minimum Io is -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. NOTE 2: The same bands apply for this requirement as for the corresponding highest accuracy requirement.. |  |  |  |  |  |

##### 9.1.1.1.2	Relative accuracy requirement

The relative accuracy of CPICH RSCP is defined as the CPICH RSCP measured from one cell compared to the CPICH RSCP measured from another cell on the same frequency

The accuracy requirements in table 9.2 are valid under the following conditions:

CPICH_RSCP1,2|dBm according to Annex B.3.2 for a corresponding Band.

# Table 9.2: CPICH_RSCP Intra frequency relative accuracy

| Parameter | Accuracy [dB] |  | Conditions |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Normal condition | Extreme condition | Operating bands | Io [dBm/3,84 [MHz] |  |
|  |  |  |  | Minimum Io | Maximum Io |
| CPICH_RSCP | 3 | 3 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -50 |
|  |  |  | II, V and VII | -92 | -50 |
|  |  |  | XXV and XXVI | -90.5 (Note 1) | -50 |
|  |  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -50 |
|  |  |  | IX | -93 | -50 |
| NOTE 1: The minimum Io is -90.5 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |  |

#### 9.1.1.2	Inter frequency measurement accuracy

The measurement period for CELL_DCH state can be found in sub clause 8.1.2.3. The measurement period for CELL_FACH state can be found in sub clause 8.4.2.3.

##### 9.1.1.2.1	Relative accuracy requirement

The relative accuracy of CPICH RSCP in inter frequency case is defined as the CPICH RSCP measured from one cell compared to the CPICH RSCP measured from another cell on a different frequency.

The accuracy requirements in table 9.3 are valid under the following conditions:

CPICH_RSCP1|dBm according to Annex B.3.3 for a corresponding Band

| Channel 1_Io|dBm/3,84 MHz -Channel 2_Io|dBm/3,84 MHz |  20 dB.

Table 9.3: CPICH_RSCP Inter frequency relative accuracy

| Parameter | Accuracy [dB] |  | Conditions |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Normal condition | Extreme condition | CPICH_RSCP is on Band | Io [dBm/3,84 [MHz] |  |
|  |  |  |  | Minimum Io | Maximum Io |
| CPICH_RSCP | 6 | 6 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -50 |
|  |  |  | II, V and VII | -92 | -50 |
|  |  |  | XXV and XXVI | -90.5 (Note 1) | -50 |
|  |  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -50 |
|  |  |  | IX | -93 | -50 |
| NOTE 1: The minimum Io is -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |  |

#### 9.1.1.3	CPICH RSCP measurement report mapping

The reporting range is for CPICH RSCP is from -120 dBm ...-25 dBm.

In table 9.4 the mapping of measured quantity is defined. The range in the signalling may be larger than the guaranteed accuracy range.

Table 9.4

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CPICH_RSCP_LEV _-05 | CPICH RSCP <-120 | dBm |
| CPICH_RSCP_LEV _-04 | -120 CPICH RSCP < -119 | dBm |
| CPICH_RSCP_LEV _-03 | -119 CPICH RSCP < -118 | dBm |
| … | … | … |
| CPICH_RSCP_LEV _89 | -27 CPICH RSCP < -26 | dBm |
| CPICH_RSCP_LEV _90 | -26 CPICH RSCP < -25 | dBm |
| CPICH_RSCP_LEV _91 | -25 CPICH RSCP | dBm |

### 9.1.2	CPICH Ec/Io

NOTE:	This measurement is for Cell selection/re-selection and for handover evaluation.

#### 9.1.2.1	Intra frequency measurements accuracy

The measurement period for CELL_DCH state can be found in sub clause 8.1.2.2. The measurement period for CELL_FACH state can be found in sub clause 8.4.2.2.

##### 9.1.2.1.1	Absolute accuracy requirement

The accuracy requirements in table 9.5 are valid under the following conditions:

CPICH_RSCP1|dBm according to Annex B.3.4 for a corresponding Band

Table 9.5: CPICH_Ec/Io Intra frequency absolute accuracy

| Parameter | Accuracy [dB] |  | Conditions |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Normal condition | Extreme condition | Operating bands | Io [dBm/3,84 [MHz] |  |
|  |  |  |  | Minimum Io | Maximum Io |
| CPICH_Ec/Io | 1.5 for -14 CPICH Ec/Io 2 for -16 CPICH Ec/Io < -14 3 for -20 CPICH Ec/Io < -16 | 3 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -50 |
|  |  |  | II, V and VII | -92 | -50 |
|  |  |  | XXV and XXVI | -90.5 (Note 1) | -50 |
|  |  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -50 |
|  |  |  | IX | -93 | -50 |
| NOTE 1: The minimum Io is -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |  |

##### 9.1.2.1.2	Relative accuracy requirement

The relative accuracy of CPICH Ec/Io is defined as the maximum allowable error in the measured difference between the CPICH Ec/Io measured from one cell compared to the CPICH Ec/Io measured from another cell on the same frequency.

The relative accuracy is defined using the lower CPICH_Ec/Io of cell 1 and cell 2.

The accuracy requirements in table 9.6 are valid under the following conditions:

CPICH_RSCP1,2|dBm according to Annex B.3.5 for a corresponding Band

Table 9.6: CPICH_Ec/Io Intra frequency relative accuracy

| Parameter | Accuracy [dB] |  | Conditions |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Normal condition | Extreme condition | Operating bands | Io [dBm/3,84 [MHz] |  |
|  |  |  |  | Minimum Io | Maximum Io |
| The lower of the CPICH_Ec/Io from cell1 and cell2 | 1.5 for -14 CPICH Ec/Io 2 for -16 CPICH Ec/Io < -14 3 for -20 CPICH Ec/Io < -16 | 3 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -50 |
|  |  |  | II, V and VII | -92 | -50 |
|  |  |  | XXV and XXVI | -90.5 (Note 1) | -50 |
|  |  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -50 |
|  |  |  | IX | -93 | -50 |
| NOTE 1: The minimum Io is -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |  |

#### 9.1.2.2	Inter frequency measurement accuracy

The measurement period for CELL_DCH state can be found in sub clause 8.1.2.3. The measurement period for CELL_FACH state can be found in sub clause 8.4.2.3.

##### 9.1.2.2.1	Absolute accuracy requirement

The accuracy requirements in table 9.7 are valid under the following conditions:

CPICH_RSCP1|dBm according to Annex B.3.6 for a corresponding Band

Table 9.7: CPICH_Ec/Io Inter frequency absolute accuracy

| Parameter | Accuracy [dB] |  | Conditions |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Normal condition | Extreme condition | Operating bands | Io [dBm/3,84 [MHz] |  |
|  |  |  |  | Minimum Io | Maximum Io |
| CPICH_Ec/Io | 1.5 for -14 CPICH Ec/Io 2 for -16 CPICH Ec/Io < -14 3 for -20 CPICH Ec/Io < -16 | 3 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -50 |
|  |  |  | II, V and VII | -92 | -50 |
|  |  |  | XXV and XXVI | -90.5 (Note 1) | -50 |
|  |  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -50 |
|  |  |  | IX | -93 | -50 |
| NOTE 1: The minimum Io is -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |  |

##### 9.1.2.2.2	Relative accuracy requirement

The relative accuracy of CPICH Ec/Io in the inter frequency case is defined as the maximum allowable error in the measured difference between the CPICH Ec/Io measured from one cell compared to the CPICH Ec/Io measured from another cell on a different frequency

The relative accuracy is defined using the lower CPICH_Ec/Io of cell 1 and cell 2.

The accuracy requirements in table 9.8 are valid under the following conditions:

CPICH_RSCP1|dBm according to Annex B.3.7 for a corresponding Band

| Channel 1_Io|dBm/3,84 MHz -Channel 2_Io|dBm/3,84 MHz |  20 dB.

# Table 9.8: CPICH_Ec/Io Inter frequency relative accuracy

| Parameter | Accuracy [dB] |  | Conditions |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Normal condition | Extreme condition | CPICH_Ec/Io is on Band | Io [dBm/3,84 [MHz] |  |
|  |  |  |  | Minimum Io | Maximum Io |
| The lower of the CPICH_Ec/Io from cell1 and cell2 | 1.5 for -14 CPICH Ec/Io 2 for -16 CPICH Ec/Io < -14 3 for -20 CPICH Ec/Io < -16 | 3 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -50 |
|  |  |  | II, V and VII | -92 | -50 |
|  |  |  | XXV and XXVI | -90.5 (Note 1) | -50 |
|  |  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -50 |
|  |  |  | IX | -93 | -50 |
| NOTE 1: The minimum condition is -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |  |

#### 9.1.2.3	CPICH Ec/Io measurement report mapping

The reporting range is for CPICH Ec/Io is from -24 ...0 dB.

In table 9.9 the mapping of measured quantity is defined. The range in the signalling may be larger than the guaranteed accuracy range.

Table 9.9

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| CPICH_Ec/No _00 | CPICH Ec/Io < -24 | dB |
| CPICH_Ec/No _01 | -24 CPICH Ec/Io < -23.5 | dB |
| CPICH_Ec/No _02 | -23.5 CPICH Ec/Io < -23 | dB |
| … | … | … |
| CPICH_Ec/No _47 | -1 CPICH Ec/Io < -0.5 | dB |
| CPICH_Ec/No _48 | -0.5 CPICH Ec/Io < 0 | dB |
| CPICH_Ec/No _49 | 0 CPICH Ec/Io | dB |

### 9.1.3	UTRA Carrier RSSI

NOTE:	This measurement is for Inter-frequency handover evaluation.

The measurement period is equal to the measurement period for UE CPICH measurements, For CELL_DCH state the measurement period can be found in sub clause 8.1.2.2 for intra frequency measurements and in sub clause 8.1.2.3 for inter frequency measurements.

#### 9.1.3.1	Absolute accuracy requirement

Table 9.10: UTRA Carrier RSSI Inter frequency absolute accuracy

| Parameter | Accuracy [dB] |  | Conditions |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Normal condition | Extreme condition | Operating bands | Io [dBm/3,84MHz] |  |
|  |  |  |  | Minimum Io | Maximum Io |
| UTRA Carrier RSSI | 4 | 7 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -70 |
|  |  |  | II, V and VII | -92 | -70 |
|  |  |  | XXV and XXVI | -90.5 (Note 1) | -70 |
|  |  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -70 |
|  |  |  | IX | -93 | -70 |
|  | 6 | 9 | Note 2 | -70 | -50 |
| NOTE 1: The minimum condition is -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. NOTE 2: The same bands apply for this requirement as for the corresponding highest accuracy requirement.. |  |  |  |  |  |

#### 9.1.3.2	Relative accuracy requirement

The relative accuracy requirement is defined as the UTRA carrier RSSI measured from one frequency compared to the UTRA carrier RSSI measured from another frequency.

The accuracy requirements in table 9.11 are valid under the following condition:

| Channel 1_Io|dBm -Channel 2_Io|dBm | < 20 dB.

Table 9.11: UTRA Carrier RSSI Inter frequency relative accuracy

| Parameter | Accuracy [dB] |  | Conditions |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Normal condition | Extreme condition | UTRA Carrier RSSI is on Band | Io [dBm/3,84 [MHz] |  |
|  |  |  |  | Minimum Io | Maximum Io |
| UTRA Carrier RSSI | 7 | 11 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -50 |
|  |  |  | II, V and VII | -92 | -50 |
|  |  |  | XXV and XXVI | -90.5 (Note 1) | -50 |
|  |  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -50 |
|  |  |  | IX | -93 | -50 |
| NOTE 1: The minimum condition is -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |  |

#### 9.1.3.3	UTRA Carrier RSSI measurement report mapping

The reporting range for UTRA carrier RSSI is from -100 ...-25 dBm.

In table 9.12 the mapping of measured quantity is defined. The range in the signalling may be larger than the guaranteed accuracy range.

Table 9.12

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| UTRA_carrier_RSSI_LEV _00 | UTRA carrier RSSI < -100 | dBm |
| UTRA_carrier_RSSI_LEV _01 | -100 UTRA carrier RSSI < -99 | dBm |
| UTRA_carrier_RSSI_LEV _02 | -99 UTRA carrier RSSI < -98 | dBm |
| … | … | … |
| UTRA_carrier_RSSI_LEV _74 | -27 UTRA carrier RSSI < -26 | dBm |
| UTRA_carrier_RSSI_LEV _75 | -26 UTRA carrier RSSI < -25 | dBm |
| UTRA_carrier_RSSI_LEV _76 | -25 UTRA carrier RSSI | dBm |

------------------------------------------------------ Unchanged text omitted ------------------------------------------------------------

### 9.1.7	SFN-CFN observed time difference

NOTE: 	This measurement is for handover timing purposes to identify active cell and neighbour cell time difference.

#### 9.1.7.1	Intra frequency measurement requirement

The measurement period for CELL_DCH state can be found in sub clause 8.1.2.2.

The accuracy requirement in table 9.16 is valid under the following conditions:

CPICH_RSCP1,2|dBm according to Annex B.3.8 for a corresponding Band

is low enough to ensure successful SFN decoding.

Table 9.16

| Parameter | Accuracy [chip] | Conditions |  |  |
| --- | --- | --- | --- | --- |
|  |  | Operating bands | Io [dBm/3,84 [MHz] |  |
|  |  |  | Minimum Io | Maximum Io |
| SFN-CFN observed time difference | 1 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -50 |
|  |  | II, V and VII | -92 | -50 |
|  |  | XXV and XXVI | -90.5 (Note 1) | -50 |
|  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -50 |
|  |  | IX | -93 | -50 |
| NOTE 1: The minimum condition is -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |

#### 9.1.7.2	Inter frequency measurement requirement

The measurement period for CELL_DCH state can be found in sub clause 8.1.2.3.

The accuracy requirement in table 9.17 is valid under the following conditions:

CPICH_RSCP1|dBm according to Annex B.3.9 for a corresponding Band.

,

| Channel 1_Io|dBm/3,84 MHz -Channel 2_Io|dBm/3,84 MHz |  20 dB,

.

Table 9.17

| Parameter | Accuracy [chip] | Conditions |  |  |
| --- | --- | --- | --- | --- |
|  |  | SFN-CFN observed time difference is on Band | Io [dBm/3,84 [MHz] |  |
|  |  |  | Minimum Io | Maximum Io |
| SFN-CFN observed time difference | 1 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -50 |
|  |  | II, V and VII | -92 | -50 |
|  |  | XXV and XXVI | -90.5 (Note 1) | -50 |
|  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -50 |
|  |  | IX | -93 | -50 |
| NOTE 1: The minimum condition is -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |

#### 9.1.7.3	SFN-CFN observed time difference measurement report mapping

The reporting range is for CFN-SFN observed time difference is from 0 ... 9830400 chip.

In table 9.18 the mapping of measured quantity is defined. The range in the signalling may be larger than the guaranteed accuracy range.

Table 9.18

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SFN-CFN_TIME _0000000 | 0 SFN-CFN observed time difference < 1 | chip |
| SFN-CFN_TIME _0000001 | 1 SFN-CFN observed time difference < 2 | chip |
| SFN-CFN_TIME _0000002 | 2 SFN-CFN observed time difference < 3 | chip |
| … | … | … |
| SFN-CFN_TIME _9830397 | 9830397 SFN-CFN observed time difference < 9830398 | chip |
| SFN-CFN_TIME _9830398 | 9830398 SFN-CFN observed time difference < 980399 | chip |
| SFN-CFN_TIME _9830399 | 9830399 SFN-CFN observed time difference < 9830400 | chip |

### 9.1.8	SFN-SFN observed time difference

#### 9.1.8.1	SFN-SFN observed time difference type 1

NOTE: 	This measurement is for identifying time difference between two cells.

##### 9.1.8.1.1	Measurement requirement

The measurement period for CELL_FACH state can be found in sub clause 8.4.2.2.

The accuracy requirement in table 9.19 is valid under the following conditions:

CPICH_RSCP1|dBm according to Annex B.3.10 for a corresponding Band

,

,

is low enough to ensure successful SFN decoding.

Table 9.19

| Parameter | Accuracy [chip] | Conditions |  |  |
| --- | --- | --- | --- | --- |
|  |  | SFN-SFN observed time difference type 1 is on Band | Io [dBm/3,84 [MHz] |  |
|  |  |  | Minimum Io | Maximum Io |
| SFN-SFN observed time difference type1 | 1 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -50 |
|  |  | II, V and VII | -92 | -50 |
|  |  | XXV and XXVI | -90.5 (Note 1) | -50 |
|  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -50 |
|  |  | IX | -93 | -50 |
| NOTE 1: The minimum condition is -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |

##### 9.1.8.1.2	SFN-SFN observed time difference type 1 measurement report mapping

The reporting range is for SFN-SFN observed time difference type 1 is from 0 ... 9830400 chip.

In table 9.20 the mapping of measured quantity is defined. The range in the signalling may be larger than the guaranteed accuracy range.

Table 9.20

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| T1_SFN-SFN_TIME _0000000 | 0 SFN-SFN observed time difference type 1 < 1 | chip |
| T1_SFN-SFN_TIME _0000001 | 1 SFN-SFN observed time difference type 1 < 2 | chip |
| T1_SFN-SFN_TIME _0000002 | 2 SFN-SFN observed time difference type 1 < 3 | chip |
| … | … | … |
| T1_SFN-SFN_TIME _9830397 | 9830397 SFN-SFN observed time difference type 1 < 9830398 | chip |
| T1_SFN-SFN_TIME _9830398 | 9830398 SFN-SFN observed time difference type 1 < 980399 | chip |
| T1_SFN-SFN_TIME _9830399 | 9830399 SFN-SFN observed time difference type 1 < 9830400 | chip |

#### 9.1.8.2	SFN-SFN observed time difference type 2

NOTE:	This measurement is for location service purposes to identify time difference between two cells.

It is optional for terminal to support the use of IPDL periods together with SFN-SFN observed time difference type 2. The support of IPDL depends on the supported UE positioning methods.

NOTE:	Requirement on the UE shall be reconsidered when the state of the art technology progress.

##### 9.1.8.2.1	Intra frequency measurement requirement accuracy without IPDL period active

The measurement period for CELL_DCH state can be found in sub clause 8.1.2.2. The measurement period for CELL_FACH state can be found in sub clause 8.4.2.2.

The accuracy requirement in table 9.21 is valid under the following conditions:

CPICH_RSCP1,2|dBm according to Annex B.3.11 for a corresponding Band

Table 9.21

| Parameter | Accuracy [chip] | Conditions |  |  |
| --- | --- | --- | --- | --- |
|  |  | Operating bands | Io [dBm/3,84 [MHz] |  |
|  |  |  | Minimum Io | Maximum Io |
| SFN-SFN observed time difference type 2 | 0.5 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -50 |
|  |  | II, V and VII | -92 | -50 |
|  |  | XXV and XXVI | -90.5 (Note 1) | -50 |
|  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -50 |
|  |  | IX | -93 | -50 |
| NOTE 1: The minimum condition is -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |

##### 9.1.8.2.2	Intra frequency measurement requirement accuracy with IPDL period active

This requirement is valid only for UEs supporting IPDL measurements.

The measurement period for CELL_DCH state can be found in sub clause 8.1.2.2. The measurement period for CELL_FACH state can be found in sub clause 8.4.2.2.

The accuracy requirement in table 9.22 is valid under the following conditions:

CPICH_RSCP1,2|dBm according to Annex B.3.11 for a corresponding Band

Additionally the accuracy requirement in table 9.22 is also valid for neighbour cells for which the following conditions apply to during idle periods provided idle periods have a length of 1 slot:

CPICH_RSCPx,y|dBm  -114 dBm.

,

where x and y represent cells measured using idle periods and Io_idle-period is the total received power during the idle period.

NOTE:	Additional general conditions are needed for the requirements in table 9.22 to be valid.

Table 9.22

| Parameter | Accuracy [chip] | Conditions |  |  |
| --- | --- | --- | --- | --- |
|  |  | Operating bands | Io [dBm/3,84 [MHz] |  |
|  |  |  | Minimum Io | Maximum Io |
| SFN-SFN observed time difference type 2 | 0.5 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -50 |
|  |  | II, V and VII | -92 | -50 |
|  |  | XXV and XXVI | -90.5 (Note 1) | -50 |
|  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -50 |
|  |  | IX | -93 | -50 |
| NOTE 1: The minimum condition is -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |

##### 9.1.8.2.3	Inter frequency measurement requirement accuracy

The measurement period for CELL_DCH state can be found in sub clause 8.1.2.3. The measurement period for CELL_FACH state can be found in sub clause 8.4.2.3.

The accuracy requirement in table 9.23 is valid under the following conditions:

CPICH_RSCP1|dBm according to Annex B.3.12 for a corresponding Band

| Channel 1_Io|dBm -Channel 2_Io|dBm |  20 dB,

.

Table 9.23

| Parameter | Accuracy [chip] | Conditions |  |  |
| --- | --- | --- | --- | --- |
|  |  | SFN-SFN observed time difference type 2 is on Band | Io [dBm/3,84 [MHz] |  |
|  |  |  | Minimum Io | Maximum Io |
| SFN-SFN observed time difference type 2 | 1 | I, IV, VI, X, XI, XIX, XXI and XXXII | -94 | -50 |
|  |  | II, V and VII | -92 | -50 |
|  |  | XXV and XXVI | -90.5 (Note 1) | -50 |
|  |  | III, VIII, XII, XIII, XIV, XX and XXII | -91 | -50 |
|  |  | IX | -93 | -50 |
| NOTE 1: The minimum condition is -92 dBm/3.84 MHz when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies. |  |  |  |  |

##### 9.1.8.2.4	SFN-SFN observed time difference type 2 measurement report mapping

The reporting range is for SFN-SFN observed time difference type 2 is from -1280 ... +1280 chip.

In table 9.24 the mapping of measured quantity is defined. The range in the signalling may be larger than the guaranteed accuracy range.

Table 9.24

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| T2_SFN-SFN_TIME _00000 | SFN-SFN observed time difference type 2 < -1280.0000 | chip |
| T2_SFN-SFN_TIME _00001 | -1280.0000 SFN-SFN observed time difference type 2 < -1279.9375 | chip |
| T2_SFN-SFN_TIME _00002 | -1279.9375 SFN-SFN observed time difference type 2 < -1279.8750 | chip |
| … | … | … |
| T2_SFN-SFN_TIME _40959 | 1279.8750 SFN-SFN observed time difference type 2 < 1279.9375 | chip |
| T2_SFN-SFN_TIME _40960 | 1279.9375 SFN-SFN observed time difference type 2 < 1280.0000 | chip |
| T2_SFN-SFN_TIME _40961 | 1280.0000 SFN-SFN observed time difference type 2 | chip |

-------------------------------------------------- Unchanged text omitted ------------------------------------------------------------

## B.3.1. Conditions for intra frequency CPICH RSCP measurements accuracy

This section defines the intra frequency CPICH RSCP applicable for a corresponding operating band.

The CPICH RSCP conditions for measurements accuracy of intra-frequency CPICH RSCP are defined in Table B.3.1-1

Table B.3.1-1. Conditions for measurements of intra-frequency CPICH RSCP

| Parameter | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Bands | Bands | Bands | Bands | Bands |
|  | I, IV, VI, X, XI, XIX, XXI, XXXII | IX | II, V, VII | III, VIII, XII, XIII, XIV, XX, XXII | XXV, XXVI |
| CPICH RSCP1¦dBm | -114 dBm | -113 dBm | -112 dBm | -111 dBm | -110.5dBm (Note 1) |

Note 1: The condition is -112 dBm when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies.

## B.3.2. Conditions for intra frequency CPICH RSCP relative measurements  accuracy

This section defines the intra frequency CPICH RSCP applicable for a corresponding operating band.

The CPICH RSCP conditions for relative measurements accuracy of intra-frequency CPICH RSCP are defined in Table B.3.2-1

Table B.3.2-1. Conditions for relative measurements of intra-frequency CPICH RSCP

| Parameter | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Bands | Bands | Bands | Bands | Bands |
|  | I, IV, VI, X, XI, XIX, XXI, XXXII | IX | II, V, VII | III, VIII, XII, XIII, XIV, XX, XXII | XXV, XXVI |
| CPICH RSCP1,2¦dBm | -114 dBm | -113 dBm | -112 dBm | -111 dBm | -110.5dBm (Note 1) |

Note 1: The condition is -112 dBm when the carrier frequency of the assigned UTRA channel is within 869-894 MHz for the UE which supports both Band V and Band XXVI operating frequencies.

------------------------------------------------------------- End of change ---------------------------------------------------------------