TSG-RAN Working Group 4 (Radio) meeting #70-bis	R4-142483

San Jose del Cabo, Mexico, 31st, March  to 4th,April 2014

Source:	NTT DOCOMO, INC.

Title:	TP for TR36.851: Co-existence study on LTE Advanced Carrier Aggregation of Band Combination (1+42)

Agenda item:				7.7.2

Document for:	Approval

1. Introduction

In RAN4#70, the sample band was agreed in [1]. This contribution is a text proposal for co-existence study to add LTE Advanced Carrier Aggregation of Band Combination (1+42) into TR36.851 [2].

2. Text Proposal

----- Unchanged sections omitted -----

7.2.2.1.2	Co-existence studies for CA_1-42

The harmonic frequencies do not fall into the frequency ranges of both bands as observed in table 7.2.2.1.2-1. Therefore we can conclude that there is no issue on harmonic interference.

Table 7.2.2.1.2-1: Impact of UL/DL Harmonic Interference

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 2nd Harmonic |  | 3rd Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 | 4220 | 4340 | 6330 | 6510 |
| 42 | 3400 | 3600 | 3400 | 3600 | 6800 | 7200 | 10200 | 10800 | 6800 | 7200 | 10200 | 10800 |

Table 7.2.2.1.2-2 shows the second and third order DL harmonics and intermodulation products when two simultaneous DLs are active in Band 1 and Band 42.

Table 7.2.2.1.2-2: Band 1 and Band 42 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2110 | 2170 | 3400 | 3600 |
| 2nd order harmonics frequency range (MHz) | 4220 | 4340 | 6800 | 7200 |
| 3rd order harmonics frequency range (MHz) | 6330 | 6510 | 10200 | 10800 |
| 2nd order IMD products | (f2_low – f1_high) | (f2_high – f1_low) | (f2_low + f1_low) | (f2_high + f1_high) |
| IMD frequency limits (MHz) | 1230 | 1490 | 5510 | 5770 |
| 3rd order IMD products | (2*f1_low – f2_high) | (2*f1_high – f2_low) | (2*f2_low – f1_high) | (2*f2_high – f1_low) |
| IMD frequency limits (MHz) | 620 | 940 | 4630 | 5090 |
| 3rd order IMD products | (2*f1_low + f2_low) | (2*f1_high + f2_high) | (2*f2_low + f1_low) | (2*f2_high + f1_high) |
| IMD frequency limits (MHz) | 7620 | 7940 | 8910 | 9370 |
| 3rd order IMD products | (f1_low – f2_high + f2_low) | (f1_high + f2_high – f2_low) | (f2_low – f1_high + f1_low) | (f2_high + f1_high – f1_low) |
| IMD frequency limits (MHz) | 1910 | 2370 | 3340 | 3660 |
| 3rd order IMD products (with maximum channel bandwidth) | (f1_low – f2_BWmax) | (f1_high + f2_BWmax) | (f2_low – f1_BWmax) | (f2_high + f1_BWmax) |
| IMD frequency limits (MHz) | 2090 | 2190 | 3380 | 3620 |

It can be seen from table 7.2.2.1.2-1 that some 2nd IMD products caused by BS supporting carrier aggregation of Band 1 and Band 42 fall into the BS receive band of Bands 11 and 21, while some 3rd IMD products fall into the BS receive band of Bands 1, 5, 6, 8, 12, 13, 14, 17, 18 , 19,  20, 22, 23, 25, 26, 27, 28, 30, 33, 34, 36, 37, 39, 40, 42, 43  and 44. Note that the calculation in table 7.2.2.1.2-1 (except the last row) assumes the BS transmits the whole 60 MHz DL frequency of Band 1 and the whole 200 MHz DL frequency of Band 42. However even if the BS only transmits up to 20 MHz DL in Band 1 and up to 20 MHz DL in Band 42 as stated in Table 7.2.2.1.1-1, the 3rd IMD products may still fall into the BS receive band of the Bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 22, 26, 27, 28, 42, 43 and 44 as shown in the last row in table 7.2.2.1.2-1.

It should be noted that Bands 12, 13, 14, 17 is not intended for use in the same geographical area as Band 1 and Band 22 is not intended for use in the same geographical area as Band 42. Consequently, the focus here will be on the harmonics and IMD products falling into Bands 5, 6, 8, 18, 19, 20, 26, 27, 28, 42, 43 and 44.

TDD BS does not transmit and receive simultaneously, so the BS’s own band 42 receiver and other synchronized band 42 receivers would not be interfered. With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 5, 6, 8, 18, 19, 20,  26, 27, 28 or unsynchronized 44 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization,

Therefore, it is recommended that Bands 1 and 42 BS transmitters should not share the same antenna with Band 5, 6, 8, 18, 19, 20, 26, 27, 28,  unsynchronized 42, unsynchronized 43 or unsynchronized 44 BS receiver, unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 5, 6, 8, 18, 19, 20, 26, 27, 28,  unsynchronized 42, unsynchronized 43 or unsynchronized 44 BS receiver desensitization.

----- Unchanged sections omitted -----

Reference

[1]	R4-141230	“Agreement for TDD-FDD band combinations and handling of WI”,  Nokia Corporation

[2] R4-140843 “TR 36.851 V0.9.0: Rel-12 Inter-band Carrier Aggregation'”, Ericsson