3GPP TSG-RAN WG4 (Radio) Meeting #70bis	R4-142546

San Jose Del Cabo, Mexico, 31 March – 4 April 2014

Agenda Item:	7.27.2

Source: 	Alcatel-Lucent, Sprint, NSN

Title: 	TP for TR 36.833-6-41: Specific BS RF requirements for LTE Advanced Non-Contiguous Carrier Aggregation in Band 41 for 3DL

Document for:	Approval

# 1.	Introduction

The revised WI proposal to support the LTE Advanced intra-band non-contiguous Carrier Aggregation in Band 41 for 3 DL was approved in RAN#63 [1]. One of the objectives of this WI is to specify the core RF requirements for 3-CC non-contiguous intra-band DL CA in Band 41. The aggregation scenarios to be specified as 2-contiguous CCs and 1 non-contiguous CC, with a single sub-block gap [1].

In this contribution, we discuss the BS RF requirements for band 41 intra-band non-contiguous carrier aggregation for 3-DL CC, and provide a text proposal to record the findings in the TR36.833-6-41 [2].

# 2.	Discussion

For BS capable of multi-carrier and/or CA operation in contiguous and non-contiguous spectrum in single band, the core and test requirements currently specified in the BS specifications (TS 36.104 and TS 36.141 [3, 4]) are agnostic to the number of CC in each of the two sub-blocks within the operating band supported by the BS in intra-band non-contiguous CA operation. This can be seen by studying the test configurations defined for multi-carrier and/or CA operation in non-contiguous spectrum in single band in subclause 4.10.3 in TS 36.141. Also Table 5.5-4 in TS 36.104 and TS 36.141 (copied below) do not specify the number of CC in each of the two sub-blocks within the intra-band non-contiguous carrier aggregation bands.

Table 5.5-4. Intra-band non-contiguous carrier aggregation bands

| CA Band | E-UTRA operating band |
| --- | --- |
| CA_3-3 | 3 |
| CA_4-4 | 4 |
| CA_7-7 | 7 |
| CA_23-23 | 23 |
| CA_25-25 | 25 |
| CA_41-41 | 41 |

Therefore, the current requirements can be applied for 3-CC non-contiguous intra-band DL CA in Band 41.

However, the acronym ‘CA_41-41’ in the table above may be mis-interpreted that two CC instead of two sub-blocks are supported within each operating band. This should be clarified as shown below.

Table 5.5-4. Intra-band non-contiguous carrier aggregation bands (with two sub-blocks)

| CA Band | E-UTRA operating band |
| --- | --- |
| CA_3-3 | 3 |
| CA_4-4 | 4 |
| CA_7-7 | 7 |
| CA_23-23 | 23 |
| CA_25-25 | 25 |
| CA_41-41 | 41 |

# 3.	Conclusion

The current requirements specified in TS 36.104 and TS 36.141 for BS capable of multi-carrier and/or CA operation in contiguous and non-contiguous spectrum in single band are agnostic to the number of CC in each of the two sub-blocks within the operating band supported by the BS in intra-band non-contiguous CA operation, and thus can be applied for 3-CC non-contiguous intra-band DL CA in Band 41. Hence no change in the BS requirements is needed.

However, the acronym ‘CA_41-41’ in the Table 5.5-4 in TS 36.104 and TS 36.141 may be mis-interpreted that two CC instead of two sub-blocks are supported within each operating band. This should be clarified in the BS specifications.

# References

[1]	RP-140100, “Revised WID - LTE Advanced intra-band contiguous Carrier Aggregation in Band 41 for 3DL”, Sprint.

[2]	R4-141399, “(Draft) TR 36.833-6-41 V0.1.0”, Sprint.

[3]	3GPP TS 36.104 v12.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception”.

[4]	3GPP TS 36.141 v12.3.0, “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

# Text Proposal

<Start of change>

## 5.4	Specific BS RF requirements

The current requirements specified in TS 36.104 and TS 36.141 for BS capable of multi-carrier and/or CA operation in contiguous and non-contiguous spectrum in single band are agnostic to the number of CC in each of the two sub-blocks within the operating band supported by the BS in intra-band non-contiguous CA operation, and thus can be applied for 3-CC non-contiguous intra-band DL CA in Band 41. Hence no change in the BS requirements is needed.

However, the acronym ‘CA_41-41’ in the Table 5.5-4 in TS 36.104 and TS 36.141 may be mis-interpreted that two CC instead of two sub-blocks are supported within each operating band. This should be clarified in the BS specifications as shown below.

Table 5.5-4. Intra-band non-contiguous carrier aggregation bands (with two sub-blocks)

| CA Band | E-UTRA operating band |
| --- | --- |
| CA_3-3 | 3 |
| CA_4-4 | 4 |
| CA_7-7 | 7 |
| CA_23-23 | 23 |
| CA_25-25 | 25 |
| CA_41-41 | 41 |

<End of change>