3GPP TSG-RAN4 Meeting #71 	R4-142785

# Seoul, Korea, May 19 - 23, 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 2376 | rev | - | Current version: | 12.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce the support of 3DL CA to TS 36.133 Section 7.8 “Interruptions with Carrier Aggregation” |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_Bx_By_Bz |  |  |  |  |  |  | Date: |  | 2014-04-27 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | Currently, the RRM requirements defined in TS 36.133 Section 7.8 “Interruptions with Carrier Aggregation” only cover carrier aggregation with two downlink componenet carriers. The requirements need to be entended to three downlink componenet carriers. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the RRM requirements defined in TS 36.133 Section 7.8 to 3DL CCs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM requirements defined in Section 7.8 do not cover 3DL CCs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 7.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of first changes>

## 7.8	Interruptions with Carrier Aggregation

### 7.8.1	Introduction

This section contains the requirements related to the interruptions on all activated serving cells, which include PCell and activated SCells if configured, when one or more SCell(s) are configured, deconfigured, activated or deactivated.

Note:	interruptions at SCell addition/release, activation/deactivation and during measurements on SCC may not be required by all UEs.

Editor’s Note:	The interruptions shall not interrupt RRC signalling or ACK/NACKs related to RRC reconfiguration procedure [2] for SCell addition/release or MAC control signalling [36.321] for SCell activation/deactivation command. How to specify this is FFS.

### 7.8.2	Requirements

#### 7.8.2.1	Interruptions at SCell addition/release for intra-band CA

When an SCell is added or released as defined in [2] the UE is allowed an interruption with a total duration of up to 5 subframes on all activated downlink and uplink serving cells in the same band during the RRC reconfiguration procedure [2].

#### 7.8.2.2	Interruptions at SCell addition/release for inter-band CA

When an SCell is added or released as defined in [2] the UE that requires interrupt is allowed an interruption with a total duration of up to 1 subframe on all activated downlink and uplink serving cells in different bandsduring the RRC reconfiguration procedure [2].

#### 7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA

When an SCell is activated or deactivated as defined in [2] the UE is allowed an interruption with a total duration of up to 5 subframes on all activated downlink and uplink serving cells in the same band during the activation/deactivation procedure [2] delay - defined in Section 7.7.

#### 7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA

When an SCell is activated or deactivated as defined in [2] the UE that requires interrupt is allowed an interruption with a total duration of up to 1 subframe on all activated downlink and uplink serving cells in the different band during the activation/deactivation procedure [2] - defined in Section 7.7.

#### 7.8.2.5	Interruptions during measurements on SCC for intra-band CA

PCell interruptions due to measurements on SCC when the SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell[2]is 640ms or longer. Each interruption shall not exceed 5 subframes.

For UE supporting more than two downlink CCs, interruptions on all activated downlink and uplink serving cells due to measurements on SCC with a deactivated SCell are allowed with up to 0.5% probability of missed ACK/NACK for each activated serving cell in the same band when the configured measCycleSCell[2]is 640ms or longer. Each interruption shall not exceed 5 subframes.

#### 7.8.2.6	Interruptions during measurements on SCC for inter-band CA

PCell interruptions due to measurements on SCC when the SCell is deactivated are allowed with up to 0.5% probability of missed ACK/NACK when the configured measCycleSCell[2]is 640ms or longer. Each interruption shall not exceed 1 subframe.

Editor’s Note: For UE supporting more than two downlink CCs, the measurements on SCC with deactivated SCell should not cause any interruption on the activated serving cells in a different band.

#### 7.8.2.7	Interruptions at addition/release of multiple SCells

When multiple downlink and/or unplink SCells are added or released in the same RRCConnectionReconfiguration message as defined in [2], the UE is allowed, during the RRC reconfiguration procedure, an interruption with a total duration of up to 5 subframes on all activated downlink and uplink serving cells that are in the same band as any of the SCells being added/released. The UE is also allowed an interruption with a total duration of up to 1 subframe on all activated downlink and uplink serving cells that are not in the same band as any of the SCells being added/released.

#### 7.8.2.8	Interruptions at activation/deactivation of multiple SCells

When multiple downlink and/or unplink SCells are activated/deactivated in the same MAC control element as defined in [17], the UE is allowed, during the Activation/Deactivation procedure, an interruption with a total duration of up to 5 subframes on all activated downlink and uplink serving cells that are in the same band as any of the SCells being activated/deactivated. The UE is also allowed an interruption with a total duration of up to 1 subframe on all activated downlink and uplink serving cells that are not in the same band as any of the SCells being activated/deactivated.

#### 7.8.2.9	Interruptions at overlapping addition/release/activation/deactivation of SCells

#### For a UE capable of supporting carrier aggregations with more than two component carries, the UE may be commanded by the network to sequentially add/release/activate/deactivate SCells. It may happen that a new procedure of addition/release/activation/deactivation of SCell(s) takes place even before the completion of previous procedure of addition/release/activation/deactivation of SCell(s). In this case, the interruptions on any of the already activated serving cells due to sequential addition/release/activation/deactivation of multiple SCells shall not exceed the sum of the allowed interruptions on the activated serving cell caused by each of the addition/release/activation/deactivation  procedures, as defined in above sections.

<End of changes>