3GPP TSG-RAN WG4 RAN4 #71                                    R4 -143174

Seoul, Korea, 19th – 23rd May, 2014

Agenda item:	7.5.2

Source: 	Samsung

Title: 			  Test case design for PUSCH3-2 CQI test in FeDL-MIMO

Document for:	   Discussion

1	Introduction

Performance requirements for eDL-MIMO work item in Rel-12 were widely discussed in recently RAN4 meeting. Regarding PUSCH 3-2 CQI test such agreements were achieved in last meeting:

- Throughput ratio of PUSCH 3-2 over PUSCH 3-1

  - TM9 with Rel-12 codebook

  - TM6 with Rel-8 codebook

  - Note: Best sub-band scheduling (for both PUSCH 3-2 and PUSCH 3-1) or random sub-band to be decided in the next meeting.

  - Antenna correlation TBD: ULA 4x2, XP High, As in B.2.4 (2-Tap)

  - Time Delay between Tx Antennas: TBD for the next meeting with options that have been proposed: 0ns, up to 65ns, and up to 130ns. TE vendors input encouraged

In this contribution, we provide initial simulation results and analysis for PUSCH 3-2 CQI test design.

- Analysis

2.1 Overview

MIMO antenna correlation and fading channel

Regarding antenna correlation, both ULA and cross polarized antenna correlation can be used in test cases. Considering Rel-12 codebook is optimized for cross polarized antenna pattern and improve test coverage with limited test cases, we propose:

Proposal1: Introduce TM6 test case with ULA low channel and TM9 test case with XP high channel.

Time delay between tx antennas

In order to ensure large performance gap between PUSCH 3-2 and PUSCH 3-1, some contributions i.e. [2], [3] proposed to introduce timing delay between transmit antenna ports to generate artificial frequency-selective correlation between antennas. However, the feasibility of this artificial time delay between transmit antennas need to be further confirmed by TE vendors. Furthermore, the time delay may affect UE implementation i.e. timing synchronization as point out in [4], we prefer not to introduce time delay between tx antennas.

Proposal2: Not introduce time delay between tx antennas.

Test metric

With PUSCH 3-2 feedback mode, UE needs to report both sub-band CQI and sub-band PMI. Unlike legacy CQI or PMI test, only CQI or PMI reporting accuracy were identified in single test by fixed PMI or fixed MCS level during test. Throughput ratio with PUSCH 3-2 over other feedback mode was proposed to verify reporting CQI and PMI accuracy. Several test metrics were proposed with this new test metrics as summarized below:

- Test metric1: TP ratio with PUSCH 3-2 over PUSCH 3-1 with best sub-band scheduling

- Test metric 2: TP ratio with PUSCH 3-2 over PUSCH 3-1 with random sub-band scheduling

- Test metric 3: TP ratio with PUSCH 3-2 with best sub-band over PUSCH 1-2 with random sub-band

With test metric1, both sub-band CQI and sub-band PMI reporting accuracy can be jointly verified. However, whether large performance gain can be guaranteed with this test metric need to be further evaluated. Test metric 2 and test metric 3 can be used to verify sub-band PMI and sub-band CQI reporting accuracy separately. Generally, two alternatives can be used:

- Alt1: Only introduce test metric 1 to jointly verify sub-band PMI and sub-band CQI accuracy

- Alt2: Introduce both test metric 2 and 3 to verify sub-band PMI and sub-band CQI accuracy separately.

In order to address the open issues for PUSCH 3-2 sub-CQI test and validate the feasibility new test metric i.e. throughput ratio of PUSCH 3-2 over PUSCH 3-1, several cases were evaluated in next chapter with different parameters combination.

2.2 Simulation results

Simulation assumption

The main simulation assumption were summarized below and detailed parameters given in table 1 below.

- Antenna correlation: 4x2 ULA

- Fading channel: EVA5Hz

- Time Delay between Tx antennas: 0ns, {+65,-65,+65,-65} ns

- CSI feedback mode: PUSCH 3-2,PUSCH 1-2,PUSCH 3-1

- PMI and CQI scheduled:

  - follow sub-band PMI and sub-band CQI for PUSCH 3-2

  - follow sub-band CQI and wideband CQI for PUSCH 3-1

  - follow wideband CQI and sub-band PMI for PUSCH 1-2

Table 1 simulation assumption

| Parameter |  | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 MHz |  |
| Transmission mode |  |  | 6 |  |
| Downlink power allocation |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
|  |  | dB | 0 |  |
| Propagation channel |  |  | Option1:EVA5Hz |  |
| Antenna configuration |  |  | Option1:Low ULA 4 x 2 |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |
| CRS reference signals |  |  | Antenna ports 0,1 |  |
| CSI reference signals |  |  | Antenna ports 15, 16,17,18 |  |
| CSI-RS periodicity and sub-frame offset TCSI-RS / ∆CSI-RS |  |  | 5/ 1 |  |
| CSI-RS reference signal configuration |  |  | 4 |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0000 FFFF |  |
| Reporting interval |  | ms | 5 |  |
| CQI delay |  | ms | 8 |  |
| Reporting mode |  |  | PUSCH 3-2 PUSCH 1-2 PUSCH 3-1 |  |
| Sub-band size |  | RB | 6 (full size) |  |
| Max number of HARQ transmissions |  |  | 1 |  |

Simulation results

Based on proposed test setup in last chapter, such test metrics were evaluated:

- TP ratio with PUSCH 3-2 over PUSCH 3-1 with best sub-band scheduling

- TP ratio with PUSCH 3-2 over PUSCH 3-1 with random sub-band scheduling

- TP ratio with PUSCH 3-2 with best sub-band over PUSCH 1-2 with random sub-band

Figures 1 below summarize TP ratios vs. SNR for above 3 test metrics with time delay between tx antennas. And figure 2 show the TP ratios vs. SNR without time delay.

Based on simulation results, such observations were found across SNR range {0~12} dB:

- For test metric 1: PUSCH 3-2 over PUSCH 3-1 with best sub-band scheduling

  - TP ratio is around 1.03~1.09 with time delay case and 1.02~1.04 for without time delay case

- For test metric 2: TP ratio with PUSCH 3-2 over PUSCH 3-1 with random sub-band scheduling

  - TP ratio is around 1.17~1.41 with time delay case and 1.11~1.23 without time delay case.

- For test metric 3: TP ratio with PUSCH 3-2 with best sub-band over PUSCH 1-2 with random sub-band

  - TP ratio is around 1.19~1.46 with time delay case and 1.18~1.47 without time delay case.

Figure 1: TP ratio vs. SNR with time delay

Figure 2: TP ratio vs. SNR without time delay

Based on simulation results and summary, we observed that the performance gain for PUSCH 3-2 over PUSCH 3-1 with best sub-band scheduling is marginal. The throughput ratio is less than 1.1 even with time delay. On the other side, TP ratio under test metric 2 and 3 is over 1.1 and feasible to introduce for PUSCH 3-2 test.

Propose 3: Introduce 2 test metrics to verify reporting sub-band CQI and sub-band PMI accuracy separately in one test:

- TP ratio with PUSCH 3-2 over PUSCH 3-1 with random sub-band scheduling

- TP ratio with PUSCH 3-2 with best sub-band over PUSCH 1-2 with random sub-band

- Conclusion

In this contribution, some open issues for PUSCH 3-2 test were analyzed, then several cases were evaluated with different parameters combination.

Based on simulation results, such observations were found:

Observation 1: The performance gain for PUSCH 3-2 over PUSCH 3-1 with best sub-band scheduling is marginal and less than 10% even with time delay.

Observation 2: The performance gain for PUSCH 3-2 over PUSCH 3-1 with random sub-band scheduling is above 10%.

Observation 3: The performance gain for PUSCH 3-2 with best sub-band over PUSCH 1-2 with random sub-band is over 18%.

Based on the observations and analysis, such detailed test case design was proposed to verify new features in this WI:

Proposal1: Introduce TM6 test case with ULA low channel and TM9 test case with XP high channel.

Proposal2: Not introduce time delay between tx antennas.

Propose 3: Introduce 2 test metrics to verify reporting sub-band CQI and sub-band PMI accuracy separately:

- TP ratio with PUSCH 3-2 over PUSCH 3-1 with random sub-band scheduling

- TP ratio with PUSCH 3-2 with best sub-band over PUSCH 1-2 with random sub-band

- Reference

- R4-142360, “eDL-MIMO Adhoc Minutes”, Alcatel-Lucent

- R4-141986, “Discussions on test for reporting mode 3-2”, MediaTek Inc.

- R4-141504, “Discussion on feedback mode PUSCH 3-2 test”, Intel Corporation

- R4-141602, “Discussion on test cases of PUSCH 3-2 feedback for downlink MIMO enhancement”, Huawei