3GPP TSG-RAN WG4 Meeting #72	R4-144173

# Dresden, Germany, 18 – 22 Aug, 2014

# Source: 	Motorola Solutions

Title: 	TP for the D2D TR section 5 (Operating bands & channel arrangements) & Annex A

# Agenda Item:	7.12.2

# Document for:	Discussion and Approval

# Introduction

In this document we present our proposal on the UE RF requirements for section 5 of TS36.101 (Operating bands and channel arrangements). In section 2 we provide some background, section 4 a text proposal for D2D TR section 5 and in section 5 a TP for D2D TR Annex A. Note we use the term ProSe instead of D2D for Annex A.

# Background / Proposal

In this section we review the current requirements from TS36.101 section 5 (using the same subclause numbering) of indentifying the requirements that may be impacted by D2D (ProSe) . We also propose new subclauses that may be needed or modified which are highlighted in red.

| 5 | Operating bands and channel arrangement |  |  |
| --- | --- | --- | --- |
| 5.1 | General |  |  |
| 5.2 | Void |  |  |
| 5.3 | Void |  |  |
| 5.4 | Void |  |  |
| 5.5 |  | Operating bands |  |  |
| 5.5A |  | Operating bands for CA |  |  |
| 5.5B |  | Operating bands for UL-MIMO |  |  |
| 5.5C |  | Operating bands for ProSe |  |  |

Proposal

- See proposal TP proposal in R4-14xxx for section 5 and Annex A.

- Rationale

- The operating bands for ProSe_discovery and ProSe_communications should be separately defined.

- It is proposed that operating bands for ProSe_communications should be in brackets to indicate which bands are prioritised pending completion of outstanding RF requirements

| 5.6 | Channel bandwidth |  |
| --- | --- | --- |
| 5.6.1 | Channel bandwidths per operating band |  |
| 5.6A | Channel bandwidth for CA |  |
| 5.6A.1 | Channel bandwidths per operating band for CA |  |
| 5.6B | Channel bandwidth for UL-MIMO |  |
| 5.6B.1 | Void |  |
| 5.6C | Channel bandwidth for ProSe |  |

Proposal

- The supported channel bandwidths should be specified in a new subclause 5.6C.

- Rationale

  - Current focus in terms of analysis and simulations are based on 10MHz channel bandwidth.

  - Resource pool has yet to be defined but would be expected to be based on 10MHz as baseline

  - The specified channel bandwidth would need to take into account in-band interference and would be expected to be based on 10MHz baseline

  - Additional channel bandwidth can be specified as the work progress to consider impact of other RF requirements which are in progress

  - It is proposed this should be specified as 10MHz for both D2D_discovery and D2D_communications in separate tables in brackets pending completion of other RF aspects

| 5.7 | Channel arrangement |  |
| --- | --- | --- |
| 5.7.1 | Channel spacing |  |
| 5.7.1A | Channel spacing for CA |  |
| 5.7.2 | Channel raster |  |
| 5.7.2A | Channel raster for CA |  |
| 5.7.3 | Carrier frequency and EARFCN |  |

Proposal

- We need to add a note to Table 5.7.3-1: E-UTRA channel number  or a new subclause  5.7.3C to indicate the URFCN for the DL for ProSe

- Rationale

  - To indicate for ProSe  the same  EARFCN UL number as specified for the corresponding E-UTRA band is also specified for the DL in the case of ProSe

  - Propose a new note to Table 5.7.3-1 ;  NOTE 3:   For ProSe the corresponding UL channel number are also specified for the DL for the associated ProSe operating bands i.e. ProSE_FUL = FUL and ProSE_FDL = FUL

| 5.7.4 | TX–RX frequency separation |  |
| --- | --- | --- |
| 5.7.4A | TX–RX frequency separation for CA |  |

# 3	References

[1]	3GPP TR 36.942: "Radio Frequency (RF) system scenarios".

[2]	ACLR Model for D2D Coexistence Simulations; Qualcomm

- TP for D2D TR 36.XXX V0.0.3 (2014-05) section 5

------------------------ START OF TP -------------------------

5	Deployment and co-existence studies

5.1	General

5.3	Channel bandwidth

Background

E-UTRA supports both flexibility bandwidths smaller than 5 MHz i.e.1.4MHz and 3MHz and scalable bandwidths of 5, 10, 15 and 20MHz. Data is allocated to the UEs in terms of resource blocks, i.e. one UE can be allocated integer multiples of one resource block in the frequency domain. In uplink, data is allocated in multiples of one resource block. Both D2D_discovery and D2D_communications assume a maximum signal bandwidth of 2RB

D2D channel bandwidth

Channel bandwidths defined in TR36.843 Table A2.1.1-1 assume a 10MHz allocation for both UL and DL for FDD (20MHz for TDD) for both the general and Public Safety scenarios.  It is proposed 10MHz channel bandwidth should be taken as the baseline for both D2D_discovery and D2D_communication.  Other channel bandwidths are FFS

Table 5.3-1:  E-UTRA band / D2D_discovery channel bandwidth

| E-UTRA band / D2D_discovery channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| [ ] |  |  |  | [Yes] |  |  |

Table 5.3-2:  E-UTRA band / D2D_communications channel bandwidth

| E-UTRA band / D2D_communications channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| [ ] |  |  |  | [Yes] |  |  |

------------------------ END OF TP -------------------------

# TP for D2D TR 36.XXX V0.0.3 (2014-05) Annex A

------------------------ START OF TP -------------------------

## 5.6B	Channel bandwidth for UL-MIMO

The requirements specified in subclause 5.6 are applicable to UE supporting UL-MIMO.

### 5.6B.1	Void

### 5.6C	Channel bandwidth for ProSe

The ProSe combination of channel bandwidths and operating bands is shown in Table 5.6C-1 and Table 5.6C-2. The transmission bandwidth configuration in Table 5.6C-1 and Table 5.6C-2 shall be supported for each of the specified channel bandwidths. The same (symmetrical) channel bandwidth is specified for both the TX and RX path.

# Table 5.6C-1: ProSe_discovery channel bandwidth

| # E-UTRA band / ProSe channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| # E-UTRA Band | # 1.4 MHz | # 3 MHz | # 5 MHz | # 10 MHz | # 15 MHz | # 20 MHz |
| # [ ] |  |  |  |  |  |  |  | # [Yes] |  |  |  |  |  |

# Table 5.6C-2: ProSe_communications channel bandwidth

| E-UTRA band / ProSe channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| [ ] |  |  |  | [Yes] |  |  |

## 5.7	Channel arrangement

### 5.7.1	Channel spacing

The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between two adjacent E-UTRA carriers is defined as following:

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective E-UTRA carriers. The channel spacing can be adjusted to optimize performance in a particular deployment scenario.

### 5.7.1A	Channel spacing for CA

For intra-band contiguous carrier aggregation bandwidth classes B and C, the nominal channel spacing between two adjacent E-UTRA component carriers is defined as the following:

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective E-UTRA component carriers according to Table 5.6-1 with values in MHz. The channel spacing for intra-band contiguous carrier aggregation can be adjusted to any multiple of 300 kHz less than the nominal channel spacing to optimize performance in a particular deployment scenario.

For intra-band non-contiguous carrier aggregation the channel spacing between two E-UTRA component carriers in different sub-blocks shall be larger than the nominal channel spacing defined in this subclause.

### 5.7.2	Channel raster

The channel raster is 100 kHz for all bands, which means that the carrier centre frequency must be an integer multiple of 100 kHz.

### 5.7.2A	Channel raster for CA

For carrier aggregation the channel raster is 100 kHz for all bands, which means that the carrier centre frequency must be an integer multiple of 100 kHz.

### 5.7.3	Carrier frequency and EARFCN

The carrier frequency in the uplink and downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 - 65535. The relation between EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL_low and NOffs-DL are given in Table 5.7.3-1 and NDL is the downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL)

The relation between EARFCN and the carrier frequency in MHz for the uplink is given by the following equation where FUL_low and NOffs-UL are given in Table 5.7.3-1 and NUL is the uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL)

# Table 5.7.3-1: E-UTRA channel numbers

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 1 | 2110 | 0 | 0 – 599 | 1920 | 18000 | 18000 – 18599 |
| 2 | 1930 | 600 | 6001199 | 1850 | 18600 | 18600 – 19199 |
| 3 | 1805 | 1200 | 1200 – 1949 | 1710 | 19200 | 19200 – 19949 |
| 4 | 2110 | 1950 | 1950 – 2399 | 1710 | 19950 | 19950 – 20399 |
| 5 | 869 | 2400 | 2400 – 2649 | 824 | 20400 | 20400 – 20649 |
| 6 | 875 | 2650 | 2650 – 2749 | 830 | 20650 | 20650 – 20749 |
| 7 | 2620 | 2750 | 2750 – 3449 | 2500 | 20750 | 20750 – 21449 |
| 8 | 925 | 3450 | 3450 – 3799 | 880 | 21450 | 21450 – 21799 |
| 9 | 1844.9 | 3800 | 3800 – 4149 | 1749.9 | 21800 | 21800 – 22149 |
| 10 | 2110 | 4150 | 4150 – 4749 | 1710 | 22150 | 22150 – 22749 |
| 11 | 1475.9 | 4750 | 4750 – 4949 | 1427.9 | 22750 | 22750 – 22949 |
| 12 | 729 | 5010 | 5010 - 5179 | 699 | 23010 | 23010 - 23179 |
| 13 | 746 | 5180 | 5180 – 5279 | 777 | 23180 | 23180 – 23279 |
| 14 | 758 | 5280 | 5280 – 5379 | 788 | 23280 | 23280 – 23379 |
| … |  |  |  |  |  |  |
| 17 | 734 | 5730 | 5730 – 5849 | 704 | 23730 | 23730 – 23849 |
| 18 | 860 | 5850 | 5850 – 5999 | 815 | 23850 | 23850 – 23999 |
| 19 | 875 | 6000 | 6000 – 6149 | 830 | 24000 | 24000 – 24149 |
| 20 | 791 | 6150 | 6150 – 6449 | 832 | 24150 | 24150 – 24449 |
| 21 | 1495.9 | 6450 | 6450 – 6599 | 1447.9 | 24450 | 24450 – 24599 |
| 22 | 3510 | 6600 | 6600 – 7399 | 3410 | 24600 | 24600 – 25399 |
| 23 | 2180 | 7500 | 7500 – 7699 | 2000 | 25500 | 25500 – 25699 |
| 24 | 1525 | 7700 | 7700 - 8039 | 1626.5 | 25700 | 25700 – 26039 |
| 25 | 1930 | 8040 | 8040 - 8689 | 1850 | 26040 | 26040 – 26689 |
| 26 | 859 | 8690 | 8690 - 9039 | 814 | 26690 | 26690 – 27039 |
| 27 | 852 | 9040 | 9040 – 9209 | 807 | 27040 | 27040 – 27209 |
| 28 | 758 | 9210 | 9210 – 9659 | 703 | 27210 | 27210 – 27659 |
| 292 | 717 | 9660 | 9660 – 9769 | N/A |  |  |
| 30 | 2350 | 9770 | 9770 – 9869 | 2305 | 27660 | 27660 – 27759 |
| 31 | 462.5 | 9870 | 9870 – 9919 | 452.5 | 27760 | 27760 – 27809 |
| … |  |  |  |  |  |  |
| 33 | 1900 | 36000 | 36000 – 36199 | 1900 | 36000 | 36000 – 36199 |
| 34 | 2010 | 36200 | 36200 – 36349 | 2010 | 36200 | 36200 – 36349 |
| 35 | 1850 | 36350 | 36350 – 36949 | 1850 | 36350 | 36350 – 36949 |
| 36 | 1930 | 36950 | 36950 – 37549 | 1930 | 36950 | 36950 – 37549 |
| 37 | 1910 | 37550 | 37550 – 37749 | 1910 | 37550 | 37550 – 37749 |
| 38 | 2570 | 37750 | 37750 – 38249 | 2570 | 37750 | 37750 – 38249 |
| 39 | 1880 | 38250 | 38250 – 38649 | 1880 | 38250 | 38250 – 38649 |
| 40 | 2300 | 38650 | 38650 – 39649 | 2300 | 38650 | 38650 – 39649 |
| 41 | 2496 | 39650 | 39650 –41589 | 2496 | 39650 | 39650 –41589 |
| 42 | 3400 | 41590 | 41590 – 43589 | 3400 | 41590 | 41590 – 43589 |
| 43 | 3600 | 43590 | 43590 – 45589 | 3600 | 43590 | 43590 – 45589 |
| 44 | 703 | 45590 | 45590 – 46589 | 703 | 45590 | 45590 – 46589 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7, 15, 25, 50, 75 and 100 channel numbers at the lower operating band edge and the last 6, 14, 24, 49, 74 and 99 channel numbers at the upper operating band edge shall not be used for channel bandwidths of 1.4, 3, 5, 10, 15 and 20 MHz respectively. NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. NOTE 3: For ProSe the corresponding UL channel number are also specified for the DL for the associated ProSe operating bands i.e. ProSE_FUL = FUL and ProSE_FDL = FUL |  |  |  |  |  |  |

### 5.7.4	TX–RX frequency separation

a)	The default E-UTRA TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation is specified in Table 5.7.4-1 for the TX and RX channel bandwidths defined in Table 5.6.1-1

# Table 5.7.4-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX - RX carrier centre frequency separation |
| --- | --- |
| 1 | 190 MHz |
| 2 | 80 MHz. |
| 3 | 95 MHz. |
| 4 | 400 MHz |
| 5 | 45 MHz |
| 6 | 45 MHz |
| 7 | 120 MHz |
| 8 | 45 MHz |
| 9 | 95 MHz |
| 10 | 400 MHz |
| 11 | 48 MHz |
| 12 | 30 MHz |
| 13 | -31 MHz |
| 14 | -30 MHz |
| 17 | 30 MHz |
| 18 | 45 MHz |
| 19 | 45 MHz |
| 20 | -41 MHz |
| 21 | 48 MHz |
| 22 | 100 MHz |
| 23 | 180 MHz |
| 24 | -101.5 MHz |
| 25 | 80 MHz |
| 26 | 45 MHz |
| 27 | 45 MHz |
| 28 | 55 MHz |
| 30 | 45 MHz |
| 31 | 10 MHz |

b)	The use of other TX channel to RX channel carrier centre frequency separation is not precluded and is intended to form part of a later release.

### 5.7.4A	TX–RX frequency separation for CA

For intra-band contiguous carrier aggregation, the same TX-RX frequency separation as specified in Table 5.7.4-1 is applied to PCC and SCC, respectively.