3GPP TSG-RAN WG4 Meeting #72                           R4-144446

Dresden, Germany, 18 – 22 Aug, 2014

Source: 	CMCC

Title: 	RSRP measurement performance of DRS for SCE

Agenda Item:	7.10.4

Document for:	Discussion

- Introduction

The new work item about small cell enhancement was approved in RP#62. The design of Discovery Reference Signal has reached a lot of progress. Several agreements were reached in RAN1 about DRS based RRM measurement. They are as follows:

- A DRS comprises following signals

  - Both PSS and SSS are transmitted

  - Additional reference signal(s) include CRS and/or CSI-RS

- UE assumes PSS/SSS/CRS in the DRS

  - Additionally CSI-RS is assumed in the DRS for measurement if configured by higher layers

In recent RAN4#71AH, the DRS based RSRP accuracy measurement assumptions were discussed. And the simulation assumptions on CRS and CSI-RS based RSRP measurement were agreed in [1].In this contribution, the link level simulation results for DRS based RSRP accuracy are provided, and some proposals based on the results are provided.

- CRS based RSRP measurement

The simulation results of RSRP accuracy based on CRS are provided in Table 1-3. Each table represents one channel condition. The simulation assumptions follows proposal [1].

The worst case represents the combination of DRS period, duration and measurement period under which the RSRP measurement requirements could be satisfied. The blank rows represent that the requirements cannot be satisfied. And the last column contains the 5% and 95% CDF point of RSRP measurement errors. In [2], it is agreed that +/-2dB is the baseband error for absolute RSRP accuracy. Thus, +/-2dB is used as the RSRP requirement in the simulation.

Table 1 CRS based RSRP measurement simulation results under EPA5

| Channel Type | Measurement bandwidth | SNR(dB) | The worst case which could satisfy the accuracy requirements |  |  | RSRP errors at 5%~95% CDF(dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | DRS periodicity | DRS duration | Measurement period |  |
| EPA5 | 6RB | -10 | -- | -- | -- | -- |
|  |  | -8 | 40 | 3 | 800 | [0.54 1.85] |
|  |  | -3 | 160 | 1 | 480 | [-1.39 1.73] |
|  |  | 0 | 160 | 1 | 480 | [-0.91 0.98] |
|  | 25RB | -10 | 40 | 3 | 800 | [0.52 1.82] |
|  |  | -8 | 80 | 1 | 800 | [-0.58 1.68] |
|  |  | -3 | 160 | 1 | 480 | [-0.88 1.06] |
|  |  | 0 | 160 | 1 | 160 | [-1.24 1.17] |
|  | 50RB | -10 | 40 | 3 | 800 | [0.72 1.98] |
|  |  | -8 | 40 | 1 | 480 | [-0.27 1.78] |
|  |  | -3 | 160 | 1 | 160 | [-1.89 1.48] |
|  |  | 0 | 160 | 1 | 160 | [-1.08 0.89] |

Table 2 CRS based RSRP measurement simulation results under ETU30

| Channel Type | Measurement bandwidth | SNR(dB) | The worst case which could satisfy the accuracy requirements |  |  | RSRP errors at 5%~95% CDF(dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | DRS periodicity | DRS duration | Measurement period |  |
| ETU30 | 6RB | -10 | -- | -- | -- | -- |
|  |  | -8 | 40 | 3 | 800 | [0.68 1.87] |
|  |  | -3 | 160 | 1 | 480 | [-1.42 1.70] |
|  |  | 0 | 160 | 1 | 160 | [-1.96 1.80] |
|  | 25RB | -10 | 80 | 5 | 800 | [0.56 1.89] |
|  |  | -8 | 80 | 1 | 800 | [-0.32 1.73] |
|  |  | -3 | 160 | 1 | 160 | [-1.69 1.64] |
|  |  | 0 | 160 | 1 | 160 | [-1.01 0.97] |
|  | 50RB | -10 | 80 | 1 | 160 | [-1.85 1.87] |
|  |  | -8 | 80 | 1 | 160 | [-1.39 1.21] |
|  |  | -3 | 160 | 1 | 160 | [-0.86 0.73] |
|  |  | 0 | 160 | 1 | 160 | [-0.49 0.45] |

Based on the simulation results in tables above, the following observation and proposal can be obtained:

Observation 1: The CRS based RSRP requirements could be hardly satisfied when the SNR is -10dB.

Proposal 1: The existing CRS based RSRP measurement requirements and side condition could be reused.

- CSI-RS based RSRP measurement

- The CSI-RS RSRP simulations results are as follows. The RSRP measurement results in EPA5, ETU30 are presented in different tables.

Table 3 CSI-RS based RSRP measurement simulation results under EPA5

| Channel Type | Measurement bandwidth | RE number per RB | SNR (dB) | The worst case which could satisfy the accuracy requirements |  |  | RSRP errors at 5%~95% CDF(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DRS periodicity | DRS duration | Measurement period |  |
| EPA5 | 6RB | 2 | -4 | 40 | 1 | 800 | [-0.64 1.97] |
|  |  |  | -3 | 160 | 3 | 800 | [-0.98 1.36] |
|  |  |  | 0 | 160 | 3 | 800 | [-0.98 1.36] |
|  |  |  | 3 | 160 | 1 | 480 | [-1.31 1.12] |
|  |  | 4 | -4 | 40 | 1 | 800 | [-0.16 1.90] |
|  |  |  | -3 | 80 | 1 | 800 | [-0.77 1.92] |
|  |  |  | 0 | 160 | 1 | 480 | [-1.70 1.90] |
|  |  |  | 3 | 160 | 1 | 480 | [-1.13 1.19] |
|  |  | 8 | -4 | 160 | 3 | 800 | [-0.44 1.59] |
|  |  |  | -3 | 160 | 1 | 800 | [-1.14 1.75] |
|  |  |  | 0 | 160 | 1 | 480 | [-1.22 1.36] |
|  |  |  | 3 | 160 | 1 | 480 | [-0.84 0.86] |
|  | 25RB | 2 | -4 | 160 | 1 | 800 | [-0.83 1.90] |
|  |  |  | -3 | 160 | 1 | 480 | [-0.42 1.60] |
|  |  |  | 0 | 160 | 1 | 160 | [-1.76 1.74] |
|  |  |  | 3 | 160 | 1 | 160 | [-1.24 1.13] |
|  |  | 4 | -4 | 160 | 1 | 480 | [-1.54 1.97] |
|  |  |  | -3 | 160 | 1 | 480 | [-1.35 1.69] |
|  |  |  | 0 | 160 | 1 | 160 | [-0.98 1.33] |
|  |  |  | 3 | 160 | 1 | 160 | [-1.29 1.19] |
|  |  | 8 | -4 | 160 | 1 | 480 | [-1.13 1.23] |
|  |  |  | -3 | 160 | 1 | 480 | [-0.87 1.06] |
|  |  |  | 0 | 160 | 1 | 160 | [-1.29 1.21] |
|  |  |  | 3 | 160 | 1 | 160 | [-0.78 0.77] |
|  | 50RB | 2 | -4 | 160 | 1 | 480 | [-1.52 1.90] |
|  |  |  | -3 | 160 | 1 | 480 | [-1.27 1.52] |
|  |  |  | 0 | 160 | 1 | 160 | [-1.84 1.66] |
|  |  |  | 3 | 160 | 1 | 160 | [-1.09 1.10] |
|  |  | 4 | -4 | 160 | 1 | 800 | [-1.01 1.85] |
|  |  |  | -3 | 160 | 1 | 480 | [-1.41 1.86] |
|  |  |  | 0 | 80 | 1 | 160 | [-1.35 1.20] |
|  |  |  | 3 | 160 | 1 | 160 | [-1.24 1.02] |
|  |  | 8 | -4 | 160 | 1 | 480 | [-1.25 1.47] |
|  |  |  | -3 | 160 | 1 | 480 | [-1.24 1.32] |
|  |  |  | 0 | 160 | 1 | 160 | [-1.48 1.21] |
|  |  |  | 3 | 160 | 1 | 160 | [-0.88 0.74] |

Table 4 CSI-RS based RSRP measurement simulation results under ETU30

| Channel Type | Measurement bandwidth | RE number per RB | SNR (dB) | The worst case which could satisfy the accuracy requirements |  |  |  | RSRP errors at 5%~95% CDF(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | DRS periodicity | DRS duration |  | Measurement period |  |
| ETU30 | 6RB | 2 | -4 | -- |  | -- | -- | -- |
|  |  |  | -3 | 40 | 3 |  | 800 | [0.48 1.88] |
|  |  |  | 0 | 80 | 1 |  | 480 | [-1.23 1.96] |
|  |  |  | 3 | 160 | 1 |  | 480 | [-1.48 1.74] |
|  |  | 4 | -4 | 80 | 3 |  | 800 | [0.14 1.86] |
|  |  |  | -3 | 160 | 3 |  | 800 | [-0.32 1.81] |
|  |  |  | 0 | 160 | 1 |  | 480 | [-1.56 1.98] |
|  |  |  | 3 | 160 | 1 |  | 480 | [-1.10 1.28] |
|  |  | 8 | -4 | 40 | 1 |  | 800 | [-0.02 1.81] |
|  |  |  | -3 | 80 | 1 |  | 800 | [-0.51 1.78] |
|  |  |  | 0 | 160 | 1 |  | 480 | [-1.16 1.62] |
|  |  |  | 3 | 160 | 1 |  | 160 | [-1.87 1.80] |
|  | 25RB | 2 | -4 | -- |  | -- | -- | -- |
|  |  |  | -3 | 40 | 5 |  | 800 | [1.18 1.97] |
|  |  |  | 3 | 160 | 1 |  | 160 | [-1.52 1.91] |
|  |  | 4 | -4 | 160 | 1 |  | 800 | [-0.80 1.88] |
|  |  |  | -3 | 160 | 1 |  | 480 | [-1.16 1.91] |
|  |  |  | 0 | 160 | 1 |  | 160 | [-1.80 1.88] |
|  |  |  | 3 | 160 | 1 |  | 160 | [-1.05 1.05] |
|  |  | 8 | -4 | 160 | 1 |  | 480 | [-1.10 0.90] |
|  |  |  | -3 | 160 | 1 |  | 160 | [-1.70 1.23] |
|  |  |  | 0 | 160 | 1 |  | 160 | [-0.96 0.81] |
|  |  |  | 3 | 160 | 1 |  | 160 | [-0.60 0.53] |
|  | 50RB | 2 | -4 | 160 | 1 |  | 480 | [-1.18 1.32] |
|  |  |  | -3 | 160 | 1 |  | 480 | [-1.10 1.12] |
|  |  |  | 0 | 160 | 1 |  | 160 | [-1.40 1.11] |
|  |  |  | 3 | 160 | 1 |  | 160 | [-0.90 0.76] |
|  |  | 4 | -4 | 160 | 1 |  | 160 | [-1.87 1.59] |
|  |  |  | -3 | 160 | 1 |  | 160 | [-1.62 1.29] |
|  |  |  | 0 | 160 | 1 |  | 160 | [-0.98 0.83] |
|  |  |  | 3 | 160 | 1 |  | 160 | [-0.61 0.55] |
|  |  | 8 | -4 | 160 | 1 |  | 160 | [-1.28 1.67] |
|  |  |  | -3 | 160 | 1 |  | 160 | [-1.07 1.36] |
|  |  |  | 0 | 160 | 1 |  | 160 | [-0.68 0.84] |
|  |  |  | 3 | 160 | 1 |  | 160 | [-0.48 0.52] |

Based on the simulation results in tables above, the following observations can be obtained: Observation 2: CSI-RS provides better performance when the RE numbers per RB increases. And CSI-RS with configuration of 8REs/RB could achieve similar performance as CRS when SINR is 0 dB and 3dB.

- Conclusion

In this contribution, simulation results of DRS RSRP measurement for SCE are provided. From the simulation results, several observations could be obtained:

Observation 1: The CRS based RSRP requirements could hardly be satisfied when the SNR is -10dB in any configuration in Table A-1.

Observation 2: CSI-RS provides better performance with the increase of RE numbers per RB. And CSI-RS with configuration of 8REs/RB could achieve similar performance as CRS when SINR is 0 dB and 3dB.

Proposal 1: The existing CRS based RSRP measurement requirements and side condition could be reused.

- Reference

- R4-71AH-0093, Simulation assumptions for DRS based RSRP accuracy for SCE, Huawei, HiSilicon.

- R4-143905, WF on tightening absolute RSRP requirements, CMCC.

- Appendix1: Simulation Assumptions

Simulation assumptions are listed in Table A-1 and Table A-2.

Table A-1: Simulation Assumptions for CRS based RSRP measurement

| Parameters | Value | Comments |
| --- | --- | --- |
| SNR | {-10, -8, -3, 0} dB |  |
| Measurement Bandwidth | {6 RBs, 25RBs, 50 RBs } |  |
| Number of Tx Antennas | {1} |  |
| Number of Rx Antennas | 2 |  |
| Antenna Correlation | Low |  |
| DRS occasion periodicity (M) | 160/80/40 | A DRS occasion is transmitted every M ms |
| DRS occasion duration (N) | 1/3/5 | A DRS occasion for a cell comprises N consecutive subframes. The cases of duration more than 1 subframe are optional for study. |
| Measurement Period | i*160ms, i={1,3,5} |  |
| Number of Samples per Measurement Period | - | Implementation-dependent (NOTE 1) |
| L3 filtering | Disable |  |
| DRX | OFF |  |
| Propagation Condition | AWGN, EPA5, ETU30 |  |
| CP Length | Normal |  |
| NOTE 1: Companies are requested to provide the details of the measurement sampling rate for interpretation and comparison of the results. |  |  |

Table A-2: Simulation Assumptions for CSI-RS based RSRP measurement

| Parameters | Value | Comments |
| --- | --- | --- |
| SINR | {-4, -3, 0, 3} dB |  |
| Measurement Bandwidth | {6 RBs, 25RBs, 50RBs} |  |
| Number of Tx Antennas | 1 |  |
| Number of Rx Antennas | 2 |  |
| Antenna Correlation | Low |  |
| CSI reference signals | Antenna port {15} |  |
| DRS occasion periodicity (M) | 160/80/40 | A DRS occasion is transmitted every M ms |
| DRS occasion duration (N) | 1/3/5 |  |
| CSI-RS reference signal configuration | config.2 with 2 REs/RB, config.2 with 8 REs/RB | config.2 with 4 REs/RB is optional for study. |
| Measurement Period | i*160ms, i={1,3,5} |  |
| Number of Samples per Measurement Period | - | Implementation-dependent (NOTE 1) |
| L3 filtering | Disable |  |
| DRX | OFF |  |
| Propagation Condition | AWGN, EPA5, ETU30 |  |
| CP Length | Normal |  |
| NOTE 1: Companies are requested to provide the details of the measurement sampling rate for interpretation and comparison of the results. |  |  |