3GPP TSG-RAN WG4 RAN4 #72                                         R4-144530

Dresden, Germany, 18 August - 23 August, 2014

Agenda item:	7.12.2

Source: 	Samsung

Title:			   Performance Evaluation of Advanced SU-MIMO Receiver

Document for:	Discussion

1	Introduction

In 3GPP RAN4 #71 meeting, RAN4 has agreed on a set of candidate test cases under single-cell high geometry scenario for performance calibration among companies as shown below.

Table 1: Single-Cell Test Cases for Performance Calibration

| Test Case | Test setup reference in 36.101 | TM | Duplex Mode | Antenna configuration | Fading channel | Mod |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 8.2.1.3.1 | TM3 | FDD | 2x2 Medium | EVA 70 | 16QAM |
| 2 | 8.2.1.4.2 | TM4 | FDD | 2x2 Medium | ETU 70 | 16QAM |
| 3 (option 1) | 8.3.1.2 | TM9 | FDD | 2x2 Medium | EPA 5 | 16QAM |
| 3 (option 2) | 8.3.1.2 | TM9 | FDD | 2x2 Medium | ETU 5 | 16QAM |

In this contribution, we presented the simulation results of single-cell cases for alignment purpose.

- Alignment Results of Single-Cell Test Case

In this Section, we presented the simulation results of the agreed single-cell cases with R-ML receiver. At the same time, the performance of legacy MMSE-IRC receiver is also provided for comparison.

The throughput performances of the agreed tests cases are listed in Figure 1 to Figure 4 as shown below.

Furthermore, the required SNR @70% throughput are also summarized in Table 2 below.

Table 2: Required SNR @70% Throughput

| Test Case | Test setup reference in 36.101 | TM | Duplex Mode | Antenna configuration | Fading channel | Mod | R-ML (dB) | MMSE (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8.2.1.3.1 | TM3 | FDD | 2x2 Medium | EVA 70 | 16QAM | 15.7 | 17.7 |
| 2 | 8.2.1.4.2 | TM4 | FDD | 2x2 Medium | ETU 70 | 16QAM | 16.3 | 18.3 |
| 3 (option 1) | 8.3.1.2 | TM9 | FDD | 2x2 Medium | EPA 5 | 16QAM | 14.9 | 16.5 |
| 3 (option 2) | 8.3.1.2 | TM9 | FDD | 2x2 Medium | ETU 5 | 16QAM | 15.9 | 17.5 |

Figure 1: Performance of Test Case 1 (TM3, 2x2, Medium, EVA 70Hz, 16QAM case)

Figure 2: Performance of Test Case 2 (TM4, 2x2, Medium, ETU 70Hz, 16QAM case)

Figure 3: Performance of Test Case 3, Option 1 (TM9, 2x2, Medium, EPA 5Hz, 16QAM case)

Figure 4: Performance of Test Case 3, Option 2 (TM9, 2x2, Medium, ETU 5Hz, 16QAM case)

- Conclusion

In this contribution, we presented the simulation results of single cell cases for alignment purpose.

- Reference

- N/A