3GPP TSG-RAN WG4 Meeting #72 	R4-145496

Dresden, Germany, 18 – 22 Aug, 2014

Source:	KDDI

Title:	TP for TR36.851: On Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (26+41)

Agenda item:	7.7.5.2

Document for:	Approval

1.	Introduction

WI for CA_B26-B41-B41 was approved in RAN#64.  Co-existence studies of harmonics and intermodulation should be done as a nominal working procedure.  This contribution provides the studies and proposes to capture results into TR36.851.

.

2.	Text Proposal

<Start of TP>

7.2.X.1.2	Co-existence studies for CA_26-41

Table 7.2.X.1.2-1gives the intermodulation products for Band 26 + Band 41 CA with 2 DLs. For the IMD analysis the maximum transmission as defined in table 7.2.X.1.1-1 is considered. It can be seen from the table that the 2nd harmonics of BS transmitting in Band 26 may fall into the BS receive band of Bands 3, 4, 9 and 10 and the 3rd harmonics of BS transmitting in Band 26 may fall into the BS receive band of Bands 38 and 41, while the 2nd IMD products caused by BS supporting CA of Band 26 and Band 41 may fall into the BS receive band of Band 3, 4, 9, 10, 22, 24 and 42, and the 3rd IMD products may fall into the BS receive band of Bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 20, 26, 27, 28, 38, 41 and 44. Therefore, sharing antenna for CA operation between Band 26 and Band 41 is not recommended.  In addition, when one deploys CA_26-41 operation in certain region, these co-existence studies should be taken into account because harmonics and IMDs could fall into twenty-two bands in total.

Table 7.2.X.1.2-1: 2DLs B26 + B41 harmonics and IMD products frequency limits

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 859 | 894 | 2496 | 2690 |
| 2nd order harmonics frequency range (MHz) | 1718 | 1788 | 4992 | 5380 |
| 3rd order harmonics frequency range (MHz) | 2577 | 2682 | 7488 | 8070 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 1602 | 1831 | 3355 | 3584 |
| 3rd order IMD products | ¦f2_high – 2*f1_low¦ | ¦f2_low – 2*f1_high¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 708 | 972 | 4098 | 4521 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 4214 | 4478 | 5851 | 6274 |
| 3rd order IMD products | ¦f1_low – f2_high + f2_low¦ | ¦f1_high + f2_high – f2_low¦ | ¦f2_low – f1_high + f1_low¦ | ¦f2_high + f1_high – f1_low¦ |
| IMD frequency limits (MHz) | 665 | 1088 | 2461 | 2725 |
| 3rd order IMD products (with maximum channel bandwidth) | ¦f1_low – max BW f2¦ | ¦f1_high + max BW f2¦ | ¦f2_low – max BW f1¦ | ¦f2_high + max BW f1¦ |
| IMD frequency limits (MHz) | 839 | 914 | 2486 | 2700 |

<End of TP>