3GPP TSG-RAN WG4 RAN4 #72                                     R4 -144515

Dresden, Germany, 18th – 22th Aug, 2014

Agenda item:	7.23.1

Source: 	Samsung

Title: 	Updated UL configuration for REFSENS on CA_25A-25A

Document for:	Discussion

- Introduction

Additional bandwidth combinations sets of 15 and 20MHz for CA_25A-25A were approved in RAN#63 and RAN4#70bis[1-2]. In last meeting, RB start and RB numbers were proposed in our former contribution by using CA_2A-2A and CA_3A-3A as reference cases[3], due to all three bands operate near 2GHz with similar passband bandwidth and duplexer gap. And in this contribution, relaxation values are additionally proposed based on our previous discussion paper.

- Discussion

In our previous contribution[3], RB allocation for CA_25A-25A were proposed based on using CA_2A-2A as reference case. Band 2 is within Band 25, as shown in Fig1. Therefore for PCC size of 15 and 20MHz for Band 25, RB allocation and Wgap mid-point could reuse CA_2A-2A as reference, which as shown in Table 1. For example, the Wgap mid-point is kept the same, but Wgap range for Band 25 is 5MHz larger than Band 2 due to the 5MHz more passband bandwidth for Band 25. As RB allocation are evaluated for the worst case, for PCC of 5 and 10MHz, the RB allocations are kept the same when SCC increases from 5 and 10MHz to 15 and 20MHz, due to the minimum WUL-DL aren’t change, while for PCC of 15 and 20MHz, RB allocations are estimated using CA_2A-2A reference cases.

Fig 2 Frequency arrangement of Band 2 and Band 25

Based on the RB allocation for CA_25A-25A in [2], the relaxation values are proposed which taking into account of narrower duplexer gap and 1.5dB looser REFSNES level compared to CA_2A-2A.

Table.1 Updated UL configuration for CA_25A-25A

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_25A-25A | 25RB+25RB | 30.0 < Wgap ≤ 55.0 | 101 | 5.0 | FDD |
|  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 25RB+50RB | 25.0< Wgap ≤ 50.0 | 101 | 4.5 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0 |  |
|  | 25RB+75RB | 20.0< Wgap ≤ 45.0 | 101 | [4.3] |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 251 | 0 |  |
|  | 25RB+100RB | 15.0< Wgap ≤ 40.0 | 101 | [4.0] |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 251 | 0 |  |
|  | 50RB+25RB | 15.0 < Wgap ≤ 50.0 | 104 | 5.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 50RB+50RB | 10.0 < Wgap ≤ 45.0 | 104 | 5.0 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 50RB+75RB | 5.0 < Wgap ≤ 40.0 | 104 | [4.5] |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 50RB+100RB | 0.0 < Wgap ≤ 35.0 | 104 | [4.1] |  |
|  | 75RB+25RB | 10.0 < Wgap ≤ 45.0 | 105 | [6.5] |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 75RB+50RB | 5.0 < Wgap ≤ 40.0 | 105 | [5.4] |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 75RB+75RB | 0.0 < Wgap ≤ 35.0 | 105 | [4.9] |  |
|  | 75RB+100RB | 0.0 < Wgap ≤ 30.0 | 105 | [4.5] |  |
|  | 100RB+25RB | 0.0 < Wgap ≤ 40.0 | 126 | [6.9] |  |
|  | 100RB+50RB | 0.0 < Wgap ≤ 35.0 | 126 | [5.8] |  |
|  | 100RB+75RB | 0.0 < Wgap ≤ 30.0 | 126 | [5.2] |  |
|  | 100RB+100RB | 0.0 < Wgap ≤ 25.0 | 126 | [4.9] |  |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The carrier center frequency of PCC in the UL operating band is configured closer to the DL operating band. NOTE 4: 4 refers to the UL resource blocks shall be located at RBstart=33. NOTE 5: 5 refers to the UL resource blocks shall be located at RBstart=56. NOTE 6: 6 refers to the UL resource blocks shall be located at RBstart=67. |  |  |  |  |  |

- Conclusion

In this contribution, updated RB allocation and relaxation values for CA_25A-25A are proposed using methodology previously used before for CA_2A-2A and CA_3A-3A, and taking CA_2A-2A as reference case.

- Reference

- RP-140452: "Additional bandwidth combinations for LTE Advanced intra-band non-contiguous Carrier Aggregation in Band 25".

- R4-142444, “Additional bandwidth combinations for intra-band NC CA in Band 25”, Huawei

- R4-143287, “UL configuration for REFSENS on CA_25A-25A considering additional bandwidth combinations”, Samsung