3GPP TSG-RAN WG4 Meeting #72 	R4-144645

Dresden, Germany, 18 – 22 Aug, 2014

Source:	KDDI

Title:	TP for TR36.851: On Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (26+41)

Agenda item:	7.7.5.2

Document for:	Approval

1.	Introduction

WI for CA_B26-B41-B41 was approved in RAN#64.  Co-existence studies of harmonics and intermodulation should be done as a nominal working procedure.  This contribution provides the studies and proposes to capture results into TR36.851.

.

2.	Text Proposal

<Start of TP>

7.2.X.1.2	Co-existence studies for CA_26-41

Table 7.2.X.1.2-1gives the intermodulation products for Band 26 + Band 41 CA with 2 DLs. For the IMD analysis the maximum transmission as defined in table 7.2.X.1.1-1 is considered. 3rd harmonics of Band 26 DL fall into the receive frequency range of Band 41. Furthermore, it can be seen that the 3rd order intermodulation products generated by two operating bands fall into the frequency ranges of both Band 26 UL and Band41 as well.  Therefore, sharing antenna for CA operation between Band 26 and Band 41 is not recommended.

Table 7.2.X.1.2-1: 2DLs B26 + B41 harmonics and IMD products frequency limits

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 859 | 894 | 2496 | 2690 |
| 2nd order harmonics frequency range (MHz) | 1718 | 1788 | 4992 | 5380 |
| 3rd order harmonics frequency range (MHz) | 2577 | 2682 | 7488 | 8070 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 1602 | 1831 | 3355 | 3584 |
| 3rd order IMD products | ¦f2_high – 2*f1_low¦ | ¦f2_low – 2*f1_high¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 708 | 972 | 4098 | 4521 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 4214 | 4478 | 5851 | 6274 |
| 3rd order IMD products | ¦f1_low – f2_high + f2_low¦ | ¦f1_high + f2_high – f2_low¦ | ¦f2_low – f1_high + f1_low¦ | ¦f2_high + f1_high – f1_low¦ |
| IMD frequency limits (MHz) | 665 | 1088 | 2461 | 2725 |
| 3rd order IMD products (with maximum channel bandwidth) | ¦f1_low – max BW f2¦ | ¦f1_high + max BW f2¦ | ¦f2_low – max BW f1¦ | ¦f2_high + max BW f1¦ |
| IMD frequency limits (MHz) | 839 | 914 | 2486 | 2700 |

<End of TP>