TSG-RAN Working Group 4 (Radio) meeting #72	R4-144693

Dresden, Germany, 18 - 22 Aug, 2014

Source:	Nokia Networks

Title:	TP for TR 36.851: The clarification of LTE TDD-FDD joint operation in BS RF requirement

Agenda item:	7.39.3.1 [TEI12]

Document for:	Approval

# Introduction

It was discussed whether multi-band configurations should be include for TDD-FDD joint operation until RAN4#7bis. The following wayforward [1] was agreed to exclude such multi-band TDD-FDD configuration.

- The discussion on feasibility of multi-band BS configuration supporting both FDD and TDD bands shall be decoupled from the completion of this WI.

  - Multi-band BS configuration supporting both FDD and TDD bands is not supported in Release 12 BS specification

It was also discussed in RAN4#71 how to include the agreement [2-5] and additional agreement [6] was also made. However the agreement was not included in the BS specs as explained in [7].

In this contribution, it is proposed to capture the agreement in TR 36.851 after the issues are sorted out as proposed in [7].

# Discussion

Because of the agreed wayfoward [1], the assumption in MB-MSR work [8] is still valid for this WI, “The MB-MSR application scenarios also cover both FDD and TDD, but are limited to MB-MSR for FDD only scenarios and TDD only scenarios respectively.”  Therefore, there is no need to include new transmitter or receiver requirement for multi-band BS configurations supporting both FDD and TDD bands in Release 12.

Thus the following text is proposed for TR36.851. The corresponding CRs [9-12] to BS specs are also submitted in this meeting.

For TDD-FDD joint operation, multi-band BS configuration supporting both TDD and FDD is excluded in Release 12. In principle, separate active RF components for TDD and FDD are assumed to support TDD-FDD CA. The RF requirements are applied separately for TDD and FDD. Therefore, the requirements in clause 6 and clause 7 of BS core specs, TS36.104 and 37.104, do not need to be modified to introduce TDD-FDD joint operation. This shall be captured in the BS core specs in the following.

- The RF requirements in the present release of this specification are not applicable for a BS with multi-band transmitter and/or multi-band receiver operating TDD and FDD bands simultaneously.

# Conclusion

It is proposed that the attached text proposal below is approved and included into TR 36.851.

# Reference

- R4-142486	Way forward on Multi-band BS for TDD-FDD joint operation including Carrier Aggregation, NTT DOCOMO, NSN, Ericsson

- R4-143710	Outstanding issues for future TDD-FDD CA work, Huawei

- R4-143433	CA TDD+FDD BS Impact, Ericsson

- R4-143488	Recommendations on LTE BS RF requirements for TDD-FDD joint operation including Carrier Aggregation, Alcatel-Lucent

- R4-143485	TP for TR 36.851: The clarification of LTE TDD-FDD joint operation in BS RF requirement, NSN

- R4-144021	Way forward on BS configurations for LTE TDD-FDD joint operation including Carrier Aggregation, NTT DOCOMO

- R4-144688	Exclusion of TDD FDD multiband BS configuration and clarification of common antenna port of multiple radios, Nokia Networks

- TR37.812 ver 11.3.0	Radio Frequency (RF) requirements for Multi-band and Multi-standard radio (MB-MSR) Base Station (BS)

- R4-144689	Clarification of multiband TDD-FDD BS in TS36.104, Nokia Networks

- R4-144690	Clarification of multiband TDD-FDD BS in TS36.141, Nokia Networks

- R4-144691	Clarification of multiband TDD-FDD BS in TS37.104, Nokia Networks

- R4-144692	Clarification of multiband TDD-FDD BS in TS37.141, Nokia Networks

TEXT PROPOSAL:

<start of text proposal >

7            TDD-FDD Inter-band Carrier aggregation

## 7.1         General part

### 7.1.1      BS specific

For TDD-FDD joint operation, multi-band BS configuration supporting both TDD and FDD is excluded in Release 12. In principle, separate active RF components for TDD and FDD are assumed to support TDD-FDD CA. The RF requirements are applied separately for TDD and FDD. Therefore, the requirements in clause 6 and clause 7 of BS core specs, TS36.104 and 37.104, do not need to be modified to introduce TDD-FDD joint operation. This shall be captured in the BS core specs in the following.

- The RF requirements in the present release of this specification are not applicable for a BS with multi-band transmitter and/or multi-band receiver operating TDD and FDD bands simultaneously.

<end of text proposal >