3GPP TSG-RAN WG4 Meeting #72	R4-145137

# Dresden, Germany, 18 – 22 August, 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.133 | CR | 1360 | rev | 1 | Current version: | 11.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for new requirements for reselection from a CSG to an inter-frequency CSG cell or inter-RAT E-UTRA CSG cell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI11 |  |  |  |  |  |  | Date: |  | 2014-xx-xx |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-11 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | As per LS from RAN5 to RAN4, in TS 34.123-1, Test Case 6.3.3.2 - Inter frequency CSG Cell Reselection / UE is in Idle, Cell_PCH and URA_PCH states – checks to verify that the UE reselects from a CSG cell to an allowed CSG cell on another frequency if the detected CSG cell is the highest ranked cell, according to clause 5.2.6.1.4 of TS 25.304 when UE is in Idle state. As per TS 25.304 Clause 5.2.6.4.3, for reselection to neighbour CSG cells listed in the system information of the serving CSG cell, the UE shall apply cell measurement rules according to clause 5.2.6. If the UE detects a suitable CSG cell that is listed in the system information of the serving cell on other frequencies, and the detected CSG cell is the strongest cell on that frequency, then the UE shall consider these frequencies to have a priority equal to the current frequency and shall follow cell reselection rules according to clauses 5.2.6.1.4 and 5.2.6.1.4a. There are no core specification requirements in TS 25.133 for Inter frequency CSG Cell reselection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add requirements for reselection from a CSG to an inter-frequency CSG cell and requirements for reselection from a CSG to an inter-RAT E-UTRA CSG cell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | As performance requirements for CSG Cell reselection is missing in 25.133 specifications, unspecified and unagreed requirements will be tested causing a unspecified behaviour to be tested on an otherwise compliant UE to fail the TS 34.123-1, Test Case 6.3.3.2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated impact analysis: there is no impact to BS implementation.. |

#### 4.2.2.11	Reselection to CSG cells

Note:	Requirements in this section are minimum requirements defined to ensure the testability of autonomous CSG search.

Reselection from non CSG to CSG cells may be performed using UE autonomous search as defined in [1] when at least one CSG ID is included in the UE’s CSG whitelist.  The requirements in this section are valid for reselection to CSG cells previously visited by the UE when the radio configuration parameters, including the carrier frequency and physical cell identity of the CSG cell, non CSG cell and other neighbour cells are unchanged from the most recent previous visit.

NOTE:	According to [1], the UE autonomous search function, per UE implementation, determines when and/or where to search for allowed CSG cells.

##### 4.2.2.11.1	Reselection from a non CSG to an inter-frequency CSG cell

The UE shall perform search and reselection to an allowed inter-frequency CSG cell that has met CSG reselection criterion defined in [1] and that is in its whitelist, within 6 minutes in the conditions shown in table 4.2.2.11.1-1. There is no need for statistical testing of this requirement.

Table 4.2.2.11.1-1: Parameters for CSG inter-frequency reselection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| UARFCN Note1 |  | Channel 1 | Channel 2 |
| CSG indicator |  | False | True |
| Primary scrambling code Note1 |  | Scrambling code 1 | Scrambling code 2 |
| CSG identity |  | Not sent | Sent (Already stored in UE whitelist from previous visit) |
| Propagation conditions |  | Static, non multipath |  |
| CSG cell previously visited by UE |  | Yes |  |
| CPICH_RSCPNote2 | dBm | -100 | -100 |
| CPICH_Ec/Ior | dB | -10 | -10 |
| PCCPCH_Ec/Ior | dB | -12 | -12 |
| SCCPCH_Ec/Ior | dB | -12 | -12 |
| AICH_Ec/Ior | dB | -15 | -15 |
| SCH_Ec/Ior | dB | -15 | -15 |
| PICH_Ec/Ior | dB | -15 | -15 |
|  | dBm/3.84 MHz | Off | Off |
| Note 1: For this requirement to be applicable, the UARFCN and scrambling code for cell 1 and cell 2 shall be unchanged from when the CSG cell was visited previously. Note 2: Chosen to ensure that CSG autonomous search has a high probability of success on every attempt made by UE. |  |  |  |

##### 4.2.2.11.2	Reselection from a non CSG to an inter-RAT E-UTRA CSG cell

The UE shall perform search and reselection to an allowed inter-RAT E-UTRA CSG cell that has met CSG reselection criterion defined in [1] and that is in its whitelist, within 6 minutes in the conditions shown in table 4.2.2.11.2-1. There is no need for statistical testing of this requirement.

Table 4.2.2.11.2-1: Parameters for CSG inter-RAT E-UTRA reselection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| UARFCN Note1 |  | Channel 1 | N/A |
| EARFCN Note1 |  | N/A | Channel 2 |
| CSG indicator |  | False | True |
| Primary scrambling code Note1 |  | Scrambling code 1 | N/A |
| Physical cell identityNote1 |  | N/A | 2 |
| CSG identity |  | Not sent | Sent (Already stored in UE whitelist from previous visit) |
| Propagation conditions |  | Static, non multipath |  |
| CSG cell previously visited by UE |  | Yes |  |
| CPICH_RSCPNote2 | dBm | -100 | N/A |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCCPCH_Ec/Ior | dB | -12 |  |
| AICH_Ec/Ior | dB | -15 |  |
| SCH_Ec/Ior | dB | -15 |  |
| PICH_Ec/Ior | dB | -15 |  |
|  | dBm/3.84 MHz | Off |  |
| PBCH_RA | dB | N/A | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm |  | -140 |
|  | dBm/15 kHz |  | Off |
| RSRP Note2 | dBm/15 KHz |  | -110 |
| Note 1: For this requirement to be applicable, the UARFCN and scrambling code for cell 1 and the EARFCN and physical cell identity for cell 2 shall be unchanged from when the CSG cell was visited previously Note 2: Chosen to ensure that CSG autonomous search has a high probability of success on every attempt made by UE |  |  |  |

## <Start of Changes>

##### 4.2.2.11.3	Reselection from a CSG to an inter-frequency CSG cell

If 0< Sx <= Sintersearch or when Sintersearch is not present, and the serving cell is the highest ranked cell on the serving frequency, the UE shall perform search and reselection to an allowed inter-frequency CSG cell that has met CSG reselection criterion defined in [1] and that is in its whitelist within 6 minutes, provided that this inter-frequency CSG cell is listed in system information without absolute priority configured.

If 0< Srxlevservingcell <= Sprioritysearch1 or 0< Squalservingcell <= Sprioritysearch2, and the serving cell is the highest ranked cell on the serving frequency, the UE shall perform search and reselection to an allowed inter-frequency CSG cell that has met CSG reselection criterion defined in [1] and that is in its whitelist within 6 minutes, provided that this inter-frequency CSG cell is listed in system information with absolute priority.

If Sx<=0 or the serving cell is not the highest ranked cell on the serving frequency, the requirements in section 4.2.2.3 applies.

There is no need for statistical testing of this requirement.

Table 4.2.2.11.3-1: Parameters for CSG inter-frequency reselection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| UARFCN Note1 |  | Channel 1 | Channel 2 |
| CSG indicator |  | True | True |
| Primary scrambling code Note1 |  | Scrambling code 1 | Scrambling code 2 |
| CSG identity |  | Not sent | Sent (Already stored in UE whitelist from previous visit) |
| Propagation conditions |  | Static, non multipath |  |
| CSG cell previously visited by UE |  | Yes |  |
| CPICH_RSCPNote2 | dBm | -100 | -100 |
| CPICH_Ec/Ior | dB | -10 | -10 |
| PCCPCH_Ec/Ior | dB | -12 | -12 |
| SCCPCH_Ec/Ior | dB | -12 | -12 |
| AICH_Ec/Ior | dB | -15 | -15 |
| SCH_Ec/Ior | dB | -15 | -15 |
| PICH_Ec/Ior | dB | -15 | -15 |
|  | dBm/3.84 MHz | Off | Off |
| Note 1: For this requirement to be applicable, the UARFCN and scrambling code for cell 1 and cell 2 shall be unchanged from when the CSG cell was visited previously. Note 2: Chosen to ensure that CSG autonomous search has a high probability of success on every attempt made by UE. |  |  |  |

##### 4.2.2.11.4	Reselection from a CSG to an inter-RAT E-UTRA CSG cell

If 0< Srxlevservingcell <= Sprioritysearch1 or 0< Squalservingcell <= Sprioritysearch2, and the serving cell is the highest ranked cell on the serving frequency, the UE shall perform search and reselection to an allowed inter-RAT E-UTRA CSG cell that has met CSG reselection criterion defined in [1] and that is in its whitelist within 6 minutes, provided that this inter-RAT E-UTRA CSG cell is listed in system information with absolute priority.

If Sx<=0 or the serving cell is not the highest ranked cell on the serving frequency, the requirements in section 4.2.2.5a applies.

There is no need for statistical testing of this requirement.

Table 4.2.2.11.4-1: Parameters for CSG inter-RAT E-UTRA reselection

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| UARFCN Note1 |  | Channel 1 | N/A |
| EARFCN Note1 |  | N/A | Channel 2 |
| CSG indicator |  | True | True |
| Primary scrambling code Note1 |  | Scrambling code 1 | N/A |
| Physical cell identityNote1 |  | N/A | 2 |
| CSG identity |  | Not sent | Sent (Already stored in UE whitelist from previous visit) |
| Propagation conditions |  | Static, non multipath |  |
| CSG cell previously visited by UE |  | Yes |  |
| CPICH_RSCPNote2 | dBm | -100 | N/A |
| CPICH_Ec/Ior | dB | -10 |  |
| PCCPCH_Ec/Ior | dB | -12 |  |
| SCCPCH_Ec/Ior | dB | -12 |  |
| AICH_Ec/Ior | dB | -15 |  |
| SCH_Ec/Ior | dB | -15 |  |
| PICH_Ec/Ior | dB | -15 |  |
|  | dBm/3.84 MHz | Off |  |
| PBCH_RA | dB | N/A | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm |  | -140 |
|  | dBm/15 kHz |  | Off |
| RSRP Note2 | dBm/15 KHz |  | -110 |
| Note 1: For this requirement to be applicable, the UARFCN and scrambling code for cell 1 and the EARFCN and physical cell identity for cell 2 shall be unchanged from when the CSG cell was visited previously Note 2: Chosen to ensure that CSG autonomous search has a high probability of success on every attempt made by UE |  |  |  |

## <End of Changes>