3GPP TSG-RAN4 Meeting #72bis                                                            R4-145720

Singapore, 6-10 Oct , 2014

Source:	LightSquared, Inc.

# Title:	Corrections and Clarifications of UE specifications in TR 36.844 [FS_LTE_FDD_1670_US]

Agenda item:	9.3

Document for:	Approval

- Introduction

During RAN#59 (Vienna), the study item titled “Expansion of LTE_FDD_1670_US to include 1670-1680 MHz Band for LTE in the US” was approved [1].  The downlink spectrum covered by this study item is 1670 to 1680 MHz, and the uplink band coincides with Band 24 UL from 1626.5 to 1660.5 MHz. This TP clarifies some ambiguities and makes some corrections in the technical report of this SI, TR 36.844 for UE specifications.

# Reference

- RP-130202:  "Study Item Proposal: Expansion of LTE_FDD_1670_US to include 1670-1680 MHz Band for LTE in the US ", RAN #59, Feb. 2013.

------------- Start of TP------------------

<First Modification>

### 2          References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

..............................................

..............................................

[13]	TR 36.821, “Extended UMTS/LTE 1500 work item technical report”, V9.1.0, Rel 9, Mar 2010, www.3GPP.org

[14]	TR 36.821: " UMTS / LTE in 800 MHz for Europe”, V9.0.0 Rel 9, Mar 2010, www.3GPP.org.

[15]	TS 36.101: " User Equipment (UE) radio transmission and reception”, V11.3.0, Rel 11, Dec 2012, www.3GPP.org

[16]	Avago ACPM-5004, http://www.avagotech.com/pages/en/rf_microwave/amplifiers/linear_power/umts_pa/acpm-5004/

[17]	R4-080710, “TP for UE Spurious emission limits”, RAN4#46bis, Shenzhen, People Republic of China, Mar 31 2008

[18]	“Cascade SKY33100-360LF BAW Filters for Enhanced Near Band Rejection, Application Note”, http://www.skyworksinc.com/uploads/documents/200627G.pdf, Skyworks Solutions Inc, August 19, 2008.

<Next Modification>

### 4	Background

Band 24 (L-Band) is standardized in 3GPP Rel-10 LTE under the work item "Adding L-Band (Band 24) LTE for Ancillary Terrestrial Component (ATC) of Mobile Satellite Service (MSS) in North America" (L_Band_LTE_ATC_MSS) UID_470010. The FCC has suspended the license to deploy ATC in both the upper and lower segments of Band-24 downlink in the United States (and the associated build-out requirements, pending further review). As a result, LightSquared has not commenced such deployment in this band, and will not use Band 24 downlink until the FCC and NTIA approve such use. On December 20, 2012, the FCC tolled (temporarily suspended) LightSquared’s ATC build-out requirement, pending action on LightSquared’s proposals to resolve concerns regarding potential interference to GPS receivers, including the proposals discussed here (Order in IB Docket No. 12-296, DA 12-2051 December 20, 2012).

Figure 4.1: LightSquared Spectrum Holding

In addition, as depicted in Figure 4.1, LightSquared has contractual and regulatory authority to use the 5 MHz of spectrum associated with the FCC’s nationwide license for 1670-1675 MHz (CH5 in Figure 4.1). FCC Rules allow 2000 Watts peak EIRP from base station, and 4 Watts peak EIRP from mobile terminals (CFR Title 47 §27.50(f) [3]).

LightSquared has requested to expand the downlink band to 10 MHz bandwidth, i.e. 1670-1680 MHz and has identified options for pairing this band with uplink spectrum at 1627.5-1637.5 or 1646.7-1656.7 MHz (see Figure 4.2). The FCC recently issued public notices for LightSquared’s requests to operate its terrestrial network using the downlink frequencies at 1670-1675 MHz and 1675-1680 MHz, and the comment cycle was completed in early January 2013 without serious objections.

<Next Modification>

### Existing Rules for 1670-1675 MHz.

FCC rules allow 2000 Watts peak EIRP from base stations, and 4 Watts peak EIRP from mobile terminals (47 CFR §27.50(f)) in the 1670-1675 MHz band.

### 5.1.3.1     Meteorological Satellite Earth Stations

Although the 1670-1675 MHz band has been generally unencumbered, there are three vital federal Geostationary Operational Environmental Satellite System (GOES) earth stations adjacent to this band (centred at 1676 MHz).  47 C.F.R §1.924(g) (1) establishes coordination zones around downlinks for these earth stations, which are located at Wallops Island, Virginia; Fairbanks, Alaska; and Greenbelt, Maryland. The coordination zones for Wallops Island and Fairbanks are specified locations bounded by circles with a radius of 100 km (62.1 miles) each, while the Greenbelt zone is a specified location bounded by a circle with a radius of 65 km (40.4 miles). For the 30 Cellular Market Areas (CMAs) defined in FCC 07-16, where the transmission power could be greater than 2 kW (i.e., high power), the radius of the circle for the coordination zone surrounding Wallops Island and Fairbanks increases to 180 km and the radius of the circle for the coordination zone for Greenbelt increases to 100 km. Even considering the larger coordination zones in Fairbanks and Greenbelt, only 4% of USA PoPs would be impacted around these three locations.

### Current Rules for 1675-1680 MHz

On November 2, 2012, LightSquared asked for FCC authority to use the 1675-1680 MHz band to provide a commercially-useable terrestrial wireless broadband service as part of a contiguous 10 MHz downlink channel.  The 5 MHz band at 1675–1680 MHz will be shared with existing federal government users.  LightSquared’s proposal is uniquely suited to protecting the integrity of essential government operations in the 1675-1680 MHz band.  The FCC has accepted public comment on the proposal, and the comment cycle has been closed.

The 1675-1680 MHz band currently is allocated on a primary basis for both non-Federal and Federal use by the Meteorological Aids (MetAids) and Meteorological-Satellite (MetSat) Services; it is not allocated for terrestrial mobile service. LightSquared’s FCC filings recognize that its proposed use of 1675-1680 MHz may require modification of the Commission’s rules in order to facilitate the prompt processing and grant of its applications.

There are two downlink MetSat services deployed in 1675-1680 MHz band that must be protected from any terrestrial operation in the same band: 1) Sensor Data Link (SD) on current GOES-NOP (centred at 1676 MHz) , and 2) Data Collection Platform Report (DCPR) on future GOES-R satellite (centred at 1679.9 MHz and 1680.2 MHz). While geographic locations for both SD and DCPR are known based on NTIA’s report, currently there are no rules specifying protection zones around these sites.

### List of band specific issues for LTE FDD in 1670-1680MHz and band 24 UL

- General issues

  - Co-existence with nearby 3GPP bands

    - Co-existence with band 24

- E-UTRA issues

  - UE Duplexer

    - Potential receiver desensitization due to UL self interference

    - Potential device receiver blocking due to self transmission in UL

  - UE REFSENS

  - Mobile to Mobile Interference

- MSR issues

### General Issues

Figure 7.1: Proposed Band including B24 UL

Figure 7.1 depicts the proposed LTE band. The LTE deployment in this band will adhere to all 3GPP terrestrial out-of-band requirements for spurious emissions including those for UE, eNB, and UE-to-UE emissions as will be defined in 3GPP TS 36.101, TS 36.104, 3GPP TS 25.101, and other relevant documents.

The downlink spectrum covered by this study item is 1670 to 1680 MHz, and the corresponding paired 10 MHz uplink band is included within band 24 UL, 1626.5 to 1660.5 MHz (Figure 7.1). In particular the proposed UL band is either 1627.5 to 1637.5 MHz, or 1646.7 to 1656.7 MHz. These configurations are depicted in Figure 7.2.

Figure 7.2: Proposed UL & DL Pairings (a) Pairing Lower UL Carrier (b) Pairing Upper UL Carrier

As depicted in Figure 7.2, the band would consider both 5 MHz and 10 MHz channel bandwidths. The 5 MHz deployment is considered for the markets with exclusion zone requirements to coordinate with weather services in 1675-1680 MHz. Tables 7.1 and 7.2 depict the proposed operating band configurations. We refer to UL pairing with smaller duplexing gap (Table 7.1) as Option 1, and the UL pairing with larger duplexing gap (Table 7.2) as Option 2.

Table 7.1: LTE proposed operating band configuration; Option 1

| E-UTRA operating Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | TRX separation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE transmit / BS receive |  |  | Channel BW [MHz] | UE receive / BS transmit |  |  | Channel BW [MHz] |  |
|  | FUL_low (MHz) – FUL_high (MHz) |  |  |  | FDL_low (MHz) – FDL_high (MHz) |  |  |  |  |
| XX | 1646.7 | - | 1656.7 | [5]/[10] | 1670 | – | 1680 | [5]/[10] | 23.3 MHz |

Table 7.2: LTE proposed operating band configuration; Option 2

| E-UTRA operating Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | TRX separation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UE transmit / BS receive |  |  | Channel BW [MHz] | UE receive / BS transmit |  |  | Channel BW [MHz] |  |
|  | FUL_low (MHz) – FUL_high (MHz) |  |  |  | FDL_low (MHz) – FDL_high (MHz) |  |  |  |  |
| XX | 1627.5 | - | 1637.5 | [5]/[10] | 1670 | – | 1680 | [5]/[10] | 42.5 MHz |

The band choices under study for 10 MHz pairing are either H and F, or H and G, as depicted in Figure 7.2. The distance between the UL and DL edges of the proposed band for 10 MHz carriers is 13.3 MHz for Option 1, and 32.5 MHz for Option 2. The passband is 10 MHz, with a UL-DL distance of either 23.3 MHz for Option 1 or 42.5 MHz for Option 2. These requirements may be challenging, but are not without precedence in 3GPP RAN4.  For example, bands 8, 26, and 28 have duplexing gap of 10 MHz, band 12 has a duplexing gap of 13 MHz, and band 25 has a duplexing gap of 15 MHz. The corresponding gaps for 5 MHz passband are 18.3 MHz for Option 1, or 37.5 MHz for Option 2.

<Next Modification>

### Study of E-UTRA specific issues for UEs

This section analyses the technical feasibilities of realizing RF components (based on currently available technologies) required for implementing the duplexing options presented in this report.  The analysis is performed by considering the potentials for both self-interference and interference from a proximate UE using the same (Band 24) UL frequencies.  A conservative model is used for the PA OOBE mask.

The UE PA OOBE mask is modelled on measurements performed at the output of an un-optimized PA, operating in Band 24, using 23 dBm transmit power, and 50 RBs for 10 MHz bandwidth UL transmission. This output is depicted in Figure 8.1. This mask presents a conservative OOBE characteristic for a Band 24 PA output, as there are existing PAs with considerably better performance (lower OOBE PSD). It should also be noted that there are existing PAs for 1700 MHz band that are known to provide approximately 4-5 dB lower OOBE PSD at a frequency offset similar to the duplexing gap (13.3 MHz offset from carrier edge) for one of the options (Option 1) for this proposed band  [16].

Figure 8.1: Un-optimized Band 24 OOB noise at PA output for LTE 1626.5-1636.5 MHz; 50 RBs; TX power 23dBm.

### UE Special Issues for Pairing Upper UL Carrier (Option 1)

If the upper UL carrier is paired with the DL carrier, the UE RX filter in the duplexer (passband of 1670 – 1680 MHz) has to sufficiently attenuate uplink transmissions from 1646.7-1656.7 MHz at a separation of 13.3 MHz from the downlink. The filters in the UE’s transmit chain and the TX-RX isolation of the duplexer must sufficiently suppress adjacent channel emissions into the downlink band, from an UL emission 13.3 MHz away. This demanding rejection requirement of the duplexer might impact the insertion loss of both TX and RX filters.

To this end, some potential receiver desensitization scenarios have to be investigated, including the followings:

- Receiver desensitization caused by UL TX OOBE into receive band (1670-1680 MHz) from own transmitter, or own receiver overload caused by emissions by own transmitter in the transmit band (1646.7-1656.7 MHz). This represents self-interference.

- Desensitization caused by OOBE or receiver overload owing to transmissions from a proximate Band 24 device(mobile to mobile interference).

Currently, the present UE Band 24 duplexer supports the 1626.5-1660.5 MHz UL band. The TX-ANT filter of this duplexer provides a sharp edge at the left side (to protect GNSS), but its isolation to protect the proposed UE DL receiver at 1670-1680 MHz is not significant. As a result, assuming that re-designing band 24 duplexer to be sharp at both sides is costly and complex, a new duplexer has to be designed for this proposed band, if Option 1 is chosen.

A duplexer has to provide high TX-RX isolation, to minimize both TX OOBE into the receive chain and overload of the RX chain by the fundamental TX signals. Toward this end, two metrics are relevant, both measured with a signal injected into the TX port and relative (input to output) attenuation measured at the RX port: (i) signal tuned to RX band – this metric is relevant to OOBE mitigation; (ii) signal tuned to TX band – this metric is relevant to overload mitigation. We set a goal of 50 dB for (i) and 55 dB for (ii) as these are typical, practical values.

The feasibility of this duplexer was assessed using simulations performed by a duplexer/filter vendor using Film Bulk Acoustic Resonator (FBAR) technology. The performance metric simulated were: TX-RX isolation, the frequency response of the TX-ANT and ANT-RX filters, TX-ANT insertion loss, and ANT-RX insertion loss. Figures 8.1.1, 8.1.2, 8.1.3, and 8.1.4 show the simulation results. In all figures, the dotted lines show the operator design objectives.

Figure 8.1.1: 1651.7 MHz (Option 1) UE Duplexer Isolation

The plots represent the typical filters at room temperature (25°C), using FBAR filter technology process. It is evident from Figure. 8.1.1 that both performance objectives (i) and (ii), indicated by dotted lines, are met with a considerable margin (greater than 20 dB for (i) and 10 dB for (ii)).

Figure 8.1.2 shows the frequency responses of the TX-ANT and ANT-RX paths, showing conformance with the stopband insertion loss requirement of 45 dB in both cases.  Figures 8.1.3 and 8.1.4 show the passband insertion losses of the TX and RX paths, respectively, showing conformance with the passband insertion loss of 3 dB in both cases.

Figure 8.1.2: 1651.7 MHz (Option 1) UE Duplexer TX-ANT and ANT-RX Frequency Responses

The vendor has confirmed that these performance objectives can be met over typical manufacturing variations and a temperature range of -20 to +85°C.  The vendor has further stated that the maximum shift in the nominal center frequency over manufacturing and temperature variations is +/- 3 MHz.  It is evident from the depicted, nominal room temperature frequency response plots that both passband insertion loss and TX-RX isolation requirements will continue to be met in spite of the above frequency shift.

Figure 8.1.3: 1651.7 MHz (Option 1) UE Duplexer TX Insertion Loss

Figure 8.1.4: 1651.7 MHz (Option 1) UE Duplexer RX Insertion Loss

Table 8.1-1 lists the performance requirements that the filter vendor has committed to meet for the temperature range of -20 to +85°C, and normal manufacturing variance using FBAR technology. All analyses in this report are based on these requirement values, which represent worst case performance, rather than the considerably superior, nominal performance characteristics shown in the above graphs.

Table 8.1-1: The duplexer performance requirements for the proposed band using FBAR technology

| Band | Max TX filter IL (dB) | Max RX filter IL (dB) | Min TX to RX isolation in RX band (dB) | Min TX to RX isolation in TX band (dB) | Min TX-ANT rejection in RX band (dB) | Min ANT-RX rejection in TX band (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1670-1680 MHz DL 1646.7-1656.7 MHz UL | 3 | 3 | 50 | 55 | 45 | 45 |

### 8.1.1    Potential receiver desensitization due to Own UL TX OOBE

Assuming the duplexer specifications in Table 8.1-1, we calculate the receiver desensitization, and additional attenuation required to achieve 1 dB and 3 dB receiver desensitization, using 9 dB UE NF (Noise Figure) in Table 8.1.1-1. The UE PA OOBE attenuation is verified by evaluating the characteristics of existing Band 24 un-optimized PA output, depicted in Figure 8.1.  Lab results show that lower OOBE values can be achieved by a PA that is optimized for Band 24.

Table 8.1.1-1: 10 MHz Receiver Desensitation for in-device isolation for Option 1 Pairing

| Parameter | Value | Unit |
| --- | --- | --- |
| Max UE PA OOBE (50 PRBs), 13.3 MHz from carrier edge | -92 | dBm/Hz |
| Duplexer isolation at 13.3 MHz from carrier edge | 50 | dB |
| TX noise at 1670-1680 MHz | -142 | dBm/Hz |
| Acceptable noise at the RX carrier for 1 dB RX desensitization | -170.9 | dBm/Hz |
| Acceptable noise at the RX carrier for 3 dB RX desensitization | -165 | dBm/Hz |
| Minimum Additional Required Isolation for 1 dB Desense. | 28.9 | dB |
| Minimum Additional Required Isolation for 3 dB Desense. | 23 | dB |

The additional required isolation must be achieved either by improving the OOBE rejection of the TX band-pass filter, or by inserting a band-pass filter similar to the duplexer transmit filter between the PA and the duplexer. By adding a 6 dB margin to the 28.9 dB additional isolation for 1 dB desensitization, we derive a requirement of 34.9 dB as the additional isolation to be provided by an external, post-PA filter.  This should be easily achievable, given that the 45 dB objective shown in Table 8.1-1 was achieved by the TX-ANT filter internal to the duplexer.

The post-PA filter incurs some passband insertion loss. Using the same filter design as the TX-ANT filter, Figure 8.1.3 shows that the insertion loss of this filter would be between 1-3 dB. Therefore, the PA has to be driven to approximately 1-3 dB higher maximum power level, which may result in some impact on the UE’s current drain relative to the case where the additional transmit filter was not required.  However, it is noteworthy that post-PA transmit filters are not uncommon in applications requiring tight control of the emission mask.

While including an additional bandpass filter in the transmit chain, it is necessary to consider the potential for frequency response distortion of both coupled filters (external and internal to the duplexer) through impedance mismatch, especially at off-main-lobe frequencies. In order to avoid such distortion, a passive matching network may be used.  Such passive networks can be constructed with an insertion loss less than 1 dB and for very low cost [18].

Table 8.1.1-2 provides the same analysis for 5 MHz UL TX channel at 1646.7-1651.7 and DL carrier at 1670-1675 MHz, using an un- optimized PA, with the OOBE characteristics depicted in Figure 8.1. Note that the PA OOBE at 18.3 MHz away from carrier edge is obtained assuming 25.8 maximum UL TX power.

Table 8.1.1-2: 5 MHz Receiver Desensitation for in-device isolation for Option 1 Pairing

| Parameter | Value | Unit |
| --- | --- | --- |
| Max UE PA OOBE (25 PRBs), 18.3 MHz from carrier edge | -125 | dBm/Hz |
| Duplexer isolation at 18.3 MHz from carrier edge | 50 | dB |
| TX noise at 1670-1675 MHz | -175 | dBm/Hz |
| Acceptable noise at the RX carrier for 1 dB RX desensitization | -170.9 | dBm/Hz |
| Acceptable noise at the RX carrier for 3 dB RX desensitization | -165 | dBm/Hz |
| Margin for 1 dB Desense. | 4.1 | dB |
| Margin for 3 dB Desense. | 10 | dB |

As seen above, for 5 MHz channel, no additional isolation is needed, and we have 4.1 and 10 dB margin for 1 dB and 3 dB RX accepted desensitization, respectively.

### Potential device receiver overload due to its UL TX power

### Mobile to Mobile OOBE Effect

Assuming that two 10 MHz UEs using this band, one transmitting in UL (1646.7-1656.7 MHz as an aggressor), and one receiving in the DL (1670 – 1680 MHz as a victim) with 1 meter separation , Table 8.1.3-1 calculates the OOBE noise level from aggressor-UE transmitter as received by the victim receiver UE. In this analysis, it is assumed that sum of antenna and body loss is 8 dB for each UE [17], with maximum 23 dBm TX power. It is assumed that the total of TX-ANT rejection of duplexer TX filter, and the Ant-RX rejection of duplexer RX filter provide at least 45 dB isolation from TX band into RX band, corresponding to the requirements stated in Table 8.1-1.

Table 8.1.3-1: Same band 10 MHz Mobile to Mobile Noise, assumiung Option 1 pairing

| Parameter | Value | Unit |
| --- | --- | --- |
| Max UE PA OOBE (50 PRBs), 13.3 MHz from carrier edge | -92 | dBm/Hz |
| Duplexer TX filter attenuation at 1670-1680 MHz (worst case) | 45 | dB |
| Path Loss at 1m | 36.6 | dB |
| Minimum Coupling Loss (MCL) | 52.6 | dB |
| TX noise at 1670-1680 MHz | -189.6 | dBm/Hz |
| Acceptable noise at the RX carrier for 1 dB RX desensitization | -170.9 | dBm/Hz |
| Acceptable noise at the RX carrier for 3 dB RX desensitization | -165 | dBm/Hz |
| Margin for 1 dB Desense. | 18.7 | dB |
| Margin for 3 dB Desense. | 24.6 | dB |

As seen above, for a 10 MHz channel, we have 18.7 and 24.6 dB margin for 1 dB and 3 dB RX accepted desensitization, respectively.

Table 8.1.3-2 provides the same analysis for 5 MHz UEs, where the margins are changed to 51.7 and 57.6 dB for 1dB and 3dB RX accepted desensitization, respectively.

Table 8.1.3-2: Same band 5 MHz Mobile to Mobile Noise, assumiung Option 1 pairing

| Parameter | Value | Unit |
| --- | --- | --- |
| Max UE PA OOBE (25PRBs), 18.3MHz from carrier edge | -125 | dBm/Hz |
| Duplexer TX filter attenuation at 1670-1675 MHz (worst case) | 45 | dB |
| Path Loss at 1m | 36.6 | dB |
| Minimum Coupling Loss (MCL) | 52.6 | dB |
| TX noise at 1670-1675 MHz | -222.6 | dBm/Hz |
| Acceptable noise at the RX carrier for 1 dB RX desensitization | -170.9 | dBm/Hz |
| Acceptable noise at the RX carrier for 3 dB RX desensitization | -165 | dBm/Hz |
| Margin for 1 dB Desense | 51.7 | dB |
| Margin for 3 dB Desense | 57.6 | dB |

The margins are even greater in this case.  The likelihood of interference is therefore considered low, in both cases.

<Next Modification>

### UE Special Issues for Pairing Lower UL Carrier (Option 2)

If the lower UL carrier is paired with the DL carrier, the edge to edge UL-DL gap is 32.5 MHz (1670 – 1637.5=32.5 MHz) for the 10 MHz wide carrier, and 37.5 MHz (1670 – 1632.5=37.5 MHz) for 5 MHz wide carrier. Consequently, the duplexer design is not as critical as the case where the upper UL carrier is used. A typical 50 dB TX-RX isolation would be enough to avoid receiver desensitization. This is because as the PA OOBE PSD level in the RX band would be much less than for Option 1.

Using the PA output of an un-optimized PA depicted in Figure 8.1, the OOBE PSD at an 32.5 MHz offset from the channel edge is -130 dBm/Hz, compared to -92 dBm/Hz for Option 1. Using the same analysis as in Table 8.1.1-1, Table 8.2-1 provides the required isolation TX-RX isolation for 1 dB and 3dB desensitization.

Table 8.2-1: 10 MHz Receiver Desensitation for in-device isolation for Option 2 Pairing

| Parameter | Value | Unit |
| --- | --- | --- |
| Max UE PA OOBE (50 PRBs), 32.5 MHz from channel edge | -130 | dBm/Hz |
| Noise Figure | 9 | dB |
| Acceptable noise at the RX carrier for 1 dB RX desensitization | -170.9 | dBm/Hz |
| Acceptable noise at the RX carrier for 3 dB RX desensitization | -165.0 | dBm/Hz |
| Required duplexer TX-RX isolation for 1 dB Desense | 40.9 | dB |
| Required duplexer TX-RX isolation for 3 dB Desense | 35.0 | dB |

For 5 MHz channel BW, the separation is 37.5 MHz, and the rejection at 37.5 MHz away from the channel edge is -132.3 dBm/Hz, as compared to -125 dBm/Hz for Option 1. Using the same analysis as in Table 8.2-1, the required TX-RX duplexer isolation for 1 dB and 3 dB desensitization would decrease to 40.9 - [-130 - (-132.3)] = 38.6 dB and 35 - [-130 - (-132.3)] = 32.7 dB, respectively.

In the case of 10 MHz channel bandwidth, for the 1 dB desensitization, the duplexer has to provide 40.9 dB rejection, at 32.5 MHz offset from the right channel edge. This is a relatively easy requirement to achieve. However, the duplexer has to provide a steep roll-off on the left to meet Band 24 uplink regulatory OOBE requirements in the GNSS band. The following discussion calculates the required duplexer rejection in the GNSS band.

We assume 0 dBi antenna gain (which is a conservative value [17]), and use the FCC requirement of Band 24 UE requirements for OOBE in GNSS band (-65 dBm/MHz at 1605 MHz as given in Section 5.3). From Figure 8.1, the PA OOBE mask at 22.5 MHz (1627.5 – 1605=22.5) offset from the left channel edge is -103 dBm/Hz. The TX-ANT rejection at 22.5 MHz offset is calculated to be -103 - (-65-60) = 22 dB. The above shows that 30 dB rejection by the TX-ANT filter at 22.5 MHz offset from channel edge provides more than 8 dB margin).

As shown in Figure 8.2.1, Band 24 SAW duplexer’s TX-ANT filter already meets this requirement, while also providing a roll-off on the right side similar to that required for meeting the isolation requirements for Option 2 (approximately 50 dB rejection at 32 MHz offset). Therefore, it is not expected that this filter will pose any significant technical challenges. Specifically, the TX-ANT filter shown in Figure 8.2.1 provides an attenuation of approximately 50 dB at a right side frequency offset greater than 32 MHz from the channel edge. If the filter can provide this selectivity while maintaining a passband width of 34 MHz, the filter complexity to realize similar roll-off (50 dB in 32 MHz) with a smaller (10 MHz) passband, as required by Option2, would be less.  It is noteworthy that filter complexity depends, in part, on the Transition Ratio (TR) of the filter, defined as (stopband width)/(passband width). A larger TR value requiring fewer poles/zeros, and therefore less implementation complexity. Hence the 50 dB isolation requirement should be achievable without exceptional technical risk.

Figure 8.2.1: EPCOS LI68A L-band duplexer TX-ANT

### 8.2.1  Mobile to Mobile OOBE Effect

With Option 2, the 10 MHz UL, 1627.5-1637.5 MHz, is paired with the 1670 – 1680 MHz DL, and the other 10 MHz UL carrier, 1646.7-1656.7 MHz, is used as a modified Band 24 UL, paired with a 1526 – 1536 MHz band DL. Assuming two UEs, one transmitting in modified Band 24 UL (aggressor) at 1646.7-1656.7 MHz, and one receiving in the proposed DL carrier (victim) at 1670-1680 MHz, are in 1 meter proximity of each other, Table 8.2.2-1 calculates the required rejection from the TX PA output to the input of receiver LNA.. In this analysis, it is assumed that antenna and body loss is 8 dB at each UE [17], with maximum 23 dBm TX power. Once again, a 9 dB NF is assumed at UE receiver.

Table 8.2.1-1: 10MHz M2M Noise

| Parameter | Value | Unit |
| --- | --- | --- |
| Max UE PA OOBE (50 PRBs), 13.3 MHz from carrier edge | -92 | dBm/Hz |
| Path Loss at 1m | 36.6 | dB |
| Minimum Coupling Loss (MCL) | 52.6 | dB |
| TX noise at 1670-1680 MHz | -144.6 | dBm/Hz |
| Acceptable noise at the RX carrier for 1 dB RX desensitization | -170.9 | dBm/Hz |
| Acceptable noise at the RX carrier for 3 dB RX desensitization | -165 | dBm/Hz |
| Minimum required isolation from TX-ANT duplexer filter for 1 dB Desense. | 26.3 | dB |
| Minimum required isolation from duplexer filter for 3 dB Desense. | 20.4 | dB |

As seen above, for a 10 MHz channel, we need at least 26.3 and 20.4 dB TX-ANT duplexer filter isolation for 1 dB and 3 dB RX desensitization levels, respectively. These isolations  could be provided by the modified Band 24 duplexer’s TX-ANT filter on the right side once we relax the sharp roll-off required on the left side to meet the regulatory OOBE requirement in the GNSS band.  We now have a transition band of 1646.7 -1605 = 41.5 MHz with a passband of 10 MHz.  A rejection of 50 dB should be realizable with this transition band, as in Section 8.2 it was shown that it is not very challenging to achieve 50 dB rejection over a transition band of 32 MHz.

<Next Modification>

### MSD and UE REFSENS for Upper UL pairing (Option 1)

Assuming [12], and the same parameters as used for Band 24, the reference sensitivity power for 5 and 10 MHz channel bandwidths could be calculated to be -100 dBm and -97 dBm, respectively. Using the UE duplexer RX insertion loss and TX-RX isolation loss in Table 8.1-1, we have listed the parameters required for calculating the MSD and UE REFSENS for both 5 and 10 MHz channel bandwidths of Option 1 in Table 8.3.2-1. Here, it is assumed that the coupling between the two RX antennas is 10 dB, and the receiver noise figure is 9 dB. The PA output OOBE PSD for 10 MHz channel BW is calculated using a linear interpolation from -92 dBm/Hz at 1670 MHz, and -104 dBm/Hz at 1680 MHz, using an un-optimized PA output, depicted in Figure 8.1.

Table 8.3.2-1: Calculation of MSD for Option 1 UL pairing (without post-PA bandpass filter)

| BW (MHz) | Lcoupling (dB) |  (dB) | PA output OOB Density (dBm/Hz) | Lrx (dB) | atx-rx (dB) | B (MHz) | NF (dB) | POOB (dBm) | Vn (dBm) | Vt (dBm) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 10 | 0.5 | -128 | 3 | 50 | 4.5 | 9 | -61.47 | -98.5 | -108.47 | -0.28 |
| 10 | 10 | 0.5 | -98.00 | 3 | 50 | 9 | 9 | -28.46 | -95.5 | -75.46 | 12.92 |

As evident from this table, the sensitivity degradation for the 5 MHz channel BW is not significant, while the maximum sensitivity degradation for 10 MHz channel, without a post-PA filter is in the range of 12.92 dB. This degradation is due to small duplexing gap between the UL and DL carrier (13.3 MHz), and the fact that the UE PA output OOBE density in the own RX channel can’t be ignored. From Table 8.1.1-1, the required additional isolation to achieve maximum 1 dB desensitization in RX noise floor per antenna branch is calculated to be 28.9 dB. This means that atx-rx must be increased to 50 + 28.9 = 78.9 dB. Assuming 78.9 dB as the isolation value, Table 8.3.2-2 shows the MSD calculatedto be -0.22   dB.

Table 8.3.2-2: Calculation of MSD for Option 1 UL pairing for 10 MHz with post-PA bandpass filter

| BW (MHz) | Lcoupling (dB) |  (dB) | PA output OOB Density (dBm/Hz) | Lrx (dB) | atx-rx (dB) | B (MHz) | NF (dB) | POOB (dBm) | Vn (dBm) | Vt (dBm) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 10 | 0.5 | -98.00 | 3 | 78.9 | 9 | 9 | -28.46 | -95.5 | -104.36 | -0.22 |

### 8.3.3    UE REFSENS for Lower UL pairing (Option 2)

The TX-RX separation for Option 2 pairing is 32.5 MHz. Using -134 dBm/Hz as the PA output density, the MSD in this case is calculated to be zero. Therefore, the reference sensitivities are the same as Band 24 values (-100 dBm for 5 MHz and -97 dBm for 10 MHz channel bandwidth).

<Next Modification>

### 8.4.3      Other expected UE TX/RX RF requirements

In Table 8.4.3-1, detailed descriptions on UE RF requirements are provided to predict any potential requirements changes for two pairing alternatives of this proposed band.

Table 8.4.3-1: Considering UE RF requirements for the proposed band

| Clause | Description | Requirement for Option 1 | Requirement for Option 2 |
| --- | --- | --- | --- |
| 6.2.2 | UE Maximum Output Power | Referring to the solution provided in Section 8.1.1 for small TX-RX duplexing separation, power class 3 can be applied to the UEs in this band. | No changes are needed for power class 3. |
| 6.2.4 | UE Maximum Output Power with additional requirements (A-MPR) | Referring to the solution provided in Section 8.1.1 for small TX-RX duplexing separation, no A-MPR is required in addition to MPR stated in [15], Table 6.2.3-1. | No changes are expected. |
| 6.6.2.2 | Additional Spectrum Emission Mask | No changes are needed to protect adjacent coexistence bands, due to no IMD and harmonic with other DL bands | No changes are needed to protect adjacent coexistence bands, due to no IMD and harmonic with other DL bands. |
| 6.6.3.2 | Spurious emission band UE coexistence | Defines SE requirements as -50 dBm/MHz to protect UE coexistence bands. SE requirements to protect self-receiver is specified in Table 8.4.2-1. | Defines SE requirements as -50 dBm/MHz to protect UE coexistence bands. For coexistence with Band 24, and SE requirements to protect self receiver is specified in Table 8.4.2-2. |
| 7.3.1 | Reference sensitivity requirement | See Section 8.3. | No changes are expected. |
| 7.5, 7.6, 7.7, and 7.8 | ACS, blocking, spurious, and intermod | . As discussed in Section 8.1.2, no additional requirements are needed as compared to Band 24. | . As discussed in Section 8.2.2, no additional requirements are needed as compared to Band 24. |

<Next Modification>

### UE Related Concluding Remarks

In Section 8, several characteristics of two UL pairing options were compared using a conservative un-optimized PA OOBE PSD mask, depicted in Figure 8.1. Option 1 (which comprises pairing the proposed DL carrier (1670-1680 MHz) with upper UL carrier (1646.7-1656.7 MHz)) is clearly more challenging than Option 2 (pairing proposed DL carrier with lower UL carrier (1626.5-1636.5).

Using conventional PAs and duplexers, with FBAR technology, which yields one of the best frequency selectivity in the present state of the art, it is not possible to achieve the isolation necessary to limit own receiver desensitization to targets below 1 dB or 3 dB. Moreover, due to self-overloading and TX OOBE, this option requires at least 12.92 dB MSD. However, there are solutions to achieve the necessary isolation. One solution is to use a post-PA bandpass filter between the PA and duplexer in the TX chain. The required selectivity (isolation) of ~35 dB in the transition band of 13.3 MHz is readily available with current filter technologies. Using this filter, the REFSENS values used for band 24 can be applied to this proposed band without MSD.

For Option 2 pairing, the technical challenge is lower than Option 1. The selectivity requirement on the right side of the UL channel is 40.9 dB isolation at a 32.5 MHz frequency separation (DL receive band) to avoid own receiver desensitization. The selectivity requirement on the left side of the UL channel is 30 dB isolation at a 22 MHz frequency separation to meet FCC’s requirement to protect the GNSS band. Both selectivity requirements are achievable with present SAW duplexer technologies for Band 24.  Hence, Option 2 is considered feasible with relatively little technical challenge

Regardless of whether Option 1 or Option 2 is selected, the present Band 24 duplexer cannot be reused.

If Option 1 is selected, then a new duplexer will be required for pairing the upper UL with the new 1670 – 1680 MHz downlink. Assuming that the lower UL carrier is paired with Band 24 lower DL (1526-1536 MHz), one may be tempted to reuse the present Band 24 duplexer for this FDD pair. However, it makes more sense to reduce the width of the present Band 24 duplexer’s passband to 10 MHz to free up greater transition bandwidth (1636.5 to 1670 MHz) to the 1670-1680 MHz of proposed DL band. This reduces the potential for interference between proximate UEs. Owing to the large transition band, a SAW filter is likely to suffice for this application.

If Option 2 were selected, the Band 24 duplexer could not be used as it would provide insufficient TX-RX isolation to the 1670 – 1680 MHz DL receiver in a mobile to mobile proximate situation.

It is worthy of mention that in both pairing options, a common duplexer cannot support this proposed band and a modified Band 24, if both the proposed band and Band 24 are supported in a given UE. Therefore for devices supporting both Band 24 (perhaps modified in future) and this proposed band, two separate duplexers will be required for the device.

<End of modifications>

------------- End of TP ------------------