3GPP TSG-RAN WG4 (Radio) Meeting #72bis	R4-146160

Singapore, 6 – 10 October 2014

Agenda Item:	7.36.2.3

Source: 	Alcatel-Lucent, AT&T

Title: 	TP for TR 36.853: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (2 + 2 + 29)

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 2 and Band 29 was approved in RAN#65 [1]. One objective of the WI is to specify the band-combination specific Radio Frequency (RF) requirements for this new Carrier Aggregation (CA) band combination.

The impacts of harmonics and IMD products caused by LTE-A BS supporting 3DL CA of Band (2 + 2 + 29) to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to record the findings in the 3 Band Carrier Aggregation Technical Report [3].

# 2.	Text Proposal

<New section>

##### 6.x.1	Co-existence studies for 1 UL/3 DL

The 3rd order IMD products caused in the BS by transmitting of Band 2, Band 2 and Band 29 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.x.1-1 below:

Table 6.x.1-1: Band 2, Band 2 and Band 29 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 717 | 728 | 1930 | 1990 | 1930 | 1990 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 657 |  |  | 788 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 657 |  |  | 788 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 3132 |  |  | 3263 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 4577 |  |  | 4708 |  |  |

It can be seen from Table 6.x.1-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (2 + 2 + 29) may fall into the BS receive band of Bands 12, 13, 17, 28 and 44. However, Bands 28 and 44 are not intended for use in the same geographical area as Bands 2 and 29. Therefore, the focus here is on the 3rd order IMD products falling into Bands 12, 13 and 17.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 13 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (2 + 2 + 29) BS transmitter does not share the same antenna with Band 13 BS receiver. On the other hand, receiver desensitization may still be an issue for Bands 12 and 17 BS receive band, which is only 1 MHz away from Band 29 transmit band.

Therefore, it is recommended that Band (2 + 2 + 29) BS transmitter should not share the same antenna with Band 12 or 17 BS receiver to prevent BS receiver desensitization, or with Band 13 BS receiver unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 13 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of section>

# References

[1]	RP-141448, “New WI proposal: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 2 and Band 29”, AT&T.

[2]	R4-146159, “Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (2 + 2 + 29)”, Alcatel-Lucent, AT&T.

[3]	RP-141217, “TR 36.853 v2.0.0 on LTE Advanced 3 Band Carrier Aggregation (3DL/1UL)”, AT&T.