3GPP TSG-RAN WG4 (Radio) Meeting #72bis	R4-146164

Singapore, 6 – 10 October 2014

Agenda Item:	7.36.2.3

Source: 	Alcatel-Lucent, Verizon

Title: 	TP for TR 36.853: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (2 + 5 + 13)

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 2, Band 5 and Band 13 was approved in RAN#63 [1]. One objective of the WI is to specify the band-combination specific Radio Frequency (RF) requirements for inter-band Carrier Aggregation (CA) of Band 2, Band 5 and Band 13 with 1 Uplink (UL) active in one of these bands.

The impacts of harmonics and IMD products caused by LTE-A BS supporting 3DL CA of Band (2 + 5 + 13) to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to record the findings in the 3 Band Carrier Aggregation Technical Report [3].

# 2.	Text Proposal

<Start of text proposal>

#### 6.29.3.1	For the BS

The 3rd order IMD products caused in the BS by transmitting of Band 2, Band 5 and Band 13 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.29.3.1-1 below:

Table 6.29.3.1-1: Band 2, Band 5 and Band 13 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 746 | 756 | 869 | 894 | 1930 | 1990 |
| 3rd order IMD products | (f3-low – f1-high – f2-high) |  |  | (f3-high – f1-low – f2-low) |  |  |
| IMD frequency limits (MHz) | 280 |  |  | 375 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 1782 |  |  | 1877 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 2043 |  |  | 2138 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 3545 |  |  | 3640 |  |  |

It can be seen from Table 6.29.3.1-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (2 + 5 + 13) may fall into the BS receive band of Bands 2, 3, 9, 25, 35, 42 and 43. However, Bands 3 and 9 are not intended for use in the same geographical area as Bands 2, 5 and 13. Moreover, co-location of Band 2 transmitter and Band 35 transceiver implies FDD/TDD co-location on adjacent frequencies which requires the use of certain site-engineering solutions to avoid mutual interference. Therefore, the focus here is on the 3rd order IMD products falling into Bands 2, 25, 42 and 43. Note that the 3rd order IMD products will not fall into the BS own (up to 20 MHz) receive block within Band 2.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 42 or 43 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (2 + 5 + 13) BS transmitter does not share the same antenna with Band 42 or 43 BS receiver. But there is not a large frequency gap between Band 2 DL and Bands 2 and 25 UL, and hence Bands 2 and 25 BS receiver desensitization may still be an issue.

Therefore, it is recommended that Band (2 + 5 + 13) BS transmitter should not share the same antenna with Band 2 or 25 BS receiver to prevent BS receiver desensitization, or with 42 or 43 BS receiver unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 42 or 43 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of text proposal>

# References

[1]	RP-140167, “New WI: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) for Band 2, Band 5, and Band 13”, Verizon, Intel.

[2]	R4-146163, “Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (2 + 5 + 13)”, Alcatel-Lucent, Verizon.

[3]	RP-141217, “TR 36.853 v2.0.0 on LTE Advanced 3 Band Carrier Aggregation (3DL/1UL)”, AT&T.