3GPP TSG-RAN WG4 (Radio) Meeting #72bis	R4-146172

Singapore, 6 – 10 October 2013

Agenda Item:	7.36.2.3

Source: 	Alcatel-Lucent, Sprint

Title: 	TP for Rel-13 3DL TR 36.8xx: Coexistence Studies of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (25 + 41 + 41)

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced inter-band DL Carrier Aggregation in Band 25A and Band 41C was approved in RAN#65 [1]. One objective of the WI is to specify the band-combinations specific Radio Frequency (RF) requirements for the new bandwidth combinations for CA_25A-41C.

The impacts of harmonics and IMD products caused by LTE-A BS supporting 3DL CA of Band (25 + 41 + 41) to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to record the findings in the 3 Band Carrier Aggregation Technical Report [3].

# 2.	Text Proposal

<New section>

##### 6.x.1	Co-existence studies for 1 UL/3 DL

The 3rd order IMD products caused in the BS by transmitting of Band 25, Band 41 and Band 41 DL carriers (i.e. 3DL CA with one component carrier in each band) can be calculated as shown in Table 6.x.1-1 below:

Table 6.x.1-1: Band 25, Band 41 and Band 41 DL 3rd order IMD products

| BS DL carriers | f1-low | f1-high | f2-low | f2-high | f3-low | f3-high |
| --- | --- | --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1930 | 1995 | 2496 | 2690 | 2496 | 2690 |
| 3rd order IMD products | (f1-low + f2-low – f3-high) |  |  | (f1-high + f2-high – f3-low) |  |  |
| IMD frequency limits (MHz) | 1736 |  |  | 2189 |  |  |
| 3rd order IMD products | (f1-low + f3-low – f2-high) |  |  | (f1-high + f3-high – f2-low) |  |  |
| IMD frequency limits (MHz) | 1736 |  |  | 2189 |  |  |
| 3rd order IMD products | (f2-low + f3-low – f1-high) |  |  | (f2-high + f3-high – f1-low) |  |  |
| IMD frequency limits (MHz) | 2997 |  |  | 3450 |  |  |
| 3rd order IMD products | (f1-low + f2-low + f3-low) |  |  | (f1-high + f2-high + f3-high) |  |  |
| IMD frequency limits (MHz) | 6922 |  |  | 7375 |  |  |

It can be seen from Table 6.x.1-1 that in addition to the impacts of harmonics and IMD products from the constituent 2DL CA configurations, the 3rd order IMD products supporting 3 DL CA of Band (25 + 41 + 41) may fall into the BS receive band of Bands 1, 2, 3, 4, 9, 10, 22, 23, 25, 33, 34, 35, 36, 37, 39 and 42. However, Bands 1, 3, 9, 10, 33, 34 and 39 are not intended for use in the same geographical area as Bands 25 and 41. Moreover, co-location of Band 25 transmitter and Band 35, 36 or 37 transceiver implies FDD/TDD co-location on adjacent frequencies which requires the use of certain site-engineering solutions to avoid mutual interference. Therefore, the focus here is on the 3rd order IMD products falling into Bands 2, 4, 22, 23, 25 and 42. Note that the 3rd order IMD products will not fall into the BS own (up to 20 MHz) receive block within Band 25.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the IMD interference generated within the Band 4, 22 or 42 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band (25 + 41 + 41) BS transmitter does not share the same antenna with Band 4, 22 or 42 BS receiver. But there is not a large frequency gap between Band 25 DL and Bands 2, 23 and 25 UL, and hence Bands 2, 23 and 25 BS receiver desensitization may still be an issue.

Therefore, it is recommended that Band (25 + 41 + 41) BS transmitter should not share the same antenna with Band 2, 23 or 25 BS receiver to prevent BS receiver desensitization, or with Band 4, 22 or 42 BS receiver unless the antenna path meets very stringent 3rd order PIM specification so that the PIM will not cause Band 4, 22 or 42 BS receiver desensitization. Note that antenna sharing may be allowed as the state-of-the-art continues to evolve in the future.

<End of section>

# References

[1]	RP-141673, “LTE Advanced inter-band DL Carrier Aggregation in Band 25A and Band 41C”, Sprint.

[2]	R4-146171, “Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (25 + 41 + 41)”, Alcatel-Lucent, Sprint.

[3]	R4-14xxxx, “TR 36.8xx v0.1.0: Rel-13 LTE Advanced inter-band CA for 3DL”, ZTE.