3GPP TSG-WG4 Meeting #72bis	R4-146434

# Singapore, 6th – 10th October, 2014

Agenda item: 	7.16.1

Source: 	Qualcomm Incorporated

Title: 	E-AGCH performance with improved granting

Document for:	Discussion

# 1		Introduction

In RAN4#72, it was agreed in [1] that new demodulation test cases, performance metric and requirements shall be defined for improved E-AGCH granting in the context of EUL enhancements WI.

Three test cases to be defined are (1) No E-AGCH (2) E-AGCH for other UEs (3) E-AGCH for UE under test, and minimum requirements shall be specified for each test case. The performance metric i.e. percentage of incorrect decisions by UE shall be a function of FA_PC/MD_PC which are rate of false alarm/mis-detection respectively based on E-AGCH presence check, and, CS/CF is E-AGCH CRC check success/failure rate respectively. The following table summarizes the three test cases and the performance metric as a function of E-AGCH presence check and E-AGCH CRC check.

| Node B transmits | E-AGCH presence check | E-AGCH CRC check | Performance Metric |
| --- | --- | --- | --- |
| No E-AGCH | False alarm ‘FA_PC’ (Detects the presence) | - | FA_PC Presence check passes |
| E-AGCH for other UEs | Mis-detection ‘MD_PC’ (Miss the presence) | CRC Pass ‘CS’ | (MD_PC) + (1-MD_PC)*CS (NOTE1) |
| E-AGCH for UE under test | Mis-detection ‘MD_PC’ (Miss the presence) | CRC Fail ‘CF’ | (MD_PC) + (1- MD_PC)*CF |
| NOTE1: (1-MD_PC)*CS might be omitted if this is negligible compared with MD_PC |  |  |  |

# 2	E-AGCH Performance with Improved Granting

## 2.1	No E-AGCH

Test Scenario:

| Node B transmits | E-AGCH presence check | E-AGCH CRC check | Performance Metric |
| --- | --- | --- | --- |
| No E-AGCH | False alarm ‘FA_PC’ (Detects the presence) | - | FA_PC Presence check passes |

Ior/Ioc (dB) is set to 0 dB

### Propagation Channel = AWGN

  - False Alarm Probability i.e. probability that E-AGCH presence check block detects the presence = 1.02%

### Propagation Channel = VA30

  - False Alarm Probability i.e. probability that E-AGCH presence Check block detects the presence = 0.95%

## 2.2	E-AGCH for other UEs

Test Scenario:

| Node B transmits | E-AGCH presence check | E-AGCH CRC check | Performance Metric |
| --- | --- | --- | --- |
| E-AGCH for other UEs | Mis-detection ‘MD_PC’ (Miss the presence) | CRC Pass ‘CS’ | (MD_PC) + (1-MD_PC)*CS (NOTE1) |
| NOTE1: (1-MD_PC)*CS might be omitted if this is negligible compared with MD_PC |  |  |  |

Ior/Ioc (dB) is set to 0 dB

E-RNTI for UE under test:    0 (0x0000)

E-RNTI for Other UE:           51966 (0xCAFE)

Results for 1.02% FA for AWGN are provided below

| E-AGCH EcIor | Missed Detection % | False CRC Pass% | Performance Metric % |
| --- | --- | --- | --- |
| -17 | 0 | 0 | 0 |
| -18 | 0 | 0 | 0 |
| -19 | 0 | 0 | 0 |
| -20 | 0 | 0 | 0 |
| -21 | 0 | 0 | 0 |
| -22 | 0.018 | 0 | 0.018 |
| -23 | 0.282 | 0 | 0.283 |
| -24 | 2.225 | 0 | 2.225 |
| -25 | 9.206 | 0 | 9.206 |
| -26 | 24.616 | 0 | 24.616 |
| -27 | 44.598 | 0 | 44.598 |
| -28 | 63.716 | 0 | 63.716 |
| -29 | 76.437 | 0 | 76.437 |
| -30 | 85.192 | 0 | 85.192 |

## 2.3	E-AGCH for UE under test

Test Scenario:

| Node B transmits | E-AGCH presence check | E-AGCH CRC check | Performance Metric |
| --- | --- | --- | --- |
| No E-AGCH | False alarm ‘FA_PC’ (Detects the presence) | - | FA_PC Presence check passes |
| E-AGCH for other UEs | Mis-detection ‘MD_PC’ (Miss the presence) | CRC Pass ‘CS’ | (MD_PC) + (1-MD_PC)*CS (NOTE1) |
| E-AGCH for UE under test | Mis-detection ‘MD_PC’ (Miss the presence) | CRC Fail ‘CF’ | (MD_PC) + (1- MD_PC)*CF |
| NOTE1: (1-MD_PC)*CS might be omitted if this is negligible compared with MD_PC |  |  |  |

Results for 1.02% FA for AWGN are provided below

| E-AGCH EcIor | Missed Detection % | Detect & CRC Fail | Perf Metric |
| --- | --- | --- | --- |
| -17 | 0 | 0 | 0 |
| -18 | 0 | 0 | 0 |
| -19 | 0 | 0.005 | 0.005 |
| -20 | 0 | 0.02 | 0.02 |
| -21 | 0.001 | 0.188 | 0.188 |
| -22 | 0.045 | 1.047 | 1.092 |
| -23 | 0.383 | 4.134 | 4.516 |
| -24 | 2.513 | 11.123 | 13.635 |
| -25 | 9.88 | 20.876 | 30.756 |
| -26 | 25.043 | 28.247 | 53.289 |
| -27 | 44.73 | 28.884 | 73.614 |
| -28 | 62.807 | 24.75 | 87.558 |
| -29 | 76.203 | 18.617 | 94.82 |
| -30 | 85.087 | 12.992 | 98.079 |

# 3		Conclusion

This contribution has presented E-AGCH performance results for the AWGN channel coditions.

# 4		Reference

- R4-145389, “WF on Test metric and simulation assumptions for E-AGCH requirements in EUL enhancements”, Qualcomm Incorporated, Ericsson, Nokia Networks