3GPP TSG-RAN WG4 Meeting #72bis	 R4-146072

# Singapore, Singapore, 6 – 10 Oct, 2014

Agenda Item:	7.9.4

Source: 	Samsung

Title: 	Simulation results for CSI-RS based RSRP measurement

Document for:	Discussion

1	Introduction

A new work item small cell enhancement for E-UTRA and E-UTRAN was approved in RAN#62 plenary. New type of discovery reference signal (DRS) was designed and introduced to facilitate TP identification and small on/off operation in RAN1. In RAN4#71 RRM AH, RSRP measurement accuracy for DRS were widely discussed. A simulation campaign for DRS based RSRP measurement accuracy was agreed in [1].

In this contribution, we provide evaluation results for CSI-RS based RSRP measurement.

- Analysis

2.1 Simulation assumption

The detailed simulation assumption was agreed in [1]. Furthermore, based on the observations from SLS evaluation for SINR distribution in [2], the SNR range = {-2,-0, 2} dB was evaluated.

Several cases with the different combination of parameters as listed below were evaluated.

- SNR: {-2,0,2} dB

- Fading channel:AWGN,EPA5Hz,ETU30Hz

- Measure period: i*160ms (i=1,2,3,4,5)

- Measurement channel bandwidth: {6RB,15RB,25RB,50RB}

2.2 Simulation results

The detailed simulation results with the 5-percentile, 50-percentile, 95-percentile performance and 90% RSRP accuracy were summarized in tables A.1~A.4 for 6RB, 15RB, 25RB and 50RB cases separately. .

Relationship of DRS periodicity and measurement period

In [3], we already evaluated the effect of DRS periodicity and measurement period. Based on results summarized in table 4 in [3], it’s observed that absolute RSRP accuracy was strongly depending on the numbder of samples collected during measurment period.

Observation1: With same measurement samples, measurement period has no obvious effect on CSI-RS based RSRP measurment accuracy

Measurement Channel BW and number of samples

Table A.5~A.9 in the annex summarized 90% RSRP accuracy with N=1, 2, 3, 4, and 5 measurement samples for different combinations of measurement channel BWs and channel models. Table 1 below summarized 90% RSRP accuracy with different configurations across all channel models.

Table 1: 90% absolute RSRP accuracy with different configurations

| Measurement BWs | 6 RB |  |  |  |  | 15 RB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | N=1 | N =2 | N =3 | N=4 | N =5 | N=1 | N =2 | N =3 | N=4 | N =5 |
| -2 | 3.8 | 3.4 | 2.9 | 2.5 | 2.3 | 3.1 | 2.4 | 2 | 1.8 | 1.6 |
| 0 | 3.6 | 2.9 | 2.4 | 2.2 | 2 | 2.8 | 2.1 | 1.7 | 1.6 | 1.5 |
| 2 | 3.3 | 2.6 | 2.2 | 1.9 | 1.7 | 2.5 | 1.9 | 1.6 | 1.5 | 1.3 |
| SNR[dB] | 25RB |  |  |  |  | 50 RB |  |  |  |  |
| -2 | 2.7 | 2 | 1.7 | 1.5 | 1.4 | 2.3 | 1.8 | 1.6 | 1.4 | 1.3 |
| 0 | 2.4 | 1.8 | 1.5 | 1.4 | 1.3 | 2.1 | 1.7 | 1.4 | 1.3 | 1.2 |
| 2 | 2.2 | 1.7 | 1.4 | 1.3 | 1.2 | 1.9 | 1.6 | 1.4 | 1.3 | 1.2 |

We observed that:

- With improved measurement BW, RSRP measurement accuracy can be improved especial with limited number of samples i.e. under 1 sample case, RSRP measurement accuracy under measurement channel BW 15RB,25RB,50RB can be improved around 0.8dB, 1.0dB and 1.5 dB separately compared 6 RB case.

- With increased number of samples, RSRP measurement accuracy can be improved i.e. under 6RB case, RSRP measurement accuracy with number of samples N =2, 3, 4, 5 can be improved around 0.5dB, 1.0dB, 1.3dB and 1.5 dB separately compared N =1 case.

Observation2: CSI-RS based RSRP measurment accuracy are strongly depending on SNR condition, number of samples and measurment bandwidths. Increased number of samples or extending measurment bandwidths is helpful for improving CSI-RS based on RSRP measurement accuracy.

Absolute RSRP accuracy comparison with legacy Rel.8 performance

As analyzed above, measurement accuracy was strongly depending on number of samples within measurement period, SNR condition, measurement bandwidths. Table below summarized number of samples needed to achieve similar performance as Rel.8 (i.e. 90% absolute RSRP accuracy within 2.5dB without impairment margin) under different channel bandwidths, SNR points.

Table 2: Number of samples needed to achieve similar performance as Rel.8 for CSI-RS RSRP

| Channel | AWGN | EPA5Hz | ETU30Hz | AWGN | EPA5Hz | ETU30Hz |
| --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | 6RB |  |  | 15RB |  |  |
| -2 | 4 | 2 | 4 | 2 | 1 | 2 |
| 0 | 2 | 1 | 3 | 1 | 1 | 2 |
| 2 | 2 | 1 | 3 | 1 | 1 | 2 |
| SNR[dB] | 25RB |  |  | 50RB |  |  |
| -2 | 1 | 1 | 2 | 1 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 2 | 1 | 1 | 1 | 1 | 1 | 1 |

Observation3: For CSI-RS RSRP measurement, in order to fulfil Rel.8 performance requirements, the number of samples needed for each combination of SNR point and measurement BW were summarized in table 3.

Table3: Number of samples needed to achieve similar performance as Rel.8

| SNR[dB] | 2 REs per DRS occasion (baseline) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 6RB | 15RB | 25RB | 50RB |
| -2 | 4 | 2 | 2 | 1 |
| 0 | 3 | 2 | 1 | 1 |
| 2 | 3 | 2 | 1 | 1 |

Based on the simulation results and summary, we can conclude that:

- Under SNR -2 dB: In order to fulfill Rel.8 requirements, N = 4,2,2,1 samples needed for 6RB, 15RB, 25 RB and 50RB cases separately.

- Under SNR 0 dB: In order to fulfill Rel.8 requirements, N = 3,2,1,1 samples needed for 6RB, 15RB, 25 RB and 50RB cases separately.

- Under SNR 2 dB: In order to fulfill Rel.8 requirements, N = 3,2,1,1 samples needed for 6RB, 15RB, 25 RB and 50RB cases separately.

- Conclusion

In this contribution, we supply initial simulation results and analysis for CSI-RS based RRM measurements.

Based on simulation results, we have such observations:

Observation1: With same measurement samples, measurement period has no obvious effect on CSI-RS based RSRP measurment accuracy

Observation2: CSI-RS based RSRP measurment accuracy are strongly depending on SNR condition, number of samples and measurment bandwidths.

Observation3: For CSI-RS RSRP measurement, in order to fulfil Rel.8 performance requirements, the number of samples needed for each combination of SNR point and measurement BW were summarized in table 3.

Table: Number of samples needed to achieve similar performance as Rel.8

| SNR[dB] | 2 REs per DRS occasion (baseline) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 6RB | 15RB | 25RB | 50RB |
| -2 | 4 | 2 | 2 | 1 |
| 0 | 3 | 2 | 1 | 1 |
| 2 | 3 | 2 | 1 | 1 |

- Reference

- R4-71AH-0103, “Simulation assumptions for DRS based RSRP accuracy for SCE”, Huawei

- R4-146589, “Updated simulation results for SCE SINR side condition”, Samsung

- R4-145368, “Overview on DRS based RRM measurement”, Samsung

Annex

Table A.1: Absolute CSI-RS based RSRP accuracy results under 6RB

|  | AWGN |  |  |  |  | EPA5Hz |  |  |  |  | ETU30Hz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | 5% | 50% | 95% | 5%~95% | 90% RSRP | 5% | 50% | 95% | 5%~95% | 90% RSRP | 5% | 50% | 95% | 5%~95% | 90% RSRP |
| Number of Sample =1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -2.3 | 1.4 | 4.2 | 6.5 | 4.2 | -3.2 | 0.4 | 4.3 | 7.5 | 4.3 | -3.8 | 0 | 3.3 | 7.1 | 3.8 |
| -2 | -2.1 | 1.2 | 3.8 | 5.9 | 3.8 | -3 | 0.3 | 3.8 | 6.8 | 3.8 | -3.8 | -0.2 | 2.8 | 6.6 | 3.8 |
| 0 | -1.7 | 0.9 | 3.2 | 4.8 | 3.2 | -2.7 | 0.2 | 2.9 | 5.6 | 2.9 | -3.6 | -0.4 | 2.1 | 5.7 | 3.6 |
| 2 | -1.2 | 0.7 | 2.6 | 3.9 | 2.6 | -2.2 | 0.1 | 2.3 | 4.5 | 2.3 | -3.3 | -0.5 | 1.5 | 4.9 | 3.3 |
| Number of Sample =2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -1.4 | 1.4 | 3.4 | 4.9 | 3.4 | -2.6 | 0.2 | 2.5 | 5.1 | 2.6 | -3.7 | -0.3 | 2 | 5.7 | 3.7 |
| -2 | -1.2 | 1.2 | 3.1 | 4.3 | 3.1 | -2.3 | 0.2 | 2.2 | 4.5 | 2.3 | -3.4 | -0.4 | 1.7 | 5.1 | 3.4 |
| 0 | -0.9 | 0.9 | 2.5 | 3.4 | 2.5 | -1.8 | 0.1 | 1.7 | 3.5 | 1.8 | -2.9 | -0.5 | 1.2 | 4.1 | 2.9 |
| 2 | -0.7 | 0.7 | 2 | 2.7 | 2 | -1.4 | 0.1 | 1.4 | 2.7 | 1.4 | -2.6 | -0.5 | 0.8 | 3.4 | 2.6 |
| Number of Sample =3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -0.9 | 1.4 | 3.1 | 3.9 | 3.1 | -2 | 0.2 | 2 | 4.1 | 2 | -3.2 | -0.3 | 1.6 | 4.7 | 3.2 |
| -2 | -0.7 | 1.2 | 2.8 | 3.5 | 2.8 | -1.8 | 0.2 | 1.8 | 3.5 | 1.8 | -2.9 | -0.4 | 1.3 | 4.3 | 2.9 |
| 0 | -0.5 | 1 | 2.3 | 2.8 | 2.3 | -1.3 | 0.1 | 1.4 | 2.7 | 1.4 | -2.4 | -0.5 | 0.9 | 3.3 | 2.4 |
| 2 | -0.4 | 0.8 | 1.8 | 2.2 | 1.8 | -1 | 0.1 | 1.1 | 2.2 | 1.1 | -2.2 | -0.5 | 0.6 | 2.9 | 2.2 |
| Number of Sample =4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -0.6 | 1.4 | 2.8 | 3.4 | 2.8 | -1.7 | 0.2 | 1.8 | 3.5 | 1.8 | -2.8 | -0.4 | 1.3 | 4.1 | 2.8 |
| -2 | -0.5 | 1.2 | 2.5 | 3 | 2.5 | -1.5 | 0.2 | 1.6 | 3 | 1.6 | -2.5 | -0.4 | 1.1 | 3.6 | 2.5 |
| 0 | -0.3 | 1 | 2.1 | 2.4 | 2.1 | -1.1 | 0.1 | 1.2 | 2.4 | 1.2 | -2.2 | -0.5 | 0.7 | 2.9 | 2.2 |
| 2 | -0.2 | 0.8 | 1.6 | 1.9 | 1.6 | -0.9 | 0.1 | 1 | 1.8 | 1 | -1.9 | -0.6 | 0.5 | 2.4 | 1.9 |
| Number of Sample =5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -0.4 | 1.4 | 2.8 | 3.1 | 2.8 | -1.4 | 0.2 | 1.6 | 3 | 1.6 | -2.5 | -0.4 | 1.1 | 3.6 | 2.5 |
| -2 | -0.3 | 1.2 | 2.5 | 2.7 | 2.5 | -1.2 | 0.2 | 1.4 | 2.6 | 1.4 | -2.2 | -0.4 | 0.9 | 3.2 | 2.2 |
| 0 | -0.2 | 1 | 2 | 2.1 | 2 | -1 | 0.1 | 1.1 | 2.1 | 1.1 | -1.9 | -0.5 | 0.6 | 2.5 | 1.9 |
| 2 | -0.1 | 0.8 | 1.6 | 1.7 | 1.6 | -0.8 | 0.1 | 0.9 | 1.6 | 0.9 | -1.7 | -0.5 | 0.4 | 2.1 | 1.7 |

Table A.2: Absolute CSI-RS based RSRP accuracy results under 15RB

|  | AWGN |  |  |  |  | EPA5Hz |  |  |  |  | ETU30Hz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | 5% | 50% | 95% | 5%~95% | 90% RSRP | 5% | 50% | 95% | 5%~95% | 90% RSRP | 5% | 50% | 95% | 5%~95% | 90% RSRP |
| Number of Sample =1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -1.5 | 0.9 | 2.8 | 4.3 | 2.8 | -2.3 | 0.2 | 2.8 | 5.1 | 2.8 | -3.3 | -0.3 | 1.9 | 5.1 | 3.8 |
| -2 | -1.3 | 0.8 | 2.6 | 3.9 | 2.6 | -2.2 | 0.1 | 2.4 | 4.6 | 2.4 | -3.1 | -0.4 | 1.6 | 4.7 | 3.8 |
| 0 | -1 | 0.6 | 2.1 | 3.1 | 2.1 | -1.8 | 0.1 | 1.8 | 3.6 | 1.8 | -2.8 | -0.5 | 1.1 | 3.9 | 3.6 |
| 2 | -0.8 | 0.5 | 1.7 | 2.5 | 1.7 | -1.4 | 0 | 1.4 | 2.9 | 1.4 | -2.5 | -0.6 | 0.8 | 3.3 | 3.3 |
| Number of Sample =2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -0.7 | 0.9 | 2.3 | 3 | 2.3 | -1.6 | 0.1 | 1.6 | 3.2 | 1.6 | -2.6 | -0.4 | 1.2 | 3.8 | 3.7 |
| -2 | -0.7 | 0.8 | 2 | 2.7 | 2 | -1.4 | 0.1 | 1.4 | 2.8 | 1.4 | -2.4 | -0.4 | 1 | 3.4 | 3.4 |
| 0 | -0.5 | 0.6 | 1.6 | 2.1 | 1.6 | -1.1 | 0 | 1.1 | 2.2 | 1.1 | -2.1 | -0.5 | 0.7 | 2.8 | 2.9 |
| 2 | -0.4 | 0.5 | 1.3 | 1.7 | 1.3 | -0.9 | 0 | 0.9 | 1.7 | 0.9 | -1.9 | -0.5 | 0.4 | 2.3 | 2.6 |
| Number of Sample =3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -0.4 | 0.9 | 2 | 2.4 | 2 | -1.2 | 0.1 | 1.3 | 2.5 | 1.3 | -2.1 | -0.4 | 0.9 | 3 | 3.2 |
| -2 | -0.4 | 0.8 | 1.8 | 2.2 | 1.8 | -1.1 | 0.1 | 1.1 | 2.2 | 1.1 | -2 | -0.4 | 0.7 | 2.8 | 2.9 |
| 0 | -0.3 | 0.6 | 1.4 | 1.7 | 1.4 | -0.8 | 0 | 0.9 | 1.7 | 0.9 | -1.7 | -0.5 | 0.5 | 2.2 | 2.4 |
| 2 | -0.2 | 0.5 | 1.2 | 1.4 | 1.2 | -0.7 | 0 | 0.7 | 1.4 | 0.7 | -1.6 | -0.5 | 0.2 | 1.8 | 2.2 |
| Number of Sample =4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -0.2 | 0.9 | 1.9 | 2.1 | 1.9 | -1.1 | 0.1 | 1.1 | 2.2 | 1.1 | -1.9 | -0.4 | 0.7 | 2.7 | 2.8 |
| -2 | -0.2 | 0.8 | 1.7 | 1.9 | 1.7 | -0.9 | 0.1 | 1 | 1.9 | 1 | -1.8 | -0.4 | 0.6 | 2.4 | 2.5 |
| 0 | -0.1 | 0.6 | 1.3 | 1.5 | 1.3 | -0.7 | 0 | 0.8 | 1.5 | 0.8 | -1.6 | -0.5 | 0.3 | 1.9 | 2.2 |
| 2 | -0.1 | 0.5 | 1.1 | 1.2 | 1.1 | -0.6 | 0 | 0.6 | 1.2 | 0.6 | -1.5 | -0.5 | 0.2 | 1.6 | 1.9 |
| Number of Sample =5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -0.1 | 0.9 | 1.8 | 1.9 | 1.8 | -0.9 | 0.1 | 1 | 1.9 | 1 | -1.7 | -0.4 | 0.6 | 2.3 | 2.5 |
| -2 | -0.1 | 0.8 | 1.6 | 1.7 | 1.6 | -0.8 | 0.1 | 0.9 | 1.7 | 0.9 | -1.6 | -0.4 | 0.5 | 2.1 | 2.2 |
| 0 | -0.1 | 0.6 | 1.3 | 1.4 | 1.3 | -0.6 | 0 | 0.7 | 1.3 | 0.7 | -1.5 | -0.5 | 0.3 | 1.7 | 1.9 |
| 2 | -0.1 | 0.5 | 1 | 1.1 | 1 | -0.5 | 0 | 0.5 | 1 | 0.5 | -1.3 | -0.5 | 0.1 | 1.4 | 1.7 |

Table A.3: Absolute CSI-RS based RSRP accuracy results under 25RB

|  | AWGN |  |  |  |  | EPA5Hz |  |  |  |  | ETU30Hz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | 5% | 50% | 95% | 5%~95% | 90% RSRP | 5% | 50% | 95% | 5%~95% | 90% RSRP | 5% | 50% | 95% | 5%~95% | 90% RSRP |
| Number of Sample =1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -1.2 | 0.7 | 2.5 | 3.8 | 2.5 | -1.9 | 0.1 | 1.9 | 3.9 | 1.9 | -2.9 | -0.4 | 1.4 | 4.2 | 2.9 |
| -2 | -1.1 | 0.6 | 2.3 | 3.4 | 2.3 | -1.7 | 0.1 | 1.7 | 3.4 | 1.7 | -2.7 | -0.5 | 1.2 | 3.9 | 2.7 |
| 0 | -0.8 | 0.5 | 1.9 | 2.7 | 1.9 | -1.4 | 0 | 1.3 | 2.7 | 1.4 | -2.4 | -0.6 | 0.8 | 3.2 | 2.4 |
| 2 | -0.6 | 0.4 | 1.5 | 2.1 | 1.5 | -1.1 | 0 | 1.1 | 2.1 | 1.1 | -2.2 | -0.6 | 0.6 | 2.7 | 2.2 |
| Number of Sample =2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -0.6 | 0.7 | 1.8 | 2.4 | 1.8 | -1.2 | 0.1 | 1.3 | 2.5 | 1.3 | -2.2 | -0.5 | 0.8 | 3 | 2.2 |
| -2 | -0.5 | 0.6 | 1.6 | 2.1 | 1.6 | -1.1 | 0.1 | 1.1 | 2.2 | 1.1 | -2 | -0.5 | 0.7 | 2.7 | 2 |
| 0 | -0.4 | 0.5 | 1.3 | 1.7 | 1.3 | -0.9 | 0 | 0.9 | 1.7 | 0.9 | -1.8 | -0.6 | 0.4 | 2.2 | 1.8 |
| 2 | -0.3 | 0.4 | 1 | 1.3 | 1 | -0.7 | 0 | 0.7 | 1.4 | 0.7 | -1.7 | -0.6 | 0.2 | 1.9 | 1.7 |
| Number of Sample =3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -0.4 | 0.7 | 1.6 | 2 | 1.6 | -0.9 | 0.1 | 1 | 2 | 1 | -1.8 | -0.5 | 0.6 | 2.4 | 1.8 |
| -2 | -0.3 | 0.6 | 1.4 | 1.7 | 1.4 | -0.8 | 0.1 | 0.9 | 1.8 | 0.9 | -1.7 | -0.5 | 0.4 | 2.1 | 1.7 |
| 0 | -0.2 | 0.5 | 1.1 | 1.4 | 1.1 | -0.6 | 0 | 0.7 | 1.3 | 0.7 | -1.5 | -0.6 | 0.2 | 1.8 | 1.5 |
| 2 | -0.2 | 0.4 | 0.9 | 1.1 | 0.9 | -0.5 | 0 | 0.6 | 1.1 | 0.6 | -1.4 | -0.6 | 0.1 | 1.5 | 1.4 |
| Number of Sample =4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -0.2 | 0.7 | 1.5 | 1.7 | 1.5 | -0.8 | 0.1 | 0.9 | 1.7 | 0.9 | -1.6 | -0.5 | 0.4 | 2.1 | 1.6 |
| -2 | -0.2 | 0.6 | 1.3 | 1.5 | 1.3 | -0.7 | 0.1 | 0.8 | 1.5 | 0.8 | -1.5 | -0.5 | 0.3 | 1.8 | 1.5 |
| 0 | -0.1 | 0.5 | 1 | 1.2 | 1 | -0.6 | 0.1 | 0.6 | 1.2 | 0.6 | -1.4 | -0.6 | 0.1 | 1.5 | 1.4 |
| 2 | -0.1 | 0.4 | 0.8 | 0.9 | 0.8 | -0.4 | 0 | 0.5 | 0.9 | 0.5 | -1.3 | -0.6 | 0 | 1.3 | 1.3 |
| Number of Sample =5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -0.1 | 0.7 | 1.4 | 1.5 | 1.4 | -0.6 | 0.1 | 0.8 | 1.5 | 0.8 | -1.5 | -0.5 | 0.4 | 1.8 | 1.5 |
| -2 | -0.1 | 0.6 | 1.3 | 1.3 | 1.3 | -0.6 | 0.1 | 0.7 | 1.3 | 0.7 | -1.4 | -0.5 | 0.2 | 1.7 | 1.4 |
| 0 | -0.1 | 0.5 | 1 | 1.1 | 1 | -0.5 | 0.1 | 0.6 | 1 | 0.6 | -1.3 | -0.6 | 0.1 | 1.4 | 1.3 |
| 2 | -0.1 | 0.4 | 0.8 | 0.8 | 0.8 | -0.4 | 0 | 0.4 | 0.8 | 0.4 | -1.2 | -0.6 | -0.1 | 1.2 | 1.2 |

Table A.4: Absolute CSI-RS based RSRP accuracy results under 50RB

|  | AWGN |  |  |  |  | EPA5Hz |  |  |  |  | ETU30Hz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | 5% | 50% | 95% | 5%~95% | 90% RSRP | 5% | 50% | 95% | 5%~95% | 90% RSRP | 5% | 50% | 95% | 5%~95% | 90% RSRP |
| Number of Sample =1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -0.8 | 0.5 | 1.8 | 2.6 | 1.8 | -1.4 | 0.1 | 1.4 | 2.7 | 1.4 | -2.4 | -0.6 | 0.8 | 3.2 | 2.4 |
| -2 | -0.7 | 0.4 | 1.6 | 2.3 | 1.6 | -1.2 | 0 | 1.2 | 2.4 | 1.2 | -2.3 | -0.6 | 0.7 | 2.9 | 2.3 |
| 0 | -0.5 | 0.3 | 1.3 | 1.8 | 1.3 | -1 | 0 | 0.9 | 1.9 | 1 | -2.1 | -0.7 | 0.4 | 2.5 | 2.1 |
| 2 | -0.4 | 0.3 | 1 | 1.4 | 1 | -0.8 | 0 | 0.7 | 1.5 | 0.8 | -1.9 | -0.7 | 0.2 | 2.2 | 1.9 |
| Number of Sample =2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -0.4 | 0.5 | 1.3 | 1.8 | 1.3 | -0.9 | 0.1 | 0.9 | 1.8 | 0.9 | -1.8 | -0.6 | 0.4 | 2.3 | 1.8 |
| -2 | -0.4 | 0.4 | 1.2 | 1.6 | 1.2 | -0.8 | 0.1 | 0.8 | 1.5 | 0.8 | -1.8 | -0.6 | 0.3 | 2.1 | 1.8 |
| 0 | -0.3 | 0.3 | 0.9 | 1.2 | 0.9 | -0.6 | 0 | 0.6 | 1.2 | 0.6 | -1.7 | -0.7 | 0.1 | 1.8 | 1.7 |
| 2 | -0.2 | 0.3 | 0.7 | 1 | 0.7 | -0.5 | 0 | 0.5 | 0.9 | 0.5 | -1.6 | -0.7 | 0 | 1.6 | 1.6 |
| -3 | -0.2 | 0.5 | 1.2 | 1.4 | 1.2 | -0.7 | 0.1 | 0.7 | 1.4 | 0.7 | -1.6 | -0.6 | 0.2 | 1.8 | 1.6 |
| -2 | -0.2 | 0.5 | 1 | 1.2 | 1 | -0.6 | 0 | 0.6 | 1.2 | 0.6 | -1.6 | -0.6 | 0.1 | 1.7 | 1.6 |
| 0 | -0.1 | 0.3 | 0.8 | 1 | 0.8 | -0.5 | 0 | 0.5 | 1 | 0.5 | -1.4 | -0.7 | 0 | 1.4 | 1.4 |
| 2 | -0.1 | 0.3 | 0.7 | 0.8 | 0.7 | -0.4 | 0 | 0.4 | 0.8 | 0.4 | -1.4 | -0.7 | -0.1 | 1.3 | 1.4 |
| Number of Sample =4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -0.1 | 0.5 | 1.1 | 1.2 | 1.1 | -0.6 | 0 | 0.6 | 1.2 | 0.6 | -1.5 | -0.6 | 0.1 | 1.6 | 1.5 |
| -2 | -0.1 | 0.4 | 1 | 1.1 | 1 | -0.5 | 0 | 0.6 | 1.1 | 0.6 | -1.4 | -0.6 | 0 | 1.4 | 1.4 |
| 0 | -0.1 | 0.3 | 0.8 | 0.8 | 0.8 | -0.4 | 0 | 0.4 | 0.8 | 0.4 | -1.3 | -0.7 | -0.1 | 1.2 | 1.3 |
| 2 | -0.1 | 0.3 | 0.6 | 0.7 | 0.6 | -0.3 | 0 | 0.3 | 0.7 | 0.3 | -1.3 | -0.7 | -0.2 | 1.1 | 1.3 |
| Number of Sample =5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| -3 | -0.1 | 0.5 | 1 | 1.1 | 1 | -0.5 | 0.1 | 0.6 | 1.1 | 0.6 | -1.4 | -0.6 | 0.1 | 1.4 | 1.4 |
| -2 | -0.1 | 0.4 | 0.9 | 1 | 0.9 | -0.4 | 0 | 0.5 | 0.9 | 0.5 | -1.3 | -0.6 | 0 | 1.3 | 1.3 |
| 0 | 0 | 0.3 | 0.7 | 0.8 | 0.7 | -0.3 | 0 | 0.4 | 0.7 | 0.4 | -1.2 | -0.7 | -0.2 | 1.1 | 1.2 |
| 2 | 0 | 0.3 | 0.6 | 0.6 | 0.6 | -0.3 | 0 | 0.3 | 0.6 | 0.3 | -1.2 | -0.7 | -0.3 | 1 | 1.2 |

Table A.5: 90% absolute RSRP accuracy with different configuration (N=1)

| Channel | AWGN | EPA5Hz | ETU30Hz | AWGN | EPA5Hz | ETU30Hz |
| --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | 6RB |  |  | 15RB |  |  |
| -2 | 3.8 | 3.8 | 3.8 | 2.6 | 2.4 | 3.1 |
| 0 | 3.2 | 2.9 | 3.6 | 2.1 | 1.8 | 2.8 |
| 2 | 2.6 | 2.3 | 3.3 | 1.7 | 1.4 | 2.5 |
| SNR[dB] | 25RB |  |  | 50RB |  |  |
| -2 | 2.3 | 1.7 | 2.7 | 1.6 | 1.2 | 2.3 |
| 0 | 1.9 | 1.4 | 2.4 | 1.3 | 1 | 2.1 |
| 2 | 1.5 | 1.1 | 2.2 | 1 | 0.8 | 1.9 |

Table A.6: 90% absolute RSRP accuracy with different configuration (N=2)

| Channel | AWGN | EPA5Hz | ETU30Hz | AWGN | EPA5Hz | ETU30Hz |
| --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | 6RB |  |  | 15RB |  |  |
| -2 | 3.1 | 2.3 | 3.4 | 2 | 1.4 | 2.4 |
| 0 | 2.5 | 1.8 | 2.9 | 1.6 | 1.1 | 2.1 |
| 2 | 2 | 1.4 | 2.6 | 1.3 | 0.9 | 1.9 |
| SNR[dB] | 25RB |  |  | 50RB |  |  |
| -2 | 1.6 | 1.1 | 2 | 1.2 | 0.8 | 1.8 |
| 0 | 1.3 | 0.9 | 1.8 | 0.9 | 0.6 | 1.7 |
| 2 | 1 | 0.7 | 1.7 | 0.7 | 0.5 | 1.6 |

Table A.7: 90% absolute RSRP accuracy with different configuration (N=3)

| Channel | AWGN | EPA5Hz | ETU30Hz | AWGN | EPA5Hz | ETU30Hz |
| --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | 6RB |  |  | 15RB |  |  |
| -2 | 2.8 | 1.8 | 2.9 | 1.8 | 1.1 | 2 |
| 0 | 2.3 | 1.4 | 2.4 | 1.4 | 0.9 | 1.7 |
| 2 | 1.8 | 1.1 | 2.2 | 1.2 | 0.7 | 1.6 |
| SNR[dB] | 25RB |  |  | 50RB |  |  |
| -2 | 1.4 | 0.9 | 1.7 | 1 | 0.6 | 1.6 |
| 0 | 1.1 | 0.7 | 1.5 | 0.8 | 0.5 | 1.4 |
| 2 | 0.9 | 0.6 | 1.4 | 0.7 | 0.4 | 1.4 |

Table A.8: 90% absolute RSRP accuracy with different configuration (N=4)

| Channel | AWGN | EPA5Hz | ETU30Hz | AWGN | EPA5Hz | ETU30Hz |
| --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | 6RB |  |  | 15RB |  |  |
| -2 | 2.5 | 1.6 | 2.5 | 1.7 | 1 | 1.8 |
| 0 | 2.1 | 1.2 | 2.2 | 1.3 | 0.8 | 1.6 |
| 2 | 1.6 | 1 | 1.9 | 1.1 | 0.6 | 1.5 |
| SNR[dB] | 25RB |  |  | 50RB |  |  |
| -2 | 1.3 | 0.8 | 1.5 | 1 | 0.6 | 1.4 |
| 0 | 1 | 0.6 | 1.4 | 0.8 | 0.4 | 1.3 |
| 2 | 0.8 | 0.5 | 1.3 | 0.6 | 0.3 | 1.3 |

Table A.9: 90% absolute RSRP accuracy with different configuration (N=5)

| Channel | AWGN | EPA5Hz | ETU30Hz | AWGN | EPA5Hz | ETU30Hz |
| --- | --- | --- | --- | --- | --- | --- |
| SNR[dB] | 6RB |  |  | 15RB |  |  |
| -2 | 2.3 | 1.4 | 2.2 | 1.6 | 0.9 | 1.6 |
| 0 | 2 | 1.1 | 1.9 | 1.3 | 0.7 | 1.5 |
| 2 | 1.6 | 0.9 | 1.7 | 1 | 0.5 | 1.3 |
| SNR[dB] | 25RB |  |  | 50RB |  |  |
| -2 | 1.3 | 0.7 | 1.4 | 0.9 | 0.5 | 1.3 |
| 0 | 1 | 0.6 | 1.3 | 0.7 | 0.4 | 1.2 |
| 2 | 0.8 | 0.4 | 1.2 | 0.6 | 0.3 | 1.2 |