3GPP TSG-RAN WG4 Meeting #72bis	 R4-146078

# Singapore, 6-10 Oct, 2014

Agenda Item:	7.14.1

Source: 	Samsung

Title: 	Updated Performance Evaluation Results for UL VoIP with Enhanced TTI

Document for:	Discussion

# 1	Introduction

In RAN4 #71 meeting, simulation assumptions for UL VoIP with enhanced TTI bundling have been agreed[1]. At last RAN4#72 meeting, the initial ideal results with VoIP packet model and full buffer models were both provided. Now that only VoIP packet model is of interest at that meeting, in this contribution, we provide the updated simulation results based on these assumptions with VoIP packet model only.

# 2  Simulation assumptions

Table 1 provides the simulation assumptions for UL FDD VoIP with enhanced TTI bundling.

Table 1  Simulation assumptions for UL FDD VoIP demodulation tests

| Parameters | Values |
| --- | --- |
| Test metric | SNR at 2% residual BLER (rBLER) - rBLER = A/B, where A is the number of incorrectly decoded TBs after retransmission and B is the number of transmitted TB (retransmitted TBs will not be counted repetitively) |
| FRC | FRC 1 in table 2 |
| PRB size and allocation | 3 contiguous PRBs in the middle of the channel bandwidth. - In case the number of RBs in the channel bandwidth is even, the 3 RBs in the middle with lower numbers are to be used. |
| Bundle size | 4 TTIs |
| HARQ combining | Incremental redundancy |
| Redundancy version sequence | 0, 2, 3, 1, 0, 2, 3, 1 |
| HARQ RTT | 12ms |
| Number of HARQ processes | 3 |
| Maximal number of HARQ transmissions (including 1st transmission and re-transmissions) | 5 |
| Channel Bandwidth | 1.4MHz, 3MHz, 5MHz, 10 MHz, 15 MHz, 20 MHz |
| CP | Normal |
| Antenna configuration and correlation matrix | 1x2 Low, 1x4 Low, 1x8 Low |
| Propagation conditions | EVA 5, ETU 300 |
| Noise model | AWGN |
| Channel estimation | Practical channel estimation and noise estimation |
| Reference receiver | Practical receiver |
| New packet generation | Transmitting a new VoIP packet every 20ms - Full buffer is not excluded |

Table 2 gives out the defined FRC for UL FDD VoIP demodulation tests.

Table 2  FRC for UL FDD VoIP demodulation tests

| Reference channel | FRC 1 |
| --- | --- |
| Allocated resource blocks | 3 |
| DFT-OFDM Symbols per subframe | 12 |
| Modulation | QPSK |
| Code rate | 11/27* |
| Payload size (bits) | 328 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Coded block size including 12bits trellis termination (bits) | 1068 |
| Total number of bits per sub-frame | 864 |
| Total symbols per sub-frame | 432 |
| Note *: code rate per TTI |  |

# 3  Simulation results

In addition, the mechanism of new packet generation is decided to be transmitting a new VoIP packet every 20ms whereas Full Buffer is not excluded as well [2]. Consequently, here the simulation results based on these 2 mechanisms are both provided.

Table 3 provides simulation results for UL FDD VoIP with enhanced TTI bundling while a new VoIP packet is transmitted every 20ms. The rBLER does not exceed 2% at the SNR given in Table 3.

Table 3 Minimum requirements for UL FDD VoIP with enhanced TTI bundling while a new VoIP packet is transmitted every 20ms

| Number of TX antennas | Number of RX antennas | Cyclic Prefix | Propagation conditions and correlation matrix | Channel Bandwidth / SNR [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 1 | 2 | Normal | EVA 5 Low | -5.12 | -5.44 | -5.47 | -5.24 | -5.28 | -5.21 |
|  |  |  | ETU 300* Low | -8.85 | -8.79 | -8.84 | -8.80 | -8.83 | -8.83 |
|  | 4 |  | EVA 5 Low | -9.27 | -9.27 | -9.22 | -9.23 | -9.29 | -9.30 |
|  |  |  | ETU 300* Low | -11.43 | -11.49 | -11.40 | -11.38 | -11.46 | -11.41 |
|  | 8 |  | EVA 5 Low | -12.17 | -12.09 | -12.26 | -11.95 | -11.98 | -12.06 |
|  |  |  | ETU 300* Low | -13.39 | -13.41 | -13.37 | -13.40 | -13.37 | -13.44 |
| Note*: Not applicable for Local Area BS and Home BS. |  |  |  |  |  |  |  |  |  |

# 4 Summary

In this contribution, we have updated the ideal simulation results on UL VoIP with Enhanced TTI with VoIP model based on the agreed simulation assumptions. It is proposed that these results should be used to finalize the demodulation performance requirements for UL VoIP with Enhanced TTI.

# Reference

- R4-143353 (2014). "Simulation assumptions for UL VoIP with enhanced TTI bundling." China Telecom, ZTE. 3GPP TSG-RAN WG4 Meeting #71, Seoul, Korea.

- R4-143845 (2014). "Way Forward on BS Test Parameters for FDD LTE Coverage Enhancements." ZTE, China Telecom, Huawei, ALU, Samsung. 3GPP TSG-RAN WG4 Meeting #71, Seoul, Korea.