3GPP TSG-RAN Working Group 4 Meeting #72bis                   R4-146084

Singapore, 6-10 Oct, 2014

Source:	Samsung

Title:	TP for Rel-13 3DL TR 36.8xx: TDD-FDD CA for CA_1A-41A-41A

Agenda item:	7.6.4.3

Document for:	Approval

- 1	Introduction

In RAN#64, the WI was approved on LTE Advanced 3 Band Carrier Aggregation(3DL/1UL) of Band 1, Band 41 and Band 41[1]. In this contribution, a relevant text proposal for Rel-13 3 DL TR TR36.8xx was provided, which include channel bandwidth combination, co-existence analysis and transmitter/receiver insertion losses, etc. It should be noted that co-existence issue of CA_1A-41A-41A is similar as CA_1A-41A, which have been described in [2]. And due to relaxation valuesΔTIB,c and ΔRIB,c dominate depend on reference architecture for H-H CA combinations which still in discussion, so are left for FFS.

- 2	Text proposal

Text Proposal for TR 36.8xx (Rel-13 3DL)

<Start of change section>

6 3 Band Carrier Aggregation with single UL: band combination specific part

---Unchanged section omitted---

6.X LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 1, Band 41 and Band 41

6.X.1 Operating bands for Band 1, Band 41 and Band 41

Table 6.X.1-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_1-41 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |

6.X.2 Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| CA_1A-41A-41A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  | 41 | See Table 5.6A.1-3 CA_41A-41A Set 1 |  |  |  |  |  |  |  |

NOTE 1: For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [4] when operating in single carrier mode.

6.X.3 Co-existence studies for Band 1, Band 41 and Band 41

6.X.3.1 For the BS

Table 6.X.3.1-1gives the intermodulation products for band 1 + band 41+ band 41 CA. For the IMD analysis the maximum transmission as defined in Table 6.X.3.1-1 is considered. None of the harmonics of one band fall into the receive band of the other. The intermodulation products generated by two operating bands do not impact the own receiver since TDD BS cannot transmit and receive simultaneously in a single band.

It can be seen in the table that 2nd order IMD products may fall into BS receive band 31. The 3rd order IMD products may fall into BS receive bands of 1, 3, 4, 7, 9, 10, 23, 24, 30, 33, 34, 36, 37, 38, 39, 40 and 41. However, when the impact of maximum bandwidth is considered, the 3rd order IMD products do not fall into the BS receive of Band 1, 23, 30, 33, 34, 36, 37, 39, 40.

It should be noted that Bands 4, 7, 10, 24, 31 and 38 are not intended for use in the same geographical area as Band 1 and Band 41. This leaves bands 3, 9 and 41 to consider for IMD products.

Currently Bands 3, and 9 are used in the same geographical area as Bands 1 and 41. It is recommended that Bands 1 and 41 BS transmitters do not share the same antenna with Band 3 or 9 BS receivers so that the antenna PIM will not cause Band 3 or 9 BS receiver desensitization. A TDD BS cannot transmit and receive simultaneously in a single band so the own B41 receiver is protected. However, it is not recommended that different band 41 operators share the same antenna as a Band 1 transmitter unless the operation is synchronized.

Table 6.X.3.1-1: B1 + B41 + B41 harmonics and IMD products frequency limits

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2110 | 2170 | 2496 | 2690 |
| 2nd order harmonics frequency range (MHz) | 4220 | 4340 | 4992 | 5380 |
| 3rd order harmonics frequency range (MHz) | 6330 | 6510 | 7488 | 8070 |
| 2nd order IMD products | ¦f2_low – f1_high¦ | ¦f2_high – f1_low¦ | ¦f2_low + f1_low¦ | ¦f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 326 | 580 | 4606 | 4860 |
| 3rd order IMD products | ¦f2_high – 2*f1_low¦ | ¦f2_low – 2*f1_high¦ | ¦2*f2_low – f1_high¦ | ¦2*f2_high – f1_low¦ |
| IMD frequency limits (MHz) | 1530 | 1844 | 2822 | 3270 |
| 3rd order IMD products | ¦2*f1_low + f2_low¦ | ¦2*f1_high + f2_high¦ | ¦2*f2_low + f1_low¦ | ¦2*f2_high + f1_high¦ |
| IMD frequency limits (MHz) | 6716 | 7030 | 7102 | 7550 |
| 3rd order IMD products | ¦f1_low – f2_high + f2_low¦ | ¦f1_high + f2_high – f2_low¦ | ¦f2_low – f1_high + f1_low¦ | ¦f2_high + f1_high – f1_low¦ |
| IMD frequency limits (MHz) | 1916 | 2364 | 2436 | 2750 |
| 3rd order IMD products (with maximum channel bandwidth) | ¦f1_low – max BW f2¦ | ¦f1_high + max BW f2¦ | ¦f2_low – max BW f1¦ | ¦f2_high + max BW f1¦ |
| IMD frequency limits (MHz) | 2090 | 2190 | 2476 | 2710 |

6.X.3.2 For the UE

Table 6.X.3.2-1 provides the harmonic products for CA_1A-41A-41A with 1 UL. None of the harmonic products fall into the B1 or B41 UE receiving bands.

Table 6.X.3.2-1: 1UL B1 + B41 + B41 harmonic products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 | 2496 | 2690 |
| 2nd order harmonics frequency range (MHz) | 3840 to 3960 |  | 4992 to 5380 |  |
| 3rd order harmonics frequency range (MHz) | 5760 to 5940 |  | 7488 to 8070 |  |

6.X.4 ∆TIB,C and ∆RIB,C

For three simultaneous DL and one UL, the TIB,c and RIB values are shown in Table 6.X.4-1 and Table 6.X.4-2 respectively.

Table 6.X.4-1 : ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1A-41A-41A | 1 | FFS |
|  | 41 | FFS |

Table 6.X.4-2: ΔRIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1A-41A-41A | 1 | FFS |
|  | 41 | FFS |

<End of change section>

---Unchanged section omitted---

Reference

- RP-140963, " New Work Item proposal: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 1, Band 41 and Band 41 " KDDI

- TR 36.851 V 0.13.0