3GPP TSG-RAN WG4 (Radio) Meeting #72bis	R4-14xxxx

Singapore, 6 – 10 October 2014

Agenda Item:	7.26.2

Source: 	Alcatel-Lucent, Intel Corporation, Verizon

Title: 	TP for TR 36.851: Corrections of Harmonics and Intermodulation Products caused by LTE Advanced Carrier Aggregation of Band Combination (5 + 13)

Document for:	Approval

# 1.	Introduction

The WI proposal to support LTE Advanced 2 Band Carrier Aggregation (2DL/1UL) of Band 5 and Band 13 was approved in RAN#63 [1]. One objective of the WI is to specify the band-combinations specific Radio Frequency (RF) requirements for this new Carrier Aggregation (CA) band combination.

The impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting carrier aggregation of this new bandwidth combination to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to correct some mismatched table entries in the Inter-band Carrier Aggregation Technical Report [3].

# 2.	Text Proposal

<Start of change>

##### 6.3.9.1.2	Co-existence studies for CA_5-13

Table 6.3.9.1.2-1 shows harmonics frequency limits intermodulation products frequency limits for CA of Band 5 and Band 13.

As shown in table 6.3.9.1.2-1, 2nd harmonics of BS transmitting in Band 5 may fall into the BS receive band of Bands 3, 4, 9 and 10; and the 3rd harmonics of Band 5 could fall into the BS receive band of Band 38, 41.

Table 6.3.9.1.2-1: DL intermodulation products frequency limits for CA of Band 5 and Band 13

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 869 | 894 | 746 | 756 |
| 2nd order harmonics frequency range (MHz) | 1738 to 1788 |  | 1492 to 1512 |  |
| 3rd order harmonics frequency range (MHz) | 2607 to 2682 |  | 2238 to 2268 |  |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 113 to 148 |  | 1615 to 1650 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high & (2*f1_low + f2_low) | 2*f1_high – f2_low & (2*f1_high + f2_high) | 2*f2_low – f1_high & (2*f2_low + f1_low) | 2* f2_high – f1_low & (2*f2_high + f1_high) |
| IMD frequency range (MHz) | 982 to 1042 2484 to 2544 |  | 598 to 643 2361 to 2406 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 859 to 904 |  | 736 to 766 |  |

It also can be seen that 2nd order intermodulation products may fall into BS receive band of Band 24. 3rd intermodulation products may fall to UL frequencies of Bands 7, 8, 20, 28, 40, 41 and 44.

It should be noted that Band 5 and Band 13 are not meant to be deployed together in the same geographic area as Bands 3, 8, 9, 20, 28, 38, 40 and 44.  Therefore the focus here will be on the harmonics of Band 5 falling into Bands 4, 10 and 41, and IMD products falling into Band 7, 24 and 41.

It is recommended that Band 5 and Band 13 BS transmitters not share an antenna with Band 4, 7, 10, 24 or 41 BS receivers.  However Band 4, 7, 10, 24 and 41 would use separate antennas from Band 5 and Band 13, due to that high/low bands use separate antennas at BS side. There is no need to address above issue in inter-band carrier aggregation context.

<End of change>

# References

[1]	RP-140169, “New WI Proposal: LTE Advanced inter-band Carrier Aggregation of Band 5 and Band 13”, Verizon, Intel.

[2]	R4-142548, “	TP for TR 36.851: LTE_CA_B5_B13 introduction”, Intel Corporation, Verizon.

[3]	R4-14xxxx, “TR 36.851 V0.12.0: Rel-12 Inter-band Carrier Aggregation”, Ericsson.