3GPP TSG-RAN WG4 (Radio) Meeting #72bis	R4-146168

Singapore, 6 – 10 October 2013

Agenda Item:	7.36.2.1

Source: 	Alcatel-Lucent, Sprint

Title: 	TP for Rel-13 3DL TR 36.8xx: Coexistence Studies of Harmonics and Intermodulation Products for Contiguous Carrier Aggregation of Band 41 for 2DL

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced inter-band DL Carrier Aggregation in Band 25A and Band 41C was approved in RAN#65 [1]. One objective of the WI is to specify the band-combinations specific Radio Frequency (RF) requirements for the new bandwidth combinations for CA_25A-41C.

The impact of Harmonics and InterModulation Distortion (IMD) products caused by LTE Advanced Base Station (BS) supporting contiguous CA in Band 41 for 2DL to the receiver of own or different BS was investigated in [2]. In this paper, we provide a text proposal to record the findings in the 3 Band Carrier Aggregation Technical Report [3].

# 2.	Text Proposal

<New section>

##### 6.x.1	Co-existence studies for 1 UL/2 DL

The 2nd and 3rd order harmonics and IMD products caused in the BS by transmitting of contiguous Band 41 DL carriers can be calculated as shown in Table 6.x.1-1 below:

Table 6.x.1-1: Band 41 DL harmonics and IMD products

| BS DL carriers | f-low | f-high |
| --- | --- | --- |
| DL frequency (MHz) | 2496 | 2690 |
| 2nd harmonics frequency limits (MHz) | 4992 | 5380 |
| 3rd harmonics frequency limits (MHz) | 7488 | 8070 |
| 2nd order IMD products | (f-low – f-high) | (f-high – f-low) |
| IMD frequency limits (MHz) | -194 | 194 |
| 3rd order IMD products | (2*f-low – f-high) | (2*f-high – f-low) |
| IMD frequency limits (MHz) | 2302 | 2884 |
| 3rd order IMD products (with maximum channel bandwidth) | (f-low – BWmax) | (f-high + BWmax) |
| IMD frequency limits (MHz) | 2456 | 2730 |

It can be seen from Table 6.x.1-1 that the 3rd order IMD products of BS transmitting contiguous carriers in Band 41 may fall into the BS receive band of Bands 7, 30, 38, 40 and 41. Note that the calculation in Table 6.x.1-1 (except the last row) assumes the BS is transmitting with the whole 194 MHz DL frequency of Band 41. If the BS is only transmitting an up to 40 MHz DL in Band 41 as stated in the WIDS, then the 3rd IMD products will not fall into the BS receive band of the Band 30 or 40 as shown in the last row in Table 6.x.1-1.

Note that Bands 7 and 38 are not intended for use in the same geographical area as Band 41. Moreover, the Band 41 BS deployed in the same geographical area are assumed to be synchronized so that the Band 41 BS receivers will not be interfered by the Band 41 BS transmission. Therefore, the 3rd IMD products of BS transmitting two contiguous carriers in Band 41will not fall into the BS receive band of any operating band currently defined in the 3GPP standards for use in the same geographical area.

<End of section>

# References

[1]	RP-141673, “LTE Advanced inter-band DL Carrier Aggregation in Band 25A and Band 41C”, Sprint.

[2]	R4-146167, “Harmonics and Intermodulation Products caused by LTE Advanced intra-band contiguous Carrier Aggregation in Band 41 for 2DL”, Alcatel-Lucent, Sprint.

[3]	R4-14xxxx, “TR 36.8xx v0.1.0: Rel-13 LTE Advanced inter-band CA for 3DL”, ZTE.