3GPP TSG-RAN WG4 (Radio) Meeting #73	R4-146840

San Francisco, CA, 17 – 21 November 2014

Agenda Item:	7.6.4.2

Source: 	Alcatel-Lucent, Sprint

Title: 	TP for Rel-13 2DL TR 36.8xx: ∆TIB and ∆RIB values of LTE-A Inter-band Carrier Aggregation of Band 25A and Band 41A (1UL)

Document for:	Approval

# 1.	Introduction

The WI proposal to support the LTE Advanced inter-band DL Carrier Aggregation (CA) in Band 25A and Band 41C was approved in RAN#65 [1]. In consequence of this WI approval, RAN4 should define the relevant requirements of the maximum output power relaxation and the reference sensitivity relaxation to support inter-band CA of Band 25A and Band 41A. In this contribution, both ∆TIB and ∆RIB values are proposed and the relaxation requirements are based on the RAN4 approved Additional framework for Interband CA relaxations [2].

# 2.	Discussion

The Inter-band Carrier Aggregation of Band 25A and Band 41A is a FDD-TDD High-High band combination. The relaxations according to the approved Additional framework for Interband CA relaxations, i.e. ΔTIB,c = 0.5dB and ΔRIB = 0dB for both bands, should be specified for inter-band CA of Band 25A and Band 41A.

# 3.	Conclusion

Based on the approved Additional framework for Interband CA relaxations, the following requirements are proposed for the inter-band CA of Band 25A and Band 41A,

Proposal: ΔTIB,c = 0.5dB and ΔRIB = 0dB should be applied to both bands for CA of Band 25A + Band 41A

This proposal should be recorded in the Rel-13 2DL/1UL Inter-band Carrier Aggregation [3].

# References

[1]	RP-141673, “LTE Advanced inter-band DL Carrier Aggregation in Band 25A and Band 41C”, Sprint.

[2]	R4-146833, “Additional framework for Interband CA relaxations”, Nokia Corporation, Telecom italia, Sprint, Ericsson, TeliaSonera, Orange.

[3]	R4-146317, “Skeleton for TR 36.8xx: Rel-13 2DL/1UL Inter-band Carrier Aggregation”, Ericsson.

# Text Proposal

<Start of change>

# 1	Scope

The present document is a technical report for Inter-band Carrier Aggregation for 2DL/1UL under Rel-13 time frame. The purpose is to gather the relevant background information and studies in order to address Inter-band Carrier Aggregation requirements.

This TR covers relevant background information and studies in order to address Inter-band Carrier Aggregation (2DL/1UL) requirements for the Rel-13 band combinations in table 1-1.

Table 1-1: Release 12 inter-band carrier aggregation combinations

| WI code | WI title | Class |
| --- | --- | --- |

The scope of the report also covers the 2 DL fallback modes for the 3 DL Carrier Aggregation combinations with single UL configuration [2]. These 3 DL/1UL combinations (with 2 DL fallback modes) are listed in table 1-2. In addition, the report also covers the LTE TDD-FDD joint operation including Carrier Aggregation combinations [3], captured in Table 1-3.

Table 1-2: Release 13 inter-band carrier aggregation combinations (3 DL/1UL)

| WI code | WI title |
| --- | --- |
| LTE_CA_25A_41C | LTE Advanced inter-band DL Carrier Aggregation in Band 25A and Band 41C |

Table 1-3: Release 12 inter-band carrier aggregation combinations (LTE TDD-FDD joint operation)

| WI code | WI title | Class |
| --- | --- | --- |

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

<New section>

### 7.2.X	LTE Advanced Carrier Aggregation of Band 25 and Band 41

CA_25-41 is designed to operate in the operating bands in table 7.2.X-1.

Table 7.2.X-1: Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) band |  |  |  | Downlink (DL) band |  |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | Channel BW (MHz) | BS transmit / UE receive |  |  | Channel BW (MHz) |  |
|  |  | FUL_low – FUL_high |  |  |  | FDL_low – FDL_high |  |  |  |  |
| CA_25-41 | 25 | 1850 MHz | – | 1915 MHz | 5, 10, 15, 20 (note 1) | 1930 MHz | – | 1995 MHz | 5, 10, 15, 20 | FDD |
|  | 41 | 2496 MHz | – | 2690 MHz | 5, 10, 15, 20 (note 1) | 2496 MHz | – | 2690 MHz | 5, 10, 15, 20 | TDD |
| NOTE 1: The WI considers only one uplink component carrier to be used in Band 25 at any time |  |  |  |  |  |  |  |  |  |  |

#### 7.2.X.1		List of specific combination issues

##### 7.2.X.1.1	Channel bandwidths per operating band for CA

Table 7.2.X.1.1-1: Supported E-UTRA bandwidths per CA configuration for inter-band CA

| E-UTRA CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_25A-41A | 25 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in table 5.6.1-1 of TS 36.101 [z] when operating in single carrier mode.

<Next changed section>

##### 7.2.X.1.3		ΔTIB,c and ΔRIB (1 UL)

For the UE which supports CA_25A-41A the ΔTIB,c is defined for applicable bands in table 7.2.X.1.3-1.

Table 7.2.X.1.3-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_25A-41A | 25 | 0.5 |
|  | 41 | 0.5 |

For the UE which supports CA_25A-41A the ΔRIB is defined for applicable bands in table 7.2.X.1.3-2.

Table 7.2.X.1.3-2: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_25A-41A | 25 | 0 |
|  | 41 | 0 |

<End of change>