3GPP TSG-RAN4 Meeting #73	 R4-146902

# San Fransisco, USA, 17-21 November, 2014

Source: 	SONY Mobile Communications Japan Ltd. Inc

Title: 	OTA measurement results on delta between BH and BHH for 20 smart-phones.

Agenda Item: 	7.1

Document for:	Discussion

# 1.		Introduction R4-125719

Twenty Sony mobile smart-phones were measured for TRP and TRS at both Beside Head condition (BH) and Beside Head and Hand SAM phantom condition (BHH). The deltas obtained between BH and BHH in these measurements represents the performance degradation contribution caused by the presence of the hand, i.e. the hand loss. The existing TS 37.144 specify TRP/TRS requirements for the SAM BH condition only for UTRA bands. By adding the average hand loss figures obtained from TRP measurements to the BH requirements for TRP, the corresponding BHH requirements can be derived.  Similarly, subtracting the hand loss from the existing TRS BH requirements may provide a basis for BHH TRS requirements. See R4-146903 for a proposal on BHH requirements.

2.	Measurement results

These are the band-average deltas in dB obtained from twenty phones by taking the difference of the BH and BHH TRP and TRS measurements respectively.

| Measurement | GSM 850 | GSM 900 | GSM 1800 | GSM 1900 | UMTS Band I | UMTS Band II | UMTS Band IV | UMTS Band V | UMTS Band VIII |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TRP | -4.0 | -4.6 | -2.7 | -2.9 | -2.9 | -2.8 | -2.4 | -4.8 | -5.2 |
| TRS | -5.1 | -5.5 | -3.3 | -2.9 | -2.9 | -3.2 | -2.7 | -5.3 | -4.2 |

3.	Conclusion

Typical deltas between BH and BHH measurements have been presented.  Using this data, one can extend the current TS 37.144 requirements to new usage positions (BHH).