3GPP TSG-RAN WG4 RAN4 #73                                         R4-147016

San Francisco, USA, 17 Nov - 21 Nov, 2014

Agenda item:			7.12.1

Source:			Samsung

Title:				Phase 1 simulation results for performance calibration

Document for:			Discussion

# 1	Introduction

In 3GPP RAN4 #72Bis meeting, RAN4 has triggered the discussion on NAICS performance part. In order to speed up the progress, it has been proposed to introduce Phase 1 performance calibration among companies, which is in parallel with the test case design, i.e.

- Phase 1: performance calibration with some simplified simulation assumption

  - Note: the simulation assumption is only for calibration purpose, not for test case design

- Phase 2: performance alignment for the agreed test cases

- Phase 3: Impairment results for the agreed test cases

Although the procedure above is not formally agreed within the group, it is good to kick-off the performance calibration as soon as possible since the results from companies seems to be somehow diverse during the core part.  Therefore, we provide our simulation results for Phase 1 performance calibration in this contribution.

# Simulation Assumption

The simulated test case is aligned with the suggestion in [1] as shown in Table 1.

Table 2 listed the assumed HL signaling on blind detection.

Table 3 listed the general simulation assumptions.

Table 1: Simulated Case

| Test | TMs | Inter. ON/OFF | MCS | Rank | Antenna Config. | Interf. Type | CRS Colliding |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | TM4/4/4 | ON/ON | MCS 5/5/5 | Rank1/1/1 | 2x2 | Fixed | Colliding |
| 2 | TM9/9/9 | ON/ON | MCS 5/14/14 | Rank1/1/1 | 2x2 | Fixed | Non-colliding |

Table 2: Simulation Assumptions on Blind Detection

| Parameter | Value |
| --- | --- |
| PA subset | {-3dB, 0dB, 3dB} |
| TM subset | {TM2, TM3, TM4, TM6, TM9} |
| Blind detection granularity | 1 PRB pair |
| PCFICH/PDCCH detection | Genie-aided |
| Serving cell PMI | Random wideband PMI for serving cell per TTI |
| Interference cell PMI | Random wideband PMI for interference cell per TTI |
| Timing Offset and frequency offset | Not configured |

Table 3: General Simulation Assumptions

| Parameter | Value |
| --- | --- |
| System bandwidth | 10 MHz |
| RB allocation | {6RB, 12RB, 25RB, 50RB} |
| Cell ID | [0, 6, 1] |
| Receiver | R-ML |
| Interference profiles | INF Level 1: INR1 = 7.77dB, INR2 = 2.29dB; INF Level 2: INR1 = 13.91dB, INR2 = 3.34dB |
| MIMO configuration | 2x2 and low correlation |
| Channel model and Doppler frequency for target and interference cells | EPA 5Hz Use different channel seed for between cells |
| CRS configuration | 2 CRS ports. |
| PA | -3dB |
| PB | 0dB |
| CSI-RS configuration | Not configured |
| Channel Estimation | Realistic Channel Estimation |
| H-ARQ | 8 HARQ processes |
| PBCH, PSS, SSS configuration | Not configured |
| PCFICH | CFI = 2 |

# Simulation Results

Figure 1 shows the simulation results with 12 RB allocation. For other RB allocation size, the results are shown in Annex A.

Figure 1: Simulation Results with 12 RB allocation

Table 4 summarized the required SNR @70% throughput. It is observed the required SNR is slightly different with different RB allocation due to the slight different coding rate and CTC gain.

Table 4: Required SNR @70% throughput of Test 1

| Test 1 | Receiver | 6RB | 12RB | 25RB | 50RB |
| --- | --- | --- | --- | --- | --- |
| INR1 = 7.77dB, INR2 = 2.29dB; | MMSE-IRC | 6.1 | 6.0 | 5.8 | 5.3 |
|  | R-ML | 3.9 | 3.8 | 3.6 | 3.1 |
| INR1 = 13.91dB, INR2 = 3.34dB; | MMSE-IRC | 10.0 | 9.8 | 9.4 | 8.9 |
|  | R-ML | 3.4 | 3.4 | 3.2 | 2.0 |

Table 5: Required SNR @70% throughput of Test 2

| Test 2 | Receiver | 6RB | 12RB | 25RB | 50RB |
| --- | --- | --- | --- | --- | --- |
| INR1 = 7.77dB, INR2 = 2.29dB; | MMSE-IRC | 5.9 | 5.8 | 5.8 | 5.2 |
|  | R-ML | 5.2 | 5.1 | 5.1 | 4.5 |
| INR1 = 13.91dB, INR2 = 3.34dB; | MMSE-IRC | 8.5 | 8.3 | 8.3 | 7.6 |
|  | R-ML | 6.2 | 6.1 | 6.1 | 5.5 |

# Conclusion

In this contribution, we provide our simulation results for Phase 1 performance calibration, which are summarized in Table 4.

# Reference

- 3GPP, R4-146812, MediaTek, “Way Forward on NAICS UE Initial Simulation Assumptions,” Oct 2014.

# Annex

Figure 2: Simulation Results with 12 RB allocation

Figure 2: Simulation Results with 25 RB allocation

Figure 2: Simulation Results with 50 RB allocation