3GPP TSG-RAN4 Meeting #72bis 	R4-147425

# Singapore, Singapore, 6-10 September, 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 2708 | rev |  | Current version: | 12.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Behaviour after Measurement Gap in CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_TDD_FDD-Core |  |  |  |  |  |  | Date: |  | 2014-11-1o |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | To define UE behaviour after the measurement gap to account for: carrier aggregation including TDD-FDD CA where PCell may be FDD or TDD, and also when different UL/DL subframe configurations are used in CA. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The requirements for UE behaviour after measurement gap was defined for different RAT (FDD and TDD) for single carrier case. The UE behaviour is defined wrt the RAT of the PCell. The same UE behaviour also applies in case UE operates in CA of the same RAT. For TDD UE behaviour after measurement gap, requirements with different TDD UL/DL subframe configurations on PCell and SCell(s) are defined. The UE behaviour is defined for the following cases in compact form: In the uplink subframe occurring immediately after the measurement gap, - the UE shall not transmit any data if any of the serving cells belong to E-UTRAN FDD - the UE shall not transmit any data if all the serving cells belong to E-UTRAN TDD and - the subframe occurring immediately before the measurement gap is a downlink subframe on any of the serving cells. - whether the UE can transmit data or not is up to the UE implementation if all the serving cells belong to E-UTRAN TDD and the subframe occurring immediately before the measurement gap is an uplink subframe or a special subframe on all the serving cells |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behaviour after measurement gap for UE operating in any CA configuration including TDD-FDD CA will be unlcear. The TDD UE behaviour on measurement gap is not clear for Rel-12 UE supporting different TDD UL/DL subframe configuration between CA carriers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections have been omitted >

#### 8.1.2.1	UE measurement capability

If the UE requires measurement gaps to identify and measure inter-frequency and/or inter-RAT cells, in order for the requirements in the following subsections to apply the E-UTRAN must provide a single measurement gap pattern with constant gap duration for concurrent monitoring of all frequency layers and RATs.

During the measurement gaps the UE:

-	shall not transmit any data

-	is not expected to tune its receiver on any of the E-UTRAN carrier frequencies of PCell and any SCell.

In the uplink subframe occurring immediately after the measurement gap,

- if the following conditions are met then the UE can transmit data or not is up to the UE implementation:

-	all the serving cells belong to E-UTRAN TDD;

- if the subframe occurring immediately before the measurement gap is an uplink subframe or a special subframe.

- Otherwise the UE shall not transmit any data.

Inter-frequency and inter-RAT measurement requirements within this clause rely on the UE being configured with one measurement gap pattern unless the UE has signaled that it is capable of conducting such measurements without gaps. UEs shall only support those measurement gap patterns listed in Table 8.1.2.1-1 that are relevant to its measurement capabilities.

Table 8.1.2.1-1: Gap Pattern Configurations supported by the UE

| Gap Pattern Id | MeasurementGap Length (MGL, ms) | Measurement Gap Repetition Period (MGRP, ms) | Minimum available time for inter-frequency and inter-RAT measurements during 480ms period (Tinter1, ms) | Measurement Purpose |
| --- | --- | --- | --- | --- |
| 0 | 6 | 40 | 60 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x |
| 1 | 6 | 80 | 30 | Inter-Frequency E-UTRAN FDD and TDD, UTRAN FDD, GERAN, LCR TDD, HRPD, CDMA2000 1x |

NOTE 1:	When inter-frequency RSTD measurements are configured and the UE requires measurement gaps for performing such measurements, only Gap Pattern 0 can be used. For defining the inter-frequency and inter-RAT requirements  Tinter1=30ms shall be assumed.

NOTE 2:	A measurement gap starts at the end of the latest subframe occurring immediately before the measurement gap.

A UE that is capable of identifying and measuring inter-frequency and/or inter-RAT cells without gaps shall follow requirements as if Gap Pattern Id #0 had been used and the minimum available time Tinter1 of 60 ms shall be assumed for the corresponding requirements.

If the UE supporting E-UTRA carrier aggregation when configured with one or two SCCs is performing measurements on cells on PCC, inter-frequency measurements, or inter-RAT measurements, and interruption occurs on PCell or any activated SCell or both due to measurements performed on cells on an SCC with a deactivated SCell according to section 8.3, then the UE shall meet the requirements specified for each measurement in Section 8 and Section 9.

< Unchanged sections have been omitted >