3GPP TSG-RAN WG4 #73	R4-147437

San Francisco, USA, 17th Nov – 21th Nov. 2014

Source: 	Ericsson

Title:	Discussion on D2D receiver regarding soft buffer sharing and soft combining issue

Agenda Item:	7.11.4

Document for:	Discussion

# Introduction

In RAN4#72bis, two LSs were received from RAN1 regarding to D2D receiving [1], [2], one is about soft buffer sharing issue [1], and one is about soft combining issue for discovery [2]. Both of them asked RAN4 to take the above into consideration for performance tests definition. In this contribution, we make our suggestion on how to address these issues in the D2D performance tests.

# Discussion

## Discussion on soft buffer sharing issue

For soft buffer sharing issue, LS [1] from RAN1 have the following agreements:

Agreement:

- No standardized mechanism is defined for D2D communication and discovery to share the soft buffer already defined for cellular communications

Agreements:

- The transmitting D2D UE always assumes that there is enough memory to always use the mother turbo code with rate 1/3

- The maximum aggregate data rate that a D2D UE is expected to receive is limited to [X]

  - X may [FFS] depend on UE D2D capability

- The number of HARQ processes per transmitting D2D UE is 1 per destination ID

  - Note that any discussion of communication with multiple destination IDs is up to RAN2.

D2D UE behaviour in cases when these limits are reached is left to D2D UE implementation

According to RAN2 agreements in [4], there is agreement:

- Intra- and inter-frequency (and inter-PLMN) ProSe reception does not affect Uu reception

The above agreement is applied to discovery. It is a natural extension for other ProSe D2D reception.

Considering RAN1 agreements that no standardized mechanism is defined for sharing the UE soft buffer between cellular communications and D2D communication and discovery, it is a natural extension that there will be no standardized mechanism sharing the processing resources in the UE between cellular communications and D2D communication and discovery either. Hence, the cellular UL and DL capabilities are unaffected by UE D2D transmissions and reception.

For RAN4, in order to verify UE whether to have enough soft buffers to handle ProSe and WAN independently, some test cases shall be introduced.

Proposal 1:  In RAN4, test case(s) shall be introduced to verify UE whether to have enough soft buffers to handle ProSe and WAN independently.

To serve this purpose, we can have SDR test. How to setup SDR test is for further study. As one example setup, it could be:

- The transmitted aggregate data rate for D2D link reception can set to exceed the limitation on purpose, and WAN are configured with maximum downlink throughput, the performance for D2D link and WAN are checked simultaneously.

## Discussion on the soft-combining issue

For soft-combing issue, the LS [2] from RAN have the following agreements:

RAN1 sees benefits from soft-combining of repeated transmissions of discovery messages within a discovery period when repeated transmissions are configured.

In Figure 1 we compare the performance for {1,2,4} (re)transmissions, with and without soft combination of the discovery packets. We consider contiguous transmissions in time with PUSCH-like frequency hopping, such that retransmissions experience diversity. The simulations are based on the EPA channel at 3km/h. For the “soft combine” case we considered IR (the performance is very similar also for CC), while for the case without soft combination we considered RV=0 for all transmissions.

Figure 1: Link level simulation results for discovery with/without soft combining

Based on the above figure, we have the following observations:

Observations:

- With 2 transmissions, the loss for not performing soft combination is ~2dB

- With 4 transmissions, the loss for not performing soft combination is ~3.5dB

In [3], the same observation is presented.  For communication, the gain of soft combining is also expected. Based on the LS and the above observation, we propose:

Proposal 2: If repetition transmission is configured for D2D transmission, Soft-combing of repeated transmissions shall be set as reference receiver.

# Conclusion

In this paper, we discuss the soft buffer sharing issue and soft-combining issue, and have the following proposals:

Proposal 1:  In RAN4, test case(s) shall be introduced to verify UE whether to have enough soft buffers to handle ProSe and WAN independently.

Proposal 2: If repetition transmission is configured for D2D transmission, Soft-combining of repeated transmissions shall be set as reference receiver.

# Reference

R1-144524, LS on soft buffer management for D2D communications, Oct. 6 ~ 10, 2014

R1- 144523, LS on soft-combining of discovery messages within a discovery period, Oct. 6~10, 2014

R1-143984, Resource allocation for Type 1 D2D discovery, Qualcomm Incorporated, Oct.6~10, 2014

R2-144101, Report of 3GPP TSG RAN WG2 meeting #87, 3GPP RAN2, August 18 ~22, 2014