3GPP TSG-RAN WG4 Meeting #73 	R4-147870

San Francisco, November 17-21, 2014

Title:	LS on measurements for MBMS support

Release:	Rel-12

Work Item:	MBMS_LTE_OS

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

#### Name:	Valentin Gheorghiu

E-mail Address:	vgheorgh@qti.qualcomm.com

1. Overall Description:

RAN4 would like to inform RAN2 that it discussed the requirements for MBSFN RSRP, MBSFN RSRQ and MCH BLER measurements and reached the following agreements:

For MBSFN RSRP and MBSFN RSRQ:

- L1 measurement period for MBSFN RSRP/RSRQ

  - Contains at least 5 decoded MBSFN subframes with MCH and the minimum measurement period is [640] ms

  - It is observed that the MBSFN RSRP/RSRQ measurment period can exceed the configured logging interval, and UE logging behavior may need to be clarified in RAN2.

- The absolute measurement accuracy requirements on any carrier (same as or different from the serving unicast carrier ) where PMCH is received will be the same as those defined for CRS RSRP/RSRQ

- MBSFN RSRP report mapping  is the same as used for CRS RSRP

- MBSFN RSRQ report mapping is from –7.5 to -23 with 0.5dB granularity (total of 32 values)

- It will be captured in 36.133 that the UE performing measurements in a MBSFN subframe on a non-serving carrier shall not cause interruptions on any serving carrier in unicast subframes

For MCH BLER:

- MBMS BLER report is accompanied by the corresponding total number of MCH transport blocks, which were received by the UE during the configured MCH BLER logging interval and used for calculating the MCH BLER value

  - 5 bits are used to encode the MCH BLER

  - 11 bits are used to encode the number of received packets

- The MCH BLER measurement period is the same as the configured logging interval

- UE measures and logs BLER per each configured logging interval.

- The MBMS BLER report mapping will be defined in 36.133.

2. Actions:

To RAN2:

ACTION: RAN4 kindly asks RAN2 to take the above information into account.

3. Date of Next TSG-RAN4 Meetings:

RAN4#74		9 - 13 Feb 2015		 Athens, Greece

RAN4#74Bis 20-24 Apr 2015          Brazil