3GPP TSG RAN WG4 Meeting #73	         R4-146946

San Francisco, CA, US, 17th to 21st, November 2014

- Agenda Item:	7.29

- Source:	LG Electronics

- Title:	Consideration on the remaining UE RF requirements for 2ULs inter-band CA

Document for:	Approval

- Introduction

In RAN4 #72bis meetings, remaining UE RF requirements for 2ULs inter-band CA were discussed without consensus on the GNSS protection and general UE RX RF test cases. RAN4 just reached consensus to define MSD level for class A4 band combinations that MSD can be defined by taking average value of the results from interesting companies. So there still remain two main issues as follow

- General UE RX RF requirements

- GNSS protection by IMD/Harmonics from 2ULs signals

In this contribution, we propose alternative solutions to close 2ULs inter-band CA WI in Rel-12.

- General UE RX requirements

RAN4 started discussion on general RX requirements from UE RF #69AH meeting in Austin [1], and the same big CR [2] for 2ULs inter-band CA has been submitted for the past 6 months without consensus on RX RF test cases that should be defined in TS36.101.

UE vendors provided some contributions [3][4][5]. In these contributions, it is proposed that the REFSENS requirements is the only test case needed since the other test cases would not be worse than the case with 1UL CA operation when total UE Transmitter power is kept the same as single uplink UE. However some interested companies provided counter contributions [6][7]. It is proposed that all Rx requirements except for spurious emissions and Receiver image for 2UL inter-band CA should be defined to guarantee UE RX performance.

Table 1 show the summary how to treat remaining RX requirements in two parts.

Table 1 Comparison of necessary of RX requirements for 2ULs inter-band CA UE

| Rx requirements |  | Proposal 1 | Proposal 2 |
| --- | --- | --- | --- |
| 7.3 REFSENS |  | 1UL/2DLs UE: Needed 2UL/2DLs UE: Needed | 1UL/2DLs UE: Needed 2UL/2DLs UE: Needed |
| 7.4 Maximum input level |  | 1UL/2DLs UE: Needed 2UL/2DLs UE: Not Needed | 1UL/2DLs UE: Needed 2UL/2DLs UE: Not Needed |
| 7.5 ACS |  | 1UL/2DLs UE: Needed 2UL/2DLs UE: Not Needed | 1UL/2DLs UE: Needed 2UL/2DLs UE: Needed for Class A4 UE |
| 7.6 Blocking characteristics | In-band | 1UL/2DLs UE: Needed 2UL/2DLs UE: Not Needed | 1UL/2DLs UE: Needed 2UL/2DLs UE: Needed for Class A4 UE |
|  | Out-of-band | 1UL/2DLs UE: Not Needed 2UL/2DLs UE: Needed | 1UL/2DLs UE: Needed 2UL/2DLs UE: Needed |
|  | Narrow band | 1UL/2DLs UE: Needed 2UL/2DLs UE: Not Needed | 1UL/2DLs UE: Needed 2UL/2DLs UE: Needed for Class A4 UE |
| 7.7 Spurious response |  | 1UL/2DLs UE: Needed 2UL/2DLs UE: Not Needed | 1UL/2DLs UE: Needed 2UL/2DLs UE: Needed |
| 7.8 Intermodulation characteristics |  | 1UL/2DLs UE: Needed 2UL/2DLs UE: Not Needed | 1UL/2DLs UE: Needed 2UL/2DLs UE: Needed for Class A4 UE |

From the analysis, we can propose 4 options to close 2UL CA WIs within Rel-12 time frame.

For the test cases, we assumed UL power as PCMAX_L,c – 7dBm for 2ULs CA test except REFSENS requirements (with PCMAX_L,c – 3dBm).

- Option1) Define REFSENS + OOB requirements in TS36.101 for 2ULs inter-band CA UE

  - 1ULs/2DLs UE: All 1UL/2DLs CA UE should be tested (Exception in OOB).

- Option2) Define REFSENS + ACS + OOB in TS36.101 for 2ULs inter-band CA UE

  - 1ULs/2DLs UE: 1UL CA UE can skip ACS, OOB tests.

- Option3) Define all RX requirements except spurious emission and receiver image in TS36.101 for 2ULs inter-band CA UE  with PCMAX_L,c – 6dBm

  - 1ULs/2DLs UE: 1UL CA UE can skip except REFSENS.

- Option4) Define all RX requirements for 2UL inter-band CA UE except maximum input level, Test cases can be selected according to inter-band CA Class

  - 1UL/2DLs UE : All 1UL/2DLs CA UE should be tested

We slightly prefer option1 and 2 to compromise with two parts and RAN4 should be close WI in rel-12. RAN4 can send LS to RAN5 which RX requirements are to be tested or skipped.

Proposal 1: RAN4 should determine one of these options to define remaining RX requirements.

Table 2 Candidate options for general RX requirements

| Rx requirements |  |  | Test Necessity |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | CA UE | Option1 | Option2 | Option3 | Option4 |
| 7.3 REFSENS |  | 1UL/2DLs UE | O | O | O | O |
|  |  | 2UL/2DLs UE | O | O | O | O |
| 7.4 Maximum input level |  | 1UL/2DLs UE | O | O | X | O |
|  |  | 2UL/2DLs UE | X | X | O | X |
| 7.5 ACS |  | 1UL/2DLs UE | O | X | X | O |
|  |  | 2UL/2DLs UE | X | O | O | O |
| 7.6 Blocking characteristics | IBB | 1UL/2DLs UE | O | O | X | O |
|  |  | 2UL/2DLs UE | X | X | O | O |
|  | OOB | 1UL/2DLs UE | X | X | X | O |
|  |  | 2UL/2DLs UE | O | O | O | O |
|  | NBB | 1UL/2DLs UE | O | O | X | O |
|  |  | 2UL/2DLs UE | X | X | O | O |
| 7.7 Spurious response |  | 1UL/2DLs UE | O | O | X | O |
|  |  | 2UL/2DLs UE | X | X | O | O |
| 7.8 Intermodulation characteristics |  | 1UL/2DLs UE | O | O | X | O |
|  |  | 2UL/2DLs UE | X | X | O | O |

- GNSS Protection issues for 2ULs CA UE

RAN4 has discussed how to define the GNSS protection in Rel-12 time frame, but this issue also has not reached consensus in last RAN4 meeting. At the evening the ad-hoc meeting, network signalling method is one possible solution, but it also has some issue such as how to define the network signalling or which network signalling is needed in RAN WG2.

Table 3 shows variable candidate solutions in last RAN4 meeting [8].

Table 3 GNSS protection methods

|  | GNSS Protection methods |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Option1 | Option2 | Option3 | Option4 | Option5 |
|  | P-MPR | Network signaling | A-MPR | Network signaling + MPR | Autonomous Scell deactivation |
| Procedure | - UE recognize the problem of GNSS protection - UE apply P-MPR not to impact on GNSS receiving | - UE recognize the problem of GNSS - UE send message to eNB for the problem of GNSS - eNB scheduler deactivated the Scell and reconfigure | - UE recognize the problem of GNSS - UE apply A-MPR not to impact in GNSS receiving | - UE recognize the problem of GNSS - UE send message to eNB for the problem of GNSS - eNB scheduler reconfigure UE - UE apply MPR | - UE recognize the problem of GNSS - UE deactived Scell - UE send message to eNB |
| Pros | - Simple - Can be defined in Rel-12 - No impact to other WGs | - Simple - Can be defined in Rel-12 - Decrease UE dependency | - Correct A-MPR by RB positioning and # of RB - No impact to other WG | - Correct MPR by RB positioning and # of RB - Best solution | - Simple - Can define in Rel-12 - No impact to other WG |
| Cons | - Depend on UE decision - Reduce cell coverage by overestimated P-MPR | - Depend on eNB decision - Impact to RAN WG2 | - Can Defer to Rel-13 due to time limitation - Not simple | - Can Defer to Rel-13 due to time limitation - Need more time and effort | - Depend on UE decision - Impact to 2ULs utility |

From these observation, we prefer option2 if network signalling method can be finalized in Rel-12 time frame.

Proposal 2: RAN4 should determine network signaling as a method to protect GNSS.

- Conclusions

In this contribution, we analyze the impact according to the variable options for remaining RX requirements and GNSS problems. From the analysis, we propose our preference and solutions to close 2ULs inter-band CA WI in rel-12 as follow

Proposal 1: RAN4 should determine one of these options to define remaining RX requirements.

Proposal 2: RAN4 should determine network signaling as a method to protect GNSS.

- Reference

- R4-141222, “UE CA issues Ad-Hoc Minutes,” Nokia

- R4-146740, “Introduction of dual uplink inter-band CA in TS 36.101 rel-12,” LG Electronics, Nokia Corporation, Intel Corporation, Qualcomm Incorporated, MediaTek and LG Uplus

- R4-145094, “Assessment on Rx requirements for 2UL inter-band CA,” MediaTek

- R4-141962, “2UL interband CA Rx test simulation results,” Nokia

- R4-146823, “WF on remaining RX RF requirements for 2ULs inter-band CA,” LG Electronics

- R4-143038, “Rx requirements for 2UL inter-band CA,” NTT DoCoMo

- R4-142016, “Specification of the complete set of RX requirements for UL inter-band CA,” Ericsson

- R4-146782, “Minutes for AH for UE CA issues,” NTT DoCoMo

- TS 36.101 v12.5.0