3GPP TSG-RAN WG4 Meeting #73	 R4-147012

# San Francisco, CA, USA, 17 – 21 Nov, 2014

Agenda Item:	7.9.3

Source: 	Samsung

Title: 	Views on 256QAM CSI test

Document for:	Discussion

1	Introduction

A new work item “Small cell enhancement” was introduced in Rel-12. 256QAM and small cell on/off were the key features for this WI. From RAN4 point of view, 256QAM is mostly related to UE demodulation performance and CSI performance part.

A way forward was agreed in RAN4#72 to evaluate such options for CQI test:

- Option 1: CQI definition PUCCH 1-0 single layer, CQI definition PUCCH 1-1 dual layer and CQI fading test TM9 PUSCH 3-1;

- Option 2: CQI definition PUCCH 1-0 single layer and CQI fading test TM9 PUSCH 3-1;

- Option 3: CQI definition PUCCH 1-1 dual layer and CQI fading test TM9 PUSCH 3-1;

- Option 4 : PUCCH 1-0 static test for TM1 and PUCCH 1-1 static test for TM9; no CQI fading test

In RAN4#72bis meeting, no consensus was reached for CQI test cases since some companies preferred only introducing static CQI test cases while other companies preferred introducing both fading CQI test and static CQI test cases.

In this contribution, further simulation results are provided for static CQI test cases including TM1, TM4 and TM9 test cases.

- Analysis

2.1 Overview

The purpose of new CQI test cases is to verify UE implementation of new CQI table mapping and CQI calculation for 256QAM.

From UE implementation point of view, this new CQI table has impact on the effective SNR calculation and CQI mapping between effective SNR and CQI index. The purpose of fading CQI test cases is to verify whether UE can effectively track channel variance in time domain and frequency domain or interference variance in frequency domain. Static CQI test cases are introduced to verify UE correctly following CQI definition for CQI calculation. Based the above consideration, it is not necessary to introduce fading CQI test since static CQI test cases have already severed test purpose well. Furthermore, both CRS based and CSI-RS based on feedback modes are needed to be introduced.

Proposal 1: Prefer option 4 to introduce CQI test cases.

In below sections, we further evaluate the existing static CQI test cases including TM1, TM4 and TM9 test cases to verify the feasibility of reusing existing test cases and select proper test points. Considering new CQI table has down-selected low CQI indexes to include 256QAM CQI indexes, both low SNR points and higher SNR points should be covered in new CQI test cases. In our simulation, the SNR range 17~24dB is evaluated to decide the proper high SNR points covering 256QAM indexes and low SNR points in existing test cases were also evaluated to cover low CQI indexes.

2.2 PUCCH 1-0 static CQI test (TM1)

Simulation assumption

The existing test parameters and methodology in section 9.2.1.1 (FDD) is reused here, as summarized in table 1 below.

Table 1: PUCCH 1-0 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 1 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | AWGN (1 x 2) |  |  |  |
| SNR (Note 2) |  | dB | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type |  |  | 4 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |  |  |
| Note 1: Reference measurement channel according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. |  |  |  |  |  |  |

Simulation results

Reporting CQI distribution and BLER performance for PUCCH1-0 test is summarized in table 2 below. Based on alignment simulation results, we observe that:

- Without receiver impairment margin, reporting CQI values will cover 256QAM under SNR range 17~24dB.

- Both CQI distribution and BLER requirements in existing static CQI test can be satisfied with proper UE implementation in both high SNR range and low SNR range i.e. SNR points in existing test case.

Table 2 Reporting Spread of the Wideband CQI and BLER performance for PUCCH1-0 test

| SNR | Median CQI Index | Percentage of CQI index in {median CQI1 -1, median CQI1 +1} | BLER Using Reported CQI Median-1 | BLER Using Reported CQI Median | BLER Using Reported CQI Median+1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 3 | 1.0000 | 0.0000 | 0.0000 | 1.0000 |
| 1 | 4 | 1.0000 | 0.0000 | 0.2180 | 1.0000 |
| 6 | 6 | 1.0000 | 0.0000 | 0.0000 | 1.0000 |
| 7 | 7 | 1.0000 | 0.0000 | 0.0000 | 1.0000 |
| 17 | 12 | 1.0000 | 0.0000 | 0.0908 | 1.0000 |
| 18 | 12 | 1.0000 | 0.0000 | 0.0000 | 1.0000 |
| 19 | 13 | 1.0000 | 0.0000 | 0.0050 | 1.0000 |
| 20 | 13 | 1.0000 | 0.0000 | 0.0000 | 0.1242 |
| 21 | 14 | 1.0000 | 0.0000 | 0.0008 | 1.0000 |
| 22 | 14 | 1.0000 | 0.0000 | 0.0000 | 1.0000 |
| 23 | 14 | 1.0000 | 0.0000 | 0.0000 | 1.0000 |
| 24 | 14 | 1.0000 | 0.0000 | 0.0000 | 1.0000 |
| 25 | 15 | 1.0000 | 0.0000 | 0.0441 | NA |

Proposal 2: SNR points {0, 1} dB and {19, 20} dB can be selected for PUCCH 1-0 test to cover both low CQI index and 256QAM CQI index.

2.3 PUCCH 1-1 static CQI test (TM4)

Simulation Assumption

The existing test parameters and methodology in section 9.2.2.1 (FDD) is reused here, as summarized in table 3 below.

Table3: PUCCH 1-1 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 010000 |  |  |  |
| SNR (Note 2) |  | dB | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |  |  |
| ri-ConfigIndex |  |  | 1 (Note 3) |  |  |  |
| Note 1: Reference measurement channel according to Table A.4-1 with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: It is intended to have UL collisions between RI reports and HARQ-ACK, since the RI reports shall not be used by the eNB in this test. |  |  |  |  |  |  |

Simulation results

Reporting CQI distribution and BLER performance for both codeword 0 and codeword 1 is summarized in table 4 below. Based on alignment simulation results, we observe that:

- Without receiver impairment margin, reporting CQI values will cover 256QAM under SNR range 20~24dB.

- Both CQI distribution and BLER requirements in existing static CQI test can be satisfied with proper UE implementation.

Table 4 Reporting Spread of the Wideband CQI and BLER performance for PUCCH1-1 test

| SNR | Median CQI Index |  | Percentage of CQI index in {median CQI1 -1, median CQI1 +1} |  | BLER Using Reported CQI Median-1 |  | BLER Using Reported CQI Median |  | BLER Using Reported CQI Median+1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CW0 | CW1 | CW0 | CW1 | CW0 | CW1 | CW0 | CW1 | CW0 | CW1 |
| 10 | 6 | 6 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.6573 | 0.6519 |
| 11 | 6 | 6 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 |
| 16 | 9 | 9 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 1.0000 | 1.0000 |
| 17 | 9 | 9 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 1.0000 | 1.0000 |
| 20 | 12 | 12 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0012 | 0.0006 | 1.0000 | 1.0000 |
| 21 | 12 | 12 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 1.0000 | 1.0000 |
| 22 | 13 | 13 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0018 | 0.0012 | 1.0000 | 1.0000 |
| 23 | 13 | 13 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 1.0000 | 1.0000 |
| 24 | 14 | 14 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.5830 | 0.5790 |

Proposal 3: SNR points {10, 11} dB and {22, 23} dB can be selected for PUCCH 1-1 test (TM4) to cover both low CQI index and 256QAM CQI index.

2.4 PUCCH 1-1 static CQI test (TM9)

Simulation Assumption

The existing test parameters and methodology in section 9.2.3.1 (FDD) is reused here, as summarized in table 3 below.

Table5: PUCCH 1-1 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 9 |  |  |  |
| Downlink power allocation |  | dB | 0 |  |  |  |
|  |  | dB | 0 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
| Cell-specific reference signals |  |  | Antenna ports 0, 1 |  |  |  |
| CSI reference signals |  |  | Antenna ports 15,…,18 |  |  |  |
| CSI-RS periodicity and subframe offset TCSI-RS / ∆CSI-RS |  |  | 5/1 |  |  |  |
| CSI reference signal configuration |  |  | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (4 x 2) |  |  |  |
| Beamforming Model |  |  | As specified in Section B.4.3 |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 0x0000 0000 0100 0000 |  |  |  |
| SNR (Note 2) |  | dB | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | TBD | TBD | TBD | TBD |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUSCH (Note3) |  |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |  |
| Physical channel for RI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| CQI delay |  | ms | 8 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 2 |  |  |  |
| ri-ConfigIndex |  |  | 1 |  |  |  |
| Note 1: Reference measurement channel according to Table A.4-1a with one sided dynamic OCNG Pattern OP.1 FDD as described in Annex A.5.1.1. Note 2: For each test, the minimum requirements shall be fulfilled for at least one of the two SNR(s) and the respective wanted signal input level. Note 3: To avoid collisions between CQI/PMI reports and HARQ-ACK it is necessary to report both on PUSCH instead of PUCCH. PDCCH DCI format 0 shall be transmitted in downlink SF#1 and #6 to allow periodic CQI/PMI to multiplex with the HARQ-ACK on PUSCH in uplink SF#0 and #5. |  |  |  |  |  |  |

Simulation results

Reporting CQI distribution and BLER performance for both codeword 0 and codeword 1 is summarized in table 6 below. Based on alignment simulation results, we observe that:

- Without receiver impairment margin, reporting CQI values will cover 256QAM under SNR range 18~24dB.

- Both CQI distribution and BLER requirements in existing static CQI test can be satisfied with proper UE implementation.

Table 6 Reporting Spread of the Wideband CQI and BLER performance for PUCCH1-1 test

| SNR | Median CQI Index |  | Percentage of CQI index in {median CQI1 -1, median CQI1 +1} |  | BLER Using Reported CQI Median-1 |  | BLER Using Reported CQI Median |  | BLER Using Reported CQI Median+1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CW0 | CW1 | CW0 | CW1 | CW0 | CW1 | CW0 | CW1 | CW0 | CW1 |
| 7 | 6 | 6 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 1.0000 | 1.0000 |
| 8 | 6 | 6 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 1.0000 | 1.0000 |
| 13 | 9 | 9 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0000 | 0.0000 | 0.8659 | 0.8645 |
| 14 | 10 | 10 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.2246 | 0.2377 | 1.0000 | 1.0000 |
| 18 | 12 | 12 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.4734 | 0.5075 | 1.0000 | 1.0000 |
| 19 | 13 | 13 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0050 | 0.0040 | 1.0000 | 1.0000 |
| 20 | 13 | 13 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0005 | 0.0010 | 1.0000 | 1.0000 |
| 21 | 14 | 14 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0171 | 0.0236 | 1.0000 | 1.0000 |
| 22 | 14 | 14 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0005 | 0.0010 | 1.0000 | 1.0000 |
| 23 | 15 | 15 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.1573 | 0.1568 | N/A | N/A |
| 24 | 15 | 15 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.1889 | 0.1935 | N/A | N/A |
| 25 | 15 | 15 | 1.0000 | 1.0000 | 0.0000 | 0.0000 | 0.0849 | 0.0889 | N/A | N/A |

Proposal 4: SNR points {7, 8} dB and {19, 20} dB can be selected for PUCCH 1-1 test (TM9) to cover both low CQI index and 256QAM CQI index.

- Conclusion

In this contribution, further simulation results are provided for the static CQI test cases including TM1, TM4 and TM9 test cases.

Based on the evaluation results, our observations are:

Observation1: It’s feasible to reuse existing test parameters and test metrics for static CQI test 1-0 (TM1).

Observation2: It’s feasible to reuse existing test parameters and test metrics for dual CWs static CQI test 1-1 (TM4).

Observation3: It’s feasible to reuse existing test parameters and test metrics for dual CWs static CQI test 1-1 (TM9).

Furthermore, it is also proposed that,

Proposal 1: Prefer option 4 i.e. PUCCH 1-0 static test for TM1 and PUCCH 1-1 static test for TM9 to introduce CQI test cases.

Proposal 2: SNR points {0, 1} dB and {19, 20} dB can be selected for PUCCH 1-0 test to cover both low CQI index and 256QAM CQI index.

Proposal 3: SNR points {10, 11} dB and {22, 23} dB can be selected for PUCCH 1-1 test (TM4) to cover both low CQI index and 256QAM CQI index.

Proposal 4: SNR points {7, 8} dB and {19, 20} dB can be selected for PUCCH 1-1 test (TM9) to cover both low CQI index and 256QAM CQI index.

- Reference

- R4-145377, “WF on 256QAM CSI test”, Huawei