3GPP TSG-RAN WG4 RAN4 #73                                         R4-147013

San Francisco, 17 Nov. - 21 Nov., 2014

Agenda item:	7.10

Source: 	Samsung

Title:	On remaining issue for SU-MIMO receiver

Document for:	Discussion

1	Introduction

In 3GPP RAN4 #72bis meeting, RAN4 has agreed that R-ML simulation results should be used for impairment performance definition, and alignment for R-ML receiver for following test cases is required for single-cell scenario [1].

Table 1: Single-Cell Test Cases for Performance Alignment

| Test Case | Test setup reference in 36.101 | TM | Duplex Mode | Antenna configuration | Fading channel | Mod |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 8.2.1.3.1 | TM3 | FDD | 2x2 Medium | EVA 70 | 16QAM |
| 2 | 8.2.1.4.2 | TM4 | FDD | 2x2 Medium | ETU 70 | 16QAM |
| 3 (option 1) | 8.3.1.2 | TM9 | FDD | 2x2 Medium | EPA 5 | 16QAM |
| 3 (option 2) | 8.3.1.2 | TM9 | FDD | 2x2 Medium | ETU 5 | 16QAM |

To verify UE proper implementation of interference and noise whitening, the following test options for multi-cell scenario should be evaluated:

  - Option 1: Test setup as illustrated in R4-144800 [2];

  - Option 2: Test setup for absolute Tput with TM3, as given below:

Table 2: Multi-Cell Whitening Verification Test

| Test Case | TM (Serving/ Interference) | Fading channel | Serving cell antenna configuration | Intf. cell antenna configuration | Serving cell MCS (CFI=2) | Intf. cell mod | INR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 2.1 | TM3/TM1 | EVA 70 | 2x2 Medium | 1x2 Low | 1/3: MCS5 for all SFs exp. SF5 | 16QAM | 6.24 |
|  |  |  |  |  | 2/5: MCS6 for all SFs exp. SF5 |  |  |
| Case 2.2 | TM3/TM1 | EVA 70 | 2x2 Medium | 1x2 Medium | 2/5: MCS6 for all SFs exp. SF5 | 16QAM | 6.24 |
|  |  |  |  |  | 3/5: MCS8 for SF0 and MCS9 for SF1,2,3,4,6,7,8,9 |  |  |

    - For option 2, interested companies can investigate higher INR values.

In this contribution, we present the simulation results of single-cell cases for impairment performance alignment, and provide our analysis on the whitening functionality verification under the multi-cell scenario.

- Impairment Results of Single-Cell Demod Test

In this Section, we present the impairment results of the agreed single-cell cases with R-ML receiver.

Proposal 1: Based on our simulation results for R-ML receivers, we present the following impairment for performance requirements definition:

Table 3: Impairment Performance Definition based on R-ML Receivers

| Test Case | Test setup reference in 36.101 | TM | Duplex Mode | Antenna configuration | Fading channel | Mod | Impairment Performance Definition (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8.2.1.3.1 | TM3 | FDD | 2x2 Medium | EVA 70 | 16QAM | 18.5 |
| 2 | 8.2.1.4.2 | TM4 | FDD | 2x2 Medium | ETU 70 | 16QAM | 19.1 |
| 3 (option 1) | 8.3.1.2 | TM9 | FDD | 2x2 Medium | EPA 5 | 16QAM | 17.4 |
| 3 (option 2) | 8.3.1.2 | TM9 | FDD | 2x2 Medium | ETU 5 | 16QAM | 18.8 |

Between the two options for TM9, we prefer the option 2 (ETU5) which is aligned with the original test setup defined in 8.3.1.2, and fully guarantee the receivers’ performance in highly time-dispersive wireless environment. Furthermore, it is observed that companies’ results are much aligned under ETU5 channel, i.e. the spread is around 1dB less than Option 2.

Proposal 2: Between the two options for TM9, we prefer the option 2 (ETU5) which is aligned with the original test setup defined in 8.3.1.2, and also achieve better performance alignment among companies.

- Results of Multi-Cell Whitening Verification

In order to verify UE proper implementation of interference and noise whitening, the following test options for multi-cell scenario should be evaluated:

  - Option 1: Test setup as illustrated in R4-144800 [2];

  - Option 2: Test setup for absolute Tput with TM3, as given in Table 2.

    - For option 2, interested companies can investigate higher INR values.

First of all, it is RAN4 principle to separate the verification of PDSCH demodulation performance and CQI reporting accuracy since Rel-8, in order to properly test and verify UE each function block. As discussed in RAN4, the purpose of multi-cell test is to verify UE correctly implement R-ML demodulator together with interference whitening block. In this sense, it is natural to prioritize the usage of PDSCH FRC test (i.e., Option 2) to serve this purpose which follows the typical RAN4 methodology. Therefore, our proposal is:

Proposal 3: Following the typical RAN4 methodology, we prioritize the usage of PDSCH FRC test (i.e., Option 2) for joint verification of R-ML receiver and interference whitening block.

Next, we analyze the feasibility of Option 2 for the joint verification of R-ML receiver and interference whitening block by performing the link level simulation.

Option 2: Test setup for absolute Tput with TM3, as given in Table 2.

  - Test Case 2.1: 1x2 Low for Interference Cell

We simulate the Test Case 2.1 as below:

Table 4: Simulation Setup for Multi-Cell Test Case 2.1

| Test Case | TM (Serving/ Interference) | Fading channel | Serving cell antenna configuration | Intf. cell antenna configuration | Serving cell MCS (CFI=2) | Intf. cell mod | INR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 2.1 | TM3/TM1 | EVA 70 | 2x2 Medium | 1x2 Low | 1/3: MCS5 for all SFs exp. SF5 | 16QAM | 6.24 |
|  |  |  |  |  | 2/5: MCS6 for all SFs exp. SF5 |  |  |

From the simulation results provided in Figure 1, it can be observed that under 1x2 low antenna configuration for interfering TM1 cells, the advanced SU-MIMO receivers with proper whitening functionality outperforms other improper implementations by approximate 1.1dB and 2.0dB for the coding rate 1/3 and 2/5 respectively.

Observation 1: Under 1x2 low antenna configuration for interfering TM1 cells, the margin achieved by the advanced SU-MIMO receivers from other improper implementations is approximate 1.1dB and 2.0dB for the coding rate 1/3 and 2/5 respectively.

Figure 1: Performance Analysis of Option 2, Test Case 2.1 (Left: MCS5, Right: MCS6)

Based on the above observation, we have the following proposal:

Proposal 4: If the test case 2.1 of Option 2 is chosen, MCS6 is preferred as the test setup candidate.

  - Test Case 2.2: 1x2 Medium for Interference Cell

We simulate the Test Case 2.2 as below:

Table 5: Simulation Setup for Multi-Cell Test Case 2.2

| Test Case | TM (Serving/ Interference) | Fading channel | Serving cell antenna configuration | Intf. cell antenna configuration | Serving cell MCS (CFI=2) | Intf. cell mod | INR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Case 2.2 | TM3/TM1 | EVA 70 | 2x2 Medium | 1x2 Medium | 2/5: MCS6 for all SFs exp. SF5 | 16QAM | 6.24 |
|  |  |  |  |  | 3/5: MCS8 for SF0 and MCS9 for SF1,2,3,4,6,7,8,9 |  |  |

From the simulation results provided in Figure 2, it can be observed that under 1x2 medium antenna configuration for interfering TM1 cells, the advanced SU-MIMO receivers with proper whitening functionality outperforms other improper implementations by approximate 0.6dB and 2.0dB for the coding rate 2/5 and 3/5 respectively.

Observation 2: Under 1x2 medium antenna configuration for interfering TM1 cells, the margin achieved by the advanced SU-MIMO receivers from other improper implementations is approximate 0.6dB and 2.0dB for the coding rate 2/5 and 3/5 respectively.

Figure 2: Performance Analysis of Option 2 with TM1 interference (Left: MCS 6, Right: MCS 8/9)

Based on the above observation, we have the following proposal:

Proposal 5: If the test case 2.2 of Option 2 is chosen, MCS8/9 is preferred as the test setup candidate.

  - Preference between Two Test Cases

As stated in [3], for multi-cell whitening verification, there should be a single beta value of the MIMO correlation for both serving and interfering cells, since the signals from both cells should arrive at the same pair of UE receive antenna. Therefore, we prioritize the utilization of test case 2.2.

Proposal 6: Between two test-case options, test case 2.2 with medium correlation for both serving and interfering cells is preferred as the test setup candidate.

- Conclusion

In this contribution, we presented the update simulation results of single-cell cases for impairment performance alignment, and provided our analysis on the whitening functionality verification under the multi-cell scenario. Our proposal and observations can be summarized as:

Proposal 1: Based on our simulation results for R-ML receivers, we present the following impairment for performance requirements definition as shown in Table 1.

Proposal 2: Between the two options for TM9, we prefer the option 2 (ETU5) which is aligned with the original test setup defined in 8.3.1.2, and also achieve better performance alignment among companies.

Proposal 3: Following the typical RAN4 methodology, we prioritize the usage of PDSCH FRC test (i.e., Option 2) for joint verification of R-ML receiver and interference whitening block.

Observation 1: Under 1x2 low antenna configuration for interfering TM1 cells, the margin achieved by the advanced SU-MIMO receivers from other improper implementations is approximate 1.1dB and 2.0dB for the coding rates 1/3 and 2/5 respectively.

Proposal 4: If the test case 2.1 of Option 2 is chosen, MCS6 is preferred as the test setup candidate.

Observation 2: Under 1x2 medium antenna configuration for interfering TM1 cells, the margin achieved by the advanced SU-MIMO receivers from other improper implementations is approximate 0.6dB and 2.0dB for the coding rates 2/5 and 3/5 respectively.

Proposal 5: If the test case 2.2 of Option 2 is chosen, MCS8/9 is preferred as the test setup candidate.

Proposal 6: Between test case 2.1 and test case 2.2, test case 2.2 with medium correlation for both serving and interfering cells is preferred as the test setup candidate.

- Reference

- 3GPP, R4-146667, Huawei, HiSilicon etc., “Way forward on SU-MIMO”, Oct. 2014.

- 3GPP, R4-144800, Ericsson, “Verification of whitening functionality for SU-MIMO receivers”, Aug. 2014.

- 3GPP, R4-146106, MediaTek, “Discussion on multiple-cell test for SU-MIMO”, Oct. 2014.