3GPP TSG-RAN WG4 Meeting #73    	R4-147333

# San Francisco, USA, November 17-21, 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 2698 | rev | - | Current version: | 12.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of TDD-FDD CA test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Networks, Nokia Corporation, … |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_CA_TDD_FDD-Perf |  |  |  |  |  |  | Date: |  | 2014-10-31 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |
| Reason for change: |  | WF for Phase II RRM test cases for TDD-FDD CA was agreed in R4-145370. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test cases are defined for: 1) E-UTRAN TDD-FDD CA Event Triggered Reporting on Deactivated SCell with PCell Interruption in Non-DRX and with PCell in FDD; 2) E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD; 3) E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD and 4) E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Phase II RRM test cases for TDD-FDD CA will not be defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section A.8.16.y1, A.8.16.y2, A.8.16.x1 and A.8.16.x2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Start of change #1>>

### A.8.16.y1	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD

#### A.8.16.y1.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.3. The test parameters are the same except those described in the following section.

In this test case there are three cells: Cell1, Cell2 and Cell3. Cell 1 is the PCell on the FDD primary component carrier, Cell 2 is the configured and deactivated SCell on the TDD secondary component carrier, and Cell 3 is the neighbor cell on the TDD secondary component carrier. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. PDCCH indicating a new transmission on the PCell shall be sent continuously to ensure that the UE sends ACK/NACKs throughout the test.

The listed parameter values in Tables A.8.16.y1.1-1 and A.8.16.y1.1-2 will add/replace the values of corresponding parameters in Tables A.8.16.3.1-1 and A.8.16.3.1-2.

Table A.8.16.y1.1-1: General test parameters for E-UTRAN TDD-FDD CA Event triggered reporting on   configured but deactivated SCell with PCell interruption in non-DRX with PCell in FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | For FDD cell: DL Reference Measurement Channel R.3 FDD For TDD cell: DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.1 (FDD) and A.3.1.1.2 (TDD) |
| PCFICH/PDCCH/PHICH parameters |  | For FDD cell: DL Reference Measurement Channel R.6 FDD For TDD cell: DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.1 (FDD) and A.3.1.2.2 (TDD) |
| Special Subframe Configuration |  | For TDD cell: 6 For FDD cell: not applicable | As specified in table 4.2.1 in TS 36.211. The same configuration applies to all TDD cells. |
| Uplink-downlink configuration |  | For TDD cell: 1 For FDD cell: not applicable |  |
| Note 1: See Table A.8.16.3.1-1 for other general test parameters. |  |  |  |

Table A.8.16.y1.1-2: Cell specific test parameters for E-UTRAN TDD-FDD CA Event triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX with PCell in FDD

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| OCNG Pattern defined in A.3.2.1.10 (OP.10 FDD) and in A.3.2.2.2 (OP.2 TDD) |  | OP.10 FDD |  | OP.2 TDD |  | OP.2 TDD |  |
| Note: See Table A.8.16.3.1-2 for other cell-specific test parameters. |  |  |  |  |  |  |  |

#### A.8.16.y1.2	Test Requirements

The test requirements defined in section A.8.16.3.2 shall apply to this test case.

<< End of change #1>>

<< Start of change #2>>

### A.8.16.y2	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD

#### A.8.16.y2.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.4. The test parameters are the same except those described in the following section.

In this test case there are three cells: Cell1, Cell2 and Cell3. Cell 1 is the PCell on the TDD primary component carrier, Cell 2 is the configured and deactivated SCell on the FDD secondary component carrier, and Cell 3 is the neighbor cell on the FDD secondary component carrier. It is indicated to the UE in the measurement control information that event-triggered reporting with Event A6 is used. The test consists of two successive time periods, with duration of T1 and T2, respectively. During T1 the UE shall not have any information of cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. PDCCH indicating a new transmission on the PCell shall be sent continuously to ensure that the UE sends ACK/NACKs throughout the test.

The listed parameter values in Tables A.8.16.y2.1-1 and A.8.16.y2.1-2 will add/replace the values of corresponding parameters in Tables A.8.16.4.1-1 and A.8.16.4.1-2.

Table A.8.16.y2.1-1: General test parameters for E-UTRAN TDD-FDD CA Event triggered reporting on   configured but deactivated SCell with PCell interruption in non-DRX with PCell in TDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | For FDD cell: DL Reference Measurement Channel R.3 FDD For TDD cell: DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.1 (FDD) and A.3.1.1.2 (TDD) |
| PCFICH/PDCCH/PHICH parameters |  | For FDD cell: DL Reference Measurement Channel R.6 FDD For TDD cell: DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.1 (FDD) and A.3.1.2.2 (TDD) |
| Special Subframe Configuration |  | For TDD cell: 6 For FDD cell: not applicable | As specified in table 4.2.1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  | For TDD cell: 1 For FDD cell: not applicable |  |
| Note 1: See Table A.8.16.4.1-1 for other general test parameters. |  |  |  |

Table A.8.16.y2.1-2: Cell specific test parameters for E-UTRAN TDD-FDD CA Event triggered reporting on configured but deactivated SCell with PCell interruption in non-DRX with PCell in TDD

| Parameter | Unit | Cell 1 |  | Cell 2 |  | Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| OCNG Pattern defined in A.3.2.1.2 (OP.2 FDD) and in A.3.2.2.1 (OP.1 TDD) |  | OP.1 TDD |  | OP.2 FDD |  | OP.2 FDD |  |
| Note: See Table A.8.16.4.1-2 for other cell-specific test parameters. |  |  |  |  |  |  |  |

#### A.8.16.y2.2	Test Requirements

The test requirements defined in section A.8.16.4.2 shall apply to this test case.

<< End of change #2>>

<< Start of change #3>>

### A.8.16.x1	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD

#### A.8.16.x1.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.1. The test parameters are the same except those described in the following section.

In this test case there are 3 cells: Cell1, Cell2 and Cell3. Cell 1 is the PCell on the FDD primary component carrier, Cell 2 is the configured and deactivated SCell on the TDD secondary component carrier, and Cell 3 is the neighbor cell on the TDD secondary component carrier. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A2 (PCell and SCell) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. At the beginning of T3 the transmission powers of cells 1, 2 and 3 are reduced below a threshold value and this shall result in reporting of Event A2 for PCell and SCell, respectively.

The listed parameter values in Tables A.8.16.x1.1-1 and A.8.16.x1.1-2 will add/replace the values of corresponding parameters in Tables A.8.16.1.1-1 and A.8.16.1.1-2.

Table A.8.16.x1.1-1: General test parameters for E-UTRAN TDD-FDD CA event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions with PCell in FDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | For FDD cell: DL Reference Measurement Channel R.0 FDD For TDD cell: DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.1 (FDD) and A.3.1.1.2 (TDD) |
| PCFICH/PDCCH/PHICH parameters |  | For FDD cell: DL Reference Measurement Channel R.6 FDD For TDD cell: DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.1 (FDD) and A.3.1.2.2 (TDD) |
| Special Subframe Configuration |  | For TDD cell: 6 For FDD cell: not applicable | As specified in table 4.2.1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  | For TDD cell: 1 For FDD cell: not applicable |  |
| Note 1: See Table A.8.16.1.1-1 for other general test parameters. |  |  |  |

Table A.8.16.x1.1-2: Cell specific test parameters for E-UTRAN TDD-FDD CA event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions with PCell in FDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| OCNG Patterns defined in A.3.2.1.1 (OP.1 FDD) and A.3.2.2.2 (OP.2 TDD) |  | OP.1 FDD |  |  | OP.2 TDD |  |  | OP.2 TDD |  |  |
| Note 1: See Table A.8.16.1.1-2 for other cell-specific test parameters. |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.x.2	Test Requirements

The test requirements defined in Section A.8.16.1 shall apply in this test case.

<< End of change #3>>

<< Start of change #4>>

### A.8.16.x2	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD

#### A.8.16.x2.1	Test Purpose and Environment

The purpose of this test case is the same as for the test defined in subclause A.8.16.2. The test parameters are the same except those described in the following section.

In this test case there are 3 cells: Cell1, Cell2 and Cell3. Cell 1 is the PCell on the TDD primary component carrier, Cell 2 is the configured and deactivated SCell on the FDD secondary component carrier, and Cell 3 is the neighbor cell on the FDD secondary component carrier. It is indicated to the UE in the measurement control information that event-triggered reporting with Events A2 (PCell and SCell) and A6 is used. The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. During T1 the UE shall not have any information on cell 3. Immediately at beginning of T2 the transmission power of cell 3 is increased to same level as for cell 2, and due to usage of an offset this shall result in reporting of Event A6. At the beginning of T3 the transmission powers of cells 1, 2 and 3 are reduced below a threshold value and this shall result in reporting of Event A2 for PCell and SCell, respectively.

The listed parameter values in Tables A.8.16.x2.1-1 and A.8.16.x2.1-2 will add/replace the values of corresponding parameters in Tables A.8.16.2.1-1 and A.8.16.2.1-2.

Table A.8.16.x2.1-1: General test parameters for E-UTRAN TDD-FDD CA event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions with PCell in TDD

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| PDSCH parameters |  | For FDD cell: DL Reference Measurement Channel R.0 FDD For TDD cell: DL Reference Measurement Channel R.0 TDD | As specified in clause A.3.1.1.1 (FDD) and A.3.1.1.2 (TDD) |
| PCFICH/PDCCH/PHICH parameters |  | For FDD cell: DL Reference Measurement Channel R.6 FDD For TDD cell: DL Reference Measurement Channel R.6 TDD | As specified in clause A.3.1.2.1 (FDD) and A.3.1.2.2 (TDD) |
| Special Subframe Configuration |  | For TDD cell: 6 For FDD cell: not applicable | As specified in table 4.2.1 in TS 36.211. The same configuration applies to all cells. |
| Uplink-downlink configuration |  | For TDD cell: 1 For FDD cell: not applicable |  |
| Note 1: See Table A.8.16.2.1-1 for other general test parameters. |  |  |  |

Table A.8.16.x2.1-2: Cell specific test parameters for E-UTRAN TDD-FDD CA event triggered reporting on configured but deactivated SCell in non-DRX under fading propagation conditions with PCell in TDD

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 | T1 | T2 | T3 |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.1.2 (OP.2 FDD) |  | OP.1 TDD |  |  | OP.2 FDD |  |  | OP.2 FDD |  |  |
| Note 1: See Table A.8.16.2.1-2 for other cell-specific test parameters |  |  |  |  |  |  |  |  |  |  |

#### A.8.16.x.2	Test Requirements

The test requirements defined in Section A.8.16.2 shall apply in this test case.

<< End of change #4>>